Lime Microsystems Limited

Surrey Tech Centre

Occam Road

The Surrey Research Park

Guildford, Surrey GU2 7YG

United Kingdom

Tel:		+44 (0) 1483 685 063

Fax:		+44 (0) 1428 656 662

e-mail:		enquiries@limemicro.com

Stream protocol

| Document version: | 01 |
| --- | --- |
| Document revision: | 00 |
| Last modified: | 18/12/2017 11:49:56 |

Contents

- Packet structure	4

1.1	Streaming configuration	5

1.1.1	12 bit compressed samples	7

1.1.2	16 bit compressed samples	7

Revision History

Version 01r00

Released: 23 Nov, 2015

Initial version.

Version 02r00

Released: 13 Jan, 2016

Updated FPGA registers map. Fixed bits indexes in samples data structures.

Version 03r00

Released: 28 Jan, 2016

Changed packet header byte indexes.

Version 04r00

Released: 17 Feb, 2016

Added timestamp reseting, timestamp synchronization individually for each packet.

Version 05r00

Released: 9 May, 2016

Updated FPGA registers map.

Version 06r00

Released: 12 July, 2017

Updated FPGA registers map for LimeSRD-USB 1v4 board.

Packet structure

The total packet size is 64 bytes. The packet structure is described in Table 1. All values are stored in little-endian format.

Table 1 Packet structure

| Byte index | Description |
| --- | --- |
| Header |  |
| 0-7 | Reserved |
| 8-15 | Allignment check: 0x00000000000000A5 |
| Payload |  |
| 16-19 | Ck-2 real part (32-bit floating-point value) |
| 20-23 | Ck-2 imaginary part (32-bit floating-point value) |
| 24-27 | Ck-1 real part (32-bit floating-point value) |
| 28-31 | Ck-1 imaginary part (32-bit floating-point value) |
| 32-35 | Ck real part (32-bit floating-point value) |
| 36-39 | Ck imaginary part (32-bit floating-point value) |
| 40-43 | Ck+1 real part (32-bit floating-point value) |
| 44-47 | Ck+1 imaginary part (32-bit floating-point value) |
| 48-51 | Ck+2 real part (32-bit floating-point value) |
| 52-55 | Ck+2 imaginary part (32-bit floating-point value) |
| 56-57 | Channel index (16-bit integer value) |
| 58-59 | Sweep point number (16-bit integer value) |
| 60-54 | Reserved |

  - Streaming configuration

Streaming configuration should be set before initiating data streaming. Protocol configuration is set by writing to board SPI registers. DNU - Do not use (register bit unsupported in device)

Table 2 Configuration registers

| Address | Def. value | Bits | Name | Description | LimeSRD-USB 1v4 |
| --- | --- | --- | --- | --- | --- |
| 0x0000 |  | 15-0 | Board ID |  | 000E |
| 0x0001 |  | 15-0 | GW version |  |  |
| 0x0002 |  | 15-0 | GW revision |  |  |
| 0x0003 |  | 15-7 | Reserved |  |  |
|  |  | 6-4 | BOM_VER |  |  |
|  |  | 3-0 | HW_VER |  |  |
| 0x0004 | 0000 | 15-0 | Reserved |  |  |
| 0x0005 | 0000 | 15-0 | DRCT_CLK_EN | Each bit enables clock to be direct clock source: 1 - enabled, 0 - disabled |  |
|  |  |  |  | [n] - nth clock |  |
|  |  |  |  | [1] - Second clock | RX clk |
|  |  |  |  | [0] - First clock | TX clk |
| 0x0006 | 0000 | 15-0 | Reserved |  |  |
| 0x0007 | 0003 | 15-0 | CH_EN | MIMO Channel enables each bit means: 1- enabled, 0-disabled |  |
|  |  |  |  | [n] - nth channel |  |
|  |  |  |  | [1] - channel 1 |  |
|  |  |  |  | [0] - channel 0 |  |
| 0x0008 | 0102 | 15-11 | Reserved |  |  |
|  |  | 10 | DLB_EN |  | DNU |
|  |  | 9 | SYNCH_DIS | Packets synchronization using timestamps: |  |
|  |  |  |  | 0 - Enabled |  |
|  |  |  |  | 1 - Disabled |  |
|  |  | 8 | MIMO_INT_EN | MIMO mode: |  |
|  |  |  |  | 0 - Disabled |  |
|  |  |  |  | 1 - Enabled |  |
|  |  | 7 | TRIQ_PULSE | TRXIQ_pulse mode: | DNU |
|  |  |  |  | 0 - OFF |  |
|  |  |  |  | 1 - ON |  |
|  |  | 6 | DDR_EN | DIQ interface mode: | DNU |
|  |  |  |  | 0 - SDR |  |
|  |  |  |  | 1 - DDR |  |
|  |  | 5 | MODE | Limelight port mode: | DNU |
|  |  |  |  | 0 - TRXIQ |  |
|  |  |  |  | 1 - JESD207 |  |
|  |  | 4-2 | Reserved |  |  |
|  |  | 1-0 | SMPL_WIDTH | Interface sample width selection: |  |
|  |  |  |  | "10" - 12bit, |  |
|  |  |  |  | "01" - 14bit, |  |
|  |  |  |  | "00" - 16bit |  |
| 0x0009 | 0003 | 15-2 | Reserved |  |  |
|  |  | 1 | TXPCT_LOSS_CLR | TX packets dropping flag clear: |  |
|  |  |  |  | 0 - Normal operation |  |
|  |  |  |  | 1 - Rising edge clears flag |  |
|  |  | 0 | SMPL_NR_CLR | Reset_timestamp: Rx and Tx should be stopped when resetting |  |
|  |  |  |  | 1 - Timestamp is cleared |  |
|  |  |  |  | 0 - Normal operation |  |
| 0x000A | 0000 | 15-10 | Reserved |  |  |
|  |  | 9 | TX_PTRN_EN | Test pattern on TX: |  |
|  |  |  |  | 0 - Disabled |  |
|  |  |  |  | 1 - Enabled |  |
|  |  | 8 | RX_PTRN_EN | Test pattern on RX: | DNU |
|  |  |  |  | 0 - Disabled |  |
|  |  |  |  | 1 - Enabled |  |
|  |  | 7-2 | Reserved |  |  |
|  |  | 1 | TX_EN | TX chain: |  |
|  |  |  |  | 0 - Disabled |  |
|  |  |  |  | 1 - Enabled |  |
|  |  | 0 | RX_EN | RX chain: |  |
|  |  |  |  | 0 - Disabled |  |
|  |  |  |  | 1 - Enabled |  |