# 8	Message functional definitions and contents

## 8.1	Overview

This clause defines the structure of the messages of the Layer 3 (L3) protocols defined in the present document. These are standard L3 messages as defined in 3GPP TS 24.007 [12].

Each definition given in the present clause includes:

a)	a brief description of the message direction and use, including whether the message has:

1.	Local significance, i.e. relevant only on the originating or terminating access;

2.	Access significance, i.e. relevant in the originating and terminating access, but not in the network;

3.	Dual significance, i.e. relevant in either the originating or terminating access and in the network; or

4.	Global significance, i.e. relevant in the originating and terminating access and in the network.

b)	a table listing the Information Elements (IE) known in the message and the order of their appearance in the message. All IEs that may be repeated are explicitly indicated (The V, LV and LV-E formatted IEs, which compose the imperative part of the message, occur before the T, TV, TLV and TLV-E formatted IEs which compose the non-imperative part of the message, see 3GPP TS 24.007 [12]). In a (maximal) sequence of consecutive IEs with half octet length, the first IE with half octet length occupies bits 1 to 4 of octet N, the second IE bits 5 to 8 of octet N, the third IE bits 1 to 4 of octet N+1 etc. Such a sequence always has an even number of elements.

For each information element the table indicates:

1.	The Information Element Identifier (IEI), in hexadecimal notation, if the IE has format T, TV, TLV or TLV-E. If the IEI has half octet length, it is specified by a notation representing the IEI as a hexadecimal digit followed by a "-" (example: B-).

NOTE 1:	The same IEI can be used for different information element types in different messages of the same protocol.

NOTE 2:	If a message includes a Type 6 IE container information element, then the same IEI can be used for different information element types in the Type 6 IE container information element and in other parts of the same message.

2.	The name of the information element (which may give an idea of the semantics of the element). The name of the information element followed by "IE" or "information element" is used in this technical report as reference to the information element within a message.

3.	The name of the type of the information element (which indicates the coding of the value part of the IE), and generally, the referenced clause of clause 9 of the present document describing the value part of the information element.

4.	The presence requirement indication (M, C, or O) for the IE as defined in 3GPP TS 24.007 [12].

5.	The format of the information element (T, V, TV, LV, TLV, LV-E or TLV-E) as defined in 3GPP TS 24.007 [12].

6.	The length of the information element (or permissible range of lengths), in octets, in the message, where "?" means that the maximum length of the IE is only constrained by link layer protocol. This indication is non-normative.

c)	clauses specifying, where appropriate, conditions for IEs with presence requirement C or O in the relevant message which together with other conditions specified in the present document define when the information elements shall be included or not, what non-presence of such IEs means, and – for IEs with presence requirement C – the static conditions for presence or non-presence of the IEs or for both cases (see 3GPP TS 24.007 [12]). If new IEs are added to a message, the respective clauses are included in the same order or sequence as the IEs appear in the message. I.e., they are appended after the existing clauses at the end of the message description or at the end of the type 6 IE container information element description, if the new IE is added to such a container information element.

## 8.2	5GS mobility management messages

### 8.2.1	Authentication request

#### 8.2.1.1	Message definition

The AUTHENTICATION REQUEST message is sent by the AMF to the UE to initiate authentication of the UE identity. See table 8.2.1.1.1.

Message type:	AUTHENTICATION REQUEST

Significance:	dual

Direction:	network to UE

Table 8.2.1.1.1: AUTHENTICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Authentication request message identity | Message type 9.7 | M | V | 1 |
|  | ngKSI | NAS key set identifier 9.11.3.32 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | ABBA | ABBA 9.11.3.10 | M | LV | 3-n |
| 21 | Authentication parameter RAND (5G authentication challenge) | Authentication parameter RAND 9.11.3.16 | O | TV | 17 |
| 20 | Authentication parameter AUTN (5G authentication challenge) | Authentication parameter AUTN 9.11.3.15 | O | TLV | 18 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |

#### 8.2.1.2	Authentication parameter RAND

Authentication parameter RAND IE is included if the AUTHENTICATION REQUEST message is used in a 5G AKA authentication procedure.

#### 8.2.1.3	Authentication parameter AUTN

Authentication parameter AUTN IE is included if the AUTHENTICATION REQUEST message is used in a 5G AKA authentication procedure.

#### 8.2.1.4	Void

#### 8.2.1.5	EAP message

EAP message IE is included if the AUTHENTICATION REQUEST message is used in an EAP based primary authentication and key agreement procedure.

### 8.2.2	Authentication response

#### 8.2.2.1	Message definition

The AUTHENTICATION RESPONSE message is sent by the UE to the AMF to deliver a calculated authentication response to the network. See table 8.2.2.1.1.

Message type:	AUTHENTICATION RESPONSE

Significance:	dual

Direction:	UE to network

Table 8.2.2.1.1: AUTHENTICATION RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Authentication response message identity | Message type 9.7 | M | V | 1 |
| 2D | Authentication response parameter | Authentication response parameter 9.11.3.17 | O | TLV | 18 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |

#### 8.2.2.2	Authentication response parameter

This IE is included if the message is sent in a 5G AKA based primary authentication and key agreement procedure.

#### 8.2.2.3	EAP message

EAP message IE is included if the EAP message received in a related AUTHENTICATION REQUEST message was an EAP-request.

### 8.2.3	Authentication result

#### 8.2.3.1	Message definition

The AUTHENTICATION RESULT message is sent by the AMF to the UE to provide result of EAP authentication of the UE identity. See table 8.2.3.1.1.

Message type:	AUTHENTICATION RESULT

Significance:	dual

Direction:	network to UE

Table 8.2.3.1.1: AUTHENTICATION RESULT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Authentication result message identity | Message type 9.7 | M | V | 1 |
|  | ngKSI | NAS key set identifier 9.11.3.32 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | EAP message | EAP message 9.11.2.2 | M | LV-E | 6-1502 |
| 38 | ABBA | ABBA 9.11.3.10 | O | TLV | 4-n |
| 55 | AUN3 device security key | AUN3 device security key 9.11.3.107 | O | TLV | 36-n |

NOTE:	It is possible for AMFs compliant with version 18.3.0 or 18.3.1 of this specification to send the AUN3 device security key IE with a fixed length of 66 octets.

#### 8.2.3.2	ABBA

This IE shall be included if the message contains an EAP message IE with an EAP-success message.

#### 8.2.3.3	AUN3 device security key

This IE shall be included when the AUTHENTICATION RESULT message is provided to a 5G-RG that is acting on behalf of an AUN3 device if the EAP message IE is set to an EAP-success message.

### 8.2.4	Authentication failure

#### 8.2.4.1	Message definition

The AUTHENTICATION FAILURE message is sent by the UE to the AMF to indicate that authentication of the network has failed. See table 8.2.4.1.1.

Message type:	AUTHENTICATION FAILURE

Significance:	dual

Direction:	UE to network

Table 8.2.4.1.1: AUTHENTICATION FAILURE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Authentication failure message identity | Message type 9.7 | M | V | 1 |
|  | 5GMM cause | 5GMM cause 9.11.3.2 | M | V | 1 |
| 30 | Authentication failure parameter | Authentication failure parameter 9.11.3.14 | O | TLV | 16 |

#### 8.2.4.2	Authentication failure parameter

This IE shall be included in a 5G AKA based primary authentication and key agreement procedure if and only if the 5GMM cause was #21 "synch failure". It shall include the response to the authentication challenge from the USIM, which is made up of the AUTS parameter (see 3GPP TS 33.501 [24]).

### 8.2.5	Authentication reject

#### 8.2.5.1	Message definition

The AUTHENTICATION REJECT message is sent by the AMF to the UE to indicate that the authentication procedure has failed and that the UE shall abort all activities. See table 8.2.5.1.1.

Message type:	AUTHENTICATION REJECT

Significance:	dual

Direction:	network to UE

Table 8.2.5.1.1: AUTHENTICATION REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Authentication reject message identity | Message type 9.7 | M | V | 1 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |

#### 8.2.5.2	EAP message

EAP message IE is included if the AUTHENTICATION REJECT message is used to convey EAP-failure message.

### 8.2.6	Registration request

#### 8.2.6.1	Message definition

The REGISTRATION REQUEST message is sent by the UE to the AMF. See table 8.2.6.1.1.

Message type:	REGISTRATION REQUEST

Significance:	dual

Direction:	UE to network

Table 8.2.6.1.1: REGISTRATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended Protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Registration request message identity | Message type 9.7 | M | V | 1 |
|  | 5GS registration type | 5GS registration type 9.11.3.7 | M | V | 1/2 |
|  | ngKSI | NAS key set identifier 9.11.3.32 | M | V | 1/2 |
|  | 5GS mobile identity | 5GS mobile identity 9.11.3.4 | M | LV-E | 6-n |
| C- | Non-current native NAS key set identifier | NAS key set identifier 9.11.3.32 | O | TV | 1 |
| 10 | 5GMM capability | 5GMM capability 9.11.3.1 | O | TLV | 3-15 |
| 2E | UE security capability | UE security capability 9.11.3.54 | O | TLV | 4-10 |
| 2F | Requested NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-74 |
| 52 | Last visited registered TAI | 5GS tracking area identity 9.11.3.8 | O | TV | 7 |
| 17 | S1 UE network capability | S1 UE network capability 9.11.3.48 | O | TLV | 4-15 |
| 40 | Uplink data status | Uplink data status 9.11.3.57 | O | TLV | 4-34 |
| 50 | PDU session status | PDU session status 9.11.3.44 | O | TLV | 4-34 |
| B- | MICO indication | MICO indication 9.11.3.31 | O | TV | 1 |
| 2B | UE status | UE status 9.11.3.56 | O | TLV | 3 |
| 77 | Additional GUTI | 5GS mobile identity 9.11.3.4 | O | TLV-E | 14 |
| 25 | Allowed PDU session status | Allowed PDU session status 9.11.3.13 | O | TLV | 4-34 |
| 18 | UE's usage setting | UE's usage setting 9.11.3.55 | O | TLV | 3 |
| 51 | Requested DRX parameters | 5GS DRX parameters 9.11.3.2A | O | TLV | 3 |
| 70 | EPS NAS message container | EPS NAS message container 9.11.3.24 | O | TLV-E | 4-n |
| 74 | LADN indication | LADN indication 9.11.3.29 | O | TLV-E | 3-811 |
| 8- | Payload container type | Payload container type 9.11.3.40 | O | TV | 1 |
| 7B | Payload container | Payload container 9.11.3.39 | O | TLV-E | 4-65538 |
| 9- | Network slicing indication | Network slicing indication 9.11.3.36 | O | TV | 1 |
| 53 | 5GS update type | 5GS update type 9.11.3.9A | O | TLV | 3 |
| 41 | Mobile station classmark 2 | Mobile station classmark 2 9.11.3.31C | O | TLV | 5 |
| 42 | Supported codecs | Supported codec list 9.11.3.51A | O | TLV | 5-n |
| 71 | NAS message container | NAS message container 9.11.3.33 | O | TLV-E | 4-n |
| 60 | EPS bearer context status | EPS bearer context status 9.11.3.23A | O | TLV | 4 |
| 6E | Requested extended DRX parameters | Extended DRX parameters 9.11.3.26A | O | TLV | 3-4 |
| 6A | T3324 value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 67 | UE radio capability ID | UE radio capability ID 9.11.3.68 | O | TLV | 3-n |
| 35 | Requested mapped NSSAI | Mapped NSSAI 9.11.3.31B | O | TLV | 3-42 |
| 48 | Additional information requested | Additional information requested 9.11.3.12A | O | TLV | 3 |
| 1A | Requested WUS assistance information | WUS assistance information 9.11.3.71 | O | TLV | 3 |
| A- | N5GC indication | N5GC indication 9.11.3.72 | O | TV | 1 |
| 30 | Requested NB-N1 mode DRX parameters | NB-N1 mode DRX parameters 9.11.3.73 | O | TLV | 3 |
| 29 | UE request type | UE request type 9.11.3.76 | O | TLV | 3 |
| 28 | Paging restriction | Paging restriction 9.11.3.77 | O | TLV | 3-35 |
| 72 | Service-level-AA container | Service-level-AA container 9.11.2.10 | O | TLV-E | 4-65538 |
| 32 | NID | NID 9.11.3.79 | O | TLV | 8 |
| 16 | UE determined PLMN with disaster condition | PLMN identity 9.11.3.85 | O | TLV | 5 |
| 2A | Requested PEIPS assistance information | PEIPS assistance information 9.11.3.80 | O | TLV | 3 |
| 3B | Requested T3512 value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 3C | Unavailability information | Unavailability information 9.11.2.20 | O | TLV | 3-9 |
| 3F | Non-3GPP path switching information | Non-3GPP path switching information 9.11.3.102 | O | TLV | 3 |
| 56 | AUN3 indication | AUN3 indication 9.11.3.104 | O | TLV | 3 |
| 64 | Requested LP-WUSPS assistance information | LP-WUSPS assistance information 9.11.3.111 | O | TLV | 3 |

#### 8.2.6.2	Non-current native NAS key set identifier

The UE shall include this IE if the UE has a valid non-current native 5G NAS security context when the UE performs an inter-system change from S1 mode to N1 mode in 5GMM-CONNECTED mode and the UE uses a mapped 5G NAS security context to protect the REGISTRATION REQUEST message.

#### 8.2.6.3	5GMM capability

The UE shall include this IE, unless the UE performs a periodic registration updating procedure.

#### 8.2.6.4	UE security capability

The UE shall include this IE, unless the UE performs a periodic registration updating procedure.

#### 8.2.6.5	Requested NSSAI

This IE is included if the UE needs to indicate the requested NSSAI to the network.

#### 8.2.6.6	Last visited registered TAI

This IE shall be included if the UE holds a valid last visited registered TAI.

#### 8.2.6.7	S1 UE network capability

A UE supporting S1 mode shall include this IE, unless the UE performs a periodic registration updating procedure.

#### 8.2.6.8	Uplink data status

This IE shall be included if the UE has uplink user data pending to be sent or the UE has active multicast MBS session(s), unless the UE performs a periodic registration updating procedure.

#### 8.2.6.9	PDU session status

This IE shall be included when the UE needs to indicate the PDU sessions that are associated with the access type that the message is sent over, that are active within the UE.

#### 8.2.6.10	MICO indication

The UE may include this IE to request the use of MICO mode.

#### 8.2.6.11	UE status

This IE shall be included if the UE in single-registration mode performs the registration procedure due to inter-system change from S1 mode to N1 mode or if the UE in dual-registration mode and EMM state EMM-REGISTERED performs initial registration.

#### 8.2.6.12	Additional GUTI

This IE shall be included:

a)	 if the UE performs the registration procedure due to inter-system change from S1 mode to N1 mode, the UE operates in single-registration mode and the UE has a valid 5G-GUTI; or

b)	the UE holds two valid native 5G-GUTIs and one of the valid native 5G-GUTI was assigned by the PLMN with which the UE is performing the registration.

#### 8.2.6.13	Allowed PDU session status

This IE shall be included if the REGISTRATION REQUEST message is sent as a response to paging with the access type indicating non-3GPP access.

#### 8.2.6.14	UE's usage setting

This IE shall be included if the UE supports IMS voice.

#### 8.2.6.15	Requested DRX parameters

If the UE wants to use or change the UE specific DRX parameters, the UE shall include the Requested DRX parameters IE in the REGISTRATION REQUEST message.

#### 8.2.6.16	EPS NAS message container

The UE operating in the single-registration mode shall include this information element as specified in clause 5.5.1.3.2 if the UE performs mobility from S1 mode to N1 mode in 5GMM-IDLE mode. The content of this message container is the complete integrity protected TRACKING AREA UPATE REQUEST message, using EPS security context.

The UE performing initial registration shall include this information element if

a)	the UE:

1)	was previously registered in S1 mode before entering state EMM-DEREGISTERED; and

2)	has received an "interworking without N26 interface not supported" indication from the network; and

b)	EPS security context and a valid 4G-GUTI are available.

The content of this message container is the complete integrity protected ATTACH REQUEST message, using EPS security context.

#### 8.2.6.17	LADN indication

The UE shall include this information element when the UE needs to request LADN information for specific LADN DNN(s) or to indicate a request for LADN information.

#### 8.2.6.17A	Payload container type

This IE shall be included if the UE includes the Payload container IE.

NOTE:	In this version of the protocol, the Payload container type IE in the REGISTRATION REQUEST message is set to "UE policy container" as described in clauses 5.5.1.2.2 and 5.5.1.3.2.

#### 8.2.6.18	Payload container

Within a PLMN, this IE shall be included if:

a)	the UE has one or more stored UE policy sections identified by a UPSI with the PLMN ID part indicating the HPLMN or the selected PLMN for the registration procedure for mobility and periodic registration update due to inter-system change from S1 mode to N1 mode of a UE operating in the single-registration mode or for the registration procedure for initial registration; or

b)	the UE does not have any stored UE policy section identified by a UPSI with the PLMN ID part indicating the HPLMN or the selected PLMN for the registration procedure for mobility and periodic registration update due to inter-system change from S1 mode to N1 mode of a UE operating in the single-registration mode or for the registration procedure for initial registration and the UE needs to send a UE policy container to the network.

Within an SNPN, this IE shall be included if:

a)	the UE has one or more stored UE policy sections for the selected SNPN for the registration procedure for initial registration; or

b)	the UE does not have any UE policy section for the selected SNPN for the registration procedure for initial registration and the UE needs to send a UE policy container to the network.

#### 8.2.6.19	Network slicing indication

This IE shall be included when a requested NSSAI is included in the REGISTRATION REQUEST message and the requested NSSAI is created from the default configured NSSAI.

#### 8.2.6.20	5GS update type

This IE shall be included when the UE is performing the registration procedure to indicate any of the following:

a)	the UE requests the use of SMS over NAS or there is a change in the UE's requirements to use SMS over NAS;

b)	a change in the UE's radio capability for NG-RAN; or

c)	the UE requests CIoT 5GS optimizations.

#### 8.2.6.21	NAS message container

This IE shall be included if the UE is sending a REGISTRATION REQUEST message as an initial NAS message, the UE has a valid 5G NAS security context and the UE needs to send non-cleartext IEs.

#### 8.2.6.22	Requested extended DRX parameters

The UE shall include this IE if the UE needs to use extended DRX or change the extended DRX parameters.

#### 8.2.6.23	EPS bearer context status

The UE shall include this IE if the UE operating in the single-registration mode performs inter-system change from S1 mode to N1 mode and the UE has locally deactivated EPS bearer context(s) for which interworking to 5GS is supported while the UE was in S1 mode without notifying the network.

#### 8.2.6.24	T3324 value

The UE may include this IE during the registration update procedure if it requests to use MICO mode and use the active time timer.

#### 8.2.6.25	Mobile station classmark 2

This IE shall be included if the UE supports 5G-SRVCC from NG-RAN to UTRAN (see 3GPP TS 23.216 [6A]).

#### 8.2.6.26	Supported codecs

This IE shall be included if the UE supports 5G-SRVCC from NG-RAN to UTRAN.

#### 8.2.6.27	UE radio capability ID

This IE shall be included if the UE is not in NB-N1 mode, the UE supports RACS and the UE needs to signal a UE radio capability ID to the network.

#### 8.2.6.28	Requested mapped NSSAI

This IE shall be included by the UE when the UE has a PDN connection or a PDU session to transfer to visited PLMN associated only with an HPLMN S-NSSAI as specified in clause 5.5.1.3.2.

#### 8.2.6.29	Additional information requested

The UE shall include this IE if the UE supports ciphered broadcast assistance data and the UE needs to obtain new ciphering keys for ciphered broadcast assistance data.

#### 8.2.6.30	Requested WUS assistance information

The UE may include this IE as specified in clause 5.3.24.

#### 8.2.6.31	Void

#### 8.2.6.32	N5GC indication

This IE shall be included in the REGISTRATION REQUEST message when the W-AGF acting on behalf of an N5GC device performs registration procedure for initial registration.

#### 8.2.6.33	Requested NB-N1 mode DRX parameters

The UE shall include this IE if the UE wants to use or change the UE specific DRX parameters for NB-N1 mode.

#### 8.2.6.34	UE request type

The UE shall include this IE if the MUSIM UE requests the release of the NAS signalling connection.

#### 8.2.6.35	Paging restriction

The UE shall include this IE if the Request type is set to "NAS signalling connection release" in the UE request type IE and the UE requests the network to restrict paging.

#### 8.2.6.35	Service-level-AA container

The UE shall include this IE if the UE supporting UAS services requests a registration for UAS services.

#### 8.2.6.36	NID

The UE shall include this IE if the 5G-GUTI in the 5GS mobile identity IE was assigned by an SNPN other than the SNPN with which the UE is registering.

#### 8.2.6.37	UE determined PLMN with disaster condition

The UE shall include this IE when the UE needs to indicate the UE determined PLMN with disaster condition determined as specified in 3GPP TS 23.122 [5].

#### 8.2.6.38	Requested PEIPS assistance information

The UE may include this IE if the UE supports PEIPS assistance information, the UE is not performing initial registration for emergency services, is not registered for emergency services and does not have an active emergency PDU session, and the UE does not perform a periodic registration updating procedure.

#### 8.2.6.38A	Void

#### 8.2.6.39	Requested T3512 value

The UE may include this IE during the registration procedure if it requests to use MICO mode and T3324 IE is included, to request a particular T3512 timer value.

#### 8.2.6.40	Unavailability information

This IE shall be included if an event is triggered in the UE that would make the UE unavailable for a certain period.

#### 8.2.6.41	Non-3GPP path switching information

The UE may include this IE during the registration procedure for mobility registration update if it requests from the network to keep using the user plane resources of the old non-3GPP access during path switching to the new non-3GPP access.

#### 8.2.6.42	AUN3 indication

This IE shall be included in the REGISTRATION REQUEST message when the 5G-RG is acting on behalf of an AUN3 device performs registration procedure for initial registration.

#### 8.2.6.43	Requested LP-WUSPS assistance information

The UE may include this IE if the UE supports the LP-WUSPS assistance information, unless the UE performs a periodic registration updating procedure.

### 8.2.7	Registration accept

#### 8.2.7.1	Message definition

The REGISTRATION ACCEPT message is sent by the AMF to the UE. See table 8.2.7.1.1.

Message type:	REGISTRATION ACCEPT

Significance:	dual

Direction:	network to UE

Table 8.2.7.1.1: REGISTRATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Registration accept message identity | Message type 9.7 | M | V | 1 |
|  | 5GS registration result | 5GS registration result 9.11.3.6 | M | LV | 2 |
| 77 | 5G-GUTI | 5GS mobile identity 9.11.3.4 | O | TLV-E | 14 |
| 4A | Equivalent PLMNs | PLMN list 9.11.3.45 | O | TLV | 5-47 |
| 54 | TAI list | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 15 | Allowed NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-74 |
| 11 | Rejected NSSAI | Rejected NSSAI 9.11.3.46 | O | TLV | 4-42 |
| 31 | Configured NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-146 |
| 21 | 5GS network feature support | 5GS network feature support 9.11.3.5 | O | TLV | 3-6 |
| 50 | PDU session status | PDU session status 9.11.3.44 | O | TLV | 4-34 |
| 26 | PDU session reactivation result | PDU session reactivation result 9.11.3.42 | O | TLV | 4-34 |
| 72 | PDU session reactivation result error cause | PDU session reactivation result error cause 9.11.3.43 | O | TLV-E | 5-515 |
| 79 | LADN information | LADN information 9.11.3.30 | O | TLV-E | 13-1715 |
| B- | MICO indication | MICO indication 9.11.3.31 | O | TV | 1 |
| 9- | Network slicing indication | Network slicing indication 9.11.3.36 | O | TV | 1 |
| 27 | Service area list | Service area list 9.11.3.49 | O | TLV | 6-114 |
| 5E | T3512 value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 5D | Non-3GPP de-registration timer value | GPRS timer 2 9.11.2.4 | O | TLV | 3 |
| 16 | T3502 value | GPRS timer 2 9.11.2.4 | O | TLV | 3 |
| 34 | Emergency number list | Emergency number list 9.11.3.23 | O | TLV | 5-50 |
| 7A | Extended emergency number list | Extended emergency number list 9.11.3.26 | O | TLV-E | 7-65538 |
| 73 | SOR transparent container | SOR transparent container 9.11.3.51 | O | TLV-E | 20-n |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |
| A- | NSSAI inclusion mode | NSSAI inclusion mode 9.11.3.37A | O | TV | 1 |
| 76 | Operator-defined access category definitions | Operator-defined access category definitions 9.11.3.38 | O | TLV-E | 3-8323 |
| 51 | Negotiated DRX parameters | 5GS DRX parameters 9.11.3.2A | O | TLV | 3 |
| D- | Non-3GPP NW policies | Non-3GPP NW provided policies 9.11.3.36A | O | TV | 1 |
| 60 | EPS bearer context status | EPS bearer context status 9.11.3.23A | O | TLV | 4 |
| 6E | Negotiated extended DRX parameters | Extended DRX parameters 9.11.3.26A | O | TLV | 3-4 |
| 6C | T3447 value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 6B | T3448 value | GPRS timer 2 9.11.2.4 | O | TLV | 3 |
| 6A | T3324 value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 67 | UE radio capability ID | UE radio capability ID 9.11.3.68 | O | TLV | 3-n |
| E- | UE radio capability ID deletion indication | UE radio capability ID deletion indication 9.11.3.69 | O | TV | 1 |
| 39 | Pending NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-146 |
| 74 | Ciphering key data | Ciphering key data 9.11.3.18C | O | TLV-E | 34-n |
| 75 | CAG information list | CAG information list 9.11.3.18A | O | TLV-E | 3-n |
| 1B | Truncated 5G-S-TMSI configuration | Truncated 5G-S-TMSI configuration 9.11.3.70 | O | TLV | 3 |
| 1C | Negotiated WUS assistance information | WUS assistance information 9.11.3.71 | O | TLV | 3 |
| 29 | Negotiated NB-N1 mode DRX parameters | NB-N1 mode DRX parameters 9.11.3.73 | O | TLV | 3 |
| 68 | Extended rejected NSSAI | Extended rejected NSSAI 9.11.3.75 | O | TLV | 5-90 |
| 7B | Service-level-AA container | Service-level-AA container 9.11.2.10 | O | TLV-E | 4-65538 |
| 33 | Negotiated PEIPS assistance information | PEIPS assistance information 9.11.3.80 | O | TLV | 3 |
| 35 | 5GS additional request result | 5GS additional request result 9.11.3.81 | O | TLV | 3 |
| 70 | NSSRG information | NSSRG information 9.11.3.82 | O | TLV-E | 7-4099 |
| 14 | Disaster roaming wait range | Registration wait range 9.11.3.84 | O | TLV | 4 |
| 2C | Disaster return wait range | Registration wait range 9.11.3.84 | O | TLV | 4 |
| 13 | List of PLMNs to be used in disaster condition | List of PLMNs to be used in disaster condition 9.11.3.83 | O | TLV | 2-n |
| 1D | Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 1E | Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 71 | Extended CAG information list | Extended CAG information list 9.11.3.86 | O | TLV-E | 3-n |
| 7C | NSAG information | NSAG information 9.11.3.87 | O | TLV-E | 9-3143 |
| 3D | Equivalent SNPNs | SNPN list 9.11.3.92 | O | TLV | 11-137 |
| 32 | NID | NID 9.11.3.79 | O | TLV | 8 |
| 7D | Registration accept type 6 IE container | Type 6 IE container 9.11.3.98 | O | TLV-E | 6-65538 |
| 4B | RAN timing synchronization | RAN timing synchronization 9.11.3.95 | O | TLV | 3 |
| 4C | Alternative NSSAI | Alternative NSSAI 9.11.3.97 | O | TLV | 2-146 |
| 4F | Discontinuous coverage maximum time offset | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 5B | S-NSSAI time validity information | S-NSSAI time validity information 9.11.3.101 | O | TLV | 23-257 |
| 3C | Unavailability configuration | Unavailability configuration 9.11.2.21 | O | TLV | 3-6 |
| 5C | Feature authorization indication | Feature authorization indication 9.11.3.105 | O | TLV | 3-257 |
| 61 | On-demand NSSAI | On-demand NSSAI 9.11.3.108 | O | TLV | 5-210 |
| 63 | Access technology utilization control | Access technology utilization control 9.11.3.110 | O | TLV | 4-5 |
| 64 | Negotiated LP-WUSPS assistance information | LP-WUSPS assistance information 9.11.3.111 | O | TLV | 3 |
| 8- | LP-WUS status | LP-WUS status 9.11.3.112 | O | TV | 1 |

#### 8.2.7.2	5G-GUTI

This IE may be included to assign a 5G-GUTI to a UE.

#### 8.2.7.3	Equivalent PLMNs

This IE may be included in order to assign an equivalent PLMNs list to a UE.

#### 8.2.7.4	TAI list

This IE may be included to assign a TAI list to a UE.

#### 8.2.7.5	Allowed NSSAI

This IE shall be included:

a)	if:

1)	one or more S-NSSAIs in the requested NSSAI of the REGISTRATION REQUEST message are allowed by the AMF for a network not supporting NSSAA;

2)	one or more S-NSSAIs in the requested NSSAI of the REGISTRATION REQUEST message are not subject to network slice-specific authentication and authorization and are allowed by the AMF; or

3)	the network slice-specific authentication and authorization has been successfully performed for one or more S-NSSAIs in the requested NSSAI of the REGISTRATION REQUEST message; or

b)	if the initial registration request is not for onboarding services in SNPN or the UE is not registered for onboarding services in SNPN, the requested NSSAI was not included in the REGISTRATION REQUEST message or none of the requested NSSAI are allowed; and

1)	the network does not support NSSAA; or

2)	the network has one or more default S-NSSAIs which are not subject to network slice-specific authentication and authorization.

#### 8.2.7.6	Rejected NSSAI

The network may include this IE to inform the UE of one or more S-NSSAIs that were included in the requested NSSAI in the REGISTRATION REQUEST message but were rejected by the network.

#### 8.2.7.7	Configured NSSAI

The network may include this IE if the network needs to provide the UE with a new configured NSSAI for the current PLMN or SNPN and the UE is neither registering nor registered for onboarding services in SNPN.

#### 8.2.7.8	5GS network feature support

The network may include this IE to inform the UE of the support of certain features. If this IE is not included then the UE shall interpret this as a receipt of an information element with all bits of the value part coded as zero.

#### 8.2.7.9	PDU session status

This IE shall be included when the network needs to indicate the PDU sessions that are associated with the access type the message is sent over, that are active in the network.

#### 8.2.7.10	PDU session reactivation result

This IE shall be included:

-	if the Uplink data status IE is included in the REGISTRATION REQUEST message; or

-	if the Allowed PDU session status IE is included in the REGISTRATION REQUEST message and there is at least one PDU session indicated in the Allowed PDU session status IE for which the user-plane resources can be re-established over 3GPP access.

#### 8.2.7.11	PDU session reactivation result error cause

This IE may be included, if the PDU session reactivation result IE is included and there exist one or more PDU sessions for which the user-plane resources cannot be re-established, to indicate the cause of failure to re-establish the user-plane resources.

#### 8.2.7.12	LADN information

The network shall include this IE if there are valid LADN service area(s) for the subscribed DNN(s) of the UE in the current registration area.

#### 8.2.7.13	MICO indication

The network shall include the MICO indication IE if:

a)-	the UE included the MICO indication IE in the REGISTRATION REQUEST message; and

b)	the network supports and accepts the use of MICO mode.

#### 8.2.7.14	Network slicing indication

This IE shall be included if the user's network slicing subscription has changed in the UDM of a PLMN or an SNPN.

#### 8.2.7.15	Service area list

This IE may be included to assign new service area restrictions to the UE.

#### 8.2.7.16	T3512 value

The AMF shall include this IE during a registration procedure over 3GPP access when the 5GS registration type IE does not indicate "periodic registration updating". The AMF may include this IE during the mobility and periodic registration update procedure over 3GPP access when the 5GS registration type IE indicates "periodic registration updating".

#### 8.2.7.17	Non-3GPP de-registration timer value

This IE may be included if the network needs to indicate to the UE registered over non-3GPP access the value of a non-3GPP de-registration timer value.

#### 8.2.7.18	T3502 value

This IE may be included to indicate a value for timer T3502.

#### 8.2.7.19	Emergency number list

This IE may be sent by the network. If this IE is sent, the contents of this IE indicates a list of emergency numbers valid within the same country as in the PLMN from which this IE is received.

#### 8.2.7.20	Extended emergency number list

This IE may be sent by the network. If this IE is sent, the contents of this IE indicates a list of emergency numbers (with URN information) valid within the same country as in the PLMN from which this IE is received or valid only in the PLMN or SNPN from which this IE is received.

#### 8.2.7.21	SOR transparent container

This IE may be sent by the network.

#### 8.2.7.22	EAP message

EAP message IE is included if the REGISTRATION ACCEPT message is sent as part of registration for emergency services and is used to convey EAP-failure message.

#### 8.2.7.23	NSSAI inclusion mode

This IE shall be included if required by operatory policy.

#### 8.2.7.24	Operator-defined access category definitions

This IE may be included to assign new operator-defined access category definitions to the UE or delete the operator-defined access category definitions at the UE side.

#### 8.2.7.25	Negotiated DRX parameters

The network shall include this IE if the Requested DRX parameters IE was included in the REGISTRATION REQUEST message.

#### 8.2.7.26	Non-3GPP NW policies

The AMF shall not include this IE during a registration procedure over non-3GPP access.

This IE is included if the network needs to indicate whether emergency numbers provided via non-3GPP access can be used to initiate UE detected emergency calls (see 3GPP TS 24.302 [16]). If this IE is not included then the UE shall interpret this as a receipt of an information element with all bits of the value part coded as zero.

NOTE:	In this version of the specification, this IE is applicable in case the UE is connected to a PLMN using an ePDG as specified in 3GPP TS 24.302 [16].

#### 8.2.7.26A	EPS bearer context status

This IE shall be included when the network generated an EPS bearer context status information for the UE during the inter-system change from S1 mode to N1 mode and the network supports N26 interface.

#### 8.2.7.27	Negotiated extended DRX parameters

The network shall include the Negotiated extended DRX parameters IE if:

-	the UE included the Requested extended DRX parameters IE in the REGISTRATION REQUEST message; and

-	the network supports eDRX and accepts the use of eDRX.

#### 8.2.7.28	T3447 value

The network may include T3447 value IE if:

-	the UE has indicated support for service gap control in the REGISTRATION REQUEST message; and

-	the 5GMM context contains a service gap time value.

#### 8.2.7.29	T3448 value

The network may include this IE if the congestion control for transport of user data via the control plane is active and the UE supports the control plane CIoT 5GS optimizations.

#### 8.2.7.30	T3324 value

The AMF shall include this IE if the UE has requested active time value in the REGISTRATION REQEUST message and the AMF decides to accept the use of MICO mode and the use of the active time.

#### 8.2.7.31	void

#### 8.2.7.32	UE radio capability ID

This IE may be included if the UE is not in NB-N1 mode, both the UE and the network support RACS and the network needs to assign a network-assigned UE radio capability ID to the UE.

#### 8.2.7.33	UE radio capability ID deletion indication

This IE may be included if the UE is not in NB-N1 mode, both the UE and the network support RACS and the network needs to trigger the UE to delete all network-assigned UE radio capability IDs stored at the UE for the serving PLMN or SNPN.

#### 8.2.7.34	Pending NSSAI

The network may include this IE to inform the UE of one or more S-NSSAIs that are pending as the network slice-specific authentication and authorization procedure is not completed.

#### 8.2.7.35	Ciphering key data

This IE is included if the network needs to send ciphering key data to the UE for ciphered broadcast assistance data.

#### 8.2.7.36	CAG information list

This IE may be included to assign a new "CAG information list" to the UE or delete the "CAG information list" at the UE side.

#### 8.2.7.37	Truncated 5G-S-TMSI configuration

The network shall include this IE if:

-	the UE is in NB-N1 mode;

-	the UE requests "control plane CIoT 5GS optimization" in the 5GS update type IE of REGISTRATION REQUEST message;

-	the AMF decides to accept the requested CIoT 5GS optimization; and

-	the network is configured to provide the truncated 5G-S-TMSI configuration for control plane CIoT 5GS optimizations.

#### 8.2.7.38	Negotiated NB-N1 mode DRX parameters

The network shall include the Negotiated NB-N1 mode DRX parameters IE if the requested NB-N1 mode DRX parameters IE was included in the REGISTRATION REQUEST message.

#### 8.2.7.39	Negotiated WUS assistance information

The network shall include the Negotiated WUS assistance information IE if:

-	the UE supports WUS assistance information;

-	the AMF supports and accepts the use of WUS assistance information;

-	the UE is not performing the initial registration for emergency services; and

-	the UE does not have an active emergency PDU session.

#### 8.2.7.40	Extended rejected NSSAI

If the UE supports Extended rejected NSSAI, the network may include this IE to inform the UE of one or more S-NSSAIs that were included in the requested NSSAI in the REGISTRATION REQUEST message but were rejected by the network.

#### 8.2.7.41	Service-level-AA container

The network shall include this IE if the UUAA procedure is triggered for the UE supporting UAS services. The network may include this IE if there is a valid successful UUAA result for the UE in the UE 5GMM context upon a registration for UAS services is requested from the UE.

#### 8.2.7.42	Negotiated PEIPS assistance information

The network shall include the Negotiated PEIPS assistance information IE if the network needs to provide the PEIPS assistance information to the UE.

#### 8.2.7.42A	Void

#### 8.2.7.43	5GS additional request result

The network may include this IE to inform the UE about the result of additional request.

#### 8.2.7.44	NSSRG information

This IE may be included to provide NSSRG information associated with the configured NSSAI.

#### 8.2.7.45	Disaster roaming wait range

This IE may be included to assign a new disaster roaming wait range to the UE.

#### 8.2.7.46	Disaster return wait range

This IE may be included to assign a new disaster return wait range to the UE.

#### 8.2.7.47	List of PLMNs to be used in disaster condition

This IE may be included by an allowed PLMN to assign a new "list of PLMN(s) to be used in disaster condition" associated with the serving PLMN to the UE.

#### 8.2.7.48	Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming"

This IE is included to indicate the forbidden TAI(s) to be stored in the list of "5GS forbidden tracking areas for roaming". This IE is included only if the message is sent via satellite NG-RAN access.

#### 8.2.7.49	Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service"

This IE is included to indicate the forbidden TAI(s) to be stored in the list of "5GS forbidden tracking areas for regional provision of service". This IE is included only if the message is sent via satellite NG-RAN access.

#### 8.2.7.50	Extended CAG information list

If the UE supports Extended CAG information list, the network may include this IE to assign a new "CAG information list" to the UE or delete the "CAG information list" at the UE side.

#### 8.2.7.51	NSAG information

If the UE has set the NSAG bit to "NSAG supported" in the 5GMM capability IE of the REGISTRATION REQUEST message over 3GPP access, the network may include this IE to provide NSAG information to the UE. Otherwise, the network shall not include this IE (see the 'comprehension required' scheme in clause 11.2.5 of 3GPP TS 24.007 [11]).

#### 8.2.7.52	Equivalent SNPNs

This IE may be included in order to assign an equivalent SNPNs list to a UE, by an AMF of an SNPN.

#### 8.2.7.53	NID

This IE shall be included if the UE supports equivalent SNPNs and the serving SNPN changes.

#### 8.2.7.54	Registration accept type 6 IE container

##### 8.2.7.54.1	General

This information element may be included only if the network knows that the UE will not treat this IE as unknown 'comprehension required' IE. Otherwise, the network shall not include this IE (see the 'comprehension required' scheme in clause 11.2.5 of 3GPP TS 24.007 [11]).

In this version of the specification, only the transfer of the information elements specified in table 8.2.7.54.1.1 is supported in the Registration accept type 6 IE container information element in the present message. For the handling of an information element with an IEI not listed in table 8.2.7.54.1.1, i.e., with an IEI unknown in the Registration accept type 6 IE container information element, see clause 7.6.4.1.

Table 8.2.7.54.1.1: Information elements and IEIs for the Registration accept type 6 IE container

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
| 01 | Extended LADN information | Extended LADN information 9.11.3.96 | O | TLV-E | 15-1787 |
| 02 | S-NSSAI location validity information | S-NSSAI location validity information 9.11.3.100 | O | TLV-E | 17-38611 |
| 03 | Partially allowed NSSAI | Partial NSSAI 9.11.3.103 | O | TLV-E | 3-808 |
| 04 | Partially rejected NSSAI | Partial NSSAI 9.11.3.103 | O | TLV-E | 3-808 |

##### 8.2.7.54.2	Extended LADN information

The network shall include this IE if the UE supports LADN per DNN and S-NSSAI and there are valid LADN service area(s) for the subscribed DNN(s) of the UE and S-NSSAI associated with the LADN in the current registration area.

##### 8.2.7.54.3	S-NSSAI location validity information

The network may include this IE to update the S-NSSAI location validity information.

##### 8.2.7.54.4	Void

##### 8.2.7.54.5	Partially allowed NSSAI

The network may include this IE to assign a partially allowed NSSAI to the UE.

##### 8.2.7.54.6	Partially rejected NSSAI

The network may include this IE to assign a partially rejected NSSAI to the UE.

#### 8.2.7.55	RAN timing synchronization

This IE may be included to provide information related to the RAN timing synchronization to a UE which has set the Reconnection to the network due to RAN timing synchronization status change (RANtiming) bit to "Reconnection to the network due to RAN timing synchronization status change supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

#### 8.2.7.56	Alternative NSSAI

The network may include this IE to provide the alternative NSSAI to the UE.

#### 8.2.7.57	Discontinuous coverage maximum time offset

This IE may be included to provide the UE with a discontinuous coverage maximum time offset.

#### 8.2.7.58	S-NSSAI time validity information

If the UE supports S-NSSAI time validity information, the network may include this IE to update the S-NSSAI time validity information.

#### 8.2.7.59	Unavailability configuration

The network should include this IE if the network needs to provide parameters related to unavailability period to the UE.

#### 8.2.7.60	Feature authorization indication

The network may include this IE to inform to the UE about the features for which the UE is authorized to use.

#### 8.2.7.61	On-demand NSSAI

The network may include this IE to provide the on-demand NSSAI to the UE.

#### 8.2.7.62	Access technology utilization control

This IE is included to indicate the restricted access technology(s).

#### 8.2.7.63	Negotiated LP-WUSPS assistance information

The network shall include the Negotiated LP-WUSPS assistance information IE if the network needs to provide the LP-WUSPS assistance information to the UE.

#### 8.2.7.64	LP-WUS status

This IE may be included to indicate LP-WUS status.

### 8.2.8	Registration complete

#### 8.2.8.1	Message definition

The REGISTRATION COMPLETE message is sent by the UE to the AMF. See table 8.2.8.1.1.

Message type:	REGISTRATION COMPLETE

Significance:	dual

Direction:	UE to network

Table 8.2.8.1.1: REGISTRATION COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Registration complete message identity | Message type 9.7 | M | V | 1 |
| 73 | SOR transparent container | SOR transparent container 9.11.3.51 | O | TLV-E | 20 |

#### 8.2.8.2	SOR transparent container

This IE may be sent by the UE. If this IE is sent, the contents of this IE indicates the UE acknowledgement of successful reception of the SOR transparent container IE in the REGISTRATION ACCEPT message. This IE shall indicate the ME support of SOR-CMCI.

### 8.2.9	Registration reject

#### 8.2.9.1	Message definition

The REGISTRATION REJECT message is sent by the AMF to the UE. See table 8.2.9.1.1.

Message type:	REGISTRATION REJECT

Significance:	dual

Direction:	network to UE

Table 8.2.9.1.1: REGISTRATION REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Registration reject message identity | Message type 9.7 | M | V | 1 |
|  | 5GMM cause | 5GMM cause 9.11.3.2 | M | V | 1 |
| 5F | T3346 value | GPRS timer 2 9.11.2.4 | O | TLV | 3 |
| 16 | T3502 value | GPRS timer 2 9.11.2.4 | O | TLV | 3 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |
| 69 | Rejected NSSAI | Rejected NSSAI 9.11.3.46 | O | TLV | 4-42 |
| 75 | CAG information list | CAG information list 9.11.3.18A | O | TLV-E | 3-n |
| 68 | Extended rejected NSSAI | Extended rejected NSSAI 9.11.3.75 | O | TLV | 5-90 |
| 2C | Disaster return wait range | Registration wait range 9.11.3.84 | O | TLV | 4 |
| 71 | Extended CAG information list | Extended CAG information list 9.11.3.86 | O | TLV-E | 3-n |
| 3A | Lower bound timer value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 1D | Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 1E | Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 3E | N3IWF identifier | N3IWF identifier 9.11.3.93 | O | TLV | 7-n |
| 4D | TNAN information | TNAN information 9.11.3.94 | O | TLV | 3-n |
| 62 | Extended 5GMM cause | Extended 5GMM cause 9.11.3.109 | O | TLV | 3 |
| 63 | Access technology utilization control | Access technology utilization control 9.11.3.110 | O | TLV | 4-5 |

NOTE:	It is possible for AMFs compliant with version 17.7.0 or 17.8.0 of this specification to send the Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" IE with IEI of value "3B" for this message or the Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" IE with IEI of value "3C" for this message.

#### 8.2.9.2	T3346 value

The AMF may include this IE when the general NAS level mobility management congestion control is active

#### 8.2.9.3	T3502 value

This IE may be included to indicate a value for timer T3502 during the initial registration.

#### 8.2.9.4	EAP message

EAP message IE is included if the REGISTRATION REJECT message is used to convey EAP-failure message.

#### 8.2.9.5	Rejected NSSAI

The network may include this IE to inform the UE of one or more S-NSSAIs that were included in the requested NSSAI in the REGISTRATION REQUEST message but were rejected by the network.

#### 8.2.9.6	CAG information list

This IE may be included to assign a new "CAG information list" to the UE or delete the "CAG information list" at the UE side.

#### 8.2.9.7	Extended rejected NSSAI

If the UE supports Extended rejected NSSAI, the network may include this IE to inform the UE of one or more S-NSSAIs that were included in the requested NSSAI in the REGISTRATION REQUEST message but were rejected by the network.

#### 8.2.9.8	Disaster return wait range

This IE may be included to assign a new disaster return wait range to the UE.

#### 8.2.9.9	Extended CAG information list

If the UE supports Extended CAG information list, the network may include this IE to assign a new "CAG information list" to the UE or delete the "CAG information list" at the UE side.

#### 8.2.9.10	Lower bound timer value

The AMF may include this IE when the 5GMM cause is set to #78 "PLMN not allowed to operate at the present UE location", to provide a minimum time value for an entry added to the list of "PLMNs not allowed to operate at the present UE location".

#### 8.2.9.11	Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming"

This IE is included to indicate the forbidden TAI(s) to be stored in the list of "5GS forbidden tracking areas for roaming". This IE is included only if the message is sent via satellite NG-RAN access.

#### 8.2.9.12	Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service"

This IE is included to indicate the forbidden TAI(s) to be stored in the list of "5GS forbidden tracking areas for regional provision of service". This IE is included only if the message is sent via satellite NG-RAN access.

#### 8.2.9.13	N3IWF identifier

The network may include this IE if the UE supports slice-based N3IWF selection over non-3GPP access network.

#### 8.2.9.14	TNAN information

The network may include this IE if the UE supports slice-based TNGF selection over trusted non-3GPP access network.

#### 8.2.9.15	Extended 5GMM cause

This IE may be included by the network to indicate additional information associated with the 5GMM cause value #15 "no suitable cells in tracking area".

#### 8.2.9.16	Access technology utilization control

This IE is included to indicate the restricted access technology(s).

### 8.2.10	UL NAS transport

#### 8.2.10.1	Message definition

The UL NAS TRANSPORT message transports message payload and associated information to the AMF. See table 8.2.10.1.1.

Message type:	UL NAS TRANSPORT

Significance:	dual

Direction:	UE to network

Table 8.2.10.1.1: UL NAS TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | UL NAS TRANSPORT message identity | Message type 9.7 | M | V | 1 |
|  | Payload container type | Payload container type 9.11.3.40 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Payload container | Payload container 9.11.3.39 | M | LV-E | 3-65537 |
| 12 | PDU session ID | PDU session identity 2 9.11.3.41 | C | TV | 2 |
| 59 | Old PDU session ID | PDU session identity 2 9.11.3.41 | O | TV | 2 |
| 8- | Request type | Request type 9.11.3.47 | O | TV | 1 |
| 22 | S-NSSAI | S-NSSAI 9.11.2.8 | O | TLV | 3-10 |
| 25 | DNN | DNN 9.11.2.1B | O | TLV | 3-102 |
| 24 | Additional information | Additional information 9.11.2.1 | O | TLV | 3-n |
| A- | MA PDU session information | MA PDU session information 9.11.3.31A | O | TV | 1 |
| F- | Release assistance indication | Release assistance indication 9.11.3.46A | O | TV | 1 |
| 4E | Non-3GPP access path switching indication | Non-3GPP access path switching indication 9.11.3.99 | O | TLV | 3 |
| 5A | Alternative S-NSSAI | S-NSSAI 9.11.2.8 | O | TLV | 3-10 |
| 9- | Payload container information | Payload container information 9.11.3.106 | O | TV | 1 |

#### 8.2.10.2	PDU session ID

The UE shall include this IE when the Payload container type IE is set to "N1 SM information" or "CIoT user data.

#### 8.2.10.3	Old PDU session ID

The UE shall include this IE if the UL NAS TRANSPORT message transports a PDU SESSION ESTABLISHMENT REQUEST message upon receiving the PDU SESSION MODIFICATION COMMAND message with the 5GSM cause IE set to #39 "reactivation requested" and the Payload container type IE is set to "N1 SM information".

#### 8.2.10.4	Request type

The UE shall include this IE when the PDU session ID IE is included and the Payload container IE contains the PDU SESSION ESTABLISHMENT REQUEST message or the PDU SESSION MODIFICATION REQUEST which is not initiated to indicate a change of 3GPP PS data off UE status associated to a PDU session.

#### 8.2.10.5	S-NSSAI

The UE may include this IE when the Request type IE is set to "initial request", "existing PDU session" or "MA PDU request", the Payload container type IE is set to "N1 SM information" and the UE is not registered for onboarding services in SNPN.

#### 8.2.10.6	DNN

The UE may include this IE when the Request type IE is set to "initial request", "existing PDU session" or "MA PDU request", the Payload container type IE is set to "N1 SM information" and the UE is not registered for onboarding services in SNPN.

#### 8.2.10.7	Additional information

This IE is included when the Payload container type IE is set to "LTE Positioning Protocol (LPP) message container"or "SLPP message container". This IE may be included when the Payload container type IE is set to "Location services message container"or "UPP-CMI container".

#### 8.2.10.8	MA PDU session information

The UE may include this IE if the Request type IE is included and is not set to "initial emergency request " or "existing emergency PDU session" in the UL NAS TRANSPORT message.

#### 8.2.10.9	Release assistance indication

The UE may include this IE to inform the network whether:

-	no further uplink and no further downlink data transmission is expected; or

-	only a single downlink data transmission (e.g. acknowledgement or response to uplink data) and no further uplink data transmission subsequent to the uplink data transmission is expected.

#### 8.2.10.10	Non-3GPP access path switching indication

The UE may include this IE when the Payload container type IE is set to "N1 SM information" to indicate whether the UE supports the non-3GPP access path switching for the PDU session.

#### 8.2.10.11	Alternative S-NSSAI

The UE shall include this IE if the UE can provide alternative S-NSSAI associated with the S-NSSAI to be replaced to the network.

#### 8.2.10.12	Payload container information

The UE may include this IE when the Payload container type IE is set to "Location services message container".

### 8.2.11	DL NAS transport

#### 8.2.11.1	Message definition

The DL NAS TRANSPORT message transports message payload and associated information to the UE. See table 8.2.11.1.1.

Message type:	DL NAS TRANSPORT

Significance:	dual

Direction:	network to UE

Table 8.2.11.1.1: DL NAS TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | DL NAS TRANSPORT message identity | Message type 9.7 | M | V | 1 |
|  | Payload container type | Payload container type 9.11.3.40 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Payload container | Payload container 9.11.3.39 | M | LV-E | 3-65537 |
| 12 | PDU session ID | PDU session identity 2 9.11.3.41 | C | TV | 2 |
| 24 | Additional information | Additional information 9.11.2.1 | O | TLV | 3-n |
| 58 | 5GMM cause | 5GMM cause 9.11.3.2 | O | TV | 2 |
| 37 | Back-off timer value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 3A | Lower bound timer value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |

#### 8.2.11.2	PDU session ID

The AMF shall include this IE when the Payload container type IE is set to "N1 SM information" or "CIoT user data container".

#### 8.2.11.3	Additional information

This IE is included when the Payload container type IE is set to "LTE Positioning Protocol (LPP) message container", "UPP-CMI container", "SLPP message container", or "Location services message container".

#### 8.2.11.4	5GMM cause

The AMF shall include this IE when the Payload container IE contains an uplink payload which was not forwarded and the Payload container type IE is not set to "Multiple payloads".

#### 8.2.11.5	Back-off timer value

The AMF may include this IE to indicate the back-off timer value when the Payload container IE is included and the Payload container type IE is not set to "Multiple payloads".

#### 8.2.11.6	Lower bound timer value

The AMF may include this IE when the 5GMM cause is set to #78 "PLMN not allowed to operate at the present UE location", to provide a minimum time value for an entry added to the list of "PLMNs not allowed to operate at the present UE location".

### 8.2.12	De-registration request (UE originating de-registration)

#### 8.2.12.1	Message definition

The DEREGISTRATION REQUEST message is sent by the UE to the AMF. See table 8.2.12.1.1.

Message type:	DEREGISTRATION REQUEST

Significance:	dual

Direction:	UE to network

Table 8.2.12.1.1: DEREGISTRATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | De-registration request message identity | Message type 9.7 | M | V | 1 |
|  | De-registration type | De-registration type 9.11.3.20 | M | V | 1/2 |
|  | ngKSI | NAS key set identifier 9.11.3.32 | M | V | 1/2 |
|  | 5GS mobile identity | 5GS mobile identity 9.11.3.4 | M | LV-E | 6-n |
| 3C | Unavailability information | Unavailability information 9.11.2.20 | O | TLV | 3-9 |
| 71 | NAS message container | NAS message container 9.11.3.33 | O | TLV-E | 4-n |

#### 8.2.12.2	Unavailability information

The UE may include this IE when an event is triggered in the UE that would make the UE unavailable for a certain period and the use of unavailability period is not due to NR satellite access discontinuous coverage.

#### 8.2.12.3	NAS message container

This IE shall be included if the UE is sending a DEREGISTRATION REQUEST message as an initial NAS message and the UE needs to send non-cleartext IEs.

### 8.2.13	De-registration accept (UE originating de-registration)

#### 8.2.13.1	Message definition

The DEREGISTRATION ACCEPT message is sent by the AMF to the UE. See table 8.2.13.1.1.

Message type:	DEREGISTRATION ACCEPT

Significance:	dual

Direction:	network to UE

Table 8.2.13.1.1: DEREGISTRATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | De-registration accept message identity | Message type 9.7 | M | V | 1 |

### 8.2.14	De-registration request (UE terminated de-registration)

#### 8.2.14.1	Message definition

The DEREGISTRATION REQUEST message is sent by the AMF to the UE. See table 8.2.14.1.1.

Message type:	DEREGISTRATION REQUEST

Significance:	dual

Direction:	network to UE

Table 8.2.14.1.1: DEREGISTRATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | De-registration request message identity | Message type 9.7 | M | V | 1 |
|  | De-registration type | De-registration type 9.11.3.20 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
| 58 | 5GMM cause | 5GMM cause 9.11.3.2 | O | TV | 2 |
| 5F | T3346 value | GPRS timer 2 9.11.2.4 | O | TLV | 3 |
| 6D | Rejected NSSAI | Rejected NSSAI 9.11.3.46 | O | TLV | 4-42 |
| 75 | CAG information list | CAG information list 9.11.3.18A | O | TLV-E | 3-n |
| 68 | Extended rejected NSSAI | Extended rejected NSSAI 9.11.3.75 | O | TLV | 5-90 |
| 2C | Disaster return wait range | Registration wait range 9.11.3.84 | O | TLV | 4 |
| 71 | Extended CAG information list | Extended CAG information list 9.11.3.86 | O | TLV-E | 3-n |
| 3A | Lower bound timer value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 1D | Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 1E | Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 63 | Access technology utilization control | Access technology utilization control 9.11.3.110 | O | TLV | 4-5 |

NOTE:	It is possible for AMFs compliant with version 17.7.0 or 17.8.0 of this specification to send the Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" IE with IEI of value "3B" for this message or the Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" IE with IEI of value "3C" for this message.

#### 8.2.14.2	5GMM cause

This information element is included if a 5GMM cause is provided.

#### 8.2.14.3	T3346 value

The AMF may include this IE when the general NAS level mobility management congestion control is active.

#### 8.2.14.4	Rejected NSSAI

The AMF may include this IE to inform the UE of one or more S-NSSAIs that were rejected by the network.

#### 8.2.14.5	CAG information list

This IE may be included to assign a new "CAG information list" to the UE or delete the "CAG information list" at the UE side.

#### 8.2.14.6	Extended rejected NSSAI

If the UE supports Extended rejected NSSAI, the AMF may include this IE to inform the UE of one or more S-NSSAIs that were rejected by the network.

#### 8.2.14.7	Disaster return wait range

This IE may be included to assign a new disaster return wait range to the UE.

#### 8.2.14.7A	Extended CAG information list

If the UE supports Extended CAG information list, the network may include this IE to assign a new "CAG information list" to the UE or delete the "CAG information list" at the UE side.

#### 8.2.14.8	Lower bound timer value

The AMF may include this IE when the 5GMM cause is set to #78 "PLMN not allowed to operate at the present UE location", to provide a minimum time value for an entry added to the list of "PLMNs not allowed to operate at the present UE location".

#### 8.2.14.9	Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming"

This IE is included to indicate the forbidden TAI(s) to be stored in the list of "5GS forbidden tracking areas for roaming". This IE is included only if the message is sent via satellite NG-RAN access.

#### 8.2.14.10	Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service"

This IE is included to indicate the forbidden TAI(s) to be stored in the list of "5GS forbidden tracking areas for regional provision of service". This IE is included only if the message is sent via satellite NG-RAN access.

#### 8.2.14.11	Access technology utilization control

This IE is included to indicate the restricted access technology(s).

### 8.2.15	De-registration accept (UE terminated de-registration)

#### 8.2.15.1	Message definition

The DEREGISTRATION ACCEPT message is sent by the UE to the AMF. See table 8.2.15.1.1.

Message type:	DEREGISTRATION ACCEPT

Significance:	dual

Direction:	UE to network

Table 8.2.15.1.1.1: DEREGISTRATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | De-registration accept message identity | Message type 9.7 | M | V | 1 |

### 8.2.16	Service request

#### 8.2.16.1	Message definition

The SERVICE REQUEST message is sent by the UE to the AMF in order to request the establishment of an N1 NAS signalling connection, to request the establishment of user-plane resources for PDU sessions which are established without user-plane resources, or both. See table 8.2.16.1.1.

Message type:	SERVICE REQUEST

Significance:	dual

Direction:	UE to network

Table 8.2.16.1.1: SERVICE REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Service request message identity | Message type 9.7 | M | V | 1 |
|  | ngKSI | NAS key set identifier 9.11.3.32 | M | V | 1/2 |
|  | Service type | Service type 9.11.3.50 | M | V | 1/2 |
|  | 5G-S-TMSI | 5GS mobile identity 9.11.3.4 | M | LV-E | 9 |
| 40 | Uplink data status | Uplink data status 9.11.3.57 | O | TLV | 4-34 |
| 50 | PDU session status | PDU session status 9.11.3.44 | O | TLV | 4-34 |
| 25 | Allowed PDU session status | Allowed PDU session status 9.11.3.13 | O | TLV | 4-34 |
| 71 | NAS message container | NAS message container 9.11.3.33 | O | TLV-E | 4-n |
| 29 | UE request type | UE request type 9.11.3.76 | O | TLV | 3 |
| 28 | Paging restriction | Paging restriction 9.11.3.77 | O | TLV | 3-35 |

#### 8.2.16.2	Uplink data status

This IE shall be included if the UE has uplink user data pending to be sent.

#### 8.2.16.3	PDU session status

This IE shall be included when the UE needs to indicate the PDU sessions that are associated with the access type that the message is sent over, that are active within the UE.

#### 8.2.16.4	Allowed PDU session status

This IE shall be included if the SERVICE REQUEST message is sent as a response to paging with the access type indicating non-3GPP access or notification via 3GPP access for PDU session(s) associated with non-3GPP access.

#### 8.2.16.5	NAS message container

This IE shall be included if the UE is sending a SERVICE REQUEST message as an initial NAS message and the UE needs to send non-cleartext IEs.

#### 8.2.16.6	UE request type

The UE shall include this IE if the MUSIM UE requests the release of the NAS signalling connection or rejects the paging request from the network.

#### 8.2.16.7	Paging restriction

The UE shall include this IE if the Request type is set to "NAS signalling connection release" or to "Rejection of paging" in the UE request type IE and the UE requests the network to restrict paging.

### 8.2.17	Service accept

#### 8.2.17.1	Message definition

The SERVICE ACCEPT message is sent by the AMF to the UE in order to accept the service request procedure. See table 8.2.17.1.1.

Message type:	SERVICE ACCEPT

Significance:	dual

Direction:	network to UE

Table 8.2.17.1.1: SERVICE ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Service accept message identity | Message type 9.7 | M | V | 1 |
| 50 | PDU session status | PDU session status 9.11.3.44 | O | TLV | 4-34 |
| 26 | PDU session reactivation result | PDU session reactivation result 9.11.3.42 | O | TLV | 4-34 |
| 72 | PDU session reactivation result error cause | PDU session reactivation result error cause 9.11.3.43 | O | TLV-E | 5-515 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |
| 6B | T3448 value | GPRS timer 2 9.11.2.4 | O | TLV | 3 |
| 34 | 5GS additional request result | 5GS additional request result 9.11.3.81 | O | TLV | 3 |
| 1D | Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 1E | Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |

#### 8.2.17.2	PDU session status

This IE shall be included when the network needs to indicate the PDU sessions that are associated with the access type that the message is sent over that are active within the network.

#### 8.2.17.3	PDU session reactivation result

This IE shall be included:

-	if the Uplink data status IE is included in the SERVICE REQUEST message; or

-	if the Allowed PDU session status IE is included in the SERVICE REQUEST message and there is at least one PDU session indicated in the Allowed PDU session status IE for which user-plane resources can be re-established over 3GPP access.

#### 8.2.17.4	PDU session reactivation result error cause

This IE may be included if the PDU session reactivation result IE is included and there exist one or more PDU sessions for which the user-plane resources cannot be re-established, to indicate the cause of failure to re-establish the user-plane resources.

#### 8.2.17.5	EAP message

EAP message IE is included if the SERVICE ACCEPT message is sent to a UE registered for emergency services and is used to convey EAP-failure message.

#### 8.2.17.6	T3448 value

The network may include this IE if the congestion control for transport of user data via the control plane is active and the UE supports the control plane CIoT 5GS optimizations.

#### 8.2.17.7	5GS additional request result

The network may include this IE to inform the UE about the result of additional request.

#### 8.2.17.8	Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming"

This IE is included to indicate the forbidden TAI(s) to be stored in the list of "5GS forbidden tracking areas for roaming". This IE is included only if the message is sent via satellite NG-RAN access.

#### 8.2.17.9	Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service"

This IE is included to indicate the forbidden TAI(s) to be stored in the list of "5GS forbidden tracking areas for regional provision of service". This IE is included only if the message is sent via satellite NG-RAN access.

### 8.2.18	Service reject

#### 8.2.18.1	Message definition

The SERVICE REJECT message is sent by the AMF to the UE in order to reject the service request procedure. See table 8.2.18.1.1.

Message type:	SERVICE REJECT

Significance:	dual

Direction:	network to UE

Table 8.2.18.1.1: SERVICE REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Service reject message identity | Message type 9.7 | M | V | 1 |
|  | 5GMM cause | 5GMM cause 9.11.3.2 | M | V | 1 |
| 50 | PDU session status | PDU session status 9.11.3.44 | O | TLV | 4-34 |
| 5F | T3346 value | GPRS timer 2 9.11.2.4 | O | TLV | 3 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |
| 6B | T3448 value | GPRS timer 2 9.11.2.4 | O | TLV | 3 |
| 75 | CAG information list | CAG information list 9.11.3.18A | O | TLV-E | 3-n |
| 2C | Disaster return wait range | Registration wait range 9.11.3.84 | O | TLV | 4 |
| 71 | Extended CAG information list | Extended CAG information list 9.11.3.86 | O | TLV-E | 3-n |
| 3A | Lower bound timer value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 1D | Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 1E | Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 63 | Access technology utilization control | Access technology utilization control 9.11.3.110 | O | TLV | 4-5 |

NOTE:	It is possible for AMFs compliant with version 17.7.0 or 17.8.0 of this specification to send the Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" IE with IEI of value "3B" for this message or the Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" IE with IEI of value "3C" for this message.

#### 8.2.18.2	PDU session status

This IE shall be included when the network needs to indicate the PDU sessions that are associated with the access type that the message is sent over, that are active within the network.

#### 8.2.18.3	T3346 value

The AMF may include this IE when the general NAS level mobility management congestion control is active.

#### 8.2.18.4	EAP message

EAP message IE is included if the SERVICE REJECT message is used to convey EAP-failure message.

#### 8.2.18.5	T3448 value

The network may include this IE if the congestion control for transport of user data via the control plane is active and the UE supports the control plane CIoT 5GS optimizations.

#### 8.2.18.6	CAG information list

This IE may be included to assign a new "CAG information list" to the UE or delete the "CAG information list" at the UE side.

#### 8.2.18.7	Disaster return wait range

This IE may be included to assign a new disaster return wait range to the UE.

#### 8.2.18.8	Extended CAG information list

If the UE supports Extended CAG information list, the network may include this IE to assign a new "CAG information list" to the UE or delete the "CAG information list" at the UE side.

#### 8.2.18.9	Lower bound timer value

The AMF may include this IE when the 5GMM cause is set to #78 "PLMN not allowed to operate at the present UE location", to provide a minimum time value for an entry added to the list of "PLMNs not allowed to operate at the present UE location".

#### 8.2.18.10	Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming"

This IE is included to indicate the forbidden TAI(s) to be stored in the list of "5GS forbidden tracking areas for roaming". This IE is included only if the message is sent via satellite NG-RAN access.

#### 8.2.18.11	Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service"

This IE is included to indicate the forbidden TAI(s) to be stored in the list of "5GS forbidden tracking areas for regional provision of service". This IE is included only if the message is sent via satellite NG-RAN access.

#### 8.2.18.12	Access technology utilization control

This IE is included to indicate the restricted access technology(s).

### 8.2.19	Configuration update command

#### 8.2.19.1	Message definition

The CONFIGURATION UPDATE COMMAND message is sent by the AMF to the UE. See table 8.2.19.1.1.

Message type:	CONFIGURATION UPDATE COMMAND

Significance:	dual

Direction:	network to UE

Table 8.2.19.1.1: CONFIGURATION UPDATE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Configuration update command message identity | Message type 9.7 | M | V | 1 |
| D- | Configuration update indication | Configuration update indication 9.11.3.18 | O | TV | 1 |
| 77 | 5G-GUTI | 5GS mobile identity 9.11.3.4 | O | TLV-E | 14 |
| 54 | TAI list | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 15 | Allowed NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-74 |
| 27 | Service area list | Service area list 9.11.3.49 | O | TLV | 6-114 |
| 43 | Full name for network | Network name 9.11.3.35 | O | TLV | 3-n |
| 45 | Short name for network | Network name 9.11.3.35 | O | TLV | 3-n |
| 46 | Local time zone | Time zone 9.11.3.52 | O | TV | 2 |
| 47 | Universal time and local time zone | Time zone and time 9.11.3.53 | O | TV | 8 |
| 49 | Network daylight saving time | Daylight saving time 9.11.3.19 | O | TLV | 3 |
| 79 | LADN information | LADN information 9.11.3.30 | O | TLV-E | 3-1715 |
| B- | MICO indication | MICO indication 9.11.3.31 | O | TV | 1 |
| 9- | Network slicing indication | Network slicing indication 9.11.3.36 | O | TV | 1 |
| 31 | Configured NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-146 |
| 11 | Rejected NSSAI | Rejected NSSAI 9.11.3.46 | O | TLV | 4-42 |
| 76 | Operator-defined access category definitions | Operator-defined access category definitions 9.11.3.38 | O | TLV-E | 3-8323 |
| F- | SMS indication | SMS indication 9.11.3.50A | O | TV | 1 |
| 6C | T3447 value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 75 | CAG information list | CAG information list 9.11.3.18A | O | TLV-E | 3-n |
| 67 | UE radio capability ID | UE radio capability ID 9.11.3.68 | O | TLV | 3-n |
| A- | UE radio capability ID deletion indication | UE radio capability ID deletion indication 9.11.3.69 | O | TV | 1 |
| 44 | 5GS registration result | 5GS registration result 9.11.3.6 | O | TLV | 3 |
| 1B | Truncated 5G-S-TMSI configuration | Truncated 5G-S-TMSI configuration 9.11.3.70 | O | TLV | 3 |
| C- | Additional configuration indication | Additional configuration indication 9.11.3.74 | O | TV | 1 |
| 68 | Extended rejected NSSAI | Extended rejected NSSAI 9.11.3.75 | O | TLV | 5-90 |
| 72 | Service-level-AA container | Service-level-AA container 9.11.2.10 | O | TLV-E | 4-65538 |
| 70 | NSSRG information | NSSRG information 9.11.3.82 | O | TLV-E | 7-4099 |
| 14 | Disaster roaming wait range | Registration wait range 9.11.3.84 | O | TLV | 4 |
| 2C | Disaster return wait range | Registration wait range 9.11.3.84 | O | TLV | 4 |
| 13 | List of PLMNs to be used in disaster condition | List of PLMNs to be used in disaster condition 9.11.3.83 | O | TLV | 2-n |
| 71 | Extended CAG information list | Extended CAG information list 9.11.3.86 | O | TLV-E | 3-n |
| 1F | Updated PEIPS assistance information | PEIPS assistance information 9.11.3.80 | O | TLV | 3 |
| 73 | NSAG information | NSAG information 9.11.3.87 | O | TLV-E | 9-3143 |
| E- | Priority indicator | Priority indicator 9.11.3.91 | O | TV | 1 |
| 4B | RAN timing synchronization | RAN timing synchronization 9.11.3.95 | O | TLV | 3 |
| 78 | Extended LADN information | Extended LADN information 9.11.3.96 | O | TLV-E | 3-1787 |
| 4C | Alternative NSSAI | Alternative NSSAI 9.11.3.97 | O | TLV | 2-146 |
| 7B | S-NSSAI location validity information | S-NSSAI location validity information 9.11.3.100 | O | TLV-E | 17-38611 |
| 5B | S-NSSAI time validity information | S-NSSAI time validity information 9.11.3.101 | O | TLV | 23-257 |
| 4F | Discontinuous coverage maximum time offset | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 74 | Partially allowed NSSAI | Partial NSSAI 9.11.3.103 | O | TLV-E | 3-808 |
| 7A | Partially rejected NSSAI | Partial NSSAI 9.11.3.103 | O | TLV-E | 3-808 |
| 5C | Feature authorization indication | Feature authorization indication 9.11.3.105 | O | TLV | 3-257 |
| 61 | On-demand NSSAI | On-demand NSSAI 9.11.3.108 | O | TLV | 5-210 |
| 63 | Access technology utilization control | Access technology utilization control 9.11.3.110 | O | TLV | 2-5 |
| 64 | Updated LP-WUSPS assistance information | LP-WUSPS assistance information 9.11.3.111 | O | TLV | 2-3 |
| 8- | LP-WUS status | LP-WUS status 9.11.3.112 | O | TV | 1 |

#### 8.2.19.2	Configuration update indication

The AMF shall include this IE if the AMF needs to request an acknowledgement or a registration procedure from the UE.

#### 8.2.19.3	5G-GUTI

This IE may be included to assign a new 5G-GUTI to the UE.

#### 8.2.19.4	TAI list

This IE may be included to assign a new TAI list to the UE.

#### 8.2.19.5	Allowed NSSAI

This IE may be included to assign a new allowed NSSAI to the UE not registered for onboarding services in SNPN.

#### 8.2.19.6	Service area list

This IE may be included to assign a new service area list to the UE.

#### 8.2.19.7	Full name for network

This IE may be included to assign a new full name for network to the UE.

#### 8.2.19.8	Short name for network

This IE may be included to assign a new short name for network to the UE.

#### 8.2.19.9	Local time zone

This IE may be included to assign a new local time zone to the UE.

#### 8.2.19.10	Universal time and local time zone

This IE may be included to assign new universal time and local time zone to the UE.

#### 8.2.19.11	Network daylight saving time

This IE may be included to assign new network daylight saving time to the UE.

#### 8.2.19.12	LADN information

This IE may be included to assign new LADN information to the UE or delete the LADN information at the UE side.

#### 8.2.19.13	MICO indication

This IE may be included to request the UE to re-negotiate MICO mode.

#### 8.2.19.14	Network slicing indication

This IE shall be included if the user's network slicing subscription has changed in the UDM of a PLMN or an SNPN.

#### 8.2.19.15	Configured NSSAI

The AMF shall include this IE when the AMF needs to provide the UE with a new configured NSSAI for the current PLMN or SNPN and the UE is neither registering nor registered for onboarding services in SNPN.

#### 8.2.19.16	Rejected NSSAI

The network may include this IE to inform the UE of one or more S-NSSAIs that were previously sent to the UE in the allowed NSSAI or the pending NSSAI, but are now considered rejected by the network.

#### 8.2.19.17	Operator-defined access category definitions

This IE may be included to assign new operator-defined access category definitions to the UE or delete the operator-defined access category definitions at the UE side.

#### 8.2.19.18	SMS indication

This IE may be included to indicate that the ability for the UE to use SMS over NAS has changed.

#### 8.2.19.19	T3447 value

This IE may be included to assign a new T3447 value to the UE.

#### 8.2.19.20	CAG information list

This IE may be included to assign new "CAG information list" to the UE or delete the "CAG information list" at the UE side.

#### 8.2.19.21	UE radio capability ID

This IE may be included if the UE is not in NB-N1 mode, both the UE and the network support RACS and the network needs to assign a network-assigned UE radio capability ID to the UE.

#### 8.2.19.22	UE radio capability ID deletion indication

This IE may be included if the UE is not in NB-N1 mode, both the UE and the network support RACS and the network needs to trigger the UE to delete all network-assigned UE radio capability IDs stored at the UE for the serving PLMN or serving SNPN.

#### 8.2.19.23	5GS registration result

This IE shall be included if the network wants to indicate to the UE that the UE is registered for emergency services.

#### 8.2.19.24	Truncated 5G-S-TMSI configuration

This IE may be included to provide a new truncated 5G-S-TMSI configuration to the UE in NB-N1 mode if the network is configured to provide the truncated 5G-S-TMSI configuration for control plane CIoT 5GS optimizations.

#### 8.2.19.25	Additional configuration indication

The network may include this IE when requesting the UE to register without the release of the N1 NAS signalling connection.

#### 8.2.19.26	Extended rejected NSSAI

If the UE supports Extended rejected NSSAI, the network may include this IE to inform the UE of one or more S-NSSAIs that were previously sent to the UE in the allowed NSSAI or the pending NSSAI, but are now considered rejected by the network.

#### 8.2.19.27	Service-level-AA container

The network shall include this IE when the AMF receives the Service-level-AA payload or the UUAA-MM result from the UAS-NF during the UUAA-MM procedure or the UUAA revocation procedure. The network shall also include this IE if the AMF receives from the UAS-NF, the CAA-level UAV ID as part of the UUAA-MM procedure.

#### 8.2.19.28	NSSRG information

This IE may be included to provide NSSRG information associated with the configured NSSAI.

#### 8.2.19.29	Disaster roaming wait range

This IE may be included to assign a new disaster roaming wait range to the UE.

#### 8.2.19.30	Disaster return wait range

This IE may be included to assign a new disaster return wait range to the UE.

#### 8.2.19.31	List of PLMNs to be used in disaster condition

This IE may be included by an allowed PLMN to assign a new "list of PLMN(s) to be used in disaster condition" associated with the serving PLMN to the UE.

#### 8.2.19.32	Extended CAG information list

If the UE supports Extended CAG information list, the network may include this IE to assign a new "CAG information list" to the UE or delete the "CAG information list" at the UE side.

#### 8.2.19.33	Updated PEIPS assistance information

The AMF may include this IE if the UE supports NR paging subgrouping, the AMF supports and accepts the use of PEIPIS assistance information for the UE, the UE is not registered for emergency services, the UE does not have an active emergency PDU session, and the network needs to update PEIPS assistance information for the UE.

#### 8.2.19.34	NSAG information

If the UE has set the NSAG bit to "NSAG supported" in the 5GMM capability IE of the REGISTRATION REQUEST message and the CONFIGURATION UPDATE COMMAND message is sent over 3GPP access, the network may include this IE to provide NSAG information to the UE.

#### 8.2.19.35	Priority indicator

The network shall include this IE when it needs to inform the UE that the use of access identity 1 is valid or is no longer valid.

#### 8.2.19.36	RAN timing synchronization

This IE may be included to provide information related to the RAN timing synchronization to a UE which has set the Reconnection to the network due to RAN timing synchronization status change (RANtiming) bit to "Reconnection to the network due to RAN timing synchronization status change supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

#### 8.2.19.37	Extended LADN information

This IE may be included to assign new extended LADN information to the UE or delete the extended LADN information at the UE side.

#### 8.2.19.38	Alternative NSSAI

The network may include this IE to provide the mapping information between the S-NSSAI to be replaced and the alternative S-NSSAI to the UE.

#### 8.2.19.39	S-NSSAI location validity information

The network may include this IE to update the S-NSSAI location validity information.

#### 8.2.19.40	S-NSSAI time validity information

The network may include this IE to update the S-NSSAI time validity information.

#### 8.2.19.41	Discontinuous coverage maximum time offset

This IE may be included to provide the UE with a discontinuous coverage maximum time offset.

#### 8.2.19.42	Partially allowed NSSAI

The network may include this IE to assign a partially allowed NSSAI to the UE.

#### 8.2.19.43	Partially rejected NSSAI

The network may include this IE to assign a partially rejected NSSAI to the UE.

#### 8.2.19.44	Feature authorization indication

The network may include this IE to inform to the UE about the features for which the UE is authorized to use.

#### 8.2.19.45	On-demand NSSAI

The network may include this IE to provide the on-demand NSSAI to the UE.

#### 8.2.19.46	Access technology utilization control

This IE is included to indicate the restricted access technology(s).

#### 8.2.19.47	Updated LP-WUSPS assistance information

The AMF shall include this IE if the network needs to update the LP-WUSPS assistance information for the UE.

#### 8.2.19.48	LP-WUS status

This IE may be included to indicate LP-WUS status.

### 8.2.20	Configuration update complete

#### 8.2.20.1	Message definition

The CONFIGURATION UPDATE COMPLETE message is sent by the UE to the AMF. See table 8.2.20.1.1.

Message type:	CONFIGURATION UPDATE COMPLETE

Significance:	dual

Direction:	UE to network

Table 8.2.20.1.1: CONFIGURATION UPDATE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Configuration update complete message identity | Message type 9.7 | M | V | 1 |

#### 8.2.20.2	Void

### 8.2.21	Identity request

#### 8.2.21.1	Message definition

The IDENTITY REQUEST message is sent by the AMF to the UE to request the UE to provide specified identity. See table 8.2.21.1.1

Message type:	IDENTITY REQUEST

Significance:	dual

Direction:	AMF to UE

Table 8.2.21.1.1: IDENTITY REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Identity request message identity | Message type 9.7 | M | V | 1 |
|  | Identity type | 5GS identity type 9.11.3.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |

### 8.2.22	Identity response

#### 8.2.22.1	Message definition

The IDENTITY RESPONSE message is sent by the UE to the AMF to provide the requested identity. See table 8.2.22.1.

Message type:	IDENTITY RESPONSE

Significance:	dual

Direction:	UE to AMF

Table 8.2.22.1.1: IDENTITY RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Identity response message identity | Message type 9.7 | M | V | 1 |
|  | Mobile identity | 5GS mobile identity 9.11.3.4 | M | LV-E | 3-n |

### 8.2.23	Notification

#### 8.2.23.1	Message definition

The NOTIFICATION message is sent by the AMF to the UE to notify the UE to initiate a service request procedure. See table 8.2.23.1.1.

Message type:	NOTIFICATION

Significance:	dual

Direction:	network to UE

Table 8.2.23.1.1: NOTIFICATION message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Notification message identity | Message type 9.7 | M | V | 1 |
|  | Access type | Access type 9.11.2.1A | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |

### 8.2.24	Notification response

#### 8.2.24.1	Message definition

The NOTIFICATION RESPONSE message is sent by the UE to the AMF to notify the failure to initiate the service request procedure as a response of notification. See table 8.2.24.1.1.

Message type:	NOTIFICATION RESPONSE

Significance:	dual

Direction:	UE to network

Table 8.2.2341.1: NOTIFICATION RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Notification response message identity | Message type 9.7 | M | V | 1 |
| 50 | PDU session status | PDU session status 9.11.3.44 | O | TLV | 4-34 |

#### 8.2.24.2	PDU session status

This information element shall be included when the UE needs to indicate over non-3GPP access the PDU sessions that are associated with the 3GPP access type that are active within the UE.

### 8.2.25	Security mode command

#### 8.2.25.1	Message definition

The SECURITY MODE COMMAND message is sent by the AMF to the UE to establish NAS signalling security. See table 8.2.25.1.1.

Message type:	SECURITY MODE COMMAND

Significance:	dual

Direction:	network to UE

Table 8.2.25.1.1: SECURITY MODE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Security mode command message identity | Message type 9.7 | M | V | 1 |
|  | Selected NAS security algorithms | NAS security algorithms 9.11.3.34 | M | V | 1 |
|  | ngKSI | NAS key set identifier 9.11.3.32 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Replayed UE security capabilities | UE security capability 9.11.3.54 | M | LV | 3-9 |
| E- | IMEISV request | IMEISV request 9.11.3.28 | O | TV | 1 |
| 57 | Selected EPS NAS security algorithms | EPS NAS security algorithms 9.11.3.25 | O | TV | 2 |
| 36 | Additional 5G security information | Additional 5G security information 9.11.3.12 | O | TLV | 3 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |
| 38 | ABBA | ABBA 9.11.3.10 | O | TLV | 4-n |
| 19 | Replayed S1 UE security capabilities | S1 UE security capability 9.11.3.48A | O | TLV | 4-7 |
| 55 | AUN3 device security key | AUN3 device security key 9.11.3.107 | O | TLV | 36-257 |

NOTE:	It is possible for AMFs compliant with version 18.3.0 or 18.3.1 of this specification to send the AUN3 device security key IE with a fixed length of 66 octets.

#### 8.2.25.2	IMEISV request

The AMF may include this information element to request the UE to send its IMEISV with the corresponding SECURITY MODE COMPLETE message.

#### 8.2.25.3	Void

#### 8.2.25.4	Selected EPS NAS security algorithms

This IE shall be included if the AMF supports N26 interface, the UE set the S1 mode bit to "S1 mode supported" in the 5GMM capability IE of the REGISTRATION REQUEST message, and the AMF needs to provide the selected EPS NAS security algorithms to the UE.

#### 8.2.25.5	Additional 5G security information

The network shall include this IE if:

a)	the network needs to provide the UE with horizontal derivation parameter; or

b)	the applicable initial NAS message (i.e. REGISTRATION REQUEST, CONTROL PLANE SERVICE REQUEST or SERVICE REQUEST) does not successfully pass the integrity check at the AMF (see clause 5.4.2.2).

#### 8.2.25.6	EAP message

This IE is included when the EAP Success message is sent as part of the EAP based primary authentication and key agreement procedure, as specified in clause 5.4.1.2.

#### 8.2.25.7	ABBA

This IE shall be included if the message contains an EAP message IE with an EAP-success message.

#### 8.2.25.8	Replayed S1 UE security capabilities

This IE shall be included if the Selected EPS NAS security algorithms information element is included.

#### 8.2.25.9	AUN3 device security key

This IE shall be included when the SECURITY MODE COMMAND message is provided to a 5G-RG that is acting on behalf of an AUN3 device if the EAP message IE is set to an EAP-success message.

### 8.2.26	Security mode complete

#### 8.2.26.1	Message definition

The SECURITY MODE COMPLETE message is sent by the UE to the AMF in response to a SECURITY MODE COMMAND message. See table 8.2.26.1.1.

Message type:	SECURITY MODE COMPLETE

Significance:	dual

Direction:	UE to network

Table 8.2.26.1.1: SECURITY MODE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Security mode complete message identity | Message type 9.6 | M | V | 1 |
| 77 | IMEISV | 5GS mobile identity 9.11.3.4 | O | TLV-E | 12 |
| 71 | NAS message container | NAS message container 9.11.3.33 | O | TLV-E | 4-n |
| 78 | non-IMEISV PEI | 5GS mobile identity 9.11.3.4 | O | TLV-E | 7-n |

#### 8.2.26.2	IMEISV

The UE shall include this information element, if the IMEISV was requested within the corresponding SECURITY MODE COMMAND message.

#### 8.2.26.3	NAS message container

The UE shall include this information element:

a)	if during an ongoing registration procedure or service request procedure, the AMF included the Additional 5G security information with the RINMR bit set to "Retransmission of the initial NAS message requested" in the SECURITY MODE COMMAND message as described in 3GPP TS 33.501 [24]; or

b)	if during an ongoing registration procedure, the UE does not have a valid 5G NAS security context.

#### 8.2.26.4	non-IMEISV PEI

The 5G-RG or the W-AGF acting on behalf of the FN-RG (or on behalf of the N5GC device) shall include this information element, if the IMEISV was requested within the corresponding SECURITY MODE COMMAND message, the IMEISV is not available but MAC address is available.

The UE shall include this information element, if the IMEISV was requested within the corresponding SECURITY MODE COMMAND message, the IMEISV is not available but EUI-64 is available.

### 8.2.27	Security mode reject

#### 8.6.27.1	Message definition

The SECURITY MODE REJECT message is sent by the UE to the AMF to indicate that the corresponding security mode command has been rejected. See table 8.2.27.1.1.

Message type:	SECURITY MODE REJECT

Significance:	dual

Direction:	UE to network

Table 8.2.27.1.1: SECURITY MODE REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Security mode reject message identity | Message type 9.6 | M | V | 1 |
|  | 5GMM cause | 5GMM cause 9.11.3.2 | M | V | 1 |

### 8.2.28	Security protected 5GS NAS message

#### 8.2.28.1	Message definition

This message is sent by the UE or the network to transfer a plain 5GS NAS message as specified in clause 8.2 together with the sequence number and the message authentication code protecting the message. See table 8.2.28.1.1.

Message type:	SECURITY PROTECTED 5GS NAS MESSAGE

Significance:	dual

Direction:	both

Table 8.2.28.1.1: SECURITY PROTECTED 5GS NAS MESSAGE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Message authentication code | Message authentication code 9.8 | M | V | 4 |
|  | Sequence number | Sequence number 9.10 | M | V | 1 |
|  | Plain 5GS NAS message | Plain 5GS NAS message 9.9 | M | V | 3-n |

NOTE:	The minimum length of Plain 5GS NAS message IE can be 2 octets if it includes a Test Mode Control message specified in 3GPP TS 38.509 [31AA].

### 8.2.29	5GMM status

#### 8.2.29.1	Message definition

The 5GMM STATUS message is sent by the UE or by the AMF at any time to report certain error conditions. See table 8.2.29.1.1.

Message type:	5GMM STATUS

Significance:	local

Direction:	both

Table 8.2.29.1.1: 5GMM STATUS message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | 5GMM STATUS message identity | Message type 9.7 | M | V | 1 |
|  | 5GMM cause | 5GMM cause 9.11.3.2 | M | V | 1 |

### 8.2.30	Control Plane Service request

#### 8.2.30.1	Message definition

The CONTROL PLANE SERVICE REQUEST message is sent by the UE to the AMF when the UE is using 5GS services with control plane CIoT 5GS optimization. See table 8.2.30.1.1.

Message type:	CONTROL PLANE SERVICE REQUEST

Significance:	dual

Direction:	UE to network

Table 8.2.30.1.1: CONTROL PLANE SERVICE REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Control plane service request message identity | Message type 9.7 | M | V | 1 |
|  | Control plane service type | Control plane service type 9.11.3.18D | M | V | 1/2 |
|  | ngKSI | NAS key set identifier 9.11.3.32 | M | V | 1/2 |
| 6F | CIoT small data container | CIoT small data container 9.11.3.18B | O | TLV | 4-257 |
| 8- | Payload container type | Payload container type 9.11.3.40 | O | TV | 1 |
| 7B | Payload container | Payload container 9.11.3.39 | O | TLV-E | 4-65538 |
| 12 | PDU session ID | PDU session identity 2 9.11.3.41 | C | TV | 2 |
| 50 | PDU session status | PDU session status 9.11.3.44 | O | TLV | 4-34 |
| F- | Release assistance indication | Release assistance indication 9.11.3.46A | O | TV | 1 |
| 40 | Uplink data status | Uplink data status 9.11.3.57 | O | TLV | 4-34 |
| 71 | NAS message container | NAS message container 9.11.3.33 | O | TLV-E | 4-n |
| 24 | Additional information | Additional information 9.11.2.1 | O | TLV | 3-n |
| 25 | Allowed PDU session status | Allowed PDU session status 9.11.3.13 | O | TLV | 4-34 |
| 29 | UE request type | UE request type 9.11.3.76 | O | TLV | 3 |
| 28 | Paging restriction | Paging restriction 9.11.3.77 | O | TLV | 3-35 |

#### 8.2.30.2	CIoT small data container

This IE shall be included if the UE needs to send uplink small user data, SMS or location services message that is not more than 254 bytes, and there is no other optional IE to be sent.

NOTE:	When the UE determines to use the CIoT small data container IE to send uplink data in this message, there is no other optional IEs in this message.

#### 8.2.30.3	Payload container type

This IE shall be included if the UE includes the Payload container IE.

#### 8.2.30.4	Payload container

This IE shall be included if the UE needs to send uplink CIoT user data, SMS or location services message.

#### 8.2.30.5	PDU session ID

The UE shall include this IE when the Payload container type IE is set to "CIoT user data container".

#### 8.2.30.6	PDU session status

This IE shall be included when the UE needs to indicate the PDU sessions that are associated with the access type that the message is sent over, that are active within the UE.

#### 8.2.30.7	Release assistance indication

The UE may include this IE to inform the network whether:

-	no further uplink and no further downlink data transmission is expected; or

-	only a single downlink data transmission (e.g. acknowledgement or response to uplink data) and no further uplink data transmission subsequent to the uplink data transmission is expected.

#### 8.2.30.8	Uplink data status

This IE shall be included if the UE has uplink user data pending to be sent over the user plane.

#### 8.2.30.9	NAS message container

This IE shall be included if the UE is sending a CONTROL PLANE SERVICE REQUEST message as an initial NAS message and the UE needs to send non-cleartext IEs.

#### 8.2.30.10	Additional information

The UE may include this IE when the Payload container type IE is set to "Location services message container".

#### 8.2.30.11	Allowed PDU session status

This IE shall be included if the CONTROL PLANE SERVICE REQUEST message is sent as a response to paging with the access type indicating non-3GPP access or notification via 3GPP access for PDU session(s) associated with non-3GPP access.

#### 8.2.30.12	UE request type

The UE shall include this IE if the MUSIM UE requests the release of the NAS signalling connection or rejects the paging request from the network.

#### 8.2.30.13	Paging restriction

The UE shall include this IE if the Request type is set to "NAS signalling connection release" or to "Rejection of paging" in the UE request type IE and the UE requests the network to restrict paging.

### 8.2.31	Network slice-specific authentication command

#### 8.2.31.1	Message definition

The NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND message is sent by the AMF to the UE for authentication of the upper layers of the UE. See table 8.2.31.1.1.

Message type:	NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND

Significance:	dual

Direction:	network to UE

Table 8.2.31.1.1: NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND message identity | Message type 9.7 | M | V | 1 |
|  | S-NSSAI | S-NSSAI 9.11.2.8 | M | LV | 2-5 |
|  | EAP message | EAP message 9.11.2.2 | M | LV-E | 6-1502 |

### 8.2.32	Network slice-specific authentication complete

#### 8.2.32.1	Message definition

The NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE message is sent by the UE to the AMF in response to the NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND message and indicates acceptance of the NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND message. See table 8.2.32.1.1.

Message type:	NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE

Significance:	dual

Direction:	UE to network

Table 8.2.32.1.1: NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE message identity | Message type 9.7 | M | V | 1 |
|  | S-NSSAI | S-NSSAI 9.11.2.8 | M | LV | 2-5 |
|  | EAP message | EAP message 9.11.2.2 | M | LV-E | 6-1502 |

### 8.2.33	Network slice-specific authentication result

#### 8.2.33.1	Message definition

The NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message is sent by the AMF to the UE for indicating the result of the network slice-specific authentication and authorization procedure. See table 8.2.33.1.1.

Message type:	NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT

Significance:	dual

Direction:	network to UE

Table 8.2.33.1.1: NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message identity | Message type 9.7 | M | V | 1 |
|  | S-NSSAI | S-NSSAI 9.11.2.8 | M | LV | 2-5 |
|  | EAP message | EAP message 9.11.2.2 | M | LV-E | 6-1502 |

### 8.2.34	Relay key request

#### 8.2.34.1	Message definition

The RELAY KEY REQUEST message is sent by the UE to the AMF for initiation of PC5 keys establishment with the 5G ProSe remote UE or the 5G ProSe end UE as specified in 3GPP TS 33.503 [56]. See table 8.2.34.1.

Message type:	RELAY KEY REQUEST

Significance:	dual

Direction:	UE to network

Table 8.2.34.1: RELAY KEY REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Relay key request message identity | Message type 9.7 | M | V | 1 |
|  | PRTI | ProSe relay transaction identity 9.11.3.88 | M | V | 1 |
|  | Relay key request parameters | Relay key request parameters 9.11.3.89 | M | LV | 22-65537 |

### 8.2.35	Relay key accept

#### 8.2.35.1	Message definition

The RELAY KEY ACCEPT message is sent by the AMF to the UE as specified in 3GPP TS 33.503 [56]. See table 8.2.35.1.

Message type:	RELAY KEY ACCEPT

Significance:	dual

Direction:	network to UE

Table 8.2.35.1: RELAY KEY ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Relay key accept message identity | Message type 9.7 | M | V | 1 |
|  | PRTI | ProSe relay transaction identity 9.11.3.88 | M | V | 1 |
|  | Relay key response parameters | Relay key response parameters 9.11.3.90 | M | LV-E | 51-65537 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |

#### 8.2.35.2	EAP message

The AMF shall include the EAP message IE if the AMF has received an EAP-success message from the AUSF.

### 8.2.36	Relay key reject

#### 8.2.36.1	Message definition

The RELAY KEY REJECT message is sent by the AMF to the UE to indicate the rejection of the relay key request. See table 8.2.36.1.

Message type:	RELAY KEY REJECT

Significance:	dual

Direction:	network to UE

Table 8.2.36.1: RELAY KEY REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Relay key reject message identity | Message type 9.7 | M | V | 1 |
|  | PRTI | ProSe relay transaction identity 9.11.3.88 | M | V | 1 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |

#### 8.2.36.2	EAP message

EAP message IE is included if the RELAY KEY REJECT message is used to convey EAP-failure message.

### 8.2.37	Relay authentication request

#### 8.2.37.1	Message definition

The RELAY AUTHENTICATION REQUEST message is sent by the network to the UE to initiate authentication of the 5G ProSe remote UE or the 5G ProSe end UE as specified in 3GPP TS 33.503 [56]. See table 8.2.37.1.

Message type:	RELAY AUTHENTICATION REQUEST

Significance:	dual

Direction:	Network to UE

Table 8.2.37.1: RELAY AUTHENTICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Relay authentication request message identity | Message type 9.7 | M | V | 1 |
|  | PRTI | ProSe relay transaction identity 9.11.3.88 | M | V | 1 |
|  | EAP message | EAP message 9.11.2.2 | M | LV-E | 7-1503 |

### 8.2.38	Relay authentication response

#### 8.2.38.1	Message definition

The RELAY AUTHENTICATION RESPONSE message is sent by the UE to the network to forward the authentication response from the 5G ProSe remote UE or the 5G ProSe end UE as specified in 3GPP TS 33.503 [56]. See table 8.2.38.1.

Message type:	RELAY AUTHENTICATION RESPONSE

Significance:	dual

Direction:	UE to network

Table 8.2.38.1: RELAY AUTHENTICATION RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Relay authentication response message identity | Message type 9.7 | M | V | 1 |
|  | PRTI | ProSe relay transaction identity 9.11.3.88 | M | V | 1 |
|  | EAP message | EAP message 9.11.2.2 | M | LV-E | 6-1502 |

## 8.3	5GS session management messages

### 8.3.1	PDU session establishment request

#### 8.3.1.1	Message definition

The PDU SESSION ESTABLISHMENT REQUEST message is sent by the UE to the SMF to initiate establishment of a PDU session. See table 8.3.1.1.1.

Message type:	PDU SESSION ESTABLISHMENT REQUEST

Significance:	dual

Direction:	UE to network

Table 8.3.1.1.1: PDU SESSION ESTABLISHMENT REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION ESTABLISHMENT REQUEST message identity | Message type 9.7 | M | V | 1 |
|  | Integrity protection maximum data rate | Integrity protection maximum data rate 9.11.4.7 | M | V | 2 |
| 9- | PDU session type | PDU session type 9.11.4.11 | O | TV | 1 |
| A- | SSC mode | SSC mode 9.11.4.16 | O | TV | 1 |
| 28 | 5GSM capability | 5GSM capability 9.11.4.1 | O | TLV | 3-15 |
| 55 | Maximum number of supported packet filters | Maximum number of supported packet filters 9.11.4.9 | O | TV | 3 |
| B- | Always-on PDU session requested | Always-on PDU session requested 9.11.4.4 | O | TV | 1 |
| 39 | SM PDU DN request container | SM PDU DN request container 9.11.4.15 | O | TLV | 3-255 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |
| 66 | IP header compression configuration | IP header compression configuration 9.11.4.24 | O | TLV | 5-257 |
| 6E | DS-TT Ethernet port MAC address | DS-TT Ethernet port MAC address 9.11.4.25 | O | TLV | 8 |
| 6F | UE-DS-TT residence time | UE-DS-TT residence time 9.11.4.26 | O | TLV | 10 |
| 74 | Port management information container | Port management information container 9.11.4.27 | O | TLV-E | 8-65538 |
| 1F | Ethernet header compression configuration | Ethernet header compression configuration 9.11.4.28 | O | TLV | 3 |
| 29 | Suggested interface identifier | PDU address 9.11.4.10 | O | TLV | 11 |
| 72 | Service-level-AA container | Service-level-AA container 9.11.2.10 | O | TLV-E | 4-65538 |
| 70 | Requested MBS container | Requested MBS container 9.11.4.30 | O | TLV-E | 8-65538 |
| 34 | PDU session pair ID | PDU session pair ID 9.11.4.32 | O | TLV | 3 |
| 35 | RSN | RSN 9.11.4.33 | O | TLV | 3 |
| 36 | URSP rule enforcement reports | URSP rule enforcement reports 9.11.4.38 | O | TLV | 4-n |

#### 8.3.1.2	PDU session type

This IE shall be included in the message when the UE requests to establish a new PDU session.

#### 8.3.1.3	SSC mode

This IE is included in the message when the UE requests to establish a new PDU session with a DN and requests an SSC mode.

#### 8.3.1.4	Maximum number of supported packet filters

This IE shall be included in the message when the selected PDU session type is "IPv4", "IPv6", "IPv4v6" or "Ethernet" and the UE can support more than 16 packet filters for this PDU session.

#### 8.3.1.5	5GSM capability

This IE is included in the message when the UE requests to establish a new PDU session or to transfer an existing PDN connection and any of the 5GSM capabilities supported by the UE is relevant for the PDU session.

#### 8.3.1.6	Void

#### 8.3.1.7	Always-on PDU session requested

The UE shall include this IE if the UE requests to establish a PDU session as an always-on PDU session.

#### 8.3.1.8	SM PDU DN request container

This IE is included in the message when the UE requests to establish a new PDU session with a DN and needs to provide information for the PDU session authentication and authorization by the external DN.

#### 8.3.1.9	Extended protocol configuration options

This IE is included in the message when the UE needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

#### 8.3.1.10	IP header compression configuration

The UE shall include the IP header compression configuration IE if:

-	the PDU session type value of the PDU session type IE is set to "IPv4", "IPv6" or "IPv4v6";

-	the UE indicates "Control Plane CIoT 5GS optimization supported" and "IP header compression for control plane CIoT 5GS optimization supported" in the 5GMM capability IE of the REGISTRATION REQUEST message; and

-	the network indicates "Control plane CIoT 5GS optimization supported" and "IP header compression for control plane CIoT 5GS optimization supported" in the 5GS network support feature IE of the REGISTRATION ACCEPT message.

#### 8.3.1.11	DS-TT Ethernet port MAC address

This IE shall be included in the message if the UE supports transfer of port management information containers and the UE requests to establish a new PDU session of "Ethernet" PDU session type.

#### 8.3.1.12	UE-DS-TT residence time

This IE shall be included in the message if:

a)	the UE supports transfer of port management information containers; and

b)	the UE-DS-TT residence time is available at the UE.

#### 8.3.1.13	Port management information container

This IE shall be included in the message if the UE supports transfer of port management information containers.

#### 8.3.1.14	Ethernet header compression configuration

The UE shall include the Ethernet header compression configuration IE if:

-	the PDU session type value of the PDU session type IE is set to "Ethernet";

-	the UE indicated "Control Plane CIoT 5GS optimization supported" and "Ethernet header compression for control plane CIoT 5GS optimization supported" in the 5GMM capability IE of the REGISTRATION REQUEST message; and

-	the network indicated "Control plane CIoT 5GS optimization supported" and "Ethernet header compression for control plane CIoT 5GS optimization supported" in the 5GS network support feature IE of the REGISTRATION ACCEPT message.

#### 8.3.1.15	Suggested interface identifier

This IE may be included by the W-AGF acting on behalf of the FN-RG.

#### 8.3.1.16	Service-level-AA container

This IE shall be included in the message when the UE needs to establish a PDU session for the UAS services.

#### 8.3.1.17	Requested MBS container

This IE is included in the message when the UE requests to join one or more multicast MBS sessions that are associated with the PDU session.

#### 8.3.1.18	PDU session pair ID

This IE shall be included in the message when the UE needs to include a PDU session pair ID.

#### 8.3.1.19	RSN

This IE shall be included in the message when the UE needs to include an RSN.

#### 8.3.1.20	URSP rule enforcement reports

This IE is included in the message when the UE sends one or more URSP rule enforcement reports to the network.

### 8.3.2	PDU session establishment accept

#### 8.3.2.1	Message definition

The PDU SESSION ESTABLISHMENT ACCEPT message is sent by the SMF to the UE in response to PDU SESSION ESTABLISHMENT REQUEST message and indicates successful establishment of a PDU session. See table 8.3.2.1.1.

Message type:	PDU SESSION ESTABLISHMENT ACCEPT

Significance:	dual

Direction:	network to UE

Table 8.3.2.1.1: PDU SESSION ESTABLISHMENT ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION ESTABLISHMENT ACCEPT message identity | Message type 9.7 | M | V | 1 |
|  | Selected PDU session type | PDU session type 9.11.4.11 | M | V | 1/2 |
|  | Selected SSC mode | SSC mode 9.11.4.16 | M | V | 1/2 |
|  | Authorized QoS rules | QoS rules 9.11.4.13 | M | LV-E | 6-65538 |
|  | Session AMBR | Session-AMBR 9.11.4.14 | M | LV | 7 |
| 59 | 5GSM cause | 5GSM cause 9.11.4.2 | O | TV | 2 |
| 29 | PDU address | PDU address 9.11.4.10 | O | TLV | 7-31 |
| 56 | RQ timer value | GPRS timer 9.11.2.3 | O | TV | 2 |
| 22 | S-NSSAI | S-NSSAI 9.11.2.8 | O | TLV | 3-10 |
| 8- | Always-on PDU session indication | Always-on PDU session indication 9.11.4.3 | O | TV | 1 |
| 75 | Mapped EPS bearer contexts | Mapped EPS bearer contexts 9.11.4.8 | O | TLV-E | 7-65538 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |
| 79 | Authorized QoS flow descriptions | QoS flow descriptions 9.11.4.12 | O | TLV-E | 6-65538 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |
| 25 | DNN | DNN 9.11.2.1B | O | TLV | 3-102 |
| 17 | 5GSM network feature support | 5GSM network feature support 9.11.4.18 | O | TLV | 3-15 |
| 18 | Serving PLMN rate control | Serving PLMN rate control 9.11.4.20 | O | TLV | 4 |
| 77 | ATSSS container | ATSSS container 9.11.4.22 | O | TLV-E | 3-65538 |
| C- | Control plane only indication | Control plane only indication 9.11.4.23 | O | TV | 1 |
| 66 | IP header compression configuration | IP header compression configuration 9.11.4.24 | O | TLV | 5-257 |
| 1F | Ethernet header compression configuration | Ethernet header compression configuration 9.11.4.28 | O | TLV | 3 |
| 72 | Service-level-AA container | Service-level-AA container 9.11.2.10 | O | TLV-E | 4-65538 |
| 71 | Received MBS container | Received MBS container 9.11.4.31 | O | TLV-E | 9-65538 |
| 70 | N3QAI | N3QAI 9.11.4.36 | O | TLV-E | 9-n |
| 73 | Protocol description | Protocol description 9.11.4.39 | O | TLV-E | 6-n |
| 38 | ECN marking for L4S indication | ECN marking for L4S indication 9.11.4.40 | O | TLV | 2-257 |

#### 8.3.2.2	5GSM cause

This IE is included when the selected PDU session type is different from the PDU session type requested by the UE.

#### 8.3.2.3	PDU address

This IE is included when the selected PDU session type is "IPv4", "IPv6" or "IPv4v6".

#### 8.3.2.4	RQ timer value

This IE is included when the network needs to provide the RQ timer value.

#### 8.3.2.5	S-NSSAI

This IE shall be included in the message when the SMF received from the AMF an S-NSSAI together with the PDU SESSION ESTABLISHMENT REQUEST message, the PDU session is a non-emergency PDU session and the UE is not registered for onboarding services in SNPN.

#### 8.3.2.6	Always-on PDU session indication

The network shall include this IE if the network decides to inform the UE whether the PDU session is established as an always-on PDU session.

#### 8.3.2.7	Mapped EPS bearer contexts

This IE is included when interworking with EPS is supported for the PDU session.

#### 8.3.2.8	EAP message

This IE is included when the external DN successfully performed authentication and authorization of the UE using EAP.

#### 8.3.2.9	Authorized QoS flow descriptions

This IE is included when the network needs to provide authorized QoS flow descriptions.

#### 8.3.2.10	Extended protocol configuration options

This IE is included in the message when the network needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

#### 8.3.2.11	DNN

The IE shall be included in the message when the PDU session is a non-emergency PDU session and the UE is not registered for onboarding services in SNPN.

#### 8.3.2.12	5GSM network feature support

This IE is included when the network needs to indicate support of 5GSM network features.

#### 8.3.2.13	Void

#### 8.3.2.14	Serving PLMN rate control

This IE shall be included when the network needs to indicate the maximum uplink control plane user data the UE is allowed to send per 6 minute interval.

#### 8.3.2.15	ATSSS container

The IE shall be included in the message when the PDU session is an MA PDU session.

#### 8.3.2.16	Control plane only indication

The network shall include the control plane only indication IE if the network determines that the associated PDU session is only for control plane CIoT 5GS optimization.

#### 8.3.2.17	IP header compression configuration

The SMF may include the IP header compression configuration IE if:

-	the network accepts an IP PDU session type;

-	control plane CIoT 5GS optimization is selected; and

-	the UE provided the IP header compression configuration IE in the PDU SESSION ESTABLISHMENT REQUEST message.

#### 8.3.2.18	Ethernet header compression configuration

The SMF may include the Ethernet header compression configuration IE if:

-	the network accepts an Ethernet PDU session type;

-	control plane CIoT 5GS optimization is selected; and

-	the UE provided the Ethernet header compression configuration IE in the PDU SESSION ESTABLISHMENT REQUEST message.

#### 8.3.2.19	Service-level-AA container

The SMF shall include the Service-level-AA container IE if the service-level authentication and authorization procedure is completed successfully by the external DN.

#### 8.3.2.20	Received MBS container

The network shall include this IE if the UE has requested to join one or more multicast MBS sessions.

#### 8.3.2.21	N3QAI

This IE is included when the network needs to provide the N3QAI to the UE.

#### 8.3.2.22	Protocol description

This IE is included when the network needs to indicate the protocol description for UL PDU set based handling to the UE.

#### 8.3.2.23	ECN marking for L4S indication

This IE is included when the network determines the ECN marking for L4S for the 5G-RG.

### 8.3.3	PDU session establishment reject

#### 8.3.3.1	Message definition

The PDU SESSION ESTABLISHMENT REJECT message is sent by the SMF to the UE in response to PDU SESSION ESTABLISHMENT REQUEST message and indicates unsuccessful establishment of a PDU session. See table 8.3.3.1.1.

Message type:	PDU SESSION ESTABLISHMENT REJECT

Significance:	dual

Direction:	network to UE

Table 8.3.3.1.1: PDU SESSION ESTABLISHMENT REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION ESTABLISHMENT REJECT message identity | Message type 9.7 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.11.4.2 | M | V | 1 |
| 37 | Back-off timer value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| F- | Allowed SSC mode | Allowed SSC mode 9.11.4.5 | O | TV | 1 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |
| 61 | 5GSM congestion re-attempt indicator | 5GSM congestion re-attempt indicator 9.11.4.21 | O | TLV | 3 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |
| 1D | Re-attempt indicator | Re-attempt indicator 9.11.4.17 | O | TLV | 3 |
| 72 | Service-level-AA container | Service-level-AA container 9.11.2.10 | O | TLV-E | 4-65538 |
| 77 | ATSSS container | ATSSS container 9.11.4.22 | O | TLV-E | 3-65538 |

#### 8.3.3.2	Back-off timer value

The network may include this IE if the 5GSM cause is not #28 "unknown PDU session type", #39 "reactivation requested", #46 "out of LADN service area", #50 "PDU session type IPv4 only allowed", #51 "PDU session type IPv6 only allowed", #54 "PDU session does not exist", #57 "PDU session type IPv4v6 only allowed", #58 "PDU session type Unstructured only allowed", #61 "PDU session type Ethernet only allowed",#68 "not supported SSC mode", or #86 "UAS services not allowed" to request a minimum time interval before procedure retry is allowed.

#### 8.3.3.3	Allowed SSC mode

This IE is included when the network rejects the PDU SESSION ESTABLISHMENT REQUEST with cause #68 "not supported SSC mode.

#### 8.3.3.4	EAP message

This IE is included when the external DN unsuccessfully performed authentication and authorization of the UE using EAP.

#### 8.3.3.4A	5GSM congestion re-attempt indicator

The network may include this IE only if it includes the Back-off timer value IE and the 5GSM cause value is either #67 "insufficient resources for specific slice and DNN" or #69 "insufficient resources for specific slice".

#### 8.3.3.5	Extended protocol configuration options

This IE is included in the message when the network needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

#### 8.3.3.6	Re-attempt indicator

The network may include this IE if the network includes the Back-off timer value IE and the 5GSM cause value is not #26 "insufficient resources", #28 "unknown PDU session type", #39 "reactivation requested", #46 "out of LADN service area", #54 "PDU session does not exist", #67 "insufficient resources for specific slice and DNN", #68 "not supported SSC mode", or #69 "insufficient resources for specific slice".

#### 8.3.3.7	Service-level-AA container

The network shall include this IE if the service-level authentication and authorization procedure has completed unsuccessfully and the 5GSM cause is #29 "user authentication or authorization failed". The network shall include the service-level-AA response if provided by the DN in the service-level-AA container.

#### 8.3.3.8	ATSSS container

The network may include this IE to indicate the reason for the failure of the MA PDU session establishment procedure as specified in 3GPP TS 24.193 [13B].

### 8.3.4	PDU session authentication command

#### 8.3.4.1	Message definition

The PDU SESSION AUTHENTICATION COMMAND message is sent by the SMF to the UE for authentication of the UE establishing the PDU session or of the UE participating in the PDU session. See table 8.3.4.1.1.

Message type:	PDU SESSION AUTHENTICATION COMMAND

Significance:	dual

Direction:	network to UE

Table 8.3.4.1.1: PDU SESSION AUTHENTICATION COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION AUTHENTICATION COMMAND message identity | Message type 9.7 | M | V | 1 |
|  | EAP message | EAP message 9.11.2.2 | M | LV-E | 6-1502 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |

#### 8.3.4.2	Extended protocol configuration options

This IE is included in the message when the network needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

NOTE:	How the Extended protocol configuration options IE is used by the network and the UE during the PDU session authentication and authorization procedure is not specified in this release of the specification.

#### 8.3.4.3	Void

### 8.3.5	PDU session authentication complete

#### 8.3.5.1	Message definition

The PDU SESSION AUTHENTICATION COMPLETE message is sent by the UE to the SMF in response to the PDU SESSION AUTHENTICATION COMMAND message and indicates acceptance of the PDU SESSION AUTHENTICATION COMMAND message. See table 8.3.5.1.1.

Message type:	PDU SESSION AUTHENTICATION COMPLETE

Significance:	dual

Direction:	UE to network

Table 8.3.5.1.1: PDU SESSION AUTHENTICATION COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION AUTHENTICATION COMPLETE message identity | Message type 9.7 | M | V | 1 |
|  | EAP message | EAP message 9.11.2.2 | M | LV-E | 6-1502 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |

#### 8.3.5.2	Extended protocol configuration options

This IE is included in the message when the UE needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

NOTE:	How the Extended protocol configuration options IE is used by the network and the UE during the PDU session authentication and authorization procedure is not specified in this release of the specification.

#### 8.3.5.3	Void

### 8.3.6	PDU session authentication result

#### 8.3.6.1	Message definition

The PDU SESSION AUTHENTICATION RESULT message is sent by the SMF to the UE for indication of successful result of authentication of the UE participating in the PDU session. See table 8.3.6.1.1.

Message type:	PDU SESSION AUTHENTICATION RESULT

Significance:	dual

Direction:	network to UE

Table 8.3.6.1.1: PDU SESSION AUTHENTICATION RESULT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION AUTHENTICATION RESULT message identity | Message type 9.7 | M | V | 1 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |

#### 8.3.6.2	EAP message

This IE shall be included when the external DN performs authentication and authorization of the UE using EAP and it completes successfully.

#### 8.3.6.3	Extended protocol configuration options

This IE is included in the message when the network needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

NOTE:	How the Extended protocol configuration options IE is used by the network and the UE during the PDU session authentication and authorization procedure is not specified in this release of the specification.

### 8.3.7	PDU session modification request

#### 8.3.7.1	Message definition

The PDU SESSION MODIFICATION REQUEST message is sent by the UE to the SMF to request a modification of a PDU session. See table 8.3.7.1.1.

Message type:	PDU SESSION MODIFICATION REQUEST

Significance:	dual

Direction:	UE to network

Table 8.3.7.1.1: PDU SESSION MODIFICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION MODIFICATION REQUEST message identity | Message type 9.7 | M | V | 1 |
| 28 | 5GSM capability | 5GSM capability 9.11.4.1 | O | TLV | 3-15 |
| 59 | 5GSM cause | 5GSM cause 9.11.4.2 | O | TV | 2 |
| 55 | Maximum number of supported packet filters | Maximum number of supported packet filters 9.11.4.9 | O | TV | 3 |
| B- | Always-on PDU session requested | Always-on PDU session requested 9.11.4.4 | O | TV | 1 |
| 13 | Integrity protection maximum data rate | Integrity protection maximum data rate 9.11.4.7 | O | TV | 3 |
| 7A | Requested QoS rules | QoS rules 9.11.4.13 | O | TLV-E | 7-65538 |
| 79 | Requested QoS flow descriptions | QoS flow descriptions 9.11.4.12 | O | TLV-E | 6-65538 |
| 75 | Mapped EPS bearer contexts | Mapped EPS bearer contexts 9.11.4.8 | O | TLV-E | 7-65538 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |
| 74 | Port management information container | Port management information container 9.11.4.27 | O | TLV-E | 4-65538 |
| 66 | IP header compression configuration | Header compression configuration 9.11.4.24 | O | TLV | 5-257 |
| 1F | Ethernet header compression configuration | Ethernet header compression configuration 9.11.4.28 | O | TLV | 3 |
| 70 | Requested MBS container | Requested MBS container 9.11.4.30 | O | TLV-E | 8-65538 |
| 72 | Service-level-AA container | Service-level-AA container 9.11.2.10 | O | TLV-E | 4-65538 |
| 73 | Non-3GPP delay budget | Non-3GPP delay budget 9.11.4.37 | O | TLV-E | 6-n |
| 36 | URSP rule enforcement reports | URSP rule enforcement reports 9.11.4.38 | O | TLV | 4-n |
| 7C | Non-3GPP device information | Non-3GPP device information 9.11.4.41 | O | TLV-E | 7-n |

NOTE:	It is possible for UEs compliant with version 15.2.1 or earlier versions of this specification to send the Mapped EPS bearer contexts IE with IEI of value "7F" for this message.

#### 8.3.7.2	5GSM capability

This IE is included in the message when UE needs to indicate the 5GSM capability IE as defined in clause 6.4.2.2.

#### 8.3.7.3	5GSM cause

This IE is included in the message to indicate the reason for the deletion of one or more non-default QoS rules, QoS flow descriptions or mapped EPS bearer contexts.

#### 8.3.7.4	Maximum number of supported packet filters

This IE shall be included in the message for a PDN connection established when in S1 mode, after an inter-system change from S1 mode to N1 mode, if the UE is a UE operating in single-registration mode in a network supporting N26 interface, the UE has not previously successfully performed the UE-requested PDU session modification to provide this capability, the PDU session type is "IPv4", "IPv6", "IPv4v6" or "Ethernet", and the UE can support more than 16 packet filters for this PDU session.

#### 8.3.7.5	Always-on PDU session requested

This IE shall be included in the message for a PDN connection established when in S1 mode, after an inter-system change from S1 mode to N1 mode, if the UE is a UE operating in single-registration mode in a network supporting N26 interface, the UE has not previously successfully performed the UE-requested PDU session modification to provide this capability, and the UE requests the PDU session to be an always-on PDU session in the 5GS.

#### 8.3.7.6	Integrity protection maximum data rate

This IE shall be included in the message for a PDN connection established when in S1 mode, after an inter-system change from S1 mode to N1 mode, if the UE is a UE operating in single-registration mode in a network supporting N26 interface, and the UE has not previously successfully performed the UE-requested PDU session modification to provide this capability.

#### 8.3.7.7	Requested QoS rules

This IE is included in the message when the UE requests a specific QoS handling.

#### 8.3.7.8	Requested QoS flow descriptions

This IE is included in the message when the UE requests a specific QoS flow descriptions.

#### 8.3.7.9	Extended protocol configuration options

This IE is included in the message when the UE needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

#### 8.3.7.10	Mapped EPS bearer contexts

This IE is included when the UE requests to delete one or more mapped EPS bearer contexts.

#### 8.3.7.11	Port management information container

This IE shall be included when the UE needs to convey a port management information container.

#### 8.3.7.12	IP header compression configuration

This IE is included in the message:

a)	if the UE wishes to re-negotiate IP header compression configuration associated to a PDU session and both the UE and the network supports Control plane CIoT 5GS optimization and IP header compression; or

b)	to negotiate IP header compression configuration associated to a PDU session after an inter-system change from S1 mode to N1 mode when both the UE and the network support control plane CIoT 5GS optimization and IP header compression, and the UE is operating in single-registration mode in the network supporting N26 interface.

#### 8.3.7.13	Ethernet header compression configuration

This IE is included in the message:

a)	if the UE wishes to re-negotiate Ethernet header compression configuration associated to a PDU session and both the UE and the network support Control plane CIoT 5GS optimization and Ethernet header compression; or

b)	to negotiate Ethernet header compression configuration associated to a PDU session after an inter-system change from S1 mode to N1 mode.

#### 8.3.7.14	Requested MBS container

This IE is included in the message when the UE requests to join or leave one or more multicast MBS sessions that are associated with the PDU session.

#### 8.3.7.15	Service-level-AA container

This IE shall be included in the message when the UE needs to modify an established PDU session for C2 communication.

#### 8.3.7.16	Non-3GPP delay budget

This IE is included in the message when the UE requests a specific QoS handling for the PDU session established for PIN.

#### 8.3.7.17	URSP rule enforcement reports

This IE is included in the message when the UE sends one or more URSP rule enforcement reports to the network.

#### 8.3.7.18	Non-3GPP device information

This IE shall be included in the message when the UE requests QoS differentiation of traffic or requests suspend of QoS differentiation of traffic from one or more non-3GPP devices connecting through the UE.

### 8.3.8	PDU session modification reject

#### 8.3.8.1	Message definition

The PDU SESSION MODIFICATION REJECT message is sent by the SMF to the UE to indicate rejection of the PDU SESSION MODIFICATION REQUEST. See table 8.3.8.1.1.

Message type:	PDU SESSION MODIFICATION REJECT

Significance:	dual

Direction:	network to UE

Table 8.3.8.1.1: PDU SESSION MODIFICATION REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION MODIFICATION REJECT message identity | Message type 9.7 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.11.4.2 | M | V | 1 |
| 37 | Back-off timer value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 61 | 5GSM congestion re-attempt indicator | 5GSM congestion re-attempt indicator 9.11.4.21 | O | TLV | 3 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |
| 1D | Re-attempt indicator | Re-attempt indicator 9.11.4.17 | O | TLV | 3 |

#### 8.3.8.2	Back-off timer value

The network may include this IE if the 5GSM cause is not #26 "insufficient resources", #37 "5GS QoS not accepted", #44 "Semantic errors in packet filter(s)", #45 "Syntactical error in packet filter(s)", #46 "out of LADN service area", #59 "unsupported 5QI value", #67 "insufficient resources for specific slice and DNN", #69 "insufficient resources for specific slice", #83 "Semantic error in the QoS operation", #84 "Syntactical error in the QoS operation" or #87 "QoS differentiation for non-3GPP device identifier(s) not available" to request a minimum time interval before procedure retry is allowed.

#### 8.3.8.2A	5GSM congestion re-attempt indicator

The network may include this IE only if it includes the Back-off timer value IE and the 5GSM cause value is either #67 "insufficient resources for specific slice and DNN" or #69 "insufficient resources for specific slice".

#### 8.3.8.3	Extended protocol configuration options

This IE is included in the message when the network needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

#### 8.3.8.4	Re-attempt indicator

The network may include this IE only if it includes the Back-off timer value IE and the 5GSM cause value is not #26 "insufficient resources", #37 "5GS QoS not accepted", #44 "Semantic errors in packet filter(s)", #45 "Syntactical error in packet filter(s)", #46 "out of LADN service area", #59 "unsupported 5QI value", #67 "insufficient resources for specific slice and DNN", #69 "insufficient resources for specific slice", #83 "Semantic error in the QoS operation", #84 "Syntactical error in the QoS operation" or #87 "QoS differentiation for non-3GPP device identifier(s) not available".

### 8.3.9	PDU session modification command

#### 8.3.9.1	Message definition

The PDU SESSION MODIFICATION COMMAND message is sent by the SMF to the UE to indicate a modification of a PDU session. See table 8.3.9.1.1.

Message type:	PDU SESSION MODIFICATION COMMAND

Significance:	dual

Direction:	network to UE

Table 8.3.9.1.1: PDU SESSION MODIFICATION COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION MODIFICATION COMMAND message identity | Message type 9.7 | M | V | 1 |
| 59 | 5GSM cause | 5GSM cause 9.11.4.2 | O | TV | 2 |
| 2A | Session AMBR | Session-AMBR 9.11.4.14 | O | TLV | 8 |
| 56 | RQ timer value | GPRS timer 9.11.2.3 | O | TV | 2 |
| 8- | Always-on PDU session indication | Always-on PDU session indication 9.11.4.3 | O | TV | 1 |
| 7A | Authorized QoS rules | QoS rules 9.11.4.13 | O | TLV-E | 7-65538 |
| 75 | Mapped EPS bearer contexts | Mapped EPS bearer contexts 9.11.4.8 | O | TLV-E | 7-65538 |
| 79 | Authorized QoS flow descriptions | QoS flow descriptions 9.11.4.12 | O | TLV-E | 6-65538 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |
| 77 | ATSSS container | ATSSS container 9.11.4.22 | O | TLV-E | 3-65538 |
| 66 | IP header compression configuration | IP header compression configuration 9.11.4.24 | O | TLV | 5-257 |
| 74 | Port management information container | Port management information container 9.11.4.27 | O | TLV-E | 4-65538 |
| 1E | Serving PLMN rate control | Serving PLMN rate control 9.11.4.20 | O | TLV | 4 |
| 1F | Ethernet header compression configuration | Ethernet header compression configuration 9.11.4.28 | O | TLV | 3 |
| 71 | Received MBS container | Received MBS container 9.11.4.31 | O | TLV-E | 9-65538 |
| 72 | Service-level-AA container | Service-level-AA container 9.11.2.10 | O | TLV-E | 4-65538 |
| 5A | Alternative S-NSSAI | S-NSSAI 9.11.2.8 | O | TLV | 3-10 |
| 70 | N3QAI | N3QAI 9.11.4.36 | O | TLV-E | 9-n |
| 73 | Protocol description | Protocol description 9.11.4.39 | O | TLV-E | 6-n |
| 38 | ECN marking for L4S indication | ECN marking for L4S indication 9.11.4.40 | O | TLV | 2-257 |

NOTE:	It is possible for networks compliant with version 15.2.1 or earlier versions of this specification to send the Mapped EPS bearer contexts IE with IEI of value "7F" for this message.

#### 8.3.9.2	5GSM cause

This IE is included when the network performs the PDU session anchor relocation for SSC mode 3.

#### 8.3.9.3	Session-AMBR

This IE is included when the session-AMBR of the PDU session is modified.

#### 8.3.9.4	RQ timer value

This IE is included when the network needs to provide the RQ timer value.

#### 8.3.9.5	Always-on PDU session indication

The network shall include this IE if the network decides to inform the UE whether the PDU session is an always-on PDU session.

#### 8.3.9.6	Authorized QoS rules

This IE is included when the authorized QoS rules of the PDU session are modified.

#### 8.3.9.7	Mapped EPS bearer contexts

This IE is included when interworking with EPS is supported for the PDU session and the mapped EPS bearer contexts is modified.

#### 8.3.9.8	Authorized QoS flow descriptions

This IE is included when the authorized QoS flow descriptions of the PDU session are modified.

#### 8.3.9.9	Extended protocol configuration options

This IE is included in the message when the network needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

#### 8.3.9.10	Void

#### 8.3.9.11	ATSSS container

The IE is included in the message when the network needs to indicate that the ATSSS parameters of the MA PDU session are modified.

#### 8.3.9.12	IP header compression configuration

This IE is included in the message if the network wishes to re-negotiate IP header compression configuration associated to a PDU session and both the UE and the network support Control plane CIoT 5GS optimization and IP header compression.

#### 8.3.9.13	Port management information container

This IE shall be included when the network needs to convey a port management information container.

#### 8.3.9.14	Serving PLMN rate control

This IE shall be included when the network needs to indicate the maximum uplink control plane user data the UE is allowed to send per 6 minute interval.

#### 8.3.9.15	Ethernet header compression configuration

This IE is included in the message if the network wishes to re-negotiate Ethernet header compression configuration associated to a PDU session and both the UE and the network support Control plane CIoT 5GS optimization and Ethernet header compression.

#### 8.3.9.16	Received MBS container

The network shall include this IE if:

-	the UE has requested to join or leave one or more multicast MBS sessions;

-	the network wants to remove joined UE from one or more multicast MBS sessions; or

-	the network wants to update the MBS service area of multicast MBS session that the UE has joined; or

-	the network wants to update the MBS security information of multicast MBS session that the UE has joined.

#### 8.3.9.17	Service-level-AA container

The SMF shall include the service-level-AA container IE if the service-level authentication and authorization procedure for re-authentication purpose is completed successfully by the external DN.

#### 8.3.9.18	Alternative S-NSSAI

This IE shall be included when the network needs to provide the alternative S-NSSAI to replace the S-NSSAI of the SSC mode 1 or SSC mode 3 PDU session.

#### 8.3.9.19	N3QAI

This IE is included when the network needs to provide the N3QAI to the UE.

#### 8.3.9.20	Protocol description

This IE is included when the network needs to provide the protocol description for UL PDU set based handling to the UE.

#### 8.3.9.21	ECN marking for L4S indication

This IE is included when the network determines the ECN marking for L4S for the 5G-RG.

### 8.3.10	PDU session modification complete

#### 8.3.10.1	Message definition

The PDU SESSION MODIFICATION COMPLETE message is sent by the UE to the SMF in response to the PDU SESSION MODIFICATION COMMAND message and indicates an acceptance of the PDU SESSION MODIFICATION COMMAND message. See table 8.3.10.1.1.

Message type:	PDU SESSION MODIFICATION COMPLETE

Significance:	dual

Direction:	UE to network

Table 8.3.10.1.1: PDU SESSION MODIFICATION COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION MODIFICATION COMPLETE message identity | Message type 9.7 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |
| 74 | Port management information container | Port management information container 9.11.4.27 | O | TLV-E | 4-65538 |

NOTE:	It is possible for UEs compliant with version 15.3.0 of this specification to include the 5GSM cause IE with IEI 59 in the PDU SESSION MODIFICATION COMPLETE message, and therefore the IEI 59 cannot be used for other optional IEs other than the 5GSM cause IE for future extensions of the PDU SESSION MODIFICATION COMPLETE message.

#### 8.3.10.2	Extended protocol configuration options

This IE is included in the message when the UE needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

#### 8.3.10.3	Port management information container

This IE shall be included when the UE needs to convey a port management information container.

### 8.3.11	PDU session modification command reject

#### 8.3.11.1	Message definition

The PDU SESSION MODIFICATION COMMAND REJECT message is sent by the UE to the SMF to indicate rejection of the PDU SESSION MODIFICATION COMMAND message. See table 8.3.11.1.1.

Message type:	PDU SESSION MODIFICATION COMMAND REJECT

Significance:	dual

Direction:	UE to network

Table 8.3.11.1.1: PDU SESSION MODIFICATION COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION MODIFICATION COMMAND REJECT message identity | Message type 9.7 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.11.4.2 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |

#### 8.3.11.2	Extended protocol configuration options

This IE is included in the message when the UE needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 8.3.12	PDU session release request

#### 8.3.12.1	Message definition

The PDU SESSION RELEASE REQUEST message is sent by the UE to the SMF to request a release of a PDU session. See table 8.3.12.1.1.

Message type:	PDU SESSION RELEASE REQUEST

Significance:	dual

Direction:	UE to network

Table 8.3.12.1.1: PDU SESSION RELEASE REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION RELEASE REQUEST message identity | Message type 9.7 | M | V | 1 |
| 59 | 5GSM cause | 5GSM cause 9.11.4.2 | O | TV | 2 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |

#### 8.3.12.2	5GSM cause

This IE is included in the message to indicate the reason for releasing the PDU session.

#### 8.3.12.3	Extended protocol configuration options

This IE is included in the message when the UE needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 8.3.13	PDU session release reject

#### 8.3.13.1	Message definition

The PDU SESSION RELEASE REJECT message is sent by the SMF to the UE to indicate rejection of request a release of a PDU session. See table 8.3.13.1.1.

Message type:	PDU SESSION RELEASE REJECT

Significance:	dual

Direction:	network to UE

Table 8.3.13.1.1: PDU SESSION RELEASE REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION RELEASE REJECT message identity | Message type 9.7 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.11.4.2 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |

#### 8.3.13.2	Extended protocol configuration options

This IE is included in the message when the network needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

### 8.3.14	PDU session release command

#### 8.3.14.1	Message definition

The PDU SESSION RELEASE COMMAND message is sent by the SMF to the UE to indicate a release of a PDU session. See table 8.3.14.1.1.

Message type:	PDU SESSION RELEASE COMMAND

Significance:	dual

Direction:	network to UE

Table 8.3.14.1.1: PDU SESSION RELEASE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION RELEASE COMMAND message identity | Message type 9.7 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.11.4.2 | M | V | 1 |
| 37 | Back-off timer value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |
| 61 | 5GSM congestion re-attempt indicator | 5GSM congestion re-attempt indicator 9.11.4.21 | O | TLV | 3 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |
| D- | Access type | Access type 9.11.2.1A | O | TV | 1 |
| 72 | Service-level-AA container | Service-level-AA container 9.11.2.10 | O | TLV-E | 4-65538 |
| 5A | Alternative S-NSSAI | S-NSSAI 9.11.2.8 | O | TLV | 3-10 |

#### 8.3.14.2	Back-off timer value

The network may include this IE to request a minimum time interval before procedure retry is allowed.

#### 8.3.14.3	EAP message

This IE is included when the external DN performs re-authentication and re-authorization of the UE using EAP and it completes unsuccessfully.

#### 8.3.14.4	Extended protocol configuration options

This IE is included in the message when the network wants to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

#### 8.3.14.5	5GSM congestion re-attempt indicator

The network may include this IE only if it includes the Back-off timer value IE and the 5GSM cause value is either #67 "insufficient resources for specific slice and DNN" or #69 "insufficient resources for specific slice".

#### 8.3.14.6	Access type

This IE is included in the message when the network releases user-plane reources of an MA PDU session specifically over either 3GPP access or non-3GPP access.

#### 8.3.14.7	Service-level-AA container

The SMF shall include the service-level-AA container IE if the service-level authentication and authorization procedure for re-authentication purpose is completed unsuccessfully by the external DN.

#### 8.3.14.8	Alternative S-NSSAI

This IE shall be included when the network needs to trigger the re-establishment of the PDU Session with the alternative S-NSSAI in SSC mode 2 or SSC mode 1.

### 8.3.15	PDU session release complete

#### 8.3.15.1	Message definition

The PDU SESSION RELEASE COMPLETE message is sent by the UE to the SMF in response to the PDU SESSION RELEASE COMMAND message and indicates an acceptance of a release of the PDU session. See table 8.3.15.1.1.

Message type:	PDU SESSION RELEASE COMPLETE

Significance:	dual

Direction:	UE to network

Table 8.3.15.1.1: PDU SESSION RELEASE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION RELEASE COMPLETE message identity | Message type 9.7 | M | V | 1 |
| 59 | 5GSM cause | 5GSM cause 9.11.4.2 | O | TV | 2 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |

#### 8.3.15.2	5GSM cause

This IE is included in the message when the UE needs to indicate to the network that an error encountered with a mandatory information element in the PDU SESSION RELEASE COMMAND message.

#### 8.3.15.3	Extended protocol configuration options

This IE is included in the message when the UE needs to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 8.3.16	5GSM status

#### 8.3.16.1	Message definition

The 5GSM STATUS message is sent by the SMF or the UE to pass information on the status of the indicated PDU session and report certain error conditions. See table 8.3.16.1.1.

Message type:	5GSM STATUS

Significance:	dual

Direction:	both

Table 8.3.16.1.1: 5GSM STATUS message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | 5GSM STATUS message identity | Message type 9.7 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.11.4.2 | M | V | 1 |

### 8.3.17	Service-level authentication command

#### 8.3.17.1	Message definition

The SERVICE-LEVEL AUTHENTICATION COMMAND message is sent by the SMF to the UE for service-level authentication and authorization procedure. See table 8.3.17.1.1.

Message type:	SERVICE-LEVEL AUTHENTICATION COMMAND

Significance:	dual

Direction:	network to UE

Table 8.3.17.1.1: SERVICE-LEVEL AUTHENTICATION COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | SERVICE-LEVEL AUTHENTICATION COMMAND message identity | Message type 9.7 | M | V | 1 |
|  | Service-level-AA container | Service-level-AA container 9.11.2.10 | M | LV-E | 5-65538 |

### 8.3.18	Service-level authentication complete

#### 8.3.18.1	Message definition

The SERVICE-LEVEL AUTHENTICATION COMPLETE message is sent by the UE to the SMF in response to the SERVICE-LEVEL AUTHENTICATION COMMAND message and indicates acceptance of the SERVICE-LEVEL AUTHENTICATION COMMAND message. See table 8.3.18.1.1.

Message type:	SERVICE-LEVEL AUTHENTICATION COMPLETE

Significance:	dual

Direction:	UE to network

Table 8.3.18.1.1: SERVICE-LEVEL AUTHENTICATION COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | SERVICE-LEVEL AUTHENTICATION COMPLETE message identity | Message type 9.7 | M | V | 1 |
|  | Service-level-AA container | Service-level-AA container 9.11.2.10 | M | LV-E | 5-65538 |

### 8.3.19	Remote UE report

#### 8.3.19.1	Message definition

The REMOTE UE REPORT message is sent by the UE to the network to report connection or disconnection of 5G ProSe remote UE(s). See table 8.3.19.1.

Message type:	REMOTE UE REPORT

Significance:	dual

Direction:	UE to network

Table 8.3.19.1: REMOTE UE REPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | Remote UE report message identity | Message type 9.7 | M | V | 1 |
| 76 | Remote UE context connected | Remote UE context list 9.11.4.29 | O | TLV-E | 16-65538 |
| 70 | Remote UE context disconnected | Remote UE context list 9.11.4.29 | O | TLV-E | 16-65538 |

#### 8.3.19.2	Remote UE context connected

This IE is included in the message by the UE acting as a 5G ProSe layer-3 UE-to-network relay UE to provide the network with newly connected 5G ProSe remote UE information as specified in 3GPP TS 23.304 [6E].

#### 8.3.19.3	Remote UE context disconnected

This IE is included in the message by the UE acting as a 5G ProSe layer-3 UE-to-network relay UE to provide the network with disconnected 5G ProSe remote UE information as specified in 3GPP TS 23.304 [6E].

### 8.3.20	Remote UE report response

#### 8.3.20.1	Message definition

The REMOTE UE REPORT RESPONSE message is sent by the network to the UE to acknowledge receipt of a remote UE report message. See table 8.3.20.1.

Message type:	REMOTE UE REPORT RESPONSE

Significance:	dual

Direction:	network to UE

Table 8.3.20.1: REMOTE UE REPORT RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | Remote UE report response message identity | Message type 9.7 | M | V | 1 |

#### 8.3.20.2	Void

#### 8.3.20.3	Void

#### 8.3.20.4	Void

# 9	General message format and information elements coding

## 9.1	Overview

### 9.1.1	NAS message format

Within the protocols defined in the present document, every 5GS NAS message is a standard L3 message as defined in 3GPP TS 24.007 [11]. This means that the message consists of the following parts:

1)	if the message is a plain 5GS NAS message:

a)	extended protocol discriminator;

b)	security header type associated with a half spare octet or PDU session identity;

c)	procedure transaction identity;

d)	message type;

e)	other information elements, as required.

2)	if the message is a security protected 5GS NAS message:

a)	extended protocol discriminator;

b)	security header type associated with a half spare octet;

c)	message authentication code;

d)	sequence number;

e)	plain 5GS NAS message, as defined in item 1

The organization of a plain 5GS NAS message is illustrated in the example shown in figure 9.1.1.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended protocol discriminator |  |  |  |  |  |  |  | octet 1 |
| Security header type associated with a spare half octet; or PDU session identity |  |  |  |  |  |  |  | octet 2 |
| Procedure transaction identity |  |  |  |  |  |  |  | octet 2a* |
| Message type |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet 4 |
| Other information elements as required |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 9.1.1.1: General message organization example for a plain 5GS NAS message

The PDU session identity and the procedure transaction identity are only used in messages with extended protocol discriminator 5GS session management. Octet 2a with the procedure transaction identity shall only be included in these messages.

The organization of a security protected 5GS NAS message is illustrated in the example shown in figure 9.1.1.2.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended protocol discriminator |  |  |  |  |  |  |  | octet 1 |
| Security header type associated with a spare half octet |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| Message authentication code |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 6 |
| Sequence number |  |  |  |  |  |  |  | octet 7 |
|  |  |  |  |  |  |  |  | octet 8 |
| Plain 5GS NAS message |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 9.1.1.2: General message organization example for a security protected 5GS NAS message

Unless specified otherwise in the message descriptions of clause 8 and annex D, a particular information element shall not be present more than once in a given message.

### 9.1.2	Field format and mapping

When a field is contained within a single octet, the lowest numbered bit of the field represents the least significant bit.

When a field extends over more than one octet, the order of bit values progressively decreases as the octet number increases. In that part of the field contained in a given octet, the lowest numbered bit represents the least significant bit. The most significant bit of the field is represented by the highest numbered bit of the lowest numbered octet of the field. The least significant bit of the field is represented by the lowest numbered bit of the highest numbered octet of the field.

For example, a bit number can be identified as a couple (o, b) where o is the octet number and b is the relative bit number within the octet. Figure 9.1.2.1 illustrates a field that spans from bit (1, 3) to bit (2, 7). The most significant bit of the field is mapped on bit (1, 3) and the least significant bit is mapped on bit (2, 7).

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 24 | 23 | 22 | 1st octet of field |
| 21 | 20 |  |  |  |  |  |  | 2nd octet of field |

Figure 9.1.2.1: Field mapping convention

## 9.2	Extended protocol discriminator

Bits 1 to 8 of the first octet of every 5GS NAS message contain the Extended protocol discriminator (EPD) IE. The EPD and its use are defined in 3GPP TS 24.007 [11]. The extended protocol discriminator in the header (see 3GPP TS 24.007 [11]) of a security protected 5GS NAS message is encoded as "5GS mobility management messages".

## 9.3	Security header type

Bits 1 to 4 of the second octet of every 5GMM message contain the Security header type IE. This IE includes control information related to the security protection of a 5GMM message. The total size of the Security header type IE is 4 bits.

The Security header type IE can take the values shown in table 9.3.1.

Table 9.3.1: Security header type

| Security header type (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | Plain 5GS NAS message, not security protected |
|  |  |  |  | Security protected 5GS NAS message: |
| 0 | 0 | 0 | 1 | Integrity protected |
| 0 | 0 | 1 | 0 | Integrity protected and ciphered |
| 0 | 0 | 1 | 1 | Integrity protected with new 5G NAS security context (NOTE 1) |
| 0 | 1 | 0 | 0 | Integrity protected and ciphered with new 5G NAS security context (NOTE 2) |
| All other values are reserved. |  |  |  |  |
| NOTE 1: This codepoint may be used only for a SECURITY MODE COMMAND message. NOTE 2: This codepoint may be used only for a SECURITY MODE COMPLETE message. |  |  |  |  |

A 5GMM message received with the security header type encoded as 0000 shall be treated as not security protected, plain 5GS NAS message. A protocol entity sending a not security protected 5GMM message shall send the message as plain 5GS NAS message and encode the security header type as 0000.

## 9.4	PDU session identity

Bits 1 to 8 of the second octet of every 5GSM message contain the PDU session identity IE. The PDU session identity and its use to identify a message flow are defined in 3GPP TS 24.007 [11].

## 9.5	Spare half octet

This element is used in the description of 5GMM and 5GSM messages when an odd number of half octet type 1 information elements are used. This element is filled with spare bits set to zero and is placed in bits 5 to 8 of the octet unless otherwise specified.

## 9.6	Procedure transaction identity

Bits 1 to 8 of the third octet of every 5GSM message contain the procedure transaction identity. Bits 1 to 8 of the first octet of every UE policy delivery message contain the procedure transaction identity. The procedure transaction identity and its use are defined in 3GPP TS 24.007 [11].

## 9.7	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [11]. Tables 9.7.1 and 9.7.2 define the value part of the message type IE used in the 5GS mobility management protocol and 5GS session management protocol.

Table 9.7.1: Message types for 5GS mobility management

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | - | - | - | - | - | - |  | 5GS mobility management messages |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Registration request |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | Registration accept |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | Registration complete |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | Registration reject |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | Deregistration request (UE originating) as specified in clause 8.2.12) |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |  | Deregistration accept (UE originating) as specified in clause 8.2.13) |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  | Deregistration request (UE terminated) as specified in clause 8.2.14) |
| 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 |  | Deregistration accept (UE terminated) as specified in clause 8.2.15) |
| 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |  | Service request |
| 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |  | Service reject |
| 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |  | Service accept |
| 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |  | Control plane service request |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |  | Network slice-specific authentication command |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  | Network slice-specific authentication complete |
| 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |  | Network slice-specific authentication result |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |  | Configuration update command |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  | Configuration update complete |
| 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |  | Authentication request |
| 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |  | Authentication response |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |  | Authentication reject |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |  | Authentication failure |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |  | Authentication result |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |  | Identity request |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |  | Identity response |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |  | Security mode command |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |  | Security mode complete |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |  | Security mode reject |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |  | 5GMM status |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |  | Notification |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |  | Notification response |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |  | UL NAS transport |
| 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |  | DL NAS transport |
| 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |  | Relay key request |
| 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 |  | Relay key accept |
| 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |  | Relay key reject |
| 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 |  | Relay authentication request |
| 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |  | Relay authentication response |

Table 9.7.2: Message types for 5GS session management

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 1 | 1 | - | - | - | - | - | - |  | 5GS session management messages |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | PDU session establishment request |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | PDU session establishment accept |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | PDU session establishment reject |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | PDU session authentication command |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |  | PDU session authentication complete |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  | PDU session authentication result |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |  | PDU session modification request |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |  | PDU session modification reject |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 |  | PDU session modification command |
| 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |  | PDU session modification complete |
| 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |  | PDU session modification command reject |
| 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  | PDU session release request |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |  | PDU session release reject |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |  | PDU session release command |
| 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |  | PDU session release complete |
| 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |  | 5GSM status |
| 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |  | Service-level authentication command |
| 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |  | Service-level authentication complete |
| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |  | Remote UE report |
| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |  | Remote UE report response |

## 9.8	Message authentication code

The message authentication code (MAC) information element contains the integrity protection information for the message. The MAC IE shall be included in the SECURITY PROTECTED 5GS NAS MESSAGE message if a valid 5G NAS security context exists and security functions are started.

The message authentication code (MAC) is also included in the Intra N1 mode NAS transparent container IE and in the S1 mode to N1 mode NAS transparent container IE.

The usage of MAC is specified in clause 4.4.3.3.

## 9.9	Plain 5GS NAS message

This IE includes a complete plain 5GS NAS message as specified in clauses 8.2 and 8.3. The SECURITY PROTECTED 5GS NAS MESSAGE (see clause 8.2.28) includes a complete plain 5GS NAS message as specified in clauses 8.2. The SECURITY PROTECTED 5GS NAS MESSAGE message (see clause 8.2.28) is not plain 5GS NAS messages and shall not be included in this IE.

## 9.10	Sequence number

This IE includes the NAS message sequence number (SN) which consists of the eight least significant bits of the NAS COUNT for a SECURITY PROTECTED 5GS NAS MESSAGE message.

The NAS message sequence number (SN) with the eight least significant bits of the NAS COUNT is also included in the Intra N1 mode NAS transparent container IE and in the N1 mode to S1 mode NAS transparent container IE.

The usage of SN is specified in clause 4.4.3.

## 9.11	Other information elements

### 9.11.1	General

The different formats (V, LV, T, TV, TLV, LV-E, TLV-E) and the five categories of information elements (type 1, 2, 3, 4 and 6) are defined in 3GPP TS 24.007 [11].

The first octet of an information element in the non-imperative part contains the IEI of the information element. If this octet does not correspond to an IEI known in the message, the receiver shall determine whether this IE is of type 1 or 2 (i.e. it is an information element of one octet length) or an IE of type 4 or 6 (i.e. that the next octet is the length indicator or, for a type 6 IE, the next 2 octets are the length indicator indicating the length of the remaining of the information element) (see 3GPP TS 24.007 [11]).

NOTE:	This requirement for the receiver is not applicable for information elements included in a Type 6 IE container information element. Any IE in the Type 6 IE container information element is of type 6 with format TLV-E; therefore, the rules for the IEI value encoding defined in 3GPP TS 24.007 [11], clause 11.2.4, are not applicable.

This allows the receiver to jump over unknown information elements and to analyse any following information elements of a particular message.

The definitions of information elements which are:

a)	common for the 5GMM and 5GSM protocols;

b)	used by access stratum protocols; or

c)	sent to upper layers

are described in clause 9.11.2.

The information elements of the 5GMM or 5GSM protocols can be defined by reference to an appropriate specification which provides the definition of the information element, e.g., "see clause 10.5.6.3A in 3GPP TS 24.008 [12]".

### 9.11.2	Common information elements

#### 9.11.2.1	Additional information

The purpose of the Additional information information element is to provide additional information to upper layers in relation to the NAS transport mechanism.

The Additional information information element is coded as shown in figure 9.11.2.1.1 and table 9.11.2.1.1.

The Additional information is a type 4 information element with a minimum length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Additional information IEI |  |  |  |  |  |  |  | octet 1 |
| Additional information length |  |  |  |  |  |  |  | octet 2 |
| Additional information value |  |  |  |  |  |  |  | octets 3-n |

Figure 9.11.2.1.1: Additional information information element

Table 9.11.2.1.1 : Additional information information element

Additional information value (octet 3 to octet n)

The coding of the additional information value is dependent on the LCS application.

#### 9.11.2.1A	Access type

The purpose of the access type information element is to indicate the access type over which the signalling or user data is pending to be sent to the UE.

The access type is a type 1 information element.

The access type information element is coded as shown in figure 9.11.2.1A.1 and table 9.11.2.1A.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Access type IEI |  |  |  | 0 spare |  | Access type |  | octet 1 |

Figure 9.11.2.1A.1: Access type information element

Table 9.11.2.1A.1: Access type information element

| Access type value (octet 1, bit 1 to bit 2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 2 | 1 |  |  |  |
| 0 | 1 |  |  | 3GPP access |
| 1 | 0 |  |  | Non-3GPP access |
| All other values are reserved. |  |  |  |  |

#### 9.11.2.1B	DNN

The purpose of the DNN information element is to identify the data network.

The DNN information element is coded as shown in figure 9.11.2.1B.1.

The DNN is a type 4 information element with a minimum length of 3 octets and a maximum length of 102 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DNN IEI |  |  |  |  |  |  |  | octet 1 |
| Length of DNN contents |  |  |  |  |  |  |  | octet 2 |
| DNN value |  |  |  |  |  |  |  | octet 3 octet n |

Figure 9.11.2.1B.1: DNN information element

A DNN value field contains an APN as defined in 3GPP TS 23.003 [4].

#### 9.11.2.2	EAP message

The purpose of the EAP message information element is to transport an EAP message as specified in IETF RFC 3748 [34].

The EAP message information element is coded as shown in figure 9.11.2.2.1 and table 9.11.2.2.1.

The EAP message is a type 6 information element with minimum length of 7 octets and maximum length of 1503 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EAP message IEI |  |  |  |  |  |  |  | octet 1 |
| Length of EAP message contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| EAP message |  |  |  |  |  |  |  | octet 4 octet n |

Figure 9.11.2.2.1: EAP message information element

Table 9.11.2.2.1: EAP message information element

EAP message (octet 4 to n)

An EAP message as specified in IETF RFC 3748 [34].

#### 9.11.2.3	GPRS timer

See clause 10.5.7.3 in 3GPP TS 24.008 [12].

#### 9.11.2.4	GPRS timer 2

See clause 10.5.7.4 in 3GPP TS 24.008 [12].

#### 9.11.2.5	GPRS timer 3

See clause 10.5.7.4a in 3GPP TS 24.008 [12].

#### 9.11.2.6	Intra N1 mode NAS transparent container

The purpose of the Intra N1 mode NAS transparent container information element is to provide the UE with parameters that enable the UE to handle the 5G NAS security context after N1 mode to N1 mode handover.

The Intra N1 mode NAS transparent container information element is coded as shown in figure 9.11.2.6.1 and table 9.11.2.6.1.

The Intra N1 mode NAS transparent container is a type 4 information element with a length of 9 octets.

The value part of the Intra N1 mode NAS transparent container information element is included in specific information elements within some RRC messages sent to the UE.

NOTE:	For these cases the coding of the information element identifier and length information of RRC is defined in 3GPP TS 38.331 [30].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intra N1 mode NAS transparent container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Intra N1 mode NAS transparent container contents |  |  |  |  |  |  |  | octet 2 |
| Message authentication code |  |  |  |  |  |  |  | octet 3 octet 6 |
| Type of ciphering algorithm |  |  |  | Type of integrity protection algorithm |  |  |  | octet 7 |
| 0 | 0 Spare | 0 | KACF | TSC | Key set identifier in 5G |  |  | octet 8 |
| Sequence number |  |  |  |  |  |  |  | octet 9 |

Figure 9.11.2.6.1: Intra N1 mode NAS transparent container information element

Table 9.11.2.6.1: Intra N1 mode NAS transparent container information element

| Message authentication code (octet 3 to 6) |  |
| --- | --- |
| This field is coded as the Message authentication code information element (see clause 9.8). |  |
| Type of integrity protection algorithm (octet 7, bit 1 to 4) and type of ciphering algorithm (octet 7, bit 5 to 8) |  |
| These fields are coded as the type of integrity protection algorithm and type of ciphering algorithm in the NAS security algorithms information element (see clause 9.11.3.34). |  |
| K_AMF_change_flag (KACF) (octet 8, bit 5) |  |
| Bit |  |
| 5 |  |
| 0 | a new KAMF has not been calculated by the network |
| 1 | a new KAMF has been calculated by the network |
| Key set identifier in 5G (octet 8, bit 1 to 3) and Type of security context flag (TSC) (octet 8, bit 4) |  |
| These fields are coded as the NAS key set identifier and type of security context flag in the NAS key set identifier information element (see clause 9.11.3.32). |  |
| Sequence number (octet 9) |  |
| This field is coded as the Sequence number information element (see clause 9.10) |  |

#### 9.11.2.7	N1 mode to S1 mode NAS transparent container

The purpose of the N1 mode to S1 mode NAS transparent container information element is to provide the UE with information that enables the UE to create a mapped EPS security context.

The N1 mode to S1 mode NAS transparent container information element is coded as shown in figure 9.11.2.7.1 and table 9.11.2.7.1.

The N1 mode to S1 mode NAS transparent container is a type 3 information element with a length of 2 octets.

The value part of the N1 mode to S1 mode NAS transparent container information element is included in specific information elements within some RRC messages sent to the UE; see 3GPP TS 38.331 [30]. For these cases the coding of the information element identifier and length information is defined in 3GPP TS 38.331 [30].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N1 mode to S1 mode NAS transparent container IEI |  |  |  |  |  |  |  | octet 1 |
| Sequence number |  |  |  |  |  |  |  | octet 2 |

Figure 9.11.2.7.1: N1 mode to S1 mode NAS transparent container information element

Table 9.11.2.7.1: N1 mode to S1 mode NAS transparent container information element

Sequence number (octet 2)

This field is coded as the Sequence number information element (see clause 9.10).

#### 9.11.2.8	S-NSSAI

The purpose of the S-NSSAI information element is to identify a network slice.

The S-NSSAI information element is coded as shown in figure 9.11.2.8.1 and table 9.11.2.8.1.

The S-NSSAI is a type 4 information element with a minimum length of 3 octets and a maximum length of 10 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of S-NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| SST |  |  |  |  |  |  |  | octet 3 |
| SD |  |  |  |  |  |  |  | octet 4* octet 6* |
| Mapped HPLMN SST |  |  |  |  |  |  |  | octet 7* |
| Mapped HPLMN SD |  |  |  |  |  |  |  | octet 8* octet 10* |

Figure 9.11.2.8.1: S-NSSAI information element

Table 9.11.2.8.1: S-NSSAI information element

| Length of S-NSSAI contents (octet 2) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| This field indicates the length of the included S-NSSAI contents, and it can have the following values. Depending on the value of the length field the following S-NSSAI contents are included: |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | SST |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | SST and mapped HPLMN SST |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | SST and SD |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | SST, SD and mapped HPLMN SST |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | SST, SD, mapped HPLMN SST and mapped HPLMN SD |
| All other values are reserved. |  |  |  |  |  |  |  |  |
| Slice/service type (SST) (octet 3) |  |  |  |  |  |  |  |  |
| This field contains the 8 bit SST value. The coding of the SST value part is defined in 3GPP TS 23.003 [4]. If this IE is included during the network slice-specific authentication and authorization procedure, this field contains the 8 bit SST value of an S-NSSAI in the S-NSSAI(s) of the HPLMN or the RSNPN. |  |  |  |  |  |  |  |  |
| Slice differentiator (SD) (octet 4 to octet 6) This field contains the 24 bit SD value. The coding of the SD value part is defined in 3GPP TS 23.003 [4]. If this IE is included during the network slice-specific authentication and authorization procedure, this field contains the 24 bit SD value of an S-NSSAI in the S-NSSAI(s) of the HPLMN or the RSNPN. |  |  |  |  |  |  |  |  |
| If the SST encoded in octet 3 is not associated with a valid SD value, and the sender needs to include a mapped HPLMN SST (octet 7) and a mapped HPLMN SD (octets 8 to 10), then the sender shall set the SD value (octets 4 to 6) to "no SD value associated with the SST". |  |  |  |  |  |  |  |  |
| mapped HPLMN Slice/service type (SST) (octet 7) |  |  |  |  |  |  |  |  |
| This field contains the 8 bit SST value of an S-NSSAI in the S-NSSAI(s) of the HPLMN to which the SST value is mapped. The coding of the SST value part is defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |
| mapped HPLMN Slice differentiator (SD) (octet 8 to octet 10) This field contains the 24 bit SD value of an S-NSSAI in the S-NSSAI(s) of the HPLMN to which the SD value is mapped. The coding of the SD value part is defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |
| NOTE 1: Octet 3 shall always be included. NOTE 2: If the octet 4 is included, then octet 5 and octet 6 shall be included. NOTE 3: If the octet 7 is included, then octets 8, 9, and 10 may be included. NOTE 4: If the octet 8 is included, then octet 9 and octet 10 shall be included. NOTE 5: If only HPLMN S-NSSAI or subscribed SNPN S-NSSAI is included, then octets 7 to 10 shall not be included. |  |  |  |  |  |  |  |  |

#### 9.11.2.9	S1 mode to N1 mode NAS transparent container

The purpose of the S1 mode to N1 mode NAS transparent container information element is to provide the UE with parameters that enable the UE to create a mapped 5G NAS security context and take this context into use after inter-system change to N1 mode in 5GMM-CONNECTED mode.

The S1 mode to N1 mode NAS transparent container information element is coded as shown in figure 9.11.2.9.1 and table 9.11.2.9.1.

The S1 mode to N1 mode NAS transparent container is a type 4 information element with a length of 10 octets.

The value part of the S1 mode to N1 mode NAS transparent container information element is included in specific information elements within some RRC messages sent to the UE.

NOTE:	For these cases the coding of the information element identifier and length information of RRC is defined in 3GPP TS 38.331 [30].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1 mode to N1 mode NAS transparent container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of S1 mode to N1 mode NAS transparent container contents |  |  |  |  |  |  |  | octet 2 |
| Message authentication code |  |  |  |  |  |  |  | octet 3 octet 6 |
| Type of ciphering algorithm |  |  |  | Type of integrity protection algorithm |  |  |  | octet 7 |
| 0 Spare | NCC |  |  | TSC | Key set identifier in 5G |  |  | octet 8 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet 9 octet 10 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare |  |

Figure 9.11.2.9.1: S1 mode to N1 mode NAS transparent container information element

Table 9.11.2.9.1: S1 mode to N1 mode NAS transparent container information element

Message authentication code (octet 3 to 6)

This field is coded as the Message authentication code information element (see clause 9.8).

Type of integrity protection algorithm (octet 7, bit 1 to 4) and type of ciphering algorithm (octet 7, bit 5 to 8)

These fields are coded as the type of integrity protection algorithm and type of ciphering algorithm in the NAS security algorithms information element (see clause 9.11.3.34).

NCC (octet 8, bits 5 to 7)

This field contains the 3 bit Next hop chaining counter (see 3GPP TS 33.501 [24])

Key set identifier in 5G (octet 8, bit 1 to 3) and type of security context flag (TSC) (octet 8, bit 4)

These fields are coded as the NAS key set identifier and type of security context flag in the NAS key set identifier information element (see clause 9.11.3.32).

Octets 9 and 10 are spare and shall be coded as zero.

NOTE: In earlier versions of this protocol, octets 9 and 10 can have any value. In this version of the protocol, octets 9 and 10 can always be ignored by the UE.

#### 9.11.2.10	Service-level-AA container

The purpose of the Service-level-AA container information element is to transfer upper layer information for authentication and authorization between the UE and the network.

The Service-level-AA container information element is coded as shown in figure 9.11.2.10.1, figure 9.11.2.10.2, figure 9.11.2.10.3, figure 9.11.2.10.4 and table 9.11.2.10.1.

The Service-level-AA container information element is a type 6 information element with a minimum length of 4 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service-level-AA container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Service-level-AA container contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet 4 |
| Service-level-AA container contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 9.11.2.10.1: Service-level-AA container information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service-level-AA parameter 1 |  |  |  |  |  |  |  | octet 4 octet x1 |
| Service-level-AA parameter 2 |  |  |  |  |  |  |  | octet x1+1* octet x2* |
| …… |  |  |  |  |  |  |  | … |
| Service-level-AA parameter n |  |  |  |  |  |  |  | octet xi +1* octet n* |

Figure 9.11.2.10.2: Service-level-AA container contents

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of service-level-AA parameter |  |  |  |  |  |  |  | octet xi +1 |
| Length of service-level-AA parameter |  |  |  |  |  |  |  | octet xi +2 |
| Value of service-level-AA parameter |  |  |  |  |  |  |  | octet xi +3 octet n |

Figure 9.11.2.10.3: Service-level-AA parameter (when the type of service-level-AA parameter field contains an IEI of a type 4 information element as specified in 3GPP TS 24.007 [11])

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of service-level-AA parameter |  |  |  |  |  |  |  | octet xi +1 |
| Length of service-level-AA parameter |  |  |  |  |  |  |  | octet xi +2 octet xi +3 |
| Value of service-level-AA parameter |  |  |  |  |  |  |  | octet xi +4 octet n |

Figure 9.11.2.10.4: Service-level-AA parameter (when the type of service-level-AA parameter field contains an IEI of a type 6 information element as specified in 3GPP TS 24.007 [11])

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service-level-AA payload type |  |  |  |  |  |  |  | octet xi +1 octet xi +3 |
| Service-level-AA payload |  |  |  |  |  |  |  | octet xi +4 octet n |

Figure 9.11.2.10.5: Service-level-AA parameter (when Service-level-AA payload type and its associated Service-level-AA payload are included in the Service-level-AA container contents)

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of service-level-AA parameter |  |  |  | Value of service-level-AA parameter |  |  |  | octet xi+1 |

Figure 9.11.2.10.6: Service-level-AA parameter (when the type of service-level-AA parameter field contains an IEI of a type 1 information element as specified in 3GPP TS 24.007 [11])

Table 9.11.2.10.1: Service-level-AA container information element

| Service-level-AA container contents (octet 4 to octet n); max value of 65535 octets |  |  |
| --- | --- | --- |
| The error handlings for service-level-AA parameters specified in clauses 7.6.1, 7.6.3 and 7.7.1 shall apply to the service-level-AA parameters included in the service-level-AA container contents. |  |  |
| Service-level-AA parameters Type of service-level-AA parameter (octet xi +1) This field contains the IEI of the service-level-AA parameter. |  |  |
| Length of service-level-AA parameter This field indicates binary coded length of the value of the service-level-AA parameter. |  |  |
| Value of service-level-AA parameter This field contains the value of the service-level-AA parameter with the value part of the referred information element based on following service-level-AA parameter reference. The receiving entity shall ignore service-level-AA parameter with type of service-level-AA parameter field containing an unknown IEI. |  |  |
| IEI (hexadecimal) | Service-level-AA parameter name | Service-level-AA parameter reference |
| 10 | Service-level device ID | Service-level device ID (see clause 9.11.2.11) |
| 20 | Service-level-AA server address | Service-level-AA server address (see clause 9.11.2.12) |
| 30 | Service-level-AA response | Service-level-AA response (see clause 9.11.2.14) |
| 40 | Service-level-AA payload type | Service-level-AA payload type (see clause 9.11.2.15) (NOTE) |
| 70 | Service-level-AA payload | Service-level-AA payload (see clause 9.11.2.13) |
| A- | Service-level-AA pending indication | Service-level-AA pending indication (see clause 9.11.2.17) |
| 50 | Service-level-AA service status indication | Service-level-AA service status indication (see clause 9.11.2.18) |
| NOTE: A service-level-AA payload type is always followed by the associated service-level-AA payload as shown in figure 9.11.2.10.5. |  |  |

#### 9.11.2.11	Service-level device ID

The purpose of the Service-level device ID information element is to carry the necessary identity for authentication and authorization by the external DN.

The Service-level device ID information element is coded as shown in figure 9.11.2.11.1 and table 9.11.2.11.1.

The Service-level device ID information element is a type 4 information element with minimum length of 3 octets and maximum length of 257 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service-level device ID IEI |  |  |  |  |  |  |  | octet 1 |
| Service-level device ID length |  |  |  |  |  |  |  | octet 2 |
| Service-level device ID |  |  |  |  |  |  |  | octets 3-y |

Figure 9.11.2.11.1: Service-level device ID information element

Table 9.11.2.11.1: Service-level device ID information element

Service-level device ID (octet 3 to octet y) A service-level device ID encoded as UTF-8 string.

#### 9.11.2.12	Service-level-AA server address

The purpose of the Service-level-AA server address information element is to carry the address of the service level authentication and authorization server.

The Service-level-AA server address information element is coded as shown in figure 9.11.2.12.1 and table 9.11.2.12.1.

The Service-level-AA server address information element is a type 4 information element with minimum length of 4 octets and maximum length of 257 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service-level-AA server address IEI |  |  |  |  |  |  |  | octet 1 |
| Service-level-AA server address length |  |  |  |  |  |  |  | octet 2 |
| Service-level-AA server address type |  |  |  |  |  |  |  | octet 3 |
| Service-level-AA server address |  |  |  |  |  |  |  | octets 4-z |

Figure 9.11.2.12.1: Service-level-AA server address information element

Table 9.11.2.12.1: Service-level-AA server address information element

| Service-level-AA server address type (octet 3): Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | IPv4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | IPv6 |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | IPv4v6 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | FQDN |
| All other values are spare. If received they shall be ignored. |  |  |  |  |  |  |  |  |  |
| If the service-level-AA server address type indicates IPv4, then the service-level-AA server address field contains an IPv4 address in octet 4 to octet 7. |  |  |  |  |  |  |  |  |  |
| If the service-level-AA server address type indicates IPv6, then the service-level-AA server address field contains an IPv6 address in octet 4 to octet 19. |  |  |  |  |  |  |  |  |  |
| If the service-level-AA server address type indicates IPv4v6, then the service-level-AA server address field contains two IP addresses. The first IP address is an IPv4 address in octet 4 to octet 7. The second IP address is an IPv6 address in octet 8 to octet 23. |  |  |  |  |  |  |  |  |  |
| If the service-level-AA server address type indicates FQDN, octet 4 to octet z is encoded as defined in clause 19.4.2.1 in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |

#### 9.11.2.13	Service-level-AA payload

The purpose of the Service-level-AA payload information element is to carry the upper layer payload for authentication and authorization between the UE and the service-level-AA server.

The Service-level-AA payload information element is coded as shown in figure 9.11.2.13.1 and table 9.11.2.13.1.

The Service-level-AA payload information element is a type 6 information element with minimum length of 4 octets and maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service-level-AA payload IEI |  |  |  |  |  |  |  | octet 1 |
| Service-level-AA payload length |  |  |  |  |  |  |  | octet 2 octet 3 |
| Service-level-AA payload |  |  |  |  |  |  |  | octets 4-s |

Figure 9.11.2.13.1: Service-level-AA payload information element

Table 9.11.2.13.1: Service-level-AA payload information element

Service-level-AA payload (octet 4 to octet s) A payload for authentication and authorization transparently transported and which is provided from/to the upper layers.

#### 9.11.2.14	Service-level-AA response

The purpose of the Service-level-AA response information element is to provide information regarding the service level authentication and authorization request, e.g. to indicate that the authentication and authorization request to the service level authentication server was successful, or to notify that service level authorization is revoked.

The Service-level-AA response information element is coded as shown in figure 9.11.2.14.1 and table 9.11.2.14.1.

The Service-level-AA response information element is a type 4 information element with length of 3 octets.

| 8 | 7 |  | 6 | 5 |  | 4 | 3 |  | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service-level-AA response IEI |  |  |  |  |  |  |  |  |  |  | octet 1 |  |  |
| Service-level-AA response length |  |  |  |  |  |  |  |  |  |  | octet 2 |  |  |
| 0 Spare |  | 0 Spare |  |  | C2AR |  |  | SLAR |  |  | octet 3 |  |  |

Figure 9.11.2.14.1: Service-level-AA response information element

Table 9.11.2.14.1: Service-level-AA response information element

| Service-level-AA result field (SLAR) (octet 3, bits 1 and 2) |  |  |  |
| --- | --- | --- | --- |
| Bits |  |  |  |
| 1 | 2 |  |  |
| 0 | 0 |  | No information |
| 0 | 1 |  | Service level authentication and authorization was successful. |
| 1 | 0 |  | Service level authentication and authorization was not successful or service level authorization is revoked. |
| 1 | 1 |  | Reserved |
| C2 authorization result field (C2AR) (octet 3, bits 3 and 4) |  |  |  |
| Bits |  |  |  |
| 3 | 4 |  |  |
| 0 | 0 |  | No information |
| 0 | 1 |  | C2 authorization was successful. |
| 1 | 0 |  | C2 authorization was not successful or C2 authorization is revoked. |
| 1 | 1 |  | Reserved |
| Bits 5 to 8 of octet 3 are spare and shall be coded as zero. |  |  |  |

#### 9.11.2.15	Service-level-AA payload type

The purpose of the Service-level-AA payload type information element is to indicates type of payload included in the Service-level-AA payload information element.

The Service-level-AA payload type information element is coded as shown in figure 9.11.2.15.1 and table 9.11.2.15.1.

The Service-level-AA payload type information element is a type 4 information element with length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service-level-AA payload type IEI |  |  |  |  |  |  |  | octet 1 |
| Service-level-AA payload type length |  |  |  |  |  |  |  | octet 2 |
| Service-level-AA payload type |  |  |  |  |  |  |  | octet 3 |

Figure 9.11.2.15.1: Service-level-AA payload type information element

Table 9.11.2.15.1: Service-level-AA payload type information element

| Service-level-AA payload type (octet 3): Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | UUAA payload (see NOTE 1) |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | C2 authorization payload (see NOTE 2) |
| All other values are spare, and the receiving entity shall ignore the service-level-AA payload type value set to a spare value. |  |  |  |  |  |  |  |  |  |
| NOTE 1: If the service-level-AA payload type indicates UUAA payload, the field for the service-level-AA payload of the Service-level AA payload information element is an application layer payload for UUAA procedure between the UE supporting UAS services and the USS. NOTE 2: If the service-level-AA payload type indicates C2 authorization payload, the field for the service-level-AA payload of the Service-level-AA payload information element is an application layer payload for C2 authorization procedure between the UE supporting UAS services and the USS. |  |  |  |  |  |  |  |  |  |

#### 9.11.2.16	Void

#### 9.11.2.17	Service-level-AA pending indication

The purpose of the Service-level-AA pending indication information element is to provide an indication that the service level authentication and authorization procedure is to be performed.

The Service-level-AA pending indication information element is coded as shown in figure 9.11.2.17.1 and table 9.11.2.17.1.

The Service-level-AA pending indication information element is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service-level-AA pending indication IEI |  |  |  | 0 Spare | 0 Spare | 0 Spare | SLAPI | octet 1 |

Figure 9.11.2.17.1: Service-level-AA pending indication

Table 9.11.2.17.1: Service-level-AA pending indication

| Service-level-AA pending indication (SLAPI) (octet 1, bit 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | reserved |
| 1 | Service-level-AA procedure is to be performed |

#### 9.11.2.18	Service-level-AA service status indication

The purpose of the Service-level-AA service status indication information element is to provide an indication of the service availability to the UE.

The Service-level-AA service status indication information element is coded as shown in figure 9.11.2.18.1 and table 9.11.2.18.1.

The Service-level-AA service status indication information element is a type 4 information element with a length of 3 octets.

|  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  |  |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service-level-AA service status indication IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |  |
| Length of Service-level-AA service status indication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | UAS |  | octet 3 |  |  |

Figure 9.11.2.18.1: Service-level-AA-service-status indication information element

Table 9.11.2.18.1: Service-level-AA-service-status indication information element

| UAS (octet 3, bit 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | UAS services not enabled |
| 1 | UAS services enabled |
| Bits 2 to 8 of octet 3 are spare and shall be encoded as zero. |  |

#### 9.11.2.19	Time duration

See clause 9.9.3.68 in 3GPP TS 24.301 [15].

#### 9.11.2.20	Unavailability information

See clause 9.9.3.69 in 3GPP TS 24.301 [15].

#### 9.11.2.21	Unavailability configuration

See clause 9.9.3.70 in 3GPP TS 24.301 [15].

### 9.11.3	5GS mobility management (5GMM) information elements

#### 9.11.3.1	5GMM capability

The purpose of the 5GMM capability information element is to provide the network with information concerning aspects of the UE related to the 5GCN or interworking with the EPS. The contents might affect the manner in which the network handles the operation of the UE.

The 5GMM capability information element is coded as shown in figure 9.11.3.1.1 and table 9.11.3.1.1.

The 5GMM capability is a type 4 information element with a minimum length of 3 octets and a maximum length of 15 octets.

|  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GMM capability IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |  |
| Length of 5GMM capability contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |  |
| SGC |  | 5G-IPHC-CP CIoT |  | N3 data |  | 5G-CP CIoT |  | RestrictEC |  | LPP |  | HO attach |  | S1 mode |  | octet 3 |  |  |
| RACS |  | NSSAA |  | 5G-LCS |  | V2XCNPC5 |  | V2XCEPC5 |  | V2X-Uu |  | 5G-UP CIoT |  | 5GSRVCC |  | octet 4* |  |  |
| 5G ProSe-l2relay |  | 5G ProSe-dc |  | 5G ProSe-dd |  | ER-NSSAI |  | 5G-EHC-CP CIoT |  | multipleUP |  | WUSA |  | CAG |  | octet 5* |  |  |
| PR |  | RPR |  | PIV |  | NCR |  | NR-PSSI |  | 5G ProSe-l3rmt |  | 5G ProSe-l2rmt |  | 5G ProSe-l3relay |  | octet 6* |  |  |
| MPSIU |  | UAS |  | NSAG |  | Ex-CAG |  | SSNPNSI |  | EventNotification |  | MINT |  | NSSRG |  | octet 7* |  |  |
| SBTS |  | NSR |  | LADN-DS |  | RAN timing |  | ECI |  | ESI |  | RCMAN |  | RCMAP |  | octet 8* |  |  |
| 5G ProSe-l2end |  | 5G ProSe-l3U2U relay |  | 5G ProSe-l2U2U relay |  | RSLPS |  | SBNS |  | UN-PER |  | A2XNPC5 |  | A2XEPC5 |  | octet 9* |  |  |
| A2X-Uu |  | SLVI |  | TempNS |  | SUPL |  | LCS-UPP |  | PNS |  | RSLP |  | 5G ProSe-l3end |  | octet 10* |  |  |
| LP-WUSPSAI-SI |  | ATUC |  | RSLPPU |  | RSLPVU |  | NSUC |  | RSLPL |  | NVL-SATNR |  | MCSIU |  | octet 11* |  |  |
| LWD |  | EF5L |  | MINT-EPS |  | 5G ProSe-L2IMrelay |  | 5G ProSe-L3IMrelay |  | MLCSUP |  | 5G ProSe-MCI |  | OPHPAE |  | octet 12* |  |  |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | octet 13*-15* |  |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 9.11.3.1.1: 5GMM capability information element

Table 9.11.3.1.1: 5GMM capability information element

|  | EPC NAS supported (S1 mode) (octet 3, bit 1) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | S1 mode not supported |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  | S1 mode supported |  |  |  |
|  | ATTACH REQUEST message containing PDN CONNECTIVITY REQUEST message for handover support (HO attach) (octet 3, bit 2) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  | ATTACH REQUEST message containing PDN CONNECTIVITY REQUEST message with request type set to "handover" or "handover of emergency bearer services" to transfer PDU session from N1 mode to S1 mode not supported |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  | ATTACH REQUEST message containing PDN CONNECTIVITY REQUEST message with request type set to "handover" or "handover of emergency bearer services" to transfer PDU session from N1 mode to S1 mode supported |  |  |  |  |
|  | LTE Positioning Protocol (LPP) capability (octet 3, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support LTE Positioning Protocol (LPP) (see 3GPP TS 37.355 [26]). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | LPP in N1 mode not supported |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  | LPP in N1 mode supported |  |  |  |
|  | Restriction on use of enhanced coverage support (RestrictEC) (octet 3, bit 4) This bit indicates the capability to support restriction on use of enhanced coverage. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Restriction on use of enhanced coverage not supported |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  | Restriction on use of enhanced coverage supported |  |  |  |
|  | Control plane CIoT 5GS optimization (5G-CP CIoT) (octet 3, bit 5) This bit indicates the capability for control plane CIoT 5GS optimization. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Control plane CIoT 5GS optimization not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Control plane CIoT 5GS optimization supported |  |  |
|  | N3 data transfer (N3 data) (octet 3, bit 6) This bit indicates the capability for N3 data transfer. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N3 data transfer supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N3 data transfer not supported |  |  |
|  | IP header compression for control plane CIoT 5GS optimization (5G-IPHC-CP CIoT) (octet 3, bit 7) This bit indicates the capability for IP header compression for control plane CIoT 5GS optimization. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | IP header compression for control plane CIoT 5GS optimization not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | IP header compression for control plane CIoT 5GS optimization supported |  |  |
|  | Service gap control (SGC) (octet 3, bit 8) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | service gap control not supported |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  | service gap control supported |  |  |  |
|  | 5G-SRVCC from NG-RAN to UTRAN (5GSRVCC) capability (octet 4, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability for 5G-SRVCC from NG-RAN to UTRAN (5GSRVCC) (see 3GPP TS 23.216 [6A]). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | 5G-SRVCC from NG-RAN to UTRAN not supported |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 5G-SRVCC from NG-RAN to UTRAN supported |  |  |  |
|  | User plane CIoT 5GS optimization (5G-UP CIoT) (octet 4, bit 2) This bit indicates the capability for user plane CIoT 5GS optimization. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User plane CIoT 5GS optimization not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User plane CIoT 5GS optimization supported |  |  |
|  | V2X-Uu capability (V2X-Uu) (octet 4, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability for V2X over Uu, as specified in 3GPP TS 24.587 [19B]. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  | V2X over Uu not supported |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  | V2X over Uu supported |  |  |  |  |
|  | V2X communication over E-UTRA-PC5 capability (V2XCEPC5) (octet 4, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability for V2X communication over E-UTRA-PC5, as specified in 3GPP TS 24.587 [19B]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  | V2X communication over E-UTRA-PC5 not supported |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  | V2X communication over E-UTRA-PC5 supported |  |  |  |  |
|  | V2X communication over NR-PC5 capability (V2XCNPC5) (octet 4, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability for V2X communication over NR-PC5, as specified in 3GPP TS 24.587 [19B]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | V2X communication over NR-PC5 not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | V2X communication over NR-PC5 supported |  |
|  | Location Services (5G-LCS) notification mechanisms capability (octet 4, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support Location Services (5G-LCS) notification mechanisms (see 3GPP TS 23.273 [6B]). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LCS notification mechanisms not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LCS notification mechanisms supported |  |
|  | Network slice-specific authentication and authorization (NSSAA) (octet 4, bit 7) This bit indicates the capability to support network slice-specific authentication and authorization. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network slice-specific authentication and authorization not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network slice-specific authentication and authorization supported |  |
|  | Radio capability signalling optimisation (RACS) capability (octet 4, bit 8) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RACS not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RACS supported |  |
|  | Closed Access Group (CAG) capability (octet 5, bit 1) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CAG not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CAG supported |  |
|  | WUS assistance (WUSA) information reception capability (octet 5, bit 2) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WUS assistance information reception not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WUS assistance information reception supported |  |
|  | Multiple user-plane resources support (multipleUP) (octet 5, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support multiple user-plane resources in NB-N1 mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multiple user-plane resources not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multiple user-plane resources supported |  |
|  | Ethernet header compression for control plane CIoT 5GS optimization (5G-EHC-CP CIoT) (octet 5, bit 4) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ethernet header compression for control plane CIoT 5GS optimization not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ethernet header compression for control plane CIoT 5GS optimization supported |  |
|  | Extended rejected NSSAI support (ER-NSSAI) (octet 5, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support extended rejected NSSAI. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Extended rejected NSSAI not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Extended rejected NSSAI supported |  |
|  | 5G ProSe direct discovery (5G ProSe-dd) (octet 5, bit 6) This bit indicates the capability for 5G ProSe direct discovery. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5G ProSe direct discovery not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5G ProSe direct discovery supported |  |
|  | 5G ProSe direct communication (5G ProSe-dc) (octet 5, bit 7) This bit indicates the capability for 5G ProSe direct communication. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5G ProSe direct communication not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5G ProSe direct communication supported |  |
|  | 5G ProSe layer-2 UE-to-network-relay (5G ProSe-l2relay) (octet 5, bit 8) This bit indicates the capability to act as a 5G ProSe layer-2 UE-to-network relay UE. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-network relay UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-network relay UE supported |  |
|  | 5G ProSe layer-3 UE-to-network-relay (5G ProSe-l3relay) (octet 6, bit 1) This bit indicates the capability to act as a 5G ProSe layer-3 UE-to-network relay UE Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 UE-to-network relay UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 UE-to-network relay UE supported |  |
|  | 5G ProSe layer-2 UE-to-network-remote (5G ProSe-l2rmt) (octet 6, bit 2) This bit indicates the capability to act as a 5G ProSe layer-2 UE-to-network remote UE Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-network remote UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-network remote UE supported |  |
|  | 5G ProSe layer-3 UE-to-network-remote (5G ProSe-l3rmt) (octet 6, bit 3) This bit indicates the capability to act as a 5G ProSe layer-3 UE-to-network remote UE Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 UE-to-network remote UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 UE-to-network remote UE supported |  |
|  | NR paging subgroup support indication (NR-PSSI) (octet 6, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the support of PEIPS assistance information for NR paging subgrouping |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NR paging subgrouping not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NR paging subgrouping supported |  |
|  | N1 NAS signalling connection release (NCR) (octet 6, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates whether N1 NAS signalling connection release is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N1 NAS signalling connection release not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N1 NAS signalling connection release supported |  |
|  | Paging indication for voice services (PIV) (octet 6, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates whether paging indication for voice services is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | paging indication for voice services not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | paging indication for voice services supported |  |
|  | Reject paging request (RPR) (octet 6, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates whether reject paging request is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | reject paging request not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | reject paging request supported |  |
|  | Paging restriction (PR) (octet 6, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates whether paging restriction is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | paging restriction not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | paging restriction supported |  |
|  | NSSRG (octet 7, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support the NSSRG. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NSSRG not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NSSRG supported |  |
|  | Minimization of service interruption (MINT) (octet 7, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support Minimization of service interruption (MINT) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MINT not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MINT supported |  |
|  | Event notification (EventNotification) (octet 7, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support event notification for upper layers Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Event notification not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Event notification supported |  |
|  | SOR-SNPN-SI (SOR SNPN SI) (octet 7, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support SOR-SNPN-SI. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SOR-SNPN-SI not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SOR-SNPN-SI supported |  |
|  | Extended CAG information list support (Ex-CAG) (octet 7, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support extended CAG information list. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Extended CAG information list not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Extended CAG information list supported |  |
|  | NSAG (octet 7, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support NSAG. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NSAG not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NSAG supported |  |
|  | UAS (octet 7, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support UAS services. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | UAS services not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | UAS services supported |  |
|  | MPS indicator update (MPSIU) (octet 7, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support MPS indicator update via the UE configuration update procedure. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MPS indicator update not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MPS indicator update supported |  |
|  | Registration complete message for acknowledging negotiated PEIPS assistance information (RCMAP) (octet 8, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability for sending REGISTRATION COMPLETE message when Negotiated PEIPS assistance information IE is included in the REGISTRATION ACCEPT message. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sending of REGISTRATION COMPLETE message for negotiated PEIPS assistance information not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sending of REGISTRATION COMPLETE message for negotiated PEIPS assistance information supported |  |
|  | Registration complete message for acknowledging NSAG information (RCMAN) (octet 8, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability for sending REGISTRATION COMPLETE message when NSAG information IE is included in the REGISTRATION ACCEPT message. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sending of REGISTRATION COMPLETE message for NSAG information not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sending of REGISTRATION COMPLETE message for NSAG information supported |  |
|  | Equivalent SNPNs indicator (ESI) (octet 8, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support equivalent SNPNs. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Equivalent SNPNs not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Equivalent SNPNs supported |  |
|  | Enhanced CAG information (ECI) (octet 8, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support enhanced CAG information. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Enhanced CAG information not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Enhanced CAG information supported |  |
|  | Reconnection to the network due to RAN timing synchronization status change (RANtiming) (octet 8, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support Reconnection to the network due to RAN timing synchronization status change. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reconnection to the network due to RAN timing synchronization status change not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reconnection to the network due to RAN timing synchronization status change supported |  |
|  | LADN per DNN and S-NSSAI support (LADN-DS) (octet 8, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support LADN per DNN and S-NSSAI. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LADN per DNN and S-NSSAI not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LADN per DNN and S-NSSAI supported |  |
|  | Network slice replacement (NSR) (octet 8, bit 7) This bit indicates the capability to support network slice replacement. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network slice replacement not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network slice replacement supported |  |
|  | Slice-based TNGF selection support (SBTS) (octet 8, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support slice-based TNGF selection. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Slice-based TNGF selection not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Slice-based TNGF selection supported |  |
|  | A2X over E-UTRA-PC5 (A2XEPC5) (octet 9, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability for A2X over E-UTRA-PC5, as specified in 3GPP TS 24.577 [60]. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A2X over E-UTRA-PC5 not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A2X over E-UTRA-PC5 supported |  |
|  | A2X over NR-PC5 (A2XNPC5) (octet 9, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability for A2X over NR-PC5, as specified in 3GPP TS 24.577 [60]. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A2X over NR-PC5 not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A2X over NR-PC5 supported |  |
|  | Unavailability period (UN-PER) (octet 9, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support unavailability period. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Unavailability period not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Unavailability period supported |  |
|  | Slice-based N3IWFselection support (SBNS) (octet 9, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support slide-based N3IWF selection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Slice-based N3IWF selection not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Slice-based N3IWF selection supported |  |
|  | SL positioning server UE (RSLPS) (octet 9, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability for SL positioning server UE, as specified in 3GPP TS 24.586 [ts23586]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning for SL positioning server UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning for SL positioning server UE supported |  |
|  | 5G ProSe layer-2 UE-to-UE relay (5G ProSe-l2U2U relay) (octet 9, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to act as a 5G ProSe layer-2 UE-to-UE relay UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-UE relay UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-UE relay UE supported |  |
|  | 5G ProSe layer-3 UE-to-UE relay (5G ProSe-l3U2U relay) (octet 9, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to act as a 5G ProSe layer-3 UE-to-UE relay UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 UE-to-UE relay UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 UE-to-UE relay UE supported |  |
|  | 5G ProSe layer-2 end UE (5G ProSe-l2end) (octet 9, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to act as a 5G ProSe layer-2 end UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-UE relay end UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-UE relay end UE supported |  |
|  | 5G ProSe layer-3 end UE (5G ProSe-l3end) (octet 10, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to act as a 5G ProSe layer-3 end UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 end UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 end UE supported |  |
|  | Ranging and sidelink positioning support (RSLP) (octet 10, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning supported |  |
|  | Partial network slice (PNS) (octet 10, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates whether the UE support partial network slice in the registration area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Partial network slice not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Partial network slice supported |  |
|  | LCS-UPP user plane positioning (LCS-UPP) (octet 10, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support LCS-UPP user plane positioning (see 3GPP TS 23.273 [6B]). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User plane positioning using LCS-UPP not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User plane positioning using LCS-UPP supported |  |
|  | SUPL user plane positioning (SUPL) (octet 10, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support SUPL user plane positioning (see 3GPP TS 38.305 [67] and 3GPP TS 23.271 [68]). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User plane positioning using SUPL not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User plane positioning using SUPL supported |  |
|  | S-NSSAI time validity information (TempNS) (octet 10, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support the S-NSSAI time validity information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S-NSSAI time validity information not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S-NSSAI time validity information supported |  |
|  | S-NSSAI location validity information (SLVI) (octet 10, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support S-NSSAI location validity information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S-NSSAI location validity information not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S-NSSAI location validity information supported |  |
|  | A2X over Uu capability (A2X-Uu) (octet 10, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability for A2X over Uu, as specified in 3GPP TS 24.577 [60]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A2X over Uu not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A2X over Uu supported |  |
|  | MCS indicator update (MCSIU) (octet 11, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support MCS indicator update via the UE configuration update procedure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MCS indicator update not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MCS indicator update supported |  |
|  | Network verified UE location over satellite NG-RAN (NVL-SATNR) (octet 11, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support network verified UE location over satellite NG-RAN as specified in 3GPP TS 23.501 [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network verified UE location over satellite NG-RAN not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network verified UE location over satellite NG-RAN supported |  |
|  | Ranging and sidelink positioning over PC5 for located UE support (RSLPL) (octet 11, bit 3) This bit indicates the capability to support ranging and sidelink positioning over PC5 for located UE support. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning for located UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning for located UE supported |  |
|  | Network slice usage control (NSUC) (octet 11, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support network slice usage control. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network slice usage control not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network slice usage control supported |  |
|  | Ranging and sidelink positioning with V2X capable UE (RSLPVU) (octet 11, bit 5) (see NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning with V2X capable UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning with V2X capable UE supported |  |
|  | Ranging and sidelink positioning with 5G ProSe capable UE support (RSLPPU) (octet 11, bit 6) (see NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning with 5G ProSe capable UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning with 5G ProSe capable UE supported |  |
|  | Access technology utilization control (ATUC) (octet 11, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the support of access technology utilization control. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | access technology utilization control not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | access technology utilization control supported |  |
|  | LP-WUSPS assistance information support indication (LP-WUSPSAI-SI) (octet 11, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the support of LP-WUSPS assistance information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LP-WUSPS assistance information not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LP-WUSPS assistance information supported |  |
|  | Operator policy for high priority access exemptions for service area restrictions (OPHPAE) (octet 12, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support operator policy for high priority access exemptions for service area restrictions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Operator policy for high priority access exemption for service area restrictions is not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Operator policy for high priority access exemption for service area restrictions is supported |  |
|  | 5G ProSe Multi-hop relay communication indication (5G ProSe-MCI) (octet 12, bit 2) (see NOTE 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the support of multi-hop relay communication capability. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multi-hop relay communication not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multi-hop relay communication supported |  |
|  | Multiple LCS secured user plane connections (MLCSUP) (octet 12, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support multiple LCS secured user plane connections (see 3GPP TS 23.273 [6B]). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multiple LCS secured user plane connections not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multiple LCS secured user plane connections supported |  |
|  | 5G ProSe layer-3 intermediate UE-to-network relay UE (5G ProSe-L3IMrelay) (octet 12, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to act as a 5G ProSe layer-3 intermediate UE-to-network relay UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 intermediate UE-to-network relay UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 intermediate UE-to-network relay UE supported |  |
|  | 5G ProSe layer-2 intermediate UE-to-network relay UE (5G ProSe-L2IMrelay) (octet 12, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to act as a 5G ProSe layer-2 intermediate UE-to-network relay UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 intermediate UE-to-network relay UE not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 intermediate UE-to-network relay UE supported |  |
|  | Minimization of service interruption support in EPS (MINT-EPS) (octet 12, bit 6) This bit indicates the capability to support MINT-EPS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MINT-EPS not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MINT-EPS supported |  |
|  | ExtendedFacility IE for 5G LCS (EF5L) (octet 12, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support ExtendedFacility IE for 5G LCS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ExtendedFacility IE for 5G LCS not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ExtendedFacility IE for 5G LCS supported |  |
|  | LP-WUS disabling (LWD) (octet 12, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This bit indicates the capability to support LP-WUS disabling. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LP-WUS disabling not supported |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LP-WUS disabling supported |  |
|  | Bits in octets 13 to 15 are spare and shall be coded as zero, if the respective octet is included in the information element. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: If both the RSLPVU bit is set to "Ranging and sidelink positioning with V2X capable UE not supported" and the RSLPPU bit is set to "Ranging and sidelink positioning with 5G ProSe capable UE not supported" in the 5GMM capability IE, then the receiving entity shall ignore the RSLPVU bit and the RSLPPU bit. NOTE 2: If the 5G ProSe-MCI bit is set to "Muti-hop relay communication supported", then at least one of the following bits should be included in the 5GMM capability IE and set to 1: "5G ProSe-l2relay", "5G ProSe-l3relay", "5G ProSe-l2rmt", "5G ProSe-l3rmt", "5G ProSe-l3U2U relay", "5G ProSe-l3end", "5G ProSe-L2IMrelay" or "5G ProSe-L3IMrelay". The UE supporting only multi-hop UE-to-network relay or multi-hop UE-to-UE relay as specified in 3GPP TS 24.554 [19E] shall set the 5G ProSe-MCI bit to "Multi-hop relay communication supported" along with the corresponding 5GMM capability IE(s). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 9.11.3.2	5GMM cause

The purpose of the 5GMM cause information element is to indicate the reason why a 5GMM request is rejected.

The 5GMM cause information element is coded as shown in figure 9.11.3.2.1 and table 9.11.3.2.1.

The 5GMM cause is a type 3 information element with length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GMM cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 9.11.3.2.1: 5GMM cause information element

Table 9.11.3.2.1: 5GMM cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | Illegal UE |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |  | PEI not accepted |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | Illegal ME |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | 5GS services not allowed |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | UE identity cannot be derived by the network |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |  | Implicitly de-registered |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |  | PLMN not allowed |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |  | Tracking area not allowed |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |  | Roaming not allowed in this tracking area |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |  | No suitable cells in tracking area |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |  | MAC failure |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |  | Synch failure |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |  | Congestion |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |  | UE security capabilities mismatch |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |  | Security mode rejected, unspecified |
| 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |  | Non-5G authentication unacceptable |
| 0 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |  | N1 mode not allowed |
| 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 |  | Restricted service area |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |  | Redirection to EPC required |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |  | IAB-node operation not authorized |
| 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |  | LADN not available |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 |  | No network slices available |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Maximum number of PDU sessions reached |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | Insufficient resources for specific slice and DNN |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | Insufficient resources for specific slice |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  | ngKSI already in use |
| 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 |  | Non-3GPP access to 5GCN not allowed |
| 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |  | Serving network not authorized |
| 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |  | Temporarily not authorized for this SNPN |
| 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 |  | Permanently not authorized for this SNPN |
| 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |  | Not authorized for this CAG or authorized for CAG cells only |
| 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |  | Wireline access area not allowed |
| 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |  | PLMN not allowed to operate at the present UE location |
| 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |  | UAS services not allowed |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |  | Disaster roaming for the determined PLMN with disaster condition not allowed |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  | Selected N3IWF is not compatible with the allowed NSSAI |
| 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |  | Selected TNGF is not compatible with the allowed NSSAI |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |  | Payload was not forwarded |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |  | DNN not supported or not subscribed in the slice |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |  | Insufficient user-plane resources for the PDU session |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |  | Onboarding services terminated |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |  | User plane positioning not authorized |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |  | Semantically incorrect message |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |  | Invalid mandatory information |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |  | Message type non-existent or not implemented |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |  | Message type not compatible with the protocol state |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |  | Information element non-existent or not implemented |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |  | Conditional IE error |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |  | Message not compatible with the protocol state |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Protocol error, unspecified |
| Any other value received by the mobile station shall be treated as 0110 1111, "protocol error, unspecified". Any other value received by the network shall be treated as 0110 1111, "protocol error, unspecified". |  |  |  |  |  |  |  |  |  |

#### 9.11.3.2A	5GS DRX parameters

The purpose of the 5GS DRX parameters information element is to indicate that the UE wants to use DRX and for the network to indicate the DRX cycle value to be used at paging.

The 5GS DRX parameters is a type 4 information element with a length of 3 octets.

The 5GS DRX parameters information element is coded as shown in figure 9.11.3.2A.1 and table 9.11.3.2A.1.

The value part of a DRX parameter information element is coded as shown in table 9.11.3.2A.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS DRX parameters IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS DRX parameters contents |  |  |  |  |  |  |  | octet 2 |
| 0 | 0 | 0 | 0 | DRX value |  |  |  |  |
| spare |  |  |  |  |  |  |  | octet 3 |

Figure 9.11.3.2A.1: 5GS DRX parameters information element

Table 9.11.3.2A.1: 5GS DRX parameters information element

| DRX value (bits 4 to 1 of octet 3) This field represents the DRX cycle parameter 'T' as defined in 3GPP TS 38.304 [28] or 3GPP TS 36.304 [25C]. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | DRX value not specified |
| 0 | 0 | 0 | 1 | DRX cycle parameter T = 32 |
| 0 | 0 | 1 | 0 | DRX cycle parameter T = 64 |
| 0 | 0 | 1 | 1 | DRX cycle parameter T = 128 |
| 0 | 1 | 0 | 0 | DRX cycle parameter T = 256 |
| All other values shall be interpreted as "DRX value not specified" by this version of the protocol. Bits 5 to 8 of octet 3 are spare and shall be coded as zero. |  |  |  |  |

#### 9.11.3.3	5GS identity type

The purpose of the 5GS identity type information element is to specify which identity is requested.

The 5GS identity type is a type 1 information element.

The 5GS identity type information element is coded as shown in figure 9.11.3.3.1 and table 9.11.3.3.1.

| 8 | 7 | 6 | 5 |  | 4 | 3 |  | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS identity type IEI |  |  |  | 0 spare |  |  | Type of identity |  |  | octet 1 |

Figure 9.11.3.3.1: 5GS identity type information element

Table 9.11.3.3.1: 5GS identity type information element

| Type of identity (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 1 |  | SUCI |
| 0 | 1 | 0 |  | 5G-GUTI |
| 0 | 1 | 1 |  | IMEI |
| 1 | 0 | 0 |  | 5G-S-TMSI |
| 1 | 0 | 1 |  | IMEISV |
| 1 | 1 | 0 |  | MAC address |
| 1 | 1 | 1 |  | EUI-64 |
| All other values are unused and shall be interpreted as "SUCI", if received by the UE. |  |  |  |  |

#### 9.11.3.4	5GS mobile identity

The purpose of the 5GS mobile identity information element is to provide either the SUCI, the 5G-GUTI, the IMEI, the IMEISV, the 5G-S-TMSI, the MAC address or the EUI-64.

The 5GS mobile identity information element is coded as shown in figures 9.11.3.4.1, 9.11.3.4.2, 9.11.3.4.3, 9.11.3.4.4, 9.11.3.4.5, 9.11.3.4.6, 9.11.3.4.8 and 9.11.3.4.7, and table 9.11.3.4.1.

The 5GS mobile identity is a type 6 information element with a minimum length of 4.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 1 | 1 | 1 | 1 | 0 spare | Type of identity |  |  | octet 4 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 5 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 6 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 7 |
| AMF Region ID |  |  |  |  |  |  |  | octet 8 |
| AMF Set ID |  |  |  |  |  |  |  | octet 9 |
| AMF Set ID (continued) |  | AMF Pointer |  |  |  |  |  | octet 10 |
| 5G-TMSI |  |  |  |  |  |  |  | octet 11 |
| 5G-TMSI (continued) |  |  |  |  |  |  |  | octet 12 |
| 5G-TMSI (continued) |  |  |  |  |  |  |  | octet 13 |
| 5G-TMSI (continued) |  |  |  |  |  |  |  | octet 14 |

Figure 9.11.3.4.1: 5GS mobile identity information element for type of identity "5G-GUTI"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Identity digit 1 |  |  |  | Odd/even indication | Type of identity |  |  | octet 4 |
| Identity digit p+1 |  |  |  | Identity digit p |  |  |  | octet 5* |

Figure 9.11.3.4.2: 5GS mobile identity information element for type of identity "IMEI" or "IMEISV"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 0 spare | SUPI format |  |  | 0 spare | Type of identity |  |  | octet 4 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 5 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 6 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 7 |
| Routing indicator digit 2 |  |  |  | Routing indicator digit 1 |  |  |  | octet 8 |
| Routing indicator digit 4 |  |  |  | Routing indicator digit 3 |  |  |  | octet 9 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | Protection scheme Id |  |  |  | octet 10 |
| Home network public key identifier |  |  |  |  |  |  |  | octet 11 |
| Scheme output |  |  |  |  |  |  |  | octet 12 - x |

Figure 9.11.3.4.3: 5GS mobile identity information element for type of identity "SUCI" and SUPI format "IMSI"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSIN digit 2 |  |  |  | MSIN digit 1 |  |  |  | octet 12 |
| … |  |  |  |  |  |  |  |  |
| MSIN digit n+1 |  |  |  | MSIN digit n |  |  |  | octet x |

Figure 9.11.3.4.3a: Scheme output for type of identity "SUCI", SUPI format "IMSI" and Protection scheme Id "Null scheme"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 0 Spare | SUPI format |  |  | 0 Spare | Type of identity |  |  | octet 4 |
| SUCI NAI |  |  |  |  |  |  |  | octet 5 - y |

Figure 9.11.3.4.4: 5GS mobile identity information element for type of identity "SUCI" and SUPI format "Network specific identifier", "GCI" or "GLI"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 1 | 1 | 1 | 1 | 0 spare | Type of identity |  |  | octet 4 |
| AMF Set ID |  |  |  |  |  |  |  | octet 5 |
| AMF Set ID (continued) |  | AMF Pointer |  |  |  |  |  | octet 6 |
| 5G-TMSI |  |  |  |  |  |  |  | octet 7 |
| 5G-TMSI (continued) |  |  |  |  |  |  |  | octet 8 |
| 5G-TMSI (continued) |  |  |  |  |  |  |  | octet 9 |
| 5G-TMSI (continued) |  |  |  |  |  |  |  | octet 10 |

Figure 9.11.3.4.5: 5GS mobile identity information element for type of identity "5G-S-TMSI"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 0 | 0 | 0 | 0 | 0 | Type of identity |  |  | octet 4 |
| spare |  |  |  |  |  |  |  |  |

Figure 9.11.3.4.6: 5GS mobile identity information element for type of identity "No identity"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 0 spare | 0 spare | 0 spare | 0 spare | MAURI | Type of identity |  |  | octet 4 |
| MAC address |  |  |  |  |  |  |  | octet 5 octet 10 |

Figure 9.11.3.4.7: 5GS mobile identity information element for type of identity "MAC address"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | Type of identity |  |  | octet 4 |
| EUI-64 |  |  |  |  |  |  |  | octet 5 octet 12 |

Figure 9.11.3.4.8: 5GS mobile identity information element for type of identity "EUI-64"

Table 9.11.3.4.1: 5GS mobile identity information element

| Type of identity (octet 4 bits 1 to 3) Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 2 | 1 |  |  |  |  |  |  |  |
| 0 | 0 | 0 | No identity (see NOTE 1) |  |  |  |  |  |  |
| 0 | 0 | 1 | SUCI |  |  |  |  |  |  |
| 0 | 1 | 0 | 5G-GUTI |  |  |  |  |  |  |
| 0 | 1 | 1 | IMEI |  |  |  |  |  |  |
| 1 | 0 | 0 | 5G-S-TMSI |  |  |  |  |  |  |
| 1 | 0 | 1 | IMEISV |  |  |  |  |  |  |
| 1 | 1 | 0 | MAC address |  |  |  |  |  |  |
| 1 | 1 | 1 | EUI-64 |  |  |  |  |  |  |
| Odd/even indication (octet 4 bit 4) Bit |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |
| 0 |  |  | even number of identity digits |  |  |  |  |  |  |
| 1 |  |  | odd number of identity digits |  |  |  |  |  |  |
| For the 5G-GUTI, then bits 5 to 8 of octet 4 are coded as "1111", octet 5 through 7 contain the MCC and MNC values as specified below, octet 8 through 10 contain the AMF Region ID, the AMF Set ID and the AMF Pointer values and octet 11 through 14 contain the 5G-TMSI as defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |
| MCC, Mobile country code (octet 5, octet 6 bits 1 to 4) The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A. |  |  |  |  |  |  |  |  |  |
| MNC, Mobile network code (octet 6 bits 5 to 8, octet 7) The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet 6 shall be coded as "1111". The MCC and MNC digits are coded as octets 6 to 8 of the Temporary mobile group identity IE in figure 10.5.154 of 3GPP TS 24.008 [12]. |  |  |  |  |  |  |  |  |  |
| AMF Region ID (octet 8) This field contains the binary encoding of the AMF Region ID. Bit 8 of octet 7 is the most significant bit and bit 1 of octet 7 is the least significant bit. AMF Set ID (octet 9, octet 10 bits 7 to 8) This field contains the binary encoding of the AMF Set ID. Bit 8 of octet 9 is the most significant bit and bit 7 of octet 10 is the least significant bit. AMF Pointer (octet 10 bits 1 to 6) This field contains the binary encoding of the AMF Pointer. Bit 6 of octet 9 is the most significant bit and bit 1 of octet 9 is the least significant bit. 5G-TMSI (octet 11 to 14) Bit 8 of octet 11 is the most significant bit and bit 1 of octet 14 is the least significant bit. |  |  |  |  |  |  |  |  |  |
| Identity digit (octet 4 bits 5 to 8, octet 5 etc.) |  |  |  |  |  |  |  |  |  |
| For the IMEI, Identity digit field is coded using BCD coding. If the number of identity digits is even then bits 5 to 8 of the last octet shall be filled with an end mark coded as "1111". The format of the IMEI is described in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |
| For the IMEISV, Identity digit field is coded using BCD coding. Bits 5 to 8 of the last octet shall be filled with an end mark coded as "1111". The format of the IMEISV is described in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |
| For the SUCI, bit 8 of octet 4 is spare and shall be coded as zero. Bits 5-7 of octet 4 contain the SUPI format and are coded as shown below. |  |  |  |  |  |  |  |  |  |
| SUPI format (octet 4, bits 5-7) Bits |  |  |  |  |  |  |  |  |  |
| 7 | 6 | 5 |  |  |  |  |  |  |  |
| 0 | 0 | 0 |  | IMSI |  |  |  |  |  |
| 0 | 0 | 1 |  | Network specific identifier |  |  |  |  |  |
| 0 | 1 | 0 |  | GCI |  |  |  |  |  |
| 0 | 1 | 1 |  | GLI |  |  |  |  |  |
| All other values are interpreted as IMSI by this version of the protocol. |  |  |  |  |  |  |  |  |  |
| For the SUCI with SUPI format "IMSI", octets 5 through 7 contain the MCC and MNC values as specified below. For subsequent fields, bit 8 of octet 8 is the most significant bit and bit 1 of the last octet the least significant bit. The required fields for the SUCI are as defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |
| MCC, Mobile country code (octet 5, octet 6 bits 1 to 4) The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A. |  |  |  |  |  |  |  |  |  |
| MNC, Mobile network code (octet 6 bits 5 to 8, octet 7) The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet 6 shall be coded as "1111". The MCC and MNC digits are coded as octets 6 to 8 of the Temporary mobile group identity IE in figure 10.5.154 of 3GPP TS 24.008 [12]. |  |  |  |  |  |  |  |  |  |
| Routing indicator (octets 8-9) Routing Indicator shall consist of 1 to 4 digits. The coding of this field is the responsibility of home network operator but BCD coding shall be used. If a network operator decides to assign less than 4 digits to Routing Indicator, the remaining digits shall be coded as "1111" to fill the 4 digits coding of Routing Indicator (see NOTE 2). If no Routing Indicator is configured in the USIM or the ME, the UE shall code bits 1 to 4 of octet 8 of the Routing Indicator as "0000" and the remaining digits as "1111". |  |  |  |  |  |  |  |  |  |
| Protection scheme identifier (octet 10 bits 1 to 4) |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 4 | 3 | 2 | 1 |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | Null scheme |  |  |  |  |  |
| 0 | 0 | 0 | 1 | ECIES scheme profile A |  |  |  |  |  |
| 0 | 0 | 1 | 0 | ECIES scheme profile B |  |  |  |  |  |
| 0 | 0 | 1 | 1 |  |  |  |  |  |  |
| to |  |  |  | Reserved |  |  |  |  |  |
| 1 | 0 | 1 | 1 |  |  |  |  |  |  |
| 1 | 1 | 0 | 0 |  |  |  |  |  |  |
| to |  |  |  | Operator-specific protection scheme |  |  |  |  |  |
| 1 | 1 | 1 | 1 |  |  |  |  |  |  |
| Bits 5-8 of octet 10 are spare and shall be coded as zero. |  |  |  |  |  |  |  |  |  |
| Home network public key identifier (octet 11) |  |  |  |  |  |  |  |  |  |
| The Home network public key identifier (PKI) field is coded as defined in 3GPP TS 23.003 [4]. Home network public key identifier shall be coded as "00000000" when Protection scheme identifier is set to "0000" (i.e. Null scheme). |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | Home network PKI value 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  |  |
| to |  |  |  |  |  |  |  |  | Home network PKI value (1-254) |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |  |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | Reserved |
| Scheme output (octets 12 to x) The Scheme output field consists of a string of characters with a variable length or hexadecimal digits as specified in 3GPP TS 23.003 [4]. If Protection scheme identifier is set to "0000" (i.e. Null scheme), then the Scheme output consists of the MSIN and is coded using BCD coding with each digit of the MSIN coded over 4 bits. If the MSIN includes an odd number of digits, bits 5 to 8 of octet x shall be coded as "1111". If Protection scheme identifier is not "0000" (i.e. ECIES scheme profile A, ECIES scheme profile B or Operator-specific protection scheme), then Scheme output is coded as hexadecimal digits. |  |  |  |  |  |  |  |  |  |
| For the SUCI with SUPI format set to "Network specific identifier", the SUCI NAI field contains an NAI constructed as specified in clause 28.7.3 of 3GPP TS 23.003 [4] and encoded as UTF-8 string. |  |  |  |  |  |  |  |  |  |
| For the SUCI with SUPI format set to "GCI", the SUCI NAI field contains an NAI constructed as specified in clause 28.15.5 of 3GPP TS 23.003 [4] and encoded as UTF-8 string. |  |  |  |  |  |  |  |  |  |
| For the SUCI with SUPI format set to "GLI", the SUCI NAI field contains an NAI constructed as specified in clause 28.16.5 of 3GPP TS 23.003 [4] and encoded as UTF-8 string. |  |  |  |  |  |  |  |  |  |
| For the 5G-S-TMSI, bits 5 to 8 of octet 4 are coded as "1111". The coding of the 5G-S-TMSI is left open for each administration. |  |  |  |  |  |  |  |  |  |
| AMF Set ID (octet 5, octet 6 bits 7 to 8) This field contains the binary encoding of the AMF Set ID. Bit 8 of octet 5 is the most significant bit and bit 7 of octet 6 is the least significant bit. AMF Pointer (octet 6 bits 1 to 6) This field contains the binary encoding of the AMF Pointer. Bit 6 of octet 6 is the most significant bit and bit 1 of octet 6 is the least significant bit. 5G-TMSI (octet 7 to 10) Bit 8 of octet 7 is the most significant bit and bit 1 of octet 10 is the least significant bit. |  |  |  |  |  |  |  |  |  |
| For Type of identity "No identity", the length of mobile identity contents parameter shall be set to 1 and the bits 4-8 of octet 4 are spare and shall be coded as zero. |  |  |  |  |  |  |  |  |  |
| MAC address usage restriction indication (MAURI) (octet 4 bit 4) |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  | No restrictions |  |  |  |  |  |
| 1 |  |  |  | MAC address is not usable as an equipment identifier |  |  |  |  |  |
| MAC address (octets 5 to 10) This field contains the MAC address as defined in clause 8 of IEEE Std 802 [43]. Bit 8 of octet 5 is the most significant bit and bit 1 of octet 10 is the least significant bit. |  |  |  |  |  |  |  |  |  |
| EUI-64 (octets 5 to 12) This field contains an EUI-64 as defined in [48]. Bit 8 of octet 5 is the most significant bit and bit 1 of octet 12 is the least significant bit. |  |  |  |  |  |  |  |  |  |
| NOTE 1: This can be used when the requested identity is not available at the UE during the identification procedure. NOTE 2: For a 3-digit Routing Indicator, e.g "567", bits 1 to 4 of octet 8 are coded as "0101", bits 5 to 8 of octet 8 are coded as "0110", bits 1 to 4 of octet 9 are coded as "0111", bits 5 to 8 of octet 9 are coded as "1111". |  |  |  |  |  |  |  |  |  |

#### 9.11.3.5	5GS network feature support

The purpose of the 5GS network feature support information element is to indicate whether certain features are supported by the network.

The 5GS network feature support information element is coded as shown in figure 9.11.3.5.1 and table 9.11.3.5.1.

The 5GS network feature support is a type 4 information element with a minimum length of 3 octets and a maximum length of 6 octets.

If:

-	the length of 5GS network feature support contents field is set to one, then the UE shall interpret this as a receipt of an information element with all bits of octet 4, octet 5 and octet 6 coded as zero.

-	the length of 5GS network feature support contents field is set to two, the UE shall interpret this as a receipt of an information element with all bits of octet 5 and octet 6 coded as zero.

-	the length of 5GS network feature support contents field is set to three, the UE shall interpret this as a receipt of an information element with all bits of octet 6 coded as zero.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS network feature support IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS network feature support contents |  |  |  |  |  |  |  | octet 2 |
| MPSI | IWK N26 | EMF |  | EMC |  | IMS- VoPS-N3GPP | IMS- VoPS-3GPP | octet 3 |
| 5G-UP CIoT | 5G-IPHC-CP CIoT | N3 data | 5G-CP CIoT | RestrictEC |  | MCSI | EMCN3 | octet 4* |
| UN-PER | PR | RPR | PIV | NCR | 5G-EHC-CP CIoT | ATS-IND | 5G-LCS | octet 5* |
| 0 spare | 0 spare | EF5L | MLCSUP | RSLP | SUPL | LCS-UPP | NAPS | octet 6* |

Figure 9.11.3.5.1: 5GS network feature support information element

Table 9.11.3.5.1: 5GS network feature support information element

| IMS voice over PS session over 3GPP access indicator (IMS-VoPS-3GPP) (octet 3, bit 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| This bit indicates the support of IMS voice over PS session over 3GPP access (see NOTE 1). |  |  |  |  |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | IMS voice over PS session not supported over 3GPP access |
| 1 |  |  |  | IMS voice over PS session supported over 3GPP access |
| IMS voice over PS session over non-3GPP access indicator (IMS-VoPS-N3GPP) (octet 3, bit 2) |  |  |  |  |
| This bit indicates the support of IMS voice over PS session over non-3GPP access. |  |  |  |  |
| Bit |  |  |  |  |
| 2 |  |  |  |  |
| 0 |  |  |  | IMS voice over PS session not supported over non-3GPP access |
| 1 |  |  |  | IMS voice over PS session supported over non-3GPP access |
| Emergency service support indicator for 3GPP access (EMC) (octet 3, bit 3 and bit 4) |  |  |  |  |
| These bits indicate the support of emergency services in 5GS for 3GPP access (see NOTE 1). |  |  |  |  |
| Bits |  |  |  |  |
| 4 | 3 |  |  |  |
| 0 | 0 |  |  | Emergency services not supported |
| 0 | 1 |  |  | Emergency services supported in NR connected to 5GCN only |
| 1 | 0 |  |  | Emergency services supported in E-UTRA connected to 5GCN only |
| 1 | 1 |  |  | Emergency services supported in NR connected to 5GCN and E-UTRA connected to 5GCN |
| Emergency services fallback indicator for 3GPP access (EMF) (octet 3, bit 5 and bit 6) |  |  |  |  |
| These bits indicate the support of emergency services fallback for 3GPP access (see NOTE 1). |  |  |  |  |
| Bits |  |  |  |  |
| 6 | 5 |  |  |  |
| 0 | 0 |  |  | Emergency services fallback not supported |
| 0 | 1 |  |  | Emergency services fallback supported in NR connected to 5GCN only |
| 1 | 0 |  |  | Emergency services fallback supported in E-UTRA connected to 5GCN only |
| 1 | 1 |  |  | Emergency services fallback supported in NR connected to 5GCN and E-UTRA connected to 5GCN |
| Interworking without N26 interface indicator (IWK N26) (octet 3, bit 7) |  |  |  |  |
| This bit indicates whether interworking without N26 interface is supported. |  |  |  |  |
| Bit |  |  |  |  |
| 7 |  |  |  |  |
| 0 |  |  |  | Interworking without N26 interface not supported |
| 1 |  |  |  | Interworking without N26 interface supported |
| MPS indicator (MPSI) (octet 3, bit 8) |  |  |  |  |
| This bit indicates the validity of MPS. |  |  |  |  |
| Bit |  |  |  |  |
| 8 |  |  |  |  |
| 0 |  |  |  | Access identity 1 not valid |
| 1 |  |  |  | Access identity 1 valid |
| Emergency service support for non-3GPP access indicator (EMCN3) (octet 4, bit 1) |  |  |  |  |
| This bit indicates the support of emergency services in 5GS for non-3GPP access. |  |  |  |  |
| Bit (see NOTE 2) |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | Emergency services not supported over non-3GPP access |
| 1 |  |  |  | Emergency services supported over non-3GPP access |
| MCS indicator (MCSI) (octet 4, bit 2) |  |  |  |  |
| This bit indicates the validity of MCS. |  |  |  |  |
| Bit |  |  |  |  |
| 2 |  |  |  |  |
| 0 |  |  |  | Access identity 2 not valid |
| 1 |  |  |  | Access identity 2 valid |
| Restriction on enhanced coverage (RestrictEC) (octet 4, bit 3 and bit 4) These bits indicate enhanced coverage restricted information. |  |  |  |  |
| In WB-N1 mode these bits are set as follows: Bits |  |  |  |  |
| 4 | 3 |  |  |  |
| 0 | 0 |  |  | Both CE mode A and CE mode B are not restricted |
| 0 | 1 |  |  | Both CE mode A and CE mode B are restricted |
| 1 | 0 |  |  | CE mode B is restricted |
| 1 | 1 |  |  | Reserved |
| In NB-N1 mode these bits are set as follows |  |  |  |  |
| Bits |  |  |  |  |
| 4 | 3 |  |  |  |
| 0 | 0 |  |  | Use of enhanced coverage is not restricted |
| 0 | 1 |  |  | Use of enhanced coverage is restricted |
| 1 | 0 |  |  | Reserved |
| 1 | 1 |  |  | Reserved |
| Control plane CIoT 5GS optimization (5G-CP CIoT) (octet 4, bit 5) |  |  |  |  |
| This bit indicates the capability for control plane CIoT 5GS optimization. |  |  |  |  |
| Bit |  |  |  |  |
| 5 |  |  |  |  |
| 0 |  |  |  | Control plane CIoT 5GS optimization not supported |
| 1 |  |  |  | Control plane CIoT 5GS optimization supported |
| N3 data transfer (N3 data) (octet 4, bit 6) |  |  |  |  |
| This bit indicates the capability for N3 data transfer. |  |  |  |  |
| Bit |  |  |  |  |
| 6 |  |  |  |  |
| 0 |  |  |  | N3 data transfer supported |
| 1 |  |  |  | N3 data transfer not supported |
| IP header compression for control plane CIoT 5GS optimization (5G-IPHC-CP CIoT) (octet 4, bit 7) |  |  |  |  |
| This bit indicates the capability for IP header compression for control plane CIoT 5GS optimization. |  |  |  |  |
| Bit |  |  |  |  |
| 7 |  |  |  |  |
| 0 |  |  |  | IP header compression for control plane CIoT 5GS optimization not supported |
| 1 |  |  |  | IP header compression for control plane CIoT 5GS optimization supported |
| User plane CIoT 5GS optimization (5G-UP CIoT) (octet 4, bit 8) |  |  |  |  |
| This bit indicates the capability for user plane CIoT 5GS optimization. |  |  |  |  |
| Bit |  |  |  |  |
| 8 |  |  |  |  |
| 0 |  |  |  | User plane CIoT 5GS optimization not supported |
| 1 |  |  |  | User plane CIoT 5GS optimization supported |
| Location Services indicator in 5GC (5G-LCS) (octet 5, bit 1) |  |  |  |  |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | Location services via 5GC not supported |
| 1 |  |  |  | Location services via 5GC supported |
| ATSSS support indicator (ATS-IND) (octet 5, bit 2) |  |  |  |  |
| This bit indicates the network support for ATSSS. |  |  |  |  |
| Bit |  |  |  |  |
| 2 |  |  |  |  |
| 0 |  |  |  | ATSSS not supported |
| 1 |  |  |  | ATSSS supported |
| Ethernet header compression for control plane CIoT 5GS optimization (5G-EHC-CP CIoT) (octet 5, bit 3) |  |  |  |  |
| This bit indicates the capability for Ethernet header compression for control plane CIoT 5GS optimization |  |  |  |  |
| Bit |  |  |  |  |
| 3 |  |  |  |  |
| 0 |  |  |  | Ethernet header compression for control plane CIoT 5GS optimization not supported |
| 1 |  |  |  | Ethernet header compression for control plane CIoT 5GS optimization supported |
| N1 NAS signalling connection release (NCR) (octet 5, bit 4) |  |  |  |  |
| This bit indicates whether N1 NAS signalling connection release is supported. |  |  |  |  |
| Bit |  |  |  |  |
| 4 |  |  |  |  |
| 0 |  |  |  | N1-NAS signalling connection release not supported |
| 1 |  |  |  | N1-NAS signalling connection release supported |
| Paging indication for voice services (PIV) (octet 5, bit 5) |  |  |  |  |
| This bit indicates whether paging indication for voice services is supported. |  |  |  |  |
| Bit |  |  |  |  |
| 5 |  |  |  |  |
| 0 |  |  |  | paging indication for voice services not supported |
| 1 |  |  |  | paging indication for voice services supported |
| Reject paging request (RPR) (octet 5, bit 6) |  |  |  |  |
| This bit indicates whether reject paging request is supported. |  |  |  |  |
| Bit |  |  |  |  |
| 6 |  |  |  |  |
| 0 |  |  |  | reject paging request not supported |
| 1 |  |  |  | reject paging request supported |
| Paging restriction (PR) (octet 5, bit 7) |  |  |  |  |
| This bit indicates whether paging restriction is supported. |  |  |  |  |
| Bit |  |  |  |  |
| 7 |  |  |  |  |
| 0 |  |  |  | paging restriction not supported |
| 1 |  |  |  | paging restriction supported |
| UN-PER (octet 5, bit 8) |  |  |  |  |
| This bit indicates the capability to support Unavailability period |  |  |  |  |
| Bit |  |  |  |  |
| 8 |  |  |  |  |
| 0; unavailability period not supported. 1; unavailability period supported. |
| Non-3GPP access path switching (NAPS) (octet 6, bit 1) |  |  |  |  |
| This bit indicates whether non-3GPP access path switching is supported. |  |  |  |  |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | non-3GPP access path switching not supported |
| 1 |  |  |  | non-3GPP access path switching supported |
| LCS-UPP user plane positioning (LCS-UPP) (octet 6, bit 2) |  |  |  |  |
| This bit indicates the capability to support LCS-UPP user plane positioning (see 3GPP TS 23.273 [6B]). |  |  |  |  |
| Bit |  |  |  |  |
| 2 |  |  |  |  |
| 0 |  |  |  | User plane positioning using LCS-UPP not supported |
| 1 |  |  |  | User plane positioning using LCS-UPP supported |
| SUPL user plane positioning (SUPL) (octet 6, bit 3) |  |  |  |  |
| This bit indicates the capability to support SUPL user plane positioning (see 3GPP TS 38.305 [67] and 3GPP TS 23.271 [68]). |  |  |  |  |
| Bit |  |  |  |  |
| 3 |  |  |  |  |
| 0 |  |  |  | User plane positioning using SUPL not supported |
| 1 |  |  |  | User plane positioning using SUPL supported |
| Ranging and sidelink positioning support (RSLP) (octet 6, bit 4) |  |  |  |  |
| This bit indicates the capability to support ranging and sidelink positioning. |  |  |  |  |
| Bit |  |  |  |  |
| 4 |  |  |  |  |
| 0 |  |  |  | Ranging and sidelink positioning not supported |
| 1 |  |  |  | Ranging and sidelink positioning supported |
| Multiple LCS secured user plane connections (MLCSUP) (octet 6, bit 5) |  |  |  |  |
| This bit indicates the capability to support multiple LCS secured user plane connections. |  |  |  |  |
| Bit |  |  |  |  |
| 5 |  |  |  |  |
| 0 |  |  |  | Multiple LCS secured user plane connections not supported |
| 1 |  |  |  | Multiple LCS secured user plane connections supported |
| ExtendedFacility IE for 5G LCS (EF5L) (octet 6, bit 6) |  |  |  |  |
| This bit indicates the capability to support ExtendedFacility IE for 5G LCS. |  |  |  |  |
| Bit |  |  |  |  |
| 6 |  |  |  |  |
| 0 |  |  |  | ExtendedFacility IE for 5G LCS not supported |
| 1 |  |  |  | ExtendedFacility IE for 5G LCS supported |
| Bits 7 to 8 of octet 6 are spare and shall be coded as zero. |  |  |  |  |
| NOTE 1: For a registration procedure over non-3GPP access, bit 1 of octet 3 and bits 3 to 7 of octet 3 are ignored. NOTE 2: For a registration procedure over 3GPP access, bit 2 of octet 3 and bit 1 of octet 4 are ignored. |  |  |  |  |

#### 9.11.3.6	5GS registration result

The purpose of the 5GS registration result information element is to specify the result of a registration procedure.

The 5GS registration result information element is coded as shown in figure 9.11.3.6.1 and table 9.11.3.6.1.

The 5GS registration result is a type 4 information element with a length of 3 octets.

|  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS registration result IEI |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |  |
| Length of 5GS registration result contents |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |  |
| 0 Spare |  | Disaster roaming registration result value |  | Emergency registered |  | NSSAA Performed |  | SMS allowed |  | 5GS registration result value |  |  |  | octet 3 |  |  |

Figure 9.11.3.6.1: 5GS registration result information element

Table 9.11.3.6.1: 5GS registration result information element

| 5GS registration result value (octet 3, bits 1 to 3) (NOTE) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |
| 3 |  | 2 | 1 |  |  |  |
| 0 |  | 0 | 1 |  | 3GPP access |  |
| 0 |  | 1 | 0 |  | Non-3GPP access |  |
| 0 |  | 1 | 1 |  | 3GPP access and non-3GPP access |  |
| 1 |  | 1 | 1 |  | reserved |  |
| All other values are unused and shall be treated as "3GPP access", if received by the UE. |  |  |  |  |  |  |
| SMS over NAS transport allowed (SMS allowed) (octet 3, bit 4) (NOTE) |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 0 |  |  |  |  | SMS over NAS not allowed |  |
| 1 |  |  |  |  | SMS over NAS allowed |  |
| Network slice-specific authentication and authorization is to be performed (NSSAA to be performed) (octet 3, bit 5) (NOTE) |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 0 |  |  |  |  | Network slice-specific authentication and authorization is not to be performed |  |
| 1 |  |  |  |  | Network slice-specific authentication and authorization is to be performed |  |
| Emergency registered (octet 3, bit 6) |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 0 |  |  |  |  | Not registered for emergency services |  |
| 1 |  |  |  |  | Registered for emergency services |  |
| Disaster roaming registration result value (octet 3, bit 7) (NOTE) |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 0 |  |  |  |  | No additional information |  |
| 1 |  |  |  |  | Request for registration for disaster roaming service accepted as registration not for disaster roaming service |  |
| Bit 8 of octet 3 is spare and shall be coded as zero. |  |  |  |  |  |  |
|  | NOTE: All bits other than bit 6 in octet 3 shall be ignored by the UE when the 5GS registration result IE is received in the CONFIGURATION UPDATE COMMAND message |  |  |  |  |  |

#### 9.11.3.7	5GS registration type

The purpose of the 5GS registration type information element is to indicate the type of the requested registration.

The 5GS registration type information element is coded as shown in figure 9.11.3.7.1 and table 9.11.3.7.1.

The 5GS registration type is a type 1 information element with a length of 1 octet.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS registration type IEI |  |  |  | FOR | 5GS registration type value |  |  | octet 1 |

Figure 9.11.3.7.1: 5GS registration type information element

Table 9.11.3.7.1: 5GS registration type information element

| 5GS registration type value (octet 1, bits 1 to 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 1 |  | initial registration |
| 0 | 1 | 0 |  | mobility registration updating |
| 0 | 1 | 1 |  | periodic registration updating |
| 1 | 0 | 0 |  | emergency registration |
| 1 | 0 | 1 |  | SNPN onboarding registration |
| 1 | 1 | 0 |  | disaster roaming mobility registration updating |
| 1 | 1 | 1 |  | disaster roaming initial registration |
| All other values are unused and shall be interpreted as "initial registration", if received by the network. |  |  |  |  |
| Follow-on request bit (FOR) (octet 1, bit 4) |  |  |  |  |
| Bit |  |  |  |  |
| 4 |  |  |  |  |
| 0 |  |  |  | No follow-on request pending |
| 1 |  |  |  | Follow-on request pending |

#### 9.11.3.8	5GS tracking area identity

The purpose of the 5GS tracking area identity information element is to provide an unambiguous identification of tracking areas within the area covered by the 5GS.

The 5GS tracking area identity information element is coded as shown in figure 9.11.3.8.1 and table 9.11.3.8.1.

The 5GS tracking area identity is a type 3 information element with a length of 7 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS tracking area identity IEI |  |  |  |  |  |  |  | octet 1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 4 |
| TAC |  |  |  |  |  |  |  | octet 5 |
| TAC (continued) |  |  |  |  |  |  |  | octet 6 |
| TAC (continued) |  |  |  |  |  |  |  | octet 7 |

Figure 9.11.3.8.1: 5GS tracking area identity information element

Table 9.11.3.8.1: 5GS tracking area identity information element

MCC, Mobile country code (octets 2 and 3) The MCC field is coded as in ITU-T Recommendation E212 [42], annex A. If the TAI is deleted the MCC and MNC shall take the value from the deleted TAI. In abnormal cases, the MCC stored in the UE can contain elements not in the set {0, 1 ... 9}. In such cases the UE should transmit the stored values using full hexadecimal encoding. When receiving such an MCC, the network shall treat the TAI as deleted. MNC, Mobile network code (octet 3 bits 5 to 8, octet 4) The coding of this field is the responsibility of each administration, but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. For PCS 1900 for NA, Federal regulation mandates that a 3-digit MNC shall be used. However, a network operator may decide to use only two digits in the MNC in the TAI over the radio interface. In this case, bits 5 to 8 of octet 3 shall be coded as "1111". Mobile equipment shall accept a TAI coded in such a way. In abnormal cases, the MNC stored in the UE can have: - digit 1 or 2 not in the set {0, 1 ... 9}, or - digit 3 not in the set {0, 1 ... 9, F} hex. In such cases the UE shall transmit the stored values using full hexadecimal encoding. When receiving such an MNC, the network shall treat the TAI as deleted. The same handling shall apply for the network, if a 3-digit MNC is sent by the UE to a network using only a 2-digit MNC. TAC, Tracking area code (octets 5 to 7) In the TAC field bit 8 of octet 5 is the most significant bit and bit 1 of octet 7 the least significant bit. The coding of the tracking area code is the responsibility of each administration except that two values are used to mark the TAC, and hence the TAI, as deleted. Coding using full hexadecimal representation may be used. The tracking area code consists of 3 octets. If a TAI has to be deleted, then all bits of the tracking area code shall be set to one with the exception of the least significant bit which shall be set to zero. If a USIM is inserted in a mobile equipment with the tracking area code containing all zeros, then the mobile equipment shall recognise this TAC as part of a deleted TAI.

#### 9.11.3.9	5GS tracking area identity list

The purpose of the 5GS tracking area identity list information element is to transfer a list of tracking areas from the network to the UE.

The coding of the information element allows combining different types of lists. The lists of type "00" and "01" allow a more compact encoding, when the different TAIs are sharing the PLMN identity.

The 5GS tracking area identity list information element is coded as shown in figure 9.11.3.8.1, figure 9.11.3.8.2, figure 9.11.3.9.3, figure 9.11.3.9.4 and table 9.11.3.9.1.

The 5GS tracking area identity list is a type 4 information element, with a minimum length of 9 octets and a maximum length of 114 octets. The list can contain a maximum of 16 different tracking area identities.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS tracking area identity list IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS tracking area identity list contents |  |  |  |  |  |  |  | octet 2 |
| Partial tracking area identity list 1 |  |  |  |  |  |  |  | octet 3 octet i |
| Partial tracking area identity list 2 |  |  |  |  |  |  |  | octet i+1* octet l* |
| … |  |  |  |  |  |  |  | octet l+1* octet m* |
| Partial tracking area identity list p |  |  |  |  |  |  |  | octet m+1* octet n* |

Figure 9.11.3.9.1: 5GS tracking area identity list information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare | Type of list |  | Number of elements |  |  |  |  | octet 1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 4 |
| TAC 1 |  |  |  |  |  |  |  | octet 5 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 6 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 7 |
| … |  |  |  |  |  |  |  | … |
| … |  |  |  |  |  |  |  | … |
| TAC k |  |  |  |  |  |  |  | octet 3k+2* |
| TAC k (continued) |  |  |  |  |  |  |  | octet 3k+3* |
| TAC k (continued) |  |  |  |  |  |  |  | octet 3k+4* |

Figure 9.11.3.9.2: Partial tracking area identity list – type of list = "00"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare | Type of list |  | Number of elements |  |  |  |  | octet 1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 4 |
| TAC 1 |  |  |  |  |  |  |  | octet 5 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 6 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 7 |

Figure 9.11.3.9.3: Partial tracking area identity list – type of list = "01"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare | Type of list |  | Number of elements |  |  |  |  | octet 1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 4 |
| TAC 1 |  |  |  |  |  |  |  | octet 5 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 6 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 7 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 8* |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 9* |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 10* |
| TAC 2 |  |  |  |  |  |  |  | octet 11* |
| TAC 2 (continued) |  |  |  |  |  |  |  | octet 12* |
| TAC 2 (continued) |  |  |  |  |  |  |  | octet 13* |
| … |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 6k-4* |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 6k-3* |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 6k-2* |
| TAC k |  |  |  |  |  |  |  | octet 6k-1* |
| TAC k (continued) |  |  |  |  |  |  |  | octet 6k* |
| TAC k (continued) |  |  |  |  |  |  |  | octet 6k+1* |

Figure 9.11.3.9.4: Partial tracking area identity list – type of list = "10"

Table 9.11.3.9.1: Tracking area identity list information element

| Value part of the Tracking area identity list information element (octets 3 to n) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The value part of the Tracking area identity list information element consists of one or several partial tracking area identity lists. The length of each partial tracking area identity list can be determined from the 'type of list' field and the 'number of elements' field in the first octet of the partial tracking area identity list. |  |  |  |  |  |
| The UE shall store the complete list received. If more than 16 TAIs are included in this information element, the UE shall store the first 16 TAIs and ignore the remaining octets of the information element. |  |  |  |  |  |
| Partial tracking area identity list: |  |  |  |  |  |
| Type of list (octet 1) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 7 | 6 |  |  |  |  |
| 0 | 0 | list of TACs belonging to one PLMN or SNPN, with non-consecutive TAC values |  |  |  |
| 0 | 1 | list of TACs belonging to one PLMN or SNPN, with consecutive TAC values |  |  |  |
| 1 | 0 | list of TAIs belonging to different PLMNs (see NOTE) |  |  |  |
| All other values are reserved. |  |  |  |  |  |
| Number of elements (octet 1) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | 0 | 1 element |
| 0 | 0 | 0 | 0 | 1 | 2 elements |
| 0 | 0 | 0 | 1 | 0 | 3 elements |
| … |  |  |  |  |  |
| 0 | 1 | 1 | 0 | 1 | 14 elements |
| 0 | 1 | 1 | 1 | 0 | 15 elements |
| 0 | 1 | 1 | 1 | 1 | 16 elements |
| All other values are unused and shall be interpreted as 16, if received by the UE. |  |  |  |  |  |
| Bit 8 of octet 1 is spare and shall be coded as zero. |  |  |  |  |  |
| For type of list = "00" and number of elements = k: |  |  |  |  |  |
| octet 2 to 4 contain the MCC+MNC, and |  |  |  |  |  |
| for j = 1, …, k: |  |  |  |  |  |
| octets 3j+2 to 3j+4 contain the TAC of the j-th TAI belonging to the partial list, |  |  |  |  |  |
| For type of list = "01" and number of elements = k: |  |  |  |  |  |
| octet 2 to 4 contain the MCC+MNC, and |  |  |  |  |  |
| octets 5 to 7 contain the TAC of the first TAI belonging to the partial list. |  |  |  |  |  |
| The TAC values of the other k-1 TAIs are TAC+1, TAC+2, …, TAC+k-1. |  |  |  |  |  |
| For type of list = "10" and number of elements = k: |  |  |  |  |  |
| for j = 1, …, k. |  |  |  |  |  |
| octets 6j-4 to 6j-2 contain the MCC+MNC, and |  |  |  |  |  |
| octets 6j-1 to 6j+1 contain the TAC of the j-th TAI belonging to the partial list. |  |  |  |  |  |
| MCC, Mobile country code |  |  |  |  |  |
| The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A. |  |  |  |  |  |
| MNC, Mobile network code |  |  |  |  |  |
| The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, MNC digit 3 shall be coded as "1111". |  |  |  |  |  |
| TAC, Tracking area code |  |  |  |  |  |
| In the TAC field bit 8 of the first octet is the most significant bit and bit 1 of third octet the least significant bit. |  |  |  |  |  |
| The coding of the tracking area code is the responsibility of each administration. Coding using full hexadecimal representation may be used. The tracking area code consists of 3 octets. |  |  |  |  |  |
| NOTE: If the "list of TAIs belonging to different PLMNs" is used, the PLMNs included in the list need to be present in the list of "equivalent PLMNs". This type of list is not applicable in an SNPN. |  |  |  |  |  |

#### 9.11.3.9A	5GS update type

The purpose of the 5GS update type IE is to allow the UE to provide additional information to the network when performing a registration procedure.

The 5GS update type information element is coded as shown in figure 9.11.3.9A.1 and table 9.11.3.9A.1.

The 5GS update type is a type 4 information element with a length of 3 octects.

|  | 8 |  | 7 |  | 6 | 5 |  | 4 | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS update type IEI |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |  |
| Length of 5GS update type |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |  |
| 0 Spare |  | 0 Spare |  | EPS- PNB-CIoT |  |  | 5GS-PNB-CIoT |  |  | NG-RAN-RCU |  | SMS requested |  | octet 3 |  |  |

Figure 9.11.3.9A.1: 5GS update type information element

Table 9.11.3.9A.1: 5GS update type information element

| SMS over NAS transport requested (SMS requested) (octet 3, bit 1) |  |  |
| --- | --- | --- |
| Bit |  |  |
| 1 |  |  |
| 0 |  | SMS over NAS not supported |
| 1 |  | SMS over NAS supported |
| NG-RAN Radio Capability Update (NG-RAN-RCU) (octet 3, bit 2) |  |  |
| Bit |  |  |
| 2 |  |  |
| 0 |  | UE radio capability update not needed |
| 1 |  | UE radio capability update needed |
| For a list of RATs for which a radio capability update can be triggered by means of this indication see clause 5.5.1.3.2, case n). |  |  |
| 5GS Preferred CIoT network behaviour (5GS PNB-CIoT) (octet 3, bits 3 and 4) |  |  |
| Bits |  |  |
| 4 | 3 |  |
| 0 | 0 | no additional information |
| 0 | 1 | control plane CIoT 5GS optimization |
| 1 | 0 | user plane CIoT 5GS optimization |
| 1 | 1 | reserved |
| EPS Preferred CIoT network behaviour (EPS-PNB-CIoT) (octet 3, bits 5 and 6) |  |  |
| Bits 6; 5. |
| 0; 0; no additional information. 0; 1; control plane CIoT EPS optimization. 1; 0; user plane CIoT EPS optimization. 1; 1; reserved. |
| Bits 7 to 8 of octet 3 are spare and shall be coded as zero. |  |  |

#### 9.11.3.10	ABBA

The purpose of the ABBA information element is to enable the bidding down protection of security features.

The ABBA information element is coded as shown in figure 9.11.3.10.1 and table 9.11.3.10.1.

The ABBA is a type 4 information element with a minimum length of 4 octets and maximum length of 257 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ABBA IEI |  |  |  |  |  |  |  | octet 1 |
| Length of ABBA contents |  |  |  |  |  |  |  | octet 2 |
| ABBA contents |  |  |  |  |  |  |  | octet 3 octet n |

Figure 9.11.3.10.1: ABBA information element

Table 9.11.3.10.1: ABBA information element

ABBA contents (octet 3-n): indicate set of security features defined for 5GS as described in 3GPP TS 33.501 [24].

NOTE 1: If the UE receives the ABBA IE with a length that is set to a value of 2 and with a value of 0000H, the UE shall use the length and the contents of the ABBA IE as received from the network. NOTE 2: If the UE receives the ABBA IE with a length that is set to a value larger than 2 or with a value that is different from 0000H, the UE shall use the length and the contents of the ABBA IE as received from the network.

#### 9.11.3.11	Void

#### 9.11.3.12	Additional 5G security information

The purpose of the Additional 5G security information information element is to provide the UE with additional security parameters (e.g. horizontal derivation parameter) or to request the UE to retransmit an initial NAS message during a security mode control procedure as defined in 3GPP TS 33.501 [24]. The UE uses these parameters for completion of security mode control procedure.

The Additional 5G security information information element is coded as shown in figure 9.11.3.12.1 and table 9.11.3.12.1.

The Additional 5G security information is a type 4 information element with a length of 3 octets.

|  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Additional 5G security information IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |  |
| Length of Additional 5G security information contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | RINMR |  | HDP |  | octet 3 |  |  |

Figure 9.11.3.12.1: Additional 5G security information information element

Table 9.11.3.12.1: Additional 5G security information information element

| Horizontal derivation parameter (HDP) (octet 3, bit 1) |  |  |
| --- | --- | --- |
| 0 |  | KAMF derivation is not required |
| 1 |  | KAMF derivation is required |
| Retransmission of initial NAS message request (octet 3, bit 2) |  |  |
| 0 |  | Retransmission of the initial NAS message not requested |
| 1 |  | Retransmission of the initial NAS message requested |
| Bits 3 to 8 of octet 3 are spare and shall be coded as zero. |  |  |

#### 9.11.3.12A	Additional information requested

The purpose of the Additional information requested information element is to enable the UE to request ciphering keys for deciphering of ciphered broadcast assistance data.

The Additional information requested information element is coded as shown in figure 9.11.3.12A.1 and table 9.11.3.12A.1.

The Additional information requested is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Additional information requested IEI |  |  |  |  |  |  |  | octet 1 |
| Length of additional information requested contents |  |  |  |  |  |  |  | octet 2 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | CipherKey | octet 3 |
| Spare |  |  |  |  |  |  |  |  |

Figure 9.11.3.12A.1: Additional information requested information element

Table 9.11.3.12A.1: Additional information requested information element

| Ciphering keys for ciphered broadcast assistance data (CipherKey) (octet 3, bit 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | ciphering keys for ciphered broadcast assistance data not requested |
| 1 |  |  |  | ciphering keys for ciphered broadcast assistance data requested |
| Bits 8 to 2 of octet 3 are spare and shall be coded as zero. |  |  |  |  |

#### 9.11.3.13	Allowed PDU session status

The purpose of the Allowed PDU session status information element is to indicate to the network user-plane resources of PDU sessions associated with non-3GPP access that are allowed to be re-established over 3GPP access or if there is no PDU session(s) for which the UE allows the user-plane resources to be re-established over 3GPP access.

NOTE:	Allowed PDU session status IE is not applicable for MA PDU session(s) in this release of specification.

The Allowed PDU session status information element is coded as shown in figure 9.11.3.13.1 and table 9.11.3.13.1.

The Allowed PDU session status is a type 4 information element with minimum length of 4 octets and maximum length of 34 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allowed PDU session status IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Allowed PDU session status contents |  |  |  |  |  |  |  | octet 2 |
| PSI (7) | PSI (6) | PSI (5) | PSI (4) | PSI (3) | PSI (2) | PSI (1) | PSI (0) | octet 3 |
| PSI (15) | PSI (14) | PSI (13) | PSI (12) | PSI (11) | PSI (10) | PSI (9) | PSI (8) | octet 4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Spare |  |  |  |  |  |  |  | octet 5* -34* |

Figure 9.11.3.13.1: Allowed PDU session status information element

Table 9.11.3.13.1: Allowed PDU session status information element

PSI(x) shall be coded as follows: PSI(0): Bit 1 octet 3 is spare and shall be coded as zero. PSI(1) – PSI(15): 0 indicates that the user-plane resources of corresponding PDU session is not allowed to be re-established over 3GPP access. 1 indicates that the user-plane resources of corresponding PDU session can be re-established over 3GPP access. If there is no PDU session for which the user-plane resources can be re-established over 3GPP access, all bits in PSI(1) – PSI(15) shall be coded as zero. All bits in octet 5 to 34 are spare and shall be coded as zero, if the respective octet is included in the information element.

#### 9.11.3.14	Authentication failure parameter

See clause 10.5.3.2.2 in 3GPP TS 24.008 [12].

#### 9.11.3.15	Authentication parameter AUTN

See clause 10.5.3.1.1 in 3GPP TS 24.008 [12].

#### 9.11.3.16	Authentication parameter RAND

See clause 10.5.3.1 in 3GPP TS 24.008 [12].

#### 9.11.3.17	Authentication response parameter

See clause 9.9.3.4 in 3GPP TS 24.301 [15].

#### 9.11.3.18	Configuration update indication

The purpose of the Configuration update indication information element is to indicate the additional information associated with the generic UE configuration update procedure.

The Configuration update indication information element is coded as shown in figure 9.11.3.18.1 and table 9.11.3.18.1.

The Configuration update indication is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration update indication IEI |  |  |  | 0 Spare | 0 Spare | RED | ACK | octet 1 |

Figure 9.11.3.18.1: Configuration update indication

Table 9.11.3.18.1: Configuration update indication

| Acknowledgement (ACK) (octet 1, bit 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | acknowledgement not requested |
| 1 | acknowledgement requested |
| Registration requested (RED) (octet 1, bit 2) |  |
| Bit |  |
| 2 |  |
| 0 | registration not requested |
| 1 | registration requested |
| Bits 3 and 4 are spare and shall be coded as zero, |  |

#### 9.11.3.18A	CAG information list

The purpose of the CAG information list information element is to provide "CAG information list" or to delete the "CAG information list" at the UE.

The CAG information list information element is coded as shown in figures 9.11.3.18A.1 and 9.11.3.18A.2 and table 9.11.3.18A.1.

The CAG information list is a type 6 information element, with a minimum length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAG information list IEI |  |  |  |  |  |  |  | octet 1 |
| Length of CAG information list contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Entry 1 |  |  |  |  |  |  |  | octet 4* octet a* |
| Entry 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* octet g* |
| Entry n |  |  |  |  |  |  |  | octet g+1* octet h* |

Figure 9.11.3.18A.1: CAG information list information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of entry contents |  |  |  |  |  |  |  | octet q |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet q+1 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet q+2 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet q+3 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | CAG only | octet q+4 |
| CAG-ID 1 |  |  |  |  |  |  |  | octet q+5* octet q+8* |
| CAG-ID 2 |  |  |  |  |  |  |  | octet q+9* octet q+12* |
| … |  |  |  |  |  |  |  | octet q+13* octet q+4m* |
| CAG-ID m |  |  |  |  |  |  |  | octet q+4m+1* octet q+4m+4* |

Figure 9.11.3.18A.2: Entry n

Table 9.11.3.18A.1: CAG information list information element

| MCC, Mobile country code (octet q+1 and bits 1 to 4 octet q+2) The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A. |  |
| --- | --- |
| MNC, Mobile network code (bits 5 to 8 of octet q+2 and octet q+3) The coding of this field is the responsibility of each administration, but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet q+2 shall be coded as "1111". |  |
| The MCC and MNC digits are coded as octets 6 to 8 of the Temporary mobile group identity IE in figure 10.5.154 of 3GPP TS 24.008 [12]. |  |
| Indication that the UE is only allowed to access 5GS via CAG cells (CAGonly) (bit 1 of octet q+4) |  |
| Bit |  |
| 1 |  |
| 0 | "Indication that the UE is only allowed to access 5GS via CAG cells" is not set (i.e., the UE is allowed to access 5GS via non-CAG cells) |
| 1 | "Indication that the UE is only allowed to access 5GS via CAG cells" is set (i.e., the UE is not allowed to access 5GS via non-CAG cells) |
| CAG-ID m (octet q+4m+1 to octet q+4m+4) This field contains the 32 bit CAG-ID. The coding of the CAG-ID is defined as the CAG-Identifier in 3GPP TS 23.003 [4]. NOTE 1: The Length of CAG information list contents shall be 0 if no subscription data for CAG information list exists. NOTE 2: The Length of entry contents shall be 4 if there is no allowed CAG-ID for the PLMN. NOTE 3: Bit 2 in octet q+4 may be set to 1 in the USIM (see 3GPP TS 31.102 [22]). NOTE 4: For a given PLMN ID, there shall be up to one Entry containing the MCC value and the MNC value of the PLMN ID. |  |

#### 9.11.3.18B	CIoT small data container

This information element is used to encapsulate the CIoT user data, SMS, or location services message with a size that is not more than 254 octets between the UE and the AMF when the UE is using control plane CIoT 5GS optimization. The CIoT small data container information element is coded as shown in figure 9.11.3.18B.1, figure 9.11.3.18B.2, figure 9.11.3.18B.3, figure 9.11.3.18B.4 and table 9.11.3.18B.1.

The CIoT small data container is a type 4 information element with a minimum length of 4 octets and a maximum length of 257 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CIoT small data container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of CIoT small data container contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| CIoT small data container contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 257 |

Figure 9.11.3.18B.1: CIoT small data container information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data type |  |  | DDX |  | PDU session identity |  |  | octet 3 |
| Data contents |  |  |  |  |  |  |  | octet 4 octet 257 |

Figure 9.11.3.18B.2: CIoT small data container contents for Data type "Control plane user data"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data type |  |  | DDX |  | 0 Spare | 0 Spare | 0 Spare | octet 3 |
| Length of additional information |  |  |  |  |  |  |  | octet 4 |
| Additional information |  |  |  |  |  |  |  | octet 5* octet m* |
| Data contents |  |  |  |  |  |  |  | octet m+1 octet n |

Figure 9.11.3.18B.3: CIoT small data container contents for Data type "Location services message container"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data type |  |  |  |  | Spare |  |  | octet 3 |
| Data contents |  |  |  |  |  |  |  | octet 4 octet n |

Figure 9.11.3.18B.4: CIoT small data container contents for Data type "SMS"

Table 9.11.3.18B.1: CIoT small data container information element

| CIoT small data container contents (octet 3 to octet 257) |  |  |  |  |
| --- | --- | --- | --- | --- |
| These octets include user data to be delivered between UE and AMF. |  |  |  |  |
| Data type (octet 3, bits 6 to 8) Bits |  |  |  |  |
| 8 | 7 | 6 |  |  |
| 0 | 0 | 0 |  | Control plane user data |
| 0 | 0 | 1 |  | SMS |
| 0 | 1 | 0 |  | Location services message container |
| All other values are spare. If received they shall be ignored. |  |  |  |  |
| When the Data type is "Control plane user data ", the PDU session identity and Downlink data expected (DDX) fields are encoded as follows: |  |  |  |  |
| PDU session identity (octet 3, bits 1 to 4) Bit |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 0 |  | No PDU session identity assigned |
| 0 | 0 | 1 |  | PDU session identity value 1 |
| 0 | 1 | 0 |  | PDU session identity value 2 |
| 0 | 1 | 1 |  | PDU session identity value 3 |
| 1 | 0 | 0 |  | PDU session identity value 4 |
| 1 | 0 | 1 |  | PDU session identity value 5 |
| 1 | 1 | 0 |  | PDU session identity value 6 |
| 1 | 1 | 1 |  | PDU session identity value 7 |
| Downlink data expected (DDX) (octet 3, bits 5 to 6) |  |  |  |  |
| Bits |  |  |  |  |
| 5 | 4 |  |  |  |
| 0 | 0 | No information available |  |  |
| 0 | 1 | No further uplink and no further downlink data transmission subsequent to the uplink data transmission is expected |  |  |
| 1 | 0 | Only a single downlink data transmission and no further uplink data transmission subsequent to the uplink data transmission is expected |  |  |
| 1 | 1 | reserved |  |  |
| NOTE: The DDX field is only used in the UE to network direction. |  |  |  |  |
| Data contents (octet 4 to octet 257) This field contains the control plane user data. |  |  |  |  |
| When the Data type is "SMS", Bits 1 to 5 of octet 3 are spare and shall be coded as zero. |  |  |  |  |
| Data contents (octet 4 to octet 257) This field contains an SMS message. When the Data type is "Location services message container": Downlink data expected (DDX) (octet 3, bits 5 to 4) This field is encoded as described above for the case when the Data type is "Control plane user data". Bits 3 to 1 of octet 3 are spare and shall be encoded as zero. Length of Additional information (octet 4) (see NOTE) Indicates the length, in octets, of the Additional information field. Additional information (octets 5 to m) Contains additional information if provided by the upper layer location services application. Data contents (octets m+1 to n) Contains the location services message payload. |  |  |  |  |
| NOTE: The Length of Additional information shall be set to zero if the upper layer location service application does not provide routing information. |  |  |  |  |

#### 9.11.3.18C	Ciphering key data

The purpose of the Ciphering key data information element is to transfer a list of ciphering data sets from the network to the UE for deciphering of ciphered assistance data.

The Ciphering key data information element is coded as shown in figure 9.11.3.18C.1, figure 9.11.3.18C.2 and table 9.11.3.18C.1.

The Ciphering key data is a type 6 information element, with a minimum length of 34 octets and a maximum length of 2675 octets. The list can contain a maximum of 16 ciphering data sets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ciphering key data IEI |  |  |  |  |  |  |  | octet 1 |
| Length of ciphering key data contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Ciphering data set 1 |  |  |  |  |  |  |  | octet 4 octet i |
| Ciphering data set 2 |  |  |  |  |  |  |  | octet i+1* octet l* |
| … |  |  |  |  |  |  |  | octet l+1* octet m* |
| Ciphering data set p |  |  |  |  |  |  |  | octet m+1* octet n* |

Figure 9.11.3.18C.1: Ciphering key data information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ciphering set ID |  |  |  |  |  |  |  | octet 1 octet 2 |
| Ciphering key |  |  |  |  |  |  |  | octet 3 octet 18 |
| 0 | 0 | 0 | c0 length |  |  |  |  | octet 19 |
| Spare |  |  |  |  |  |  |  |  |
| c0 |  |  |  |  |  |  |  | octet 20 octet k |
| 0 | 0 | 0 | 0 | E-UTRA posSIB length |  |  |  | octet k+1 |
| Spare |  |  |  |  |  |  |  |  |
| PosSIBType1-1 | PosSIBType1-2 | PosSIBType1-3 | PosSIBType1-4 | PosSIBType1-5 | PosSIBType1-6 | PosSIBType1-7 | PosSIBType1-8 | octet (k+2)* octet (k+3)* octet (k+4)* octet (k+5)* |
| PosSIBType2-1 | PosSIBType2-2 | PosSIBType2-3 | PosSIBType2-4 | PosSIBType2-5 | PosSIBType2-6 | PosSIBType2-7 | PosSIBType2-8 |  |
| PosSIBType2-9 | PosSIBType2-10 | PosSIBType2-11 | PosSIBType2-12 | PosSIBType2-13 | PosSIBType2-14 | PosSIBType2-15 | PosSIBType2-16 |  |
| PosSIBType2-17 | PosSIBType2-18 | PosSIBType2-19 | PosSIBType2-20 | PosSIBType2-21 | PosSIBType2-22 | PosSIBType2-23 | PosSIBType2-24 |  |
| PosSIBType2-25 | PosSIBType3-1 | PosSIBType4-1 | PosSIBType5-1 | PosSIBType1-9 | PosSIBType1-10 | PosSIBType2-17a | PosSIBType2-18a | octet (k+6)* |
| PosSIBType2-20a | PosSIBType2-26 | PosSIBType2-27 | PosSIBType1-11 | PosSIBType1-12 | 0 Spare | 0 Spare | 0 Spare | octet p* |
| 0 | 0 | 0 | 0 | NR posSIB length |  |  |  | octet p+1 |
| Spare |  |  |  |  |  |  |  |  |
| PosSIBType1-1 | PosSIBType1-2 | PosSIBType1-3 | PosSIBType1-4 | PosSIBType1-5 | PosSIBType1-6 | PosSIBType1-7 | PosSIBType1-8 | octet (p+2)* octet (p+3)* octet (p+4)* octet (p+5)* octet (p+6)* |
| PosSIBType2-1 | PosSIBType2-2 | PosSIBType2-3 | PosSIBType2-4 | PosSIBType2-5 | PosSIBType2-6 | PosSIBType2-7 | PosSIBType2-8 |  |
| PosSIBType2-9 | PosSIBType2-10 | PosSIBType2-11 | PosSIBType2-12 | PosSIBType2-13 | PosSIBType2-14 | PosSIBType2-15 | PosSIBType2-16 |  |
| PosSIBType2-17 | PosSIBType2-18 | PosSIBType2-19 | PosSIBType2-20 | PosSIBType2-21 | PosSIBType2-22 | PosSIBType2-23 | PosSIBType3-1 |  |
| PosSIBType4-1 | PosSIBType5-1 | PosSIBType6-1 | PosSIBType6-2 | PosSIBType6-3 | PosSIBType6-4 | PosSIBType6-5 | PosSIBType6-6 |  |
| PosSIBType1-9 | PosSIBType1-10 | PosSIBType2-24 | PosSIBType2-25 | PosSIBType2-17a | PosSIBType2-18a | PosSIBType2-20a | PosSIBType2-26 | octet (p+7)* |
| PosSIBType2-27 | PosSIBType1-11 | PosSIBType1-12 | PosSIBType6-7 | PosSIBType7-1 | PosSIBType7-2 | PosSIBType7-3 | PosSIBType7-4 | octet q* |
| Validity start time |  |  |  |  |  |  |  | octet q+1 octet q+5 |
| Validity duration |  |  |  |  |  |  |  | octet q+6 octet q+7 |
| TAIs list |  |  |  |  |  |  |  | octet q+8 octet r |

Figure 9.11.3.18C.2: Ciphering data set

Table 9.11.3.18C.1: Ciphering key data information element

| Value part of the Ciphering key data information element (octets 4 to n) |  |  |  |  |
| --- | --- | --- | --- | --- |
| The value part of the Ciphering key data information element consists of one or several ciphering data sets. |  |  |  |  |
| The UE shall store the complete list received. If more than 16 ciphering data sets are included in this information element, the UE shall store the first 16 ciphering data sets and ignore the remaining octets of the information element. |  |  |  |  |
| Ciphering data set: |  |  |  |  |
| Ciphering set ID (octets 1 to 2) |  |  |  |  |
| This field contains the binary encoding of the ID identifying the ciphering set. |  |  |  |  |
| Ciphering key (octets 3 to octet 18) |  |  |  |  |
| This field contains the 128 bit ciphering key. |  |  |  |  |
| c0 length (octet 19, bits 5 to 1) This field contains the binary encoding of the length, in octets, of the c0 counter. The maximum value for the length of the c0 counter is 16 octets. |  |  |  |  |
| Bits 8 to 6 of octect 19 are spare and shall be coded as zero. |  |  |  |  |
| c0 (octets 20 to k) |  |  |  |  |
| This field contains the binary encoding of the c0 counter. |  |  |  |  |
| E-UTRA posSIB length (octet k+1, bits 4 to 1) This field contains the length in octets of the E -UTRA Positioning SIB types. A length of zero means E -UTRA Positioning SIB types are not included (see NOTE). E-UTRA Positioning SIB types for which the ciphering data set is applicable (octets k+2 to p). Unassigned bits shall be ignored by a UE. Non-included bits shall be assumed to be zero by a UE. |  |  |  |  |
| Ciphering data set applicable for positioning SIB type 1-1 (octet k+2, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-1 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-1 |
| Ciphering data set applicable for positioning SIB type 1-2 (octet k+2, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-2 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-2 |
| Ciphering data set applicable for positioning SIB type 1-3 (octet k+2, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-3 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-3 |
| Ciphering data set applicable for positioning SIB type 1-4 (octet k+2, bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-4 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-4 |
| Ciphering data set applicable for positioning SIB type 1-5 (octet k+2, bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-5 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-5 |
| Ciphering data set applicable for positioning SIB type 1-6 (octet k+2, bit 3) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-6 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-6 |
| Ciphering data set applicable for positioning SIB type 1-7 (octet k+2, bit 2) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-7 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-7 |
| Ciphering data set applicable for positioning SIB type 1-8 (octet k+2, bit 1) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-8 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-8 |
| Ciphering data set applicable for positioning SIB type 2-1 (octet k+3, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-1 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-1 |
| Ciphering data set applicable for positioning SIB type 2-2 (octet k+3, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-2 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-2 |
| Ciphering data set applicable for positioning SIB type 2-3 (octet k+3, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-3 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-3 |
| Ciphering data set applicable for positioning SIB type 2-4 (octet k+3, bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-4 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-4 |
| Ciphering data set applicable for positioning SIB type 2-5 (octet k+3, bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-5 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-5 |
| Ciphering data set applicable for positioning SIB type 2-6 (octet k+3, bit 3) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-6 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-6 |
| Ciphering data set applicable for positioning SIB type 2-7 (octet k+3, bit 2) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-7 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-7 |
| Ciphering data set applicable for positioning SIB type 2-8 (octet k+3, bit 1) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-8 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-8 |
| Ciphering data set applicable for positioning SIB type 2-9 (octet k+4, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-9 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-9 |
| Ciphering data set applicable for positioning SIB type 2-10 (octet k+4, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-10 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-10 |
| Ciphering data set applicable for positioning SIB type 2-11 (octet k+4, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-11 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-11 |
| Ciphering data set applicable for positioning SIB type 2-12 (octet k+4, bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-12 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-12 |
| Ciphering data set applicable for positioning SIB type 2-13 (octet k+4, bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-13 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-13 |
| Ciphering data set applicable for positioning SIB type 2-14 (octet k+4, bit 3) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-14 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-14 |
| Ciphering data set applicable for positioning SIB type 2-15 (octet k+4, bit 2) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-15 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-15 |
| Ciphering data set applicable for positioning SIB type 2-16 (octet k+4, bit 1) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-16 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-16 |
| Ciphering data set applicable for positioning SIB type 2-17 (octet k+5, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-17 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-17 |
| Ciphering data set applicable for positioning SIB type 2-18 (octet k+5, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-18 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-18 |
| Ciphering data set applicable for positioning SIB type 2-19 (octet k+5, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-19 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-19 |
| Ciphering data set applicable for positioning SIB type 2-20 (octet k+5, bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-20 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-20 |
| Ciphering data set applicable for positioning SIB type 2-21 (octet k+5, bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-21 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-21 |
| Ciphering data set applicable for positioning SIB type 2-22 (octet k+5, bit 3) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-22 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-22 |
| Ciphering data set applicable for positioning SIB type 2-23 (octet k+5, bit 2) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-23 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-23 |
| Ciphering data set applicable for positioning SIB type 2-24 (octet k+5, bit 1) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-24 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-24 |
| Ciphering data set applicable for positioning SIB type 2-25 (octet k+6, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-25 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-25 |
| Ciphering data set applicable for positioning SIB type 3-1 (octet k+6, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 3-1 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 3-1 |
| Ciphering data set applicable for positioning SIB type 4-1 (octet k+6, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 4-1 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 4-1 |
| Ciphering data set applicable for positioning SIB type 5-1 (octet k+6, bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 5-1 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 5-1 |
| Ciphering data set applicable for positioning SIB type 1-9 (octet k+6, bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-9 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-9 |
| Ciphering data set applicable for positioning SIB type 1-10 (octet k+6, bit 3) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-10 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-10 |
| Ciphering data set applicable for positioning SIB type 2-17a (octet k+6, bit 2) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-17a |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-17a |
| Ciphering data set applicable for positioning SIB type 2-18a (octet k+6, bit 1) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-18a |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-18a |
| Ciphering data set applicable for positioning SIB type 2-20a (octet k+7, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-20a |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-20a |
| Ciphering data set applicable for positioning SIB type 2-26 (octet k+7, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-26 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-26 |
| Ciphering data set applicable for positioning SIB type 2-27 (octet k+7, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-27 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-27 |
| Ciphering data set applicable for positioning SIB type 1-11 (octet k+7 bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-11 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-11 |
| Ciphering data set applicable for positioning SIB type 1-12 (octet k+7, bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-12 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-12 |
| Any unassigned bits shall be coded as zero. |  |  |  |  |
| NR posSIB length (octet p+1, bits 4 to 1) |  |  |  |  |
| This field contains the length in octets of the NR Positioning SIB types. A length of zero means NR Positioning SIB types are not included (see NOTE). |  |  |  |  |
| NR Positioning SIB types for which the ciphering data set is applicable (octets p+2 to q) |  |  |  |  |
| Unasssigned bits shall be ignored. Non-included bits shall be assumed to be zero. |  |  |  |  |
| Ciphering data set applicable for positioning SIB type 1-1 (octet p+2, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-1 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-1 |
| Ciphering data set applicable for positioning SIB type 1-2 (octet p+2, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-2 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-2 |
| Ciphering data set applicable for positioning SIB type 1-3 (octet p+2, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-3 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-3 |
| Ciphering data set applicable for positioning SIB type 1-4 (octet p+2, bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-4 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-4 |
| Ciphering data set applicable for positioning SIB type 1-5 (octet p+2, bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-5 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-5 |
| Ciphering data set applicable for positioning SIB type 1-6 (octet p+2, bit 3) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-6 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-6 |
| Ciphering data set applicable for positioning SIB type 1-7 (octet p+2, bit 2) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-7 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-7 |
| Ciphering data set applicable for positioning SIB type 1-8 (octet p+2, bit 1) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-8 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-8 |
| Ciphering data set applicable for positioning SIB type 2-1 (octet p+3, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-1 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-1 |
| Ciphering data set applicable for positioning SIB type 2-2 (octet p+3, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-2 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-2 |
| Ciphering data set applicable for positioning SIB type 2-3 (octet p+3, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-3 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-3 |
| Ciphering data set applicable for positioning SIB type 2-4 (octet p+3, bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-4 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-4 |
| Ciphering data set applicable for positioning SIB type 2-5 (octet p+3, bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-5 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-5 |
| Ciphering data set applicable for positioning SIB type 2-6 (octet p+3, bit 3) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-6 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-6 |
| Ciphering data set applicable for positioning SIB type 2-7 (octet p+3, bit 2) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-7 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-7 |
| Ciphering data set applicable for positioning SIB type 2-8 (octet p+3, bit 1) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-8 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-8 |
| Ciphering data set applicable for positioning SIB type 2-9 (octet p+4, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-9 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-9 |
| Ciphering data set applicable for positioning SIB type 2-10 (octet p+4, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-10 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-10 |
| Ciphering data set applicable for positioning SIB type 2-11 (octet p+4, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-11 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-11 |
| Ciphering data set applicable for positioning SIB type 2-12 (octet p+4, bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-12 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-12 |
| Ciphering data set applicable for positioning SIB type 2-13 (octet p+4, bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-13 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-13 |
| Ciphering data set applicable for positioning SIB type 2-14 (octet p+4, bit 3) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-14 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-14 |
| Ciphering data set applicable for positioning SIB type 2-15 (octet p+4, bit 2) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-15 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-15 |
| Ciphering data set applicable for positioning SIB type 2-16 (octet p+4, bit 1) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-16 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-16 |
| Ciphering data set applicable for positioning SIB type 2-17 (octet p+5, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-17 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-17 |
| Ciphering data set applicable for positioning SIB type 2-18 (octet p+5, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-18 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-18 |
| Ciphering data set applicable for positioning SIB type 2-19 (octet p+5, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-19 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-19 |
| Ciphering data set applicable for positioning SIB type 2-20 (octet p+5, bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-20 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-20 |
| Ciphering data set applicable for positioning SIB type 2-21 (octet p+5, bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-21 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-21 |
| Ciphering data set applicable for positioning SIB type 2-22 (octet p+5, bit 3) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-22 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-22 |
| Ciphering data set applicable for positioning SIB type 2-23 (octet p+5, bit 2) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-23 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-23 |
| Ciphering data set applicable for positioning SIB type 3-1 (octet p+5, bit 1) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 3-1 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 3-1 |
| Ciphering data set applicable for positioning SIB type 4-1 (octet p+6, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 4-1 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 4-1 |
| Ciphering data set applicable for positioning SIB type 5-1 (octet p+6, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 5-1 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 5-1 |
| Ciphering data set applicable for positioning SIB type 6-1 (octet p+6, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 6-1 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 6-1 |
| Ciphering data set applicable for positioning SIB type 6-2 (octet p+6, bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 6-2 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 6-3 |
| Ciphering data set applicable for positioning SIB type 6-3 (octet p+6, bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 6-3 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 6-3 |
| Ciphering data set applicable for positioning SIB type 6-4 (octet p+6, bit 3) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 6-4 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 6-4 |
| Ciphering data set applicable for positioning SIB type 6-5 (octet p+6, bit 2) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 6-5 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 6-5 |
| Ciphering data set applicable for positioning SIB type 6-6 (octet p+6, bit 1) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 6-6 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 6-6 |
| Ciphering data set applicable for positioning SIB type 1-9 (octet p+7, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-9 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-9 |
| Ciphering data set applicable for positioning SIB type 1-10 (octet p+7, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-10 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-10 |
| Ciphering data set applicable for positioning SIB type 2-24 (octet p+7, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-24 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-24 |
| Ciphering data set applicable for positioning SIB type 2-25 (octet p+7, bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-25 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-25 |
| Ciphering data set applicable for positioning SIB type 2-17a (octet p+7 bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-17a |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-17a |
| Ciphering data set applicable for positioning SIB type 2-18a (octet p+7, bit 3) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-18a |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-18a |
| Ciphering data set applicable for positioning SIB type 2-20a (octet p+7, bit 2) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-20a |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-20a |
| Ciphering data set applicable for positioning SIB type 2-26 (octet p+7, bit 1) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-26 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-26 |
| Ciphering data set applicable for positioning SIB type 2-27 (octet p+8, bit 8) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 2-27 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 2-27 |
| Ciphering data set applicable for positioning SIB type 1-11 (octet p+8, bit 7) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-11 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-11 |
| Ciphering data set applicable for positioning SIB type 1-12 (octet p+8, bit 6) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 1-12 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 1-12 |
| Ciphering data set applicable for positioning SIB type 6-7 (octet p+8, bit 5) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 6-7 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 6-7 |
| Ciphering data set applicable for positioning SIB type 7-1 (octet p+8, bit 4) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 7-1 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 7-1 |
| Ciphering data set applicable for positioning SIB type 7-2 (octet p+8, bit 3) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 7-2 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 7-2 |
| Ciphering data set applicable for positioning SIB type 7-3 (octet p+8, bit 2) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 7-3 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 7-3 |
| Ciphering data set applicable for positioning SIB type 7-4 (octet p+8, bit 1) |  |  |  |  |
| 0 |  |  |  | Ciphering data set not applicable to positioning SIB type 7-4 |
| 1 |  |  |  | Ciphering data set applicable to positioning SIB type 7-4 |
| Any unassigned bits shall be coded as zero. |  |  |  |  |
| Validity start time (octets q+1 to q+5) |  |  |  |  |
| This field contains the UTC time when the ciphering data set becomes valid, encoded as octets 2 to 6 of the Time zone and time IE specified in 3GPP TS 24.008 [12]. |  |  |  |  |
| Validity duration (octets q+6 to q+7) |  |  |  |  |
| This field contains the duration for which the ciphering data set is valid after the validity start time, in units of minutes. |  |  |  |  |
| TAIs list (octets q+8 to r) |  |  |  |  |
| This field contains the list of tracking areas for which the ciphering data set is applicable, encoded as octets 2 to n of the Tracking area identity list IE as specified in clause 9.11.3.9. If the TAIs list is empty (as indicated by a zero length), the ciphering data set is applicable to the entire serving PLMN. NOTE: The ciphering data set is always applicable to at least one of the E -UTRA Positioning SIB types or the NR Positioning SIB types. |  |  |  |  |

#### 9.11.3.18D	Control plane service type

The purpose of the Control plane service type information element is to specify the purpose of the CONTROL PLANE SERVICE REQUEST message.

The Control plane service type information element is coded as shown in figure 9.11.3.18D.1 and table 9.11.3.18D.1.

The Control plane service type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Control plane service type IEI |  |  |  | 0 Spare | Control plane service type value |  |  | octet 1 |

Figure 9.9.3.18D.1: Control plane service type information element

Table 9.9.3.18D.1: Control plane service type information element

| Control plane service type value (octet 1, bit 1 to 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 0 |  | mobile originating request |
| 0 | 0 | 1 |  | mobile terminating request |
| 0 | 1 | 0 |  | emergency services |
| 0 | 1 | 1 |  | emergency services fallback |
| 1 | 0 | 0 |  |  |
|  | to |  |  | unused; shall be interpreted as " mobile originating request", if received by the network. |
| 1 | 1 | 1 |  |  |

#### 9.11.3.19	Daylight saving time

See clause 10.5.3.12 in 3GPP TS 24.008 [12].

#### 9.11.3.20	De-registration type

The purpose of the De-registration type information element is to indicate the type of de-registration.

The De-registration type information element is coded as shown in figure 9.11.3.20.1 and table 9.11.3.20.1.

The De-registration type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| De-registration type IEI |  |  |  | Switch off | Re-registration required | Access type |  | octet 1 |

Figure 9.11.3.20.1: Deregistration type information element

Table 9.11.3.20.1: Deregistration type information element

| Switch off (octet 1, bit 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| In the UE to network direction: |  |  |  |  |
| Bit |  |  |  |  |
| 4 |  |  |  |  |
| 0 |  |  |  | Normal de-registration |
| 1 |  |  |  | Switch off |
| In the network to UE direction bit 4 is spare. The network shall set this bit to zero. |  |  |  |  |
| Re-registration required (octet 1, bit 3) |  |  |  |  |
| In the network to UE direction: |  |  |  |  |
| Bit |  |  |  |  |
| 3 |  |  |  |  |
| 0 |  |  |  | re-registration not required |
| 1 |  |  |  | re-registration required |
| In the UE to network direction bit 3 is spare. The UE shall set this bit to zero. |  |  |  |  |
| Access type (octet 1,bit 2, bit 1) |  |  |  |  |
| Bit |  |  |  |  |
| 2 | 1 |  |  |  |
| 0 | 1 |  |  | 3GPP access |
| 1 | 0 |  |  | Non-3GPP access |
| 1 | 1 |  |  | 3GPP access and non-3GPP access |
| All other values are reserved. |  |  |  |  |

#### 9.11.3.21	Void

#### 9.11.3.22	Void

#### 9.11.3.23	Emergency number list

See clause 10.5.3.13 in 3GPP TS 24.008 [12].

#### 9.11.3.23A	EPS bearer context status

See clause 9.9.2.1 in 3GPP TS 24.301 [15].

#### 9.11.3.24	EPS NAS message container

The purpose of the EPS NAS message container information element is to transport an EPS NAS message as specified in 3GPP TS 24.301 [15].

The EPS NAS message container information element is coded as shown in figure 9.11.3.24.1 and table 9.11.3.24.1.

The EPS NAS message container is a type 6 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EPS NAS message container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of EPS NAS message container contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| EPS NAS message container |  |  |  |  |  |  |  | octet 4 octet n |

Figure 9.11.3.24.1: EPS NAS message container information element

Table 9.11.3.24.1: EPS NAS message container information element

EPS NAS message container (octet 4 to n)

An EPS NAS message as specified in 3GPP TS 24.301 [15].

#### 9.11.3.25	EPS NAS security algorithms

See clause 9.9.3.23 in 3GPP TS 24.301 [15].

#### 9.11.3.26	Extended emergency number list

See clause 9.9.3.37A in 3GPP TS 24.301 [15].

#### 9.11.3.26A	Extended DRX parameters

See clause 10.5.5.32 in 3GPP TS 24.008 [12].

#### 9.11.3.27	Void

#### 9.11.3.28	IMEISV request

See clause 10.5.5.10 in 3GPP TS 24.008 [12].

#### 9.11.3.29	LADN indication

The purpose of the LADN indication information element is to request the network for LADN information for specific LADN DNN(s) or to indicate a request for LADN information.

The LADN indication information element is coded as shown in figure 9.11.3.29.1 and table 9.11.3.29.1.

The LADN indication is a type 6 information element with a minimum length of 3 octets and a maximum length of 811 octets.

The LADN indication information element can contain a minimum of 0 and a maximum of 8 different LADN DNN values.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LADN indication IEI |  |  |  |  |  |  |  | octet 1 |
| Length of LADN indication contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| LADN DNN value 1 |  |  |  |  |  |  |  | octet 4* octet a* |
| LADN DNN value 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* octet g* |
| LADN DNN value n |  |  |  |  |  |  |  | octet g+1* octet h* |

Figure 9.11.3.29.1: LADN indication information element

Table 9.11.3.29.1: LADN indication information element

Value part of the LADN indication information element (octet 4 to h): The value part of the LADN indication information element consists of zero or more LADN DNN values. If the LADN indication information element conveys more than 8 LADN DNN values in this information element, the network shall consider the first 8 LADN DNN values and ignore the remaining octets of the information element. LADN DNN value: LADN DNN value is coded as the length and value part of DNN information element as specified in clause 9.11.2.1B starting with the second octet.

#### 9.11.3.30	LADN information

The purpose of the LADN information information element is to provide the UE with the LADN service area for each available LADN in the current registration area or to delete the LADN information at the UE.

The LADN information information element is coded as shown in figure 9.11.3.30.1, figure 9.11.3.30.2 and table 9.11.3.30.1.

The LADN information is a type 6 information element with a minimum length of 3 octets and a maximum length of 1715 octets.

The LADN information information element can contain a minimum of 0 and a maximum of 8 different LADNs each including a DNN and a 5GS tracking area identity list.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LADN information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of LADN information contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| LADN 1 |  |  |  |  |  |  |  | octet 4 octet a |
| LADN 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* octet g* |
| LADN n |  |  |  |  |  |  |  | octet g+1* octet h* |

Figure 9.11.3.30.1: LADN information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of DNN value |  |  |  |  |  |  |  | octet 4 |
| DNN value |  |  |  |  |  |  |  | octet 5 octet m |
| 5GS tracking area identity list |  |  |  |  |  |  |  | octet m+1 octet a |

Figure 9.11.3.30.2: LADN

Table 9.11.3.30.1: LADN information information element

Value part of the LADN information information element (octet 4 to octet h) The value part of the LADN information information element consists of one or several LADNs. Each LADN (e.g. octet 4 to octet a) consists one DNN value and one 5GS tracking area identity list. The length of each LADN is determined by the length of DNN value field and the length of 5GS tracking area identity list field. The UE shall store the complete list as received. If more than 8 LADNs are included in this information element, the UE shall store the first 8 LADNs and ignore the remaining octets of the information element. DNN value (octet 5 to octet m): DNN value field is coded as DNN value part of DNN information element as specified in clause 9.11.2.1B starting with the third octet.

5GS tracking area identity list (octet m+1 to octet a):

5GS tracking area identity list field is coded as the length and the value part of the 5GS Tracking area identity list information element as specified in clause 9.11.3.9 starting with the second octet.

#### 9.11.3.31	MICO indication

The purpose of the MICO indication information element is to indicate the use of MICO mode or the re-negotiation of MICO mode.

The MICO indication information element is coded as shown in figure 9.11.3.31.1 and table 9.11.3.31.1.

The MICO indication is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MICO indication IEI |  |  |  | 0 Spare | 0 Spare | SPRTI | RAAI | octet 1 |

Figure 9.11.3.31.1: MICO indication

Table 9.11.3.31.1: MICO indication

| Registration Area Allocation Indication (RAAI) (octet 1, bit 1) |  |  |
| --- | --- | --- |
| In the network to UE direction: |  |  |
| Bit |  |  |
| 1 |  |  |
| 0 | all PLMN registration area not allocated |  |
| 1 | all PLMN registration area allocated |  |
| In the UE to network direction bit 1 is spare. The UE shall set this bit to zero. |  |  |
| Strictly Periodic Registration Timer Indication (SPRTI) (octet 1, bit 2) |  |  |
| In the network to UE and the UE to network direction: |  |  |
| Bit |  |  |
| 2 |  |  |
| 0 | strictly periodic registration timer not supported |  |
| 1 | strictly periodic registration timer supported |  |
| Bits 3 and 4 are spare and shall be coded as zero. NOTE: In the network to UE direction in the CONFIGURATION UPDATE COMMAND message, bits 1 and 2 shall be coded as zero. |  |  |

#### 9.11.3.31A	MA PDU session information

The purpose of the MA PDU session information information element is to convey the MA-related information for the PDU session.

The MA PDU session information information element is coded as shown in figure 9.11.3.31A.1 and table 9.11.3.31A.1.

The MA PDU session information is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MA PDU session information IEI |  |  |  | MA PDU session information value |  |  |  | octet 1 |

Figure 9.11.3.31A.1: MA PDU session information information element

Table 9.11.3.31A.1: MA PDU session information information element

| MA PDU session information value (octet 1, bit 1 to bit 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | No additional information |
| 0 | 0 | 0 | 1 | MA PDU session network upgrade is allowed |
| All other values are spare. If received they shall be ignored. |  |  |  |  |

#### 9.11.3.31B	Mapped NSSAI

The purpose of the Mapped NSSAI information element is to transfer S-NSSAI(s) applicable in the HPLMN to the visited PLMN.

The Mapped NSSAI information element is coded as shown in figure 9.11.3.31B.1, figure 9.11.3.31B.2 and table 9.11.3.31B.1.

The Mapped NSSAI is a type 4 information element with a minimum length of 4 octets and a maximum length of 42 octets.

NOTE 1:	The total number of S-NSSAI values in a requested mapped NSSAI cannot exceed eight.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mapped NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Mapped NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| Mapped S-NSSAI content 1 |  |  |  |  |  |  |  | octet 3 octet m |
| Mapped S-NSSAI content 2 |  |  |  |  |  |  |  | octet m+1* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* octet u* |
| Mapped S-NSSAI content n |  |  |  |  |  |  |  | octet u+1* octet v* |

Figure 9.11.3.31B.1: Mapped NSSAI information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Mapped S-NSSAI content |  |  |  |  |  |  |  | octet 3 |
| Mapped HPLMN SST |  |  |  |  |  |  |  | octet 4 |
| Mapped HPLMN SD |  |  |  |  |  |  |  | octet 5* octet 7* |

Figure 9.11.3.31B.2: Mapped S-NSSAI content

Table 9.11.3.31B.1: Mapped NSSAI information element

Value part of the Mapped NSSAI information element (octet 3 to v) The value part of the Mapped NSSAI information element consists of one or more mapped S-NSSAI contents. Mapped S-NSSAI content: Length of S-NSSAI contents (octet 3) Mapped HPLMN Slice/service type (SST) (octet 4) This field contains the 8 bit SST value of an S-NSSAI in the S-NSSAI(s) of the HPLMN to which the SST value is mapped. The coding of the SST value part is defined in 3GPP TS 23.003 [4]. NOTE 1: Octet 4 (i.e. mapped HPLMN SST) shall always be included. Mapped HPLMN Slice differentiator (SD) (octet 5 to octet 7) This field contains a 24-bit SD value of an S-NSSAI in the S-NSSAI(s) of the HPLMN to which the SD value is mapped. The coding of the SD value part is defined in 3GPP TS 23.003 [4]. NOTE 2: If the octet 5 is included, then octet 6 and octet 7 shall be included.

#### 9.11.3.31C	Mobile station classmark 2

See clause 10.5.1.6 in 3GPP TS 24.008 [12].

#### 9.11.3.32	NAS key set identifier

The NAS key set identifier is allocated by the network.

The NAS key set identifier information element is coded as shown in figure 9.11.3.32.1 and table 9.11.3.32.1.

The NAS key set identifier is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAS key set identifier IEI |  |  |  | TSC | NAS key set identifier |  |  | octet 1 |

Figure 9.11.3.32.1: NAS key set identifier information element

Table 9.11.3.32.1: NAS key set identifier information element

| Type of security context flag (TSC) (octet 1) |  |  |  |
| --- | --- | --- | --- |
| Bit |  |  |  |
| 4 |  |  |  |
| 0 |  |  | native security context (for KSIAMF) |
| 1 |  |  | mapped security context (for KSIASME) |
| TSC does not apply for NAS key set identifier value "111". |  |  |  |
| NAS key set identifier (octet 1) |  |  |  |
| Bits |  |  |  |
| 3 | 2 | 1 |  |
| 0 | 0 | 0 |  |
| through |  |  | possible values for the NAS key set identifier |
| 1 | 1 | 0 |  |
| 1 | 1 | 1 | no key is available (UE to network); |
|  |  |  | reserved (network to UE) |

#### 9.11.3.33	NAS message container

The purpose of the NAS message container IE is to encapsulate a plain 5GS NAS REGISTRATION REQUEST, DEREGISTRATION REQUEST, or SERVICE REQUEST message, or to encapsulate non-cleartext IEs of a CONTROL PLANE SERVICE REQUEST message.

The NAS message container information element is coded as shown in figure 9.11.3.33.1 and table 9.11.3.33.1.

The NAS message container is a type 6 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAS message container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of NAS message container contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet 4 |
| NAS message container contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 9.11.3.33.1: NAS message container information element

Table 9.11.3.33.1: NAS message container information element

NAS message container contents (octet 4 to octet n); Max value of 65535 octets

This IE can contain a REGISTRATION REQUEST message as defined in clause 5.5.1, DEREGISTRATION REQUEST message as defined in clause 5.5.2.2.1, or a SERVICE REQUEST message as defined in clause 5.6.1, or non-cleartext IEs of a CONTROL PLANE SERVICE REQUEST message as defined in clause 5.6.1.

#### 9.11.3.34	NAS security algorithms

The purpose of the NAS security algorithms information element is to indicate the 5G algorithms to be used for ciphering and integrity protection.

The NAS security algorithms information element is coded as shown in figure 9.11.3.34.1 and table 9.11.3.34.1.

The NAS security algorithms is a type 3 information element with a length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAS security algorithms IEI |  |  |  |  |  |  |  | octet 1 |
| Type of ciphering algorithm |  |  |  | Type of integrity protection algorithm |  |  |  | octet 2 |

Figure 9.11.3.34.1: NAS security algorithms information element

Table 9.11.3.34.1: NAS security algorithms information element

| Type of integrity protection algorithm (octet 2, bit 1 to 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | 5G integrity algorithm 5G-IA0 (null integrity protection algorithm) |
| 0 | 0 | 0 | 1 | 5G integrity algorithm 128-5G-IA1 |
| 0 | 0 | 1 | 0 | 5G integrity algorithm 128-5G-IA2 |
| 0 | 0 | 1 | 1 | 5G integrity algorithm 128-5G-IA3 |
| 0 | 1 | 0 | 0 | 5G integrity algorithm 5G-IA4 |
| 0 | 1 | 0 | 1 | 5G integrity algorithm 5G-IA5 |
| 0 | 1 | 1 | 0 | 5G integrity algorithm 5G-IA6 |
| 0 | 1 | 1 | 1 | 5G integrity algorithm 5G-IA7 |
| All other values are reserved. |  |  |  |  |
| Type of ciphering algorithm (octet 2, bit 5 to 7) |  |  |  |  |
| Bits |  |  |  |  |
| 8 | 7 | 6 | 5 |  |
| 0 | 0 | 0 | 0 | 5G encryption algorithm 5G-EA0 (null ciphering algorithm) |
| 0 | 0 | 0 | 1 | 5G encryption algorithm 128-5G-EA1 |
| 0 | 0 | 1 | 0 | 5G encryption algorithm 128-5G-EA2 |
| 0 | 0 | 1 | 1 | 5G encryption algorithm 128-5G-EA3 |
| 0 | 1 | 0 | 0 | 5G encryption algorithm 5G-EA4 |
| 0 | 1 | 0 | 1 | 5G encryption algorithm 5G-EA5 |
| 0 | 1 | 1 | 0 | 5G encryption algorithm 5G-EA6 |
| 0 | 1 | 1 | 1 | 5G encryption algorithm 5G-EA7 |
| All other values are reserved. |  |  |  |  |

#### 9.11.3.35	Network name

See clause 10.5.3.5a in 3GPP TS 24.008 [12].

#### 9.11.3.36	Network slicing indication

The purpose of the Network slicing indication information element is to indicate additional information associated with network slicing in the generic UE configuration update procedure and the registration procedure, other than the user's configured NSSAI, allowed NSSAI, pending NSSAI and rejected NSSAI information.

The Network slicing indication information element is coded as shown in figure 9.11.3.36.1 and table 9.11.3.36.1.

The Network slicing indication is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Network slicing indication IEI |  |  |  | 0 Spare | 0 Spare | DCNI | NSSCI | octet 1 |

Figure 9.11.3.36.1: Network slicing indication

Table 9.11.3.36.1: Network slicing indication

| Network slicing subscription change indication (NSSCI) (octet 1, bit 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | Network slicing subscription not changed |
| 1 | Network slicing subscription changed |
| Default configured NSSAI indication (DCNI) (octet 1, bit 2) |  |
| Bit |  |
| 2 |  |
| 0 | Requested NSSAI not created from default configured NSSAI |
| 1 | Requested NSSAI created from default configured NSSAI |
| In the UE to network direction bit 1 is spare. The UE shall set this bit to zero. In the network to UE direction bit 2 is spare. The network shall set this bit to zero. Bits 3 and 4 are spare and shall be coded as zero. |  |

#### 9.11.3.36A	Non-3GPP NW provided policies

See clause 10.5.5.37 in 3GPP TS 24.008 [12].

#### 9.11.3.37	NSSAI

The purpose of the NSSAI information element is to identify a collection of S-NSSAIs

The NSSAI information element is coded as shown in figure 9.11.3.37.1 and table 9.11.3.37.1.

The NSSAI is a type 4 information element with a minimum length of 4 octets and a maximum length of 146 octets.

NOTE:	More than one S-NSSAIs in an NSSAI can have the same SST values, and optionally same SD values, which are associated with different mapped HPLMN SST values and optionally mapped HPLMN SD values.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| S-NSSAI value 1 |  |  |  |  |  |  |  | octet 3 octet m |
| S-NSSAI value 2 |  |  |  |  |  |  |  | octet m+1* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* octet u* |
| S-NSSAI value n |  |  |  |  |  |  |  | octet u+1* octet v* |

Figure 9.11.3.37.1: NSSAI information element

Table 9.11.3.37.1: NSSAI information element

Value part of the NSSAI information element (octet 3 to v) The value part of the NSSAI information element consists of one or more S-NSSAI values. Each S-NSSAI value consists of one S-NSSAI and optionally one mapped S-NSSAI. The recipient of this information element shall store the complete list received (NOTE 1, NOTE 2, NOTE 3). If the NSSAI information element conveys an allowed NSSAI and more than 8 S-NSSAI values are included in this information element, the UE shall store the first 8 S-NSSAI values and ignore the remaining octets of the information element. If the NSSAI information element conveys a configured NSSAI (including the default configured NSSAI) or pending NSSAI and more than 16 S-NSSAI values are included in this information element, the UE shall store the first 16 S-NSSAI values and ignore the remaining octets of the information element. S-NSSAI value: S-NSSAI value is coded as the length and value part of S-NSSAI information element as specified in clause 9.11.2.8 starting with the second octet.

NOTE 1: The total number of S-NSSAI values in a requested NSSAI shall not exceed eight. NOTE 2: The number of S-NSSAI values in an allowed NSSAI shall not exceed eight. NOTE 3: The number of S-NSSAI values in a configured NSSAI (including the default configured NSSAI) or pending NSSAI shall not exceed sixteen.

#### 9.11.3.37A	NSSAI inclusion mode

The purpose of the NSSAI inclusion mode information element is to indicate the NSSAI inclusion mode in which the UE shall operate.

The NSSAI inclusion mode is a type 1 information element.

The NSSAI inclusion mode information element is coded as shown in figure 9.11.3.37A.1 and table 9.11.3.37A.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NSSAI inclusion mode IEI |  |  |  | 0 spare | 0 spare | NSSAI inclusion mode |  | octet 1 |

Figure 9.11.3.37A.1: NSSAI inclusion mode information element

Table 9.11.3.37A.1: NSSAI inclusion mode information element

| NSSAI inclusion mode (octet 1, bit 1 to bit 2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 2 | 1 |  |  |  |
| 0 | 0 |  |  | NSSAI inclusion mode A |
| 0 | 1 |  |  | NSSAI inclusion mode B |
| 1 | 0 |  |  | NSSAI inclusion mode C |
| 1 | 1 |  |  | NSSAI inclusion mode D |

#### 9.11.3.38	Operator-defined access category definitions

The purpose of the Operator-defined access category definitions information element is to provide the UE with the operator-defined access category definitions or to delete the operator-defined access category definitions at the UE.

The Operator-defined access category definitions information element is coded as shown in figure 9.11.3.38.1, figure 9.11.3.38.2 and table 9.11.3.38.1.

The Operator-defined access category definitions is a type 6 information element with a minimum length of 3 octets, and maximum length of 8323 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operator-defined access category definitions IEI |  |  |  |  |  |  |  | octet 1 |
| Length of operator-defined access category definitions contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Operator-defined access category definition 1 |  |  |  |  |  |  |  | octet 4* octet a* |
| Operator-defined access category definition 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* octet g* |
| Operator-defined access category definition n |  |  |  |  |  |  |  | octet g+1* octet h* |

Figure 9.11.3.38.1: Operator-defined access category definitions information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of operator-defined access category definition contents |  |  |  |  |  |  |  | octet 4 |
| Precedence value |  |  |  |  |  |  |  | octet 5 |
| PSAC | 0 Spare | 0 Spare | Operator-defined access category number |  |  |  |  | octet 6 |
| Length of criteria |  |  |  |  |  |  |  | octet 7 |
| Criteria |  |  |  |  |  |  |  | octet 8 octet a-1 |
| 0 Spare | 0 Spare | 0 Spare | Standardized access category |  |  |  |  | octet a* |

Figure 9.11.3.38.2: Operator-defined access category definition

Table 9.11.3.38.1: Operator-defined access category definitions information element

| Value part of the Operator-defined access category definitions information element (octet 4 to h) The value part of the Operator-defined access category definitions information element consists of zero or no more than 32 operator-defined access category definition fields. Each operator-defined access category definition field is coded as described in figure 9.11.3.38.2. The length of each operator-defined access category definition field is determined by the length of operator-defined access category definition contents field. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operator-defined access category definition (octet 4 to octet a): |  |  |  |  |  |  |  |  |  |
| Length of operator-defined access category definition contents (octet 4) Length of operator-defined access category definition contents indicates binary coded length of the operator-defined access category definition value field (octet 5 to octet a). Precedence value (octet 5) |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | Precedence value 0 |
| to |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | Precedence value 255 |
| Operator-defined access category number (bits 5 to 1 of octet 6) |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 |  |  |  | Access category number 32 |  |
| to |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 | 1 |  |  |  | Access category number 63 |  |
| Presence of standardized access category (PSAC) (bit 8 of octet 6) |  |  |  |  |  |  |  |  |  |
| PSAC field indicates whether the standardized access category field is present or absent. |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |
| 0 |  | Standardized access category field is not included |  |  |  |  |  |  |  |
| 1 |  | Standardized access category field is included |  |  |  |  |  |  |  |
| Length of criteria (octet 7) |  |  |  |  |  |  |  |  |  |
| Length of criteria field indicates binary coded length of the criteria field. |  |  |  |  |  |  |  |  |  |
| Criteria (octets 8 to octet a-1) |  |  |  |  |  |  |  |  |  |
| The criteria field contains one or more criteria components fields. Each criteria component field shall be encoded as a sequence of a one octet criteria type field and zero or more octets criteria value field. The criteria type field shall be transmitted first. |  |  |  |  |  |  |  |  |  |
| Criteria type |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | DNN type |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | OS id + OS App Id type |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | S-NSSAI type |
| All other values are reserved. |  |  |  |  |  |  |  |  |  |
| For "DNN type", the criteria value field shall be encoded as a sequence of one octet DNN length-value pair count field and one or more DNN length-value pair fields. The DNN length-value pair count field indicates the number of included DNN length-value pair fields. Each DNN length-value pair field is coded as a sequence of one octet DNN value length field and a DNN value field. The DNN value length field indicates the length in octets of the DNN value field. The DNN value field contains an APN as specified in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |
| For "OS Id + OS App Id type", the criteria value field shall be encoded as a sequence of one octet app id value count field and one or more app id value fields. The app id value count field indicates the number of included app id value fields. Each app id value field is coded as a sequence of a sixteen octet OS id value field, one octet OS app id value length field and an OS app id value field. The OS app id value length field indicates the length in octets of the OS app id value field. The OS id value field contains a Universally Unique IDentifier (UUID) as specified in IETF RFC 9562 [35A]. The OS app id value field contains an OS specific application identifier. Coding of the OS app id value field is outside the scope of the present document. |  |  |  |  |  |  |  |  |  |
| For "S-NSSAI type", the criteria value field shall be encoded as a sequence of one octet S-NSSAI length-value pair count field and one or more S-NSSAI length-value value fields. The S-NSSAI length-value pair count field indicates the number of included S-NSSAI length-value pair fields. Each S-NSSAI length-value pair field is coded as a sequence of one octet S-NSSAI value length field and an S-NSSAI value field. The S-NSSAI value length field indicates the length in octets of the S-NSSAI value field. The S-NSSAI value field contains one octet SST field optionally followed by three octets SD field. The SST field contains a SST. The SD field contains an SD. SST and SD are specified in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |
| Standardized access category (bits 5 to 1 of octet a) |  |  |  |  |  |  |  |  |  |
| Standardized access category field indicates the access category number of the standardized access category that is used in combination with the access identities to determine the establishment cause. |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 |  |  |  | Access category number 0 |  |
| to |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 1 | 1 | 1 |  |  |  | Access category number 7 |  |
| 0 | 1 | 0 | 0 | 1 |  |  |  | Access category number 9 |  |
| 0 | 1 | 0 | 1 | 0 |  |  |  | Access category number 10 |  |
| All other values are reserved. |  |  |  |  |  |  |  |  |  |

#### 9.11.3.39	Payload container

The purpose of the Payload container information element is to transport one or multiple payloads. If multiple payloads are transported, the associated information of each payload are also transported together with the payload.

The Payload container information element is coded as shown in figure 9.11.3.39.1, figure 9.11.3.39.1A, figure 9.11.3.39.1B, figure 9.11.3.39.2, figure 9.11.3.39.3, figure 9.11.3.39.4 and table 9.11.3.39.1.

The Payload container information element is a type 6 information element with a minimum length of 4 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Payload container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of payload container contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet 4 |
| Payload container contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 9.11.3.39.1: Payload container information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of event notification indicators |  |  |  |  |  |  |  | octet 4 |
| Event notification indicator 1 |  |  |  |  |  |  |  | octet 5 octet j |
| Event notification indicator 2 |  |  |  |  |  |  |  | octet j+1* octet k* |
| … |  |  |  |  |  |  |  | octet k+1* … octet l* |
| Event notification indicator n |  |  |  |  |  |  |  | octet l+1* octet m* |

Figure 9.11.3.39.1A: Payload container contents with Payload container type "Event notification"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of event notification indicator n |  |  |  |  |  |  |  | octet l+1 |
| Length of event notification indicator n |  |  |  |  |  |  |  | octet l+2 |
| Value of event notification indicator n |  |  |  |  |  |  |  | octet l+3* octet m* |

Figure 9.11.3.39.1B: Even notification indicator n

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of entries |  |  |  |  |  |  |  | octet 4 |
| Payload container entry 1 |  |  |  |  |  |  |  | octet 5 octet x2 |
| Payload container entry 2 |  |  |  |  |  |  |  | octet x2+1 octet x3 |
| …… |  |  |  |  |  |  |  | … |
| Payload container entry i |  |  |  |  |  |  |  | octet xi +1 octet n |

Figure 9.11.3.39.2: Payload container contents with Payload container type "Multiple payloads"

| 8 | 7 | 6 | 5 |  | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Payload container entry |  |  |  |  |  |  |  |  | octet xi +1 octet xi +2 |
| Number of optional IEs |  |  |  | Payload container type |  |  |  |  | octet xi +3 |
| Optional IE 1 |  |  |  |  |  |  |  |  | octet xi +4 octet y2 |
| Optional IE 2 |  |  |  |  |  |  |  |  | octet y2+1 octet y3 |
| … |  |  |  |  |  |  |  |  |  |
| Optional IE j |  |  |  |  |  |  |  |  | octet yj+1 octet z |
| Payload container entry contents |  |  |  |  |  |  |  |  | octet z+1 octet n |

Figure 9.11.3.39.3: Payload container entry

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of optional IE |  |  |  |  |  |  |  | octet xi +4 |
| Length of optional IE |  |  |  |  |  |  |  | octet xi +5 |
| Value of optional IE |  |  |  |  |  |  |  | octet xi +6 octet y2 |

Figure 9.11.3.39.4: Optional IE

Table 9.11.3.39.1: Payload container information element

| Payload container contents (octet 4 to octet n); max value of 65535 octets |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| If the payload container type is set to "N1 SM information" and is included in the UL NAS TRANSPORT or DL NAS TRANSPORT message, the payload container contents contain a 5GSM message as defined in clause 8.3. If the payload container type is set to "SOR transparent container" and is included in the DL NAS TRANSPORT message, the payload container contents are coded the same way as the contents of the SOR transparent container IE (see clause 9.11.3.51) for SOR data type is set to value "0" except that the first three octets are not included. If the payload container type is set to "SOR transparent container" and is included in the UL NAS TRANSPORT message, the payload container contents are coded the same way as the contents of the SOR transparent container IE (see clause 9.11.3.51) for SOR data type is set to value "1" except that the first three octets are not included. If the payload container type is set to "UE policy container" and is included in the DL NAS TRANSPORT, UL NAS TRANSPORT or REGISTRATION REQUEST message, the payload container contents are coded as defined in Annex D. If the payload container type is set to "UE parameters update transparent container" and is included in the DL NAS TRANSPORT message, the payload container contents are coded the same way as the contents of the UE parameters update transparent container IE (see clause 9.11.3.53A) for UE parameters update data type is set to value "0" except that the first three octets are not included. If the payload container type is set to "UE parameters update transparent container" and is included in the UL NAS TRANSPORT message, the payload container contents are coded the same way as the contents of the UE parameters update transparent container IE (see clause 9.11.3.53A) for UE parameters update data type is set to value "1" except that the first three octets are not included. If the payload container type is set to "SMS" and is included in the UL NAS TRANSPORT or DL NAS TRANSPORT message, the payload container contents contain an SMS message (i.e. CP-DATA, CP-ACK or CP-ERROR) as defined in clause 7.2 in 3GPP TS 24.011 [13]. If the payload container type is set to "CIoT user data container" and is included in the UL NAS TRANSPORT, DL NAS TRANSPORT or CONTROL PLANE SERVICE REQUEST message, the payload container contents are coded the same way as the contents of the user data container IE (see clause 9.9.4.24 in 3GPP TS 24.301 [15]) except that the first three octets are not included. If the payload container type is set to "SMS" and is included in the CONTROL PLANE SERVICE REQUEST message, the payload container contents are coded the same way as the contents of the NAS message container IE (see clause 9.9.3.22 in 3GPP TS 24.301 [15]) except that the first two octets are not included. If the payload container type is set to "Location services message container" and is included in the UL NAS TRANSPORT, DL NAS TRANSPORT or CONTROL PLANE SERVICE REQUEST message, the payload container contents include location services message payload. If the payload container type is set to "LTE Positioning Protocol (LPP) message container" and is included in the UL NAS TRANSPORT or DL NAS TRANSPORT message, the payload container contents include LPP message payload. If the payload container type is set to "SLPP message container" and is included in the UL NAS TRANSPORT or DL NAS TRANSPORT message, the payload container contents include SLPP message payload. If the payload container type is set to "Service-level-AA container" and is included in the UL NAS TRANSPORT or DL NAS TRANSPORT message, the payload container contents are coded the same way as the contents of service-level-AA container (see clause 9.11.2.10). If the payload container type is set to "Event notification", the payload container contents include one or more event notification indicators. |  |  |  |  |  |  |  |  |  |
| Type of event notification indicator n (octet l+1) Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | "SRVCC handover cancelled, IMS session re-establishment required" indicator |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  |  |
| to |  |  |  |  |  |  |  |  | Unused, shall be ignored if received by the UE |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |
| If the type of an event notification indicator is set to "SRVCC handover cancelled, IMS session re-establishment required" indicator, the value of the event notification indicator shall not be included. |  |  |  |  |  |  |  |  |  |
| If the payload container type is set to "UPP-CMI container" and is included in the UL NAS TRANSPORT or DL NAS TRANSPORT message, the payload container contents include UPP-CMI messages as specified in 3GPP TS 24.572 [64]. |  |  |  |  |  |  |  |  |  |
| If the payload container type is set to "Multiple payloads", the number of entries field represents the total number of payload container entries, and the payload container entry contents field is coded as a list of payload container entry according to figure 9.11.3.39.2, with each payload container entry is coded according to figure 9.11.3.39.3 and figure 9.11.3.39.4. |  |  |  |  |  |  |  |  |  |
| The coding of Payload container contents is dependent on the particular application. |  |  |  |  |  |  |  |  |  |
| Payload container entry For each payload container entry, the payload container type field represents the payload container type value as described in clause 9.11.3.40, the coding of payload container contents field is dependent on the particular application, and the number of optional IEs field represents the total number of optional IEs associated with the payload container entry contents field in the payload container entry. The error handlings for optional IEs specified in clauses 7.6.3 and 7.7.1 shall apply to the optional IEs included in the payload container entry. The receiving entity shall ignore Optional IEs with type of optinal IE parameter field containing an unknown IEI. |  |  |  |  |  |  |  |  |  |
| Optional IEs Type of optional IE (octet xi +4) This field contains the IEI of the optional IE. Length of optional IE (octet xi+5) This field indicates binary coded length of the value of the optional IE entry. Value of optional IE (octet xi+6 to octet y2) This field contains the value of the optional IE entry with the value part of the referred information element based on following optional IE reference. If the Request type is included, the value part of the Request type shall be encoded in the bits 1 to 4 and bits 5 to 8 shall be coded as zero. If the Release assistance indication is included, the value part of the Release assistance indication shall be encoded in the bits 1 to 4 and bits 5 to 8 shall be coded as zero. If the MA PDU session information is included, the value part of the MA PDU session information shall be encoded in the bits 1 to 4 and bits 5 to 8 shall be coded as zero. |  |  |  |  |  |  |  |  |  |
| IEI |  | Optional IE name |  |  |  |  |  |  | Optional IE reference |
| 12 |  | PDU session ID |  |  |  |  |  |  | PDU session identity 2 (see clause 9.11.3.41) |
| 24 |  | Additional information |  |  |  |  |  |  | Additional information (see clause 9.11.2.1) |
| 58 |  | 5GMM cause |  |  |  |  |  |  | 5GMM cause (see clause 9.11.3.2) |
| 37 |  | Back-off timer value |  |  |  |  |  |  | GPRS timer 3 (see clause 9.11.2.5) |
| 59 |  | Old PDU session ID |  |  |  |  |  |  | PDU session identity 2 (see clause 9.11.3.41) |
| 80 |  | Request type |  |  |  |  |  |  | Request type (see clause 9.11.3.47) |
| 22 |  | S-NSSAI |  |  |  |  |  |  | S-NSSAI (see clause 9.11.2.8) |
| 25 |  | DNN |  |  |  |  |  |  | DNN (see clause 9.11.2.1B) |
| F0 |  | Release assistance indication |  |  |  |  |  |  | Release assistance indication (see clause 9.11.3.46A) |
| A0 |  | MA PDU session information |  |  |  |  |  |  | MA PDU session information (see clause 9.11.3.31A) |

#### 9.11.3.40	Payload container type

The purpose of the Payload container type information element indicates type of payload included in the payload container information element.

The Payload container type information element is coded as shown in figure 9.11.3.40.1 and table 9.11.3.40.1.

The Payload container type information element is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Payload container type IEI |  |  |  |  | Payload container type value |  |  |  |  | octet 1 |  |

Figure 9.11.3.40.1: Payload container type information element

Table 9.11.3.40.1: Payload container type information element

| Payload container type value (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | N1 SM information |
| 0 | 0 | 1 | 0 | SMS |
| 0 | 0 | 1 | 1 | LTE Positioning Protocol (LPP) message container |
| 0 | 1 | 0 | 0 | SOR transparent container |
| 0 | 1 | 0 | 1 | UE policy container |
| 0 | 1 | 1 | 0 | UE parameters update transparent container |
| 0 | 1 | 1 | 1 | Location services message container (see 3GPP TS 23.273 [6B]) |
| 1 | 0 | 0 | 0 | CIoT user data container |
| 1 | 0 | 0 | 1 | Service-level-AA container |
| 1 | 0 | 1 | 0 | Event notification |
| 1 | 0 | 1 | 1 | UPP-CMI container |
| 1 | 1 | 0 | 0 | SLPP message container |
| 1 | 1 | 1 | 1 | Multiple payloads |
| All other values are reserved. |  |  |  |  |
| NOTE: The value "Multiple payloads" is only used when the Payload container contents in figure 9.11.3.39.1 contains multiple payloads as shown in figure 9.11.3.39.2. |  |  |  |  |

#### 9.11.3.41	PDU session identity 2

The purpose of the PDU session identity 2 information element is to indicate the identity of a PDU session in a 5GMM message.

The PDU session identity 2 information element is coded as shown in figure 9.11.3.41.1 and table 9.11.3.41.1.

The PDU session identity 2 is a type 3 information element with a length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU session identity 2 IEI |  |  |  |  |  |  |  | octet 1 |
| PDU session identity 2 value |  |  |  |  |  |  |  | octet 2 |

Figure 9.11.3.41.1: PDU session identity 2 information element

Table 9.11.3.41.1: PDU session identity 2 information element

PDU session identity 2 value (octet 2) The coding of the PDU session identity 2 value is identical to the coding of the PDU session identity value as defined in 3GPP TS 24.007 [11] .

#### 9.11.3.42	PDU session reactivation result

The purpose of the PDU session reactivation result information element is to indicate the result of establishments of user-plane resources of PDU sessions.

The PDU session reactivation result information element is coded as shown in figure 9.11.3.42.1 and table 9.11.3.42.1.

The PDU session reactivation result is a type 4 information element with minimum length of 4 octets and maximum length of 34 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU session reactivation result IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PDU session reactivation result |  |  |  |  |  |  |  | octet 2 |
| PSI (7) | PSI (6) | PSI (5) | PSI (4) | PSI (3) | PSI (2) | PSI (1) | PSI (0) | octet 3 |
| PSI (15) | PSI (14) | PSI (13) | PSI (12) | PSI (11) | PSI (10) | PSI (9) | PSI (8) | octet 4 |
| 0; 0; 0; 0; 0; 0; 0; 0. Spare. | octet 5* -34* |

Figure 9.11.3.42.1: PDU session reactivation result information element

Table 9.11.3.42.1: PDU session reactivation result information element

PSI(x) shall be coded as follows: PSI(0): Bit 1 of octet 3 is spare and shall be coded as zero. PSI(1) – PSI(15): 0 indicates establishment of user-plane resources of the PDU session was not requested in the Uplink data status IE or establishment of user-plane resources of the PDU session was not allowed in the Allowed PDU session status IE or establishment of user-plane resource of the PDU session is successful. 1 indicates either establishment of user-plane resources of the PDU session was requested in the Uplink data status IE but establishment of user-plane resource of the PDU session is not successful or indicates establishment of user-plane resources of the PDU session was allowed in the Allowed PDU session status IE but establishment of user-plane resource of the PDU session is either not performed or not successful. All bits in octet 5 to 34 are spare and shall be coded as zero, if the respective octet is included in the information element.

#### 9.11.3.43	PDU session reactivation result error cause

The purpose of the PDU session reactivation result error cause information element is to indicate error causes for PDU session ID(s) where there was a failure to establish the user-plane resources.

The PDU session reactivation result error cause information element is coded as shown in figure 9.11.3.43.1 and table 9.11.3.43.1.

The PDU session reactivation result error cause is a type 6 information element with a minimum length of 5 octets and a maximum length of 515 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU session reactivation result error cause IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PDU session reactivation result error cause |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| PDU session ID |  |  |  |  |  |  |  | octet 4 |
| cause value |  |  |  |  |  |  |  | octet 5 |
| …. |  |  |  |  |  |  |  |  |
| PDU session ID |  |  |  |  |  |  |  | octet 514* |
| cause value |  |  |  |  |  |  |  | octet 515* |

Figure 9.11.3.43.1: PDU session reactivation result error cause information element

Table 9.11.3.43.1: PDU session reactivation result error cause information element

PDU session ID is coded same as PDU session ID IE (see clause 9.4). The cause value is coded same as second octet of 5GMM cause information element (see clause 9.11.3.2).

#### 9.11.3.44	PDU session status

The purpose of the PDU session status information element is to indicate the state of each PDU session that can be identified by a PDU session identity.

The PDU session status information element is coded as shown in figure 9.11.3.44.1 and table 9.11.3.44.1.

The PDU session status information element is a type 4 information element with minimum length of 4 octets and a maximum length of 34 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU session status IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PDU session status contents |  |  |  |  |  |  |  | octet 2 |
| PSI (7) | PSI (6) | PSI (5) | PSI (4) | PSI (3) | PSI (2) | PSI (1) | PSI (0) | octet 3 |
| PSI (15) | PSI (14) | PSI (13) | PSI (12) | PSI (11) | PSI (10) | PSI (9) | PSI (8) | octet 4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | octet 5*- |
| spare |  |  |  |  |  |  |  | 34* |

Figure 9.11.3.44.1: PDU session status information element

Table 9.11.3.44.1: PDU session status information element

PSI(x) shall be coded as follows: PSI(0): Bit 1 of octet 3 is spare and shall be coded as zero. PSI(1) – PSI(15): 0 indicates that the 5GSM state of the corresponding PDU session is PDU SESSION INACTIVE. 1 indicates that the 5GSM state of the corresponding PDU session is not PDU SESSION INACTIVE All bits in octet 5 to 34 are spare and shall be coded as zero, if the respective octet is included in the information element.

#### 9.11.3.45	PLMN list

See clause 10.5.1.13 in 3GPP TS 24.008 [12].

#### 9.11.3.46	Rejected NSSAI

The purpose of the Rejected NSSAI information element is to identify a collection of rejected S-NSSAIs.

The Rejected NSSAI information element is coded as shown in figure 9.11.3.46.1, figure 9.11.3.46.2 and table 9.11.3.46.1.

The Rejected NSSAI is a type 4 information element with a minimum length of 4 octets and a maximum length of 42 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rejected NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Rejected NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| Rejected S-NSSAI 1 |  |  |  |  |  |  |  | octet 3 octet m |
| Rejected S-NSSAI 2 |  |  |  |  |  |  |  | octet m+1* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* octet u* |
| Rejected S-NSSAI n |  |  |  |  |  |  |  | octet u+1* octet v* |

Figure 9.11.3.46.1: Rejected NSSAI information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of rejected S-NSSAI |  |  |  | Cause value |  |  |  | octet 3 |
| SST |  |  |  |  |  |  |  | octet 4 |
| SD |  |  |  |  |  |  |  | octet 5* octet 7* |

Figure 9.11.3.46.2: Rejected S-NSSAI

Table 9.11.3.46.1: Rejected NSSAI information element

| Value part of the Rejected NSSAI information element (octet 3 to v) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The value part of the Rejected NSSAI information element consists of one or more rejected S-NSSAIs. Each rejected S-NSSAI consists of one S-NSSAI and an associated cause value. The length of each rejected S-NSSAI can be determined by the 'length of rejected S-NSSAI' field in the first octet of the rejected S-NSSAI. |  |  |  |  |  |
| The UE shall store the complete list received (NOTE 0). If more than 8 rejected S-NSSAIs are included in this information element, the UE shall store the first 8 rejected S-NSSAIs and ignore the remaining octets of the information element. |  |  |  |  |  |
| Rejected S-NSSAI: |  |  |  |  |  |
| Cause value (octet 3) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 |  | S-NSSAI not available in the current PLMN or SNPN |
| 0 | 0 | 0 | 1 |  | S-NSSAI not available in the current registration area |
| 0 | 0 | 1 | 0 |  | S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization |
| All other values are reserved. |  |  |  |  |  |
| Slice/service type (SST) (octet 4) |  |  |  |  |  |
| This field contains the 8 bit SST value. The coding of the SST value part is defined in 3GPP TS 23.003 [4]. (NOTE 2) |  |  |  |  |  |
| Slice differentiator (SD) (octet 5 to octet 7) |  |  |  |  |  |
| This field contains the 24 bit SD value. The coding of the SD value part is defined in 3GPP TS 23.003 [4]. (NOTE 3) |  |  |  |  |  |
| NOTE 0: The number of rejected S-NSSAI(s) shall not exceed eight. NOTE 1: If octet 5 is included, then octet 6 and octet 7 shall be included. NOTE 2: If the Cause value is "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization", this field shall contain the 8 bit SST value of an S-NSSAI in the S-NSSAI(s) of the HPLMN. NOTE 3: If the Cause value is "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization", this field, if included, shall contain the 24 bit SD value of an S-NSSAI in the S-NSSAI(s) of the HPLMN. |  |  |  |  |  |

#### 9.11.3.46A	Release assistance indication

See clause 9.9.4.25 in 3GPP TS 24.301 [15].

#### 9.11.3.47	Request type

The purpose of the Request type information element is to indicate the type of the 5GSM message.

The Request type information element is coded as shown in figure 9.11.3.47.1 and table 9.11.3.47.1.

The Request type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request type IEI |  |  |  | 0 spare | Request type value |  |  | octet 1 |

Figure 9.11.3.47.1: Request type information element

Table 9.11.3.47.1: Request type information element

| Request type value (octet 1, bit 1 to bit 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 1 |  | initial request |
| 0 | 1 | 0 |  | existing PDU session |
| 0 | 1 | 1 |  | initial emergency request |
| 1 | 0 | 0 |  | existing emergency PDU session |
| 1 | 0 | 1 |  | modification request |
| 1 | 1 | 0 |  | MA PDU request (NOTE) |
| 1 | 1 | 1 |  | reserved |
| All other values are unused and shall be interpreted as "initial request", if received by the network. |  |  |  |  |
| NOTE: This value shall be interpreted as "initial request", if received by a network not supporting MA PDU sessions. |  |  |  |  |

#### 9.11.3.48	S1 UE network capability

See clause 9.9.3.34 in 3GPP TS 24.301 [15].

#### 9.11.3.48A	S1 UE security capability

See clause 9.9.3.36 in 3GPP TS 24.301 [15].

#### 9.11.3.49	Service area list

The purpose of the Service area list information element is to transfer a list of allowed tracking areas for an allowed area or a list of non-allowed tracking areas for a non-allowed area from the network to the UE.

The coding of the information element allows combining different types of lists. The lists of type "00" and "01" allow a more compact encoding, when the different TAIs are sharing the PLMN identity. The lists of type "11" indicate all TAIs of the PLMNs in the registration area are allowed area.

The Service area list information element is coded as shown in figure 9.11.3.49.1, figure 9.11.3.49.2, figure 9.11.3.49.3, figure 9.11.3.49.4, figure 9.11.3.49.5 and table 9.11.3.49.1.

The Service area list is a type 4 information element with a minimum length of 6 octets and a maximum length of 114 octets. The list can contain a maximum of 16 different tracking area identities.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service area list IEI |  |  |  |  |  |  |  | octet 1 |
| Length of service area list contents |  |  |  |  |  |  |  | octet 2 |
| Partial service area list 1 |  |  |  |  |  |  |  | octet 3 octet i |
| Partial service area list 2 |  |  |  |  |  |  |  | octet i+1* octet l* |
| … |  |  |  |  |  |  |  | octet l+1* octet m* |
| Partial service area list p |  |  |  |  |  |  |  | octet m+1* octet n* |

Figure 9.11.3.49.1: Service area list information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allowed type | Type of list |  | Number of elements |  |  |  |  | octet 1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 4 |
| TAC 1 |  |  |  |  |  |  |  | octet 5 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 6 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 7 |
| … |  |  |  |  |  |  |  | … |
| TAC k |  |  |  |  |  |  |  | octet 3k+2* |
| TAC k (continued) |  |  |  |  |  |  |  | octet 3k+3* |
| TAC k (continued) |  |  |  |  |  |  |  | octet 3k+4* |

Figure 9.11.3.49.2: Partial service area list – type of list = "00"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allowed type | Type of list |  | Number of elements |  |  |  |  | octet 1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 4 |
| TAC 1 |  |  |  |  |  |  |  | octet 5 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 6 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 7 |

Figure 9.11.3.49.3: Partial service area list – type of list = "01"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allowed type | Type of list |  | Number of elements |  |  |  |  | octet 1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 4 |
| TAC 1 |  |  |  |  |  |  |  | octet 5 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 6 |
| TAC 1 (continued) |  |  |  |  |  |  |  | octet 7 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 8* |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 9* |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 10* |
| TAC 2 |  |  |  |  |  |  |  | octet 11* |
| TAC 2 (continued) |  |  |  |  |  |  |  | octet 12* |
| TAC 2 (continued) |  |  |  |  |  |  |  | octet 13* |
| … |  |  |  |  |  |  |  |  |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 6k-4* |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 6k-3* |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 6k-2* |
| TAC k |  |  |  |  |  |  |  | octet 6k*-1 |
| TAC k (continued) |  |  |  |  |  |  |  | octet 6k* |
| TAC k (continued) |  |  |  |  |  |  |  | octet 6k+1* |

Figure 9.11.3.49.4: Partial service area list – type of list = "10"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allowed type | Type of list |  | Number of elements |  |  |  |  | octet 1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 4 |

Figure 9.11.3.49.5: Partial service area list – type of list = "11"

Table 9.11.3.49.1: Service area list information element

| Value part of the Service area list information element (octets 3 to n) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The value part of the Service area list information element consists of one or several partial service area lists. The length of each partial service area list can be determined from the 'type of list' field and the 'number of elements' field in the first octet of the partial service area list. |  |  |  |  |  |
| The "Allowed type" fields in all the partial service area lists shall have the same value. For allowed type "0", TAIs contained in all partial service area lists are in the allowed area. For allowed type "1", TAIs contained in all partial service area lists are in the non-allowed area. The UE shall store the complete list received. If more than 16 TAIs are included in this information element, the UE shall store the first 16 TAIs and ignore the remaining octets of the information element. |  |  |  |  |  |
| Partial service area list: |  |  |  |  |  |
| Allowed type (octet 1) |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 0 |  | TAIs in the list are in the allowed area |  |  |  |
| 1 |  | TAIs in the list are in the non-allowed area |  |  |  |
| Type of list (octet 1) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 7 | 6 |  |  |  |  |
| 0 | 0 | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |  |  |
| 0 | 1 | list of TACs belonging to one PLMN, with consecutive TAC values |  |  |  |
| 1 | 0 | list of TAIs belonging to different PLMNs (see NOTE) |  |  |  |
| 1 | 1 | All TAIs belonging to the PLMNs in the registration area are in the allowed area |  |  |  |
| Number of elements (octet 1) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | 0 | 1 element |
| 0 | 0 | 0 | 0 | 1 | 2 elements |
| 0 | 0 | 0 | 1 | 0 | 3 elements |
| to |  |  |  |  |  |
| 0 | 1 | 1 | 0 | 1 | 14 elements |
| 0 | 1 | 1 | 1 | 0 | 15 elements |
| 0 | 1 | 1 | 1 | 1 | 16 elements |
| All other values are unused and shall be interpreted as 16, if received by the UE. |  |  |  |  |  |
| For type of list = "00" and number of elements = k: |  |  |  |  |  |
| octets 2 to 4 contain the MCC+MNC, and |  |  |  |  |  |
| for j = 1, …, k: |  |  |  |  |  |
| octets 3j+2 to 3j+4 contain the TAC of the j-th TAI belonging to the partial list, |  |  |  |  |  |
| For type of list = "01" and number of elements = k: |  |  |  |  |  |
| octets 2 to 4 contain the MCC+MNC, and |  |  |  |  |  |
| octets 5 to 7 contain the TAC of the first TAI belonging to the partial list. |  |  |  |  |  |
| The TAC values of the other k-1 TAIs are TAC+1, TAC+2, …, TAC+k-1. |  |  |  |  |  |
| For type of list = "10" and number of elements = k: |  |  |  |  |  |
| for j = 1, …, k. |  |  |  |  |  |
| octets 6j-4 to 6j-2 contain the MCC+MNC, and |  |  |  |  |  |
| octets 6j-1 to 6j+1 contain the TAC of the j-th TAI belonging to the partial list. |  |  |  |  |  |
| For type of list = "11": |  |  |  |  |  |
| Allowed type shall be coded as "0" and number of elements shall be ignored, and octets 2 to 4 containing the MCC+MNC can be ignored. If allowed type is coded as "1", it shall be interpreted as "0". |  |  |  |  |  |
| MCC, Mobile country code The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A. |  |  |  |  |  |
| MNC, Mobile network code |  |  |  |  |  |
| The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, MNC digit 3 shall be coded as "1111". |  |  |  |  |  |
| TAC, Tracking area code |  |  |  |  |  |
| In the TAC field bit 8 of the first octet is the most significant bit and bit 1 of the third octet the least significant bit. |  |  |  |  |  |
| The coding of the tracking area code is the responsibility of each administration. Coding using full hexadecimal representation may be used. The tracking area code consists of 3 octets. |  |  |  |  |  |
| NOTE: If the "list of TAIs belonging to different PLMNs" is used, the PLMNs included in the list need to be present in the list of equivalent PLMNs. This type is not applicable in an SNPN. |  |  |  |  |  |

#### 9.11.3.50	Service type

The purpose of the service type information element is to specify the purpose of the service request procedure.

The service type is a type 1 information element.

The service type information element is coded as shown in figure 9.11.3.50.1 and table 9.11.3.50.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service type IEI |  |  |  | Service type value |  |  |  | octet 1 |

Figure 9.11.3.50.1: Service type information element

Table 9.11.3.50.1: Service type information element

| Service type value (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Service type value |  |  |  |  |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | signalling |
| 0 | 0 | 0 | 1 | data |
| 0 | 0 | 1 | 0 | mobile terminated services |
| 0 | 0 | 1 | 1 | emergency services |
| 0 | 1 | 0 | 0 | emergency services fallback |
| 0 | 1 | 0 | 1 | high priority access |
| 0 | 1 | 1 | 0 | elevated signalling |
| 0 | 1 | 1 | 1 | unused; shall be interpreted as "signalling", if received by the network |
| 1 | 0 | 0 | 0 | unused; shall be interpreted as "signalling", if received by the network |
| 1 | 0 | 0 | 1 | unused; shall be interpreted as "data", if received by the network |
| 1 | 0 | 1 | 0 | unused; shall be interpreted as "data", if received by the network |
| 1 | 0 | 1 | 1 | unused; shall be interpreted as "data", if received by the network |
| All other values are reserved. |  |  |  |  |

#### 9.11.3.50A	SMS indication

The purpose of the SMS indication information element is to indicate that the ability for the UE to use SMS over NAS has changed.

The SMS indication information element is coded as shown in figure 9.11.3.50A.1 and table 9.11.3.50A.1.

The SMS indication is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SMS indication IEI |  |  |  | 0 Spare | 0 Spare | 0 Spare | SAI | octet 1 |

Figure 9.11.3.50A.1: SMS indication

Table 9.11.3.50A.1: SMS indication

| SMS availability indication (SAI) (octet 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | SMS over NAS not available |
| 1 | SMS over NAS available |
| Bits 2, 3 and 4 are spare and shall be coded as zero, |  |

#### 9.11.3.51	SOR transparent container

The purpose of the SOR transparent container information element in the REGISTRATION ACCEPT message is to provide the list of preferred PLMN/access technology combinations (or HPLMN indication that 'no change of the "Operator Controlled PLMN Selector with Access Technology" list stored in the UE is needed and thus no list of preferred PLMN/access technology combinations is provided'), or a secured packet (see 3GPP TS 23.122 [5] annex C) and optional indication of an acknowledgement request, SOR-CMCI, request the storage of the received SOR-CMCI in the ME, SOR-SNPN-SI (or subscribed SNPN or HPLMN indication that 'no change of the SOR-SNPN-SI stored in the UE is needed and thus no SOR-SNPN-SI is provided'), and SOR-SNPN-SI-LS. The purpose of the SOR transparent container information element in the REGISTRATION COMPLETE message is to indicate the UE acknowledgement of successful reception of the SOR transparent container IE in the REGISTRATION ACCEPT message as well as to indicate the ME support of SOR-CMCI, the ME support of SOR-SNPN-SI and the ME support of SOR-SNPN-SI-LS.

NOTE 1:	When used in NAS transport procedure, the contents of the SOR transparent container information element in the Payload container IE of the DL NAS TRANSPORT message are used to provide the list of preferred PLMN/access technology combinations and optional indication of an acknowledgement request, SOR-CMCI, request the storage of the received SOR-CMCI in the ME, SOR-SNPN-SI, and SOR-SNPN-SI-LS. The contents of the SOR transparent container information element in the Payload container IE of the UL NAS TRANSPORT message are used to indicate the UE acknowledgement of successful reception of the SOR transparent container IE in the DL NAS TRANSPORT message as well as to indicate the ME support of SOR-CMCI, the ME support of SOR-SNPN-SI and the ME support of SOR-SNPN-SI-LS.

NOTE 2:	The "Operator controlled signal threshold per access technology" content to update the USIM file EFOCST (see 3GPP TS 31.102 [22]) can be included in the secured packet of the SOR transparent container.

The SOR transparent container information element is coded as shown in figure 9.11.3.51.1, figure 9.11.3.51.2, figure 9.11.3.51.3, figure 9.11.3.51.4, figure 9.11.3.51.5, figure 9.11.3.51.6, figure 9.11.3.51.7, figure 9.11.3.51.8, figure 9.11.3.51.9, figure 9.11.3.51.9A, figure 9.11.3.51.10, figure 9.11.3.51.11, figure 9.11.3.51.11A, figure 9.11.3.51.11B, figure 9.11.3.51.11C, figure 9.11.3.51.11D, figure 9.11.3.51.11E, figure 9.11.3.51.11F, figure 9.11.3.51.11G, figure 9.11.3.51.11H, figure 9.11.3.51.11I, figure 9.11.3.51.12, figure 9.11.3.51.13, table 9.11.3.51.1, table 9.11.3.51.2, table 9.11.3.51.3, table 9.11.3.51.4, table 9.11.3.51.4A, table 9.11.3.51.5 and table 9.11.3.51.6.

The SOR transparent container is a type 6 information element with a minimum length of 20 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SOR transparent container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of SOR transparent container contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| SOR header |  |  |  |  |  |  |  | octet 4 |
| SOR-MAC-IAUSF |  |  |  |  |  |  |  | octet 5-20 |
| CounterSOR |  |  |  |  |  |  |  | octet 21-22 |
| Secured packet |  |  |  |  |  |  |  | octet 23* - n* |

Figure 9.11.3.51.1: SOR transparent container information element for list type with value "0" and SOR data type with value "0"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SOR transparent container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of SOR transparent container contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| SOR header |  |  |  |  |  |  |  | octet 4 |
| SOR-MAC-IAUSF |  |  |  |  |  |  |  | octet 5-20 |
| CounterSOR |  |  |  |  |  |  |  | octet 21-22 |
| PLMN ID and access technology list |  |  |  |  |  |  |  | octet 23*-m* |

Figure 9.11.3.51.2: SOR transparent container information element for list type with value "1", SOR data type with value "0", and additional parameters with value "0"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SOR transparent container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of SOR transparent container contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| SOR header |  |  |  |  |  |  |  | octet 4 |
| SOR-MAC-IAUSF |  |  |  |  |  |  |  | octet 5 octet 20 |
| CounterSOR |  |  |  |  |  |  |  | octet 21 octet 22 |
| Length of PLMN ID and access technology list |  |  |  |  |  |  |  | octet 23* |
| PLMN ID and access technology list |  |  |  |  |  |  |  | octet 24* octet m* |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | SSSLI | SSSI | SSCMI | SI | octet o |
| SOR-CMCI |  |  |  |  |  |  |  | octet (o+1)* octet p* |
| SOR-SNPN-SI |  |  |  |  |  |  |  | octet (p+1)* octet u* |
| SOR-SNPN-SI-LS |  |  |  |  |  |  |  | octet (u+1)* octet v* |

Figure 9.11.3.51.2A: SOR transparent container information element for list type with value "1", SOR data type with value "0", additional parameters with value "1"

| PLMN ID 1 | octet 23*- 25* |
| --- | --- |
| access technology identifier 1 | octet 26*- 27* |
| … |  |
| PLMN ID n | octet (18+5*n)*-(20+5*n)* |
| access technology identifier n | octet (21+5*n)*-(22+5*n)* |

Figure 9.11.3.51.3: PLMN ID and access technology list (m=22+5*n)

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SOR transparent container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of SOR transparent container contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| SOR header |  |  |  |  |  |  |  | octet 4 |
| SOR-MAC-IUE |  |  |  |  |  |  |  | octet 5 - 20 |

Figure 9.11.3.51.4: SOR transparent container information element for SOR data type with value "1"

|  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | AP |  | ACK |  | List type |  | List indication |  | SOR data type |  | octet 4 |  |  |

Figure 9.11.3.51.5: SOR header for SOR data type with value "0"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | MSSSNPNSILS | MSSNPNSI | MSSI | SOR data type | octet 4 |

Figure 9.11.3.51.6: SOR header for SOR data type with value "1"

Table 9.11.3.51.1: SOR transparent container information element

| SOR-MAC-IAUSF (see NOTE 1), SOR-MAC-IUE (see NOTE 2) and CounterSOR (see NOTE 1) are coded as specified in 3GPP TS 33.501 [24]. |  |
| --- | --- |
| SOR data type (octet 4, bit 1) |  |
| 0 | The SOR transparent container carries steering of roaming information. |
| 1 | The SOR transparent container carries acknowledgement of successful reception of the steering of roaming information. |
| List indication (octet 4, bit 2) (see NOTE 1 and NOTE 5) |  |
| 0 | HPLMN indication that 'no change of the "Operator Controlled PLMN Selector with Access Technology" list stored in the UE is needed and thus no list of preferred PLMN/access technology combinations is provided' |
| 1 | list of preferred PLMN/access technology combinations is provided |
| List type (octet 4, bit 3) (see NOTE 1) |  |
| 0 | The list type is a secured packet. |
| 1 | The list type is a "PLMN ID and access technology list". |
| Acknowledgement (ACK) value (octet 4, bit 4) (see NOTE 1) |  |
| 0 | acknowledgement not requested |
| 1 | acknowledgement requested |
| Additional parameters (AP) value (octet 4, bit 5) |  |
| Bit |  |
| 5 |  |
| 0 | Additional parameters not included |
| 1 | Additional parameters included (see NOTE 3) |
| If the SOR data type is set to value "0", the list type bit is set to value "1", and the additional parameters bit is set to value "1" then: - the octet o is present. - if the list indication bit is set to "0" then the PLMN ID and access technology list field and the length of PLMN ID and access technology list field are absent. - if the list indication bit is set to "1" then the PLMN ID and access technology list field and the length of PLMN ID and access technology list field are present. |  |
| The secure packet is coded as specified in 3GPP TS 31.115 [22B]. (see NOTE 1) |  |
| The PLMN ID and access technology list consists of PLMN ID and access technology identifier and are coded as specified in 3GPP TS 31.102 [22] clause 4.2.5. The PLMN ID and access technology identifier are provided in decreasing order of priority, i.e. PLMN ID 1 indicates highest priority and PLMN ID n indicates lowest priority. The PLMN ID and access technology list contains at minimum zero and at maximum 16 (decimal) PLMN IDs and access technology identifiers. (see NOTE 1) |  |
| ME support of SOR-CMCI indicator (MSSI) value (octet 4, bit 2) (see NOTE 2, NOTE 4) |  |
| 0 | SOR-CMCI not supported by the ME |
| 1 | SOR-CMCI supported by the ME |
| ME support of SOR-SNPN-SI indicator (MSSNPNSI) value (octet 4, bit 3) (see NOTE 2, NOTE 6) |  |
| 0 | SOR-SNPN-SI not supported by the ME |
| 1 | SOR-SNPN-SI supported by the ME |
| MS support of SOR-SNPN-SI-LS indicator (MSSNPNSILS) value (octet 4, bit 4) (see NOTE 2) |  |
| Bit |  |
| 4 |  |
| 0 | SOR-SNPN-SI-LS not supported by the ME |
| 1 | SOR-SNPN-SI-LS supported by the ME |
| SOR-CMCI indicator (SI) value (octet o, bit 1) Bit |  |
| 1 |  |
| 0 | SOR-CMCI absent |
| 1 | SOR-CMCI present |
| If the SOR-CMCI indicator bit is set to "SOR-CMCI present", the SOR-CMCI field is present. If the SI bit is set to "SOR-CMCI absent", the SOR-CMCI field is absent. |  |
| Store SOR-CMCI in ME indicator (SSCMI) value (octet o, bit 2) Bit |  |
| 2 |  |
| 0 | Do not store SOR-CMCI in ME |
| 1 | Store SOR-CMCI in ME |
| SOR-CMCI (octet o+1 to octet p) The SOR-CMCI field is coded according to figure 9.11.3.51.7 and table 9.11.3.51.2. |  |
| SOR-SNPN-SI indicator (SSSI) value (octet o, bit 3) Bit |  |
| 3 |  |
| 0 | subscribed SNPN or HPLMN indication that 'no change of the SOR-SNPN-SI stored in the UE is needed and thus no SOR-SNPN-SI is provided' |
| 1 | SOR-SNPN-SI present |
| If the SSSI bit is set to "SOR-SNPN-SI present", the SOR-SNPN-SI field is present. If the SSSI bit is set to "subscribed SNPN or HPLMN indication that 'no change of the SOR-SNPN-SI stored in the UE is needed and thus no SOR-SNPN-SI is provided'", the SOR-SNPN-SI is absent. |  |
| SOR-SNPN-SI-LS indicator (SSSLI) value (octet o, bit 4) Bit |  |
| 4 |  |
| 0 | SOR-SNPN-SI-LS absent |
| 1 | SOR-SNPN-SI-LS present |
| If the SSSLI bit is set to "SOR-SNPN-SI-LS present", the SOR-SNPN-SI-LS field is present. If the SSSLI bit is set to "SOR-SNPN-SI-LS absent", the SOR-SNPN-SI-LS is absent. |  |
| NOTE 1: This bit or field applies for SOR header with SOR data type with value "0". NOTE 2: This bit or field applies for SOR header with SOR data type with value "1". NOTE 3: Additional parameters can be set to value "1" only when the ME supports SOR-CMCI, SOR-SNPN-SI or SOR-SNPN-SI-LS, and the list type bit is set to value "1". The ME supporting SOR-SNPN-SI-LS supports SOR-SNPN-SI as specified in 3GPP TS 23.122 [5]. NOTE 4: The "SOR-CMCI supported by the ME" is not set by a UE compliant to an earlier release of the specification. NOTE 5: This bit or field applies for SOR header with list type with value "1". NOTE 6: The "SOR-SNPN-SI supported by the ME" may only be set by a UE which supports access to an SNPN using credentials from a credentials holder and which is not operating in SNPN access operation mode. |  |

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of SOR-CMCI contents |  |  |  |  |  |  |  | octet (o+1) octet (o+2) |
| SOR-CMCI rule 1 |  |  |  |  |  |  |  | octet (o+3)* octet q* |
| SOR-CMCI rule 2 |  |  |  |  |  |  |  | octet (q+1)* octet r* |
| ... |  |  |  |  |  |  |  | octet (r+1)* octet s* |
| SOR-CMCI rule n |  |  |  |  |  |  |  | octet (s+1)* octet p* |

Figure 9.11.3.51.7: SOR-CMCI

Table 9.11.3.51.2: SOR-CMCI

SOR-CMCI rule: The SOR-CMCI rule is coded according to figure 9.11.3.51.8 and table 9.11.3.51.3.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of SOR-CMCI rule contents |  |  |  |  |  |  |  | octet q+1 octet q+2 |
| Tsor-cm timer value |  |  |  |  |  |  |  | octet q+3 |
| Criterion type |  |  |  |  |  |  |  | octet q+4 |
| Criterion value |  |  |  |  |  |  |  | octet (q+5)* octet r* |

Figure 9.11.3.51.8: SOR-CMCI rule

Table 9.11.3.51.3: SOR-CMCI rule

Tsor-cm timer value The Tsor-cm timer value field is coded according to octet 2 of the GPRS timer information element as specified in 3GPP TS 24.008 [12] clause 10.5.7.3 and indicates the Tsor-cm timer value. When the unit field of the Tsor-cm timer value field indicates that the timer is deactivated, the receiving entity shall consider that Tsor-cm timer value is set to the infinity value.

Criterion type

Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 1 DNN 0 0 0 0 0 0 1 0 S-NSSAI SST 0 0 0 0 0 0 1 1 S-NSSAI SST and SD 0 0 0 0 0 1 0 0 IMS registration related signalling 0 0 0 0 0 1 0 1 MMTEL voice call 0 0 0 0 0 1 1 0 MMTEL video call 0 0 0 0 0 1 1 1 SMS over NAS or SMSoIP 0 0 0 0 1 0 0 0 SOR security check not successful 1 1 1 1 1 1 1 1 match all All other values are spare.

The receiving entity shall ignore SOR-CMCI rule with criterion of criterion type set to a spare value.

For "DNN", the criterion value field is coded as the length and value part of DNN information element as specified in clause 9.11.2.1B starting with the second octet. For "S-NSSAI SST", the criterion value field shall be encoded as one octet SST field. For "S-NSSAI SST and SD", the criterion value field shall be encoded as a sequence of one octet SST field and three octets SD field. The SST field shall be transmitted first. The SST field contains SST of HPLMN's S-NSSAI. The SD field contains SD of HPLMN's S-NSSAI. For "match all", "SOR security check not successful", "IMS registration related signalling", "MMTEL voice call", "MMTEL video call", and "SMS over NAS or SMSoIP", the criterion value field is zero octets long.

If the length of SOR-CMCI rule contents field indicates a length bigger than indicated in figure 9.11.3.51.8, receiving entity shall ignore any superfluous octets located at the end of the SOR-CMCI rule. The UE applies SOR-CMCI rules as described in 3GPP TS 23.122 [5] annex C.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of SOR-SNPN-SI contents |  |  |  |  |  |  |  | octet (p+1) octet (p+2) |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | CLGI | CLSI | octet (p+3) |
| CH controlled prioritized list of preferred SNPNs |  |  |  |  |  |  |  | octet (p+4)* octet t* |
| CH controlled prioritized list of GINs |  |  |  |  |  |  |  | octet (t+1)* octet u* |

Figure 9.11.3.51.9: SOR-SNPN-SI

Table 9.11.3.51.4: SOR-SNPN-SI

| CH controlled prioritized list of preferred SNPNs indicator (CLSI) value (octet p+3, bit 1) Bit |  |
| --- | --- |
| 1 |  |
| 0 | CH controlled prioritized list of preferred SNPNs absent |
| 1 | CH controlled prioritized list of preferred SNPNs present |
| If the CLSI bit is set to "CH controlled prioritized list of preferred SNPNs present", the CH controlled prioritized list of preferred SNPNs field is present. If the CLSI bit is set to "CH controlled prioritized list of preferred SNPNs absent", the CH controlled prioritized list of preferred SNPNs field is absent. |  |
| CH controlled prioritized list of GINs indicator (CLGI) value (octet p+3, bit 2) Bit |  |
| 2 |  |
| 0 | CH controlled prioritized list of GINs absent |
| 1 | CH controlled prioritized list of GINs present |
| If the CLGI bit is set to "CH controlled prioritized list of GINs present", the CH controlled prioritized list of GINs field is present. If the CLGI bit is set to "CH controlled prioritized list of GINs absent", the CH controlled prioritized list of GINs field is absent. |  |
| If the length of SOR-SNPN-SI contents field indicates a length bigger than indicated in figure 9.11.3.51.9, receiving entity shall ignore any superfluous octets located at the end of the SOR-SNPN-SI. |  |

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of SOR-SNPN-SI-LS contents |  |  |  |  |  |  |  | octet (u+1) octet (u+2) |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | CLGLSI | CLSLSI | octet (u+3) |
| CH controlled prioritized list of preferred SNPNs for access for localized services in SNPN |  |  |  |  |  |  |  | octet (w+1)* octet x* |
| CH controlled prioritized list of preferred GINs for access for localized services in SNPN |  |  |  |  |  |  |  | octet (x+1)* octet v* |

Figure 9.11.3.51.9A: SOR-SNPN-SI-LS

Table 9.11.3.51.4A: SOR-SNPN-SI-LS

| CH controlled prioritized list of preferred SNPNs for access for localized services in SNPN indicator (CLSLSI) value (octet u+3, bit 1) Bit |  |
| --- | --- |
| 1 |  |
| 0 | CH controlled prioritized list of preferred SNPNs for access for localized services in SNPN absent |
| 1 | CH controlled prioritized list of preferred SNPNs for access for localized services in SNPN present |
| If the CLSLSI bit is set to " CH controlled prioritized list of preferred SNPNs for access for localized services in SNPN present", the CH controlled prioritized list of preferred SNPNs for access for localized services in SNPN field is present. If the CLSLSI bit is set to " CH controlled prioritized list of preferred SNPNs for access for localized services in SNPN absent", the CH controlled prioritized list of preferred SNPNs for access for localized services in SNPN field is absent. |  |
| CH controlled prioritized list of preferred GINs for access for localized services in SNPN indicator (CLGLSI) value (octet u+3, bit 2) Bit |  |
| 2 |  |
| 0 | CH controlled prioritized list of preferred GINs for access for localized services in SNPN absent |
| 1 | CH controlled prioritized list of preferred GINs for access for localized services in SNPN present |
| If the CLGLSI bit is set to " CH controlled prioritized list of preferred GINs for access for localized services in SNPN present", the CH controlled prioritized list of preferred GINs for access for localized services in SNPN field is present. If the CLGLSI bit is set to " CH controlled prioritized list of preferred GINs for access for localized services in SNPN absent", the CH controlled prioritized list of preferred GINs for access for localized services in SNPN field is absent. |  |
| If the length of SOR-SNPN-SI-LS contents field indicates a length bigger than indicated in figure 9.11.3.51.9A, receiving entity shall ignore any superfluous octets located at the end of the SOR-SNPN-SI. |  |

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of CH controlled prioritized list of preferred SNPNs contents |  |  |  |  |  |  |  | octet p+4 octet p+5 |
| SNPN identity 1 |  |  |  |  |  |  |  | octet (p+6)* octet (p+14)* |
| SNPN identity 2 |  |  |  |  |  |  |  | octet (p+15)* octet (p+23)* |
| ... |  |  |  |  |  |  |  | octet (p+24)* octet (p+n*9-2)* |
| SNPN identity n |  |  |  |  |  |  |  | octet (p+n*9-3)* octet (p+n*9+5)* = octet t* |

Figure 9.11.3.51.10: CH controlled prioritized list of preferred SNPNs

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of CH controlled prioritized list of preferred SNPNs for access for localized services in SNPN contents |  |  |  |  |  |  |  | octet u+1 octet u+2 |
| SNPN info 1 |  |  |  |  |  |  |  | octet (u+3)* octet u1* |
| SNPN info 2 |  |  |  |  |  |  |  | octet (u1+1)* octet u2* |
| ... |  |  |  |  |  |  |  | octet (u2+1)* octet u3* |
| SNPN info n |  |  |  |  |  |  |  | octet (u3+1)* octet v* |

Figure 9.11.3.51.10A: CH controlled prioritized list of preferred SNPNs for access for localized services in SNPN

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Length of SNPN info |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet u+3 octet u+4 |  |
|  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | LVII |  | LAII |  | Time Ind |  | octet u+5 |  |
|  | SNPN identity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (u+6)* octet (u+14)* |  |
|  | Time validity information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (u+15)* octet s* |  |
|  | Location assistance information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (s+1)* octet q* |  |
|  | Location validity information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (q+1)* octet u1* |  |

Figure 9.11.3.51.10B: SNPN info

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet p+15 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet p+16 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet p+17 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | NID assignment mode |  |  |  | octet p+18 |
| NID value digit 2 |  |  |  | NID value digit 1 |  |  |  | octet p+19 |
| NID value digit 4 |  |  |  | NID value digit 3 |  |  |  | octet p+20 |
| NID value digit 6 |  |  |  | NID value digit 5 |  |  |  | octet p+21 |
| NID value digit 8 |  |  |  | NID value digit 7 |  |  |  | octet p+22 |
| NID value digit 10 |  |  |  | NID value digit 9 |  |  |  | octet p+23 |

Figure 9.11.3.51.11: SNPN identity

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of time periods |  |  |  |  |  |  |  | octet (p+18) or (t+15) |
| Time period 1 |  |  |  |  |  |  |  | octet (p+19) or (t+16) octet (p+34) or (t+31) |
| Time period 2 |  |  |  |  |  |  |  | octet (p+35)* or (t+32)* octet (p+50)* or (t+47)* |
| ... |  |  |  |  |  |  |  | octet (p+51)* or (t+48)* octet (p+2+o*16)* or (t-1+o*16)* |
| Time period o |  |  |  |  |  |  |  | octet (p+3+o*16)* or (t+o*16) octet (p+18+o*16)* or (t+15+o*16) = octet s* |

Figure 9.11.3.51.11A: Time validity information

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | TAISI |  | TAIPI |  | GADI |  | octet (s+1) |  |
|  | Geographical area descriptions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (s+2)* octet s3* |  |
|  | Tracking area information of PLMNs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (s3+1)* octet s4* |  |
|  | Tracking area information of SNPNs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (s4+1)* octet s5* = octet q* |  |

Figure 9.11.3.51.11B: Location assistance information

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Number of geographical area descriptions |  |  |  |  |  |  |  |  | octet (s+2) |  |
|  | Type of shape 1 |  |  |  |  |  |  |  |  | octet (s+3) |  |
|  | Shape description 1 |  |  |  |  |  |  |  |  | octet (s+4) octet s1 |  |
|  | … |  |  |  |  |  |  |  |  | octet (s1+1)* octet s2* |  |
|  | Type of shape n |  |  |  |  |  |  |  |  | octet (s2+1)* |  |
|  | Shape description n |  |  |  |  |  |  |  |  | octet (s2+2)* octet s3* |  |

Figure 9.11.3.51.11C: Geographical area descriptions

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Number of tracking area information of PLMNs |  |  |  |  |  |  |  |  | octet s3+1 |  |
|  | Tracking area information of PLMN 1 |  |  |  |  |  |  |  |  | octet (s3+2) octet s8 |  |
|  | Tracking area information of PLMN 2 |  |  |  |  |  |  |  |  | octet (s8+1)* octet s9* |  |
|  | ... |  |  |  |  |  |  |  |  | octet (s9+1)* octet s10* |  |
|  | Tracking area information of PLMN n |  |  |  |  |  |  |  |  | octet (s10+1)* octet s4* |  |

Figure 9.11.3.51.11D: Tracking area information of PLMNs

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet s8+1 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet s8+2 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet s8+3 |
| TAC list |  |  |  |  |  |  |  | octet s8+4 octet s9 |

Figure 9.11.3.51.11E: Tracking area information of PLMN

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Number of TACs |  |  |  |  |  |  |  |  | octet s14 = octet s8+4 or octet s11+10 |  |
|  | TAC 1 |  |  |  |  |  |  |  |  | octet s14+1 octet s14+3 |  |
|  | TAC 2 |  |  |  |  |  |  |  |  | octet (s14+4)* octet (s14+6)* |  |
|  | ... |  |  |  |  |  |  |  |  |  |  |
|  | TAC m |  |  |  |  |  |  |  |  | octet (s14+m*3-2)* octet (s14+m*3)* = octet s9 or octet s12 |  |

Figure 9.11.3.51.11F: TAC list

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Number of tracking area information of SNPNs |  |  |  |  |  |  |  |  | octet s4+1 |  |
|  | Tracking area information of SNPN 1 |  |  |  |  |  |  |  |  | octet (s4+2) octet s11 |  |
|  | Tracking area information of SNPN 2 |  |  |  |  |  |  |  |  | octet (s11+1)* octet s12* |  |
|  | ... |  |  |  |  |  |  |  |  | octet (s12+1)* octet s13* |  |
|  | Tracking area information of SNPN n |  |  |  |  |  |  |  |  | octet (s13+1)* octet s5* |  |

Figure 9.11.3.51.11G: Tracking area information of SNPNs

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNPN identity |  |  |  |  |  |  |  |  | octet s11+1 octet s11+9 |  |
|  | TAC list |  |  |  |  |  |  |  |  | octet s11+10 octet s12 |  |

Figure 9.11.3.51.11H: Tracking area information of SNPN

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | TACI |  | octet (q+1) |  |
|  | TAC list |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (q+2)* octet u1* |  |

Figure 9.11.3.51.11I: Location validity information

Table 9.11.3.51.5: CH controlled prioritized list of preferred SNPNs and CH controlled prioritized list of preferred SNPNs for access for localized services in SNPN

| Mobile country code (MCC): The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A. |  |  |
| --- | --- | --- |
| Mobile network code (MNC): The coding of MNC field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, MNC digit 3 shall be coded as "1111". |  |  |
| NID assignment mode (see NOTE 1) |  |  |
| NID assignment mode is coded as specified in 3GPP TS 23.003 [4]. |  |  |
| NID value (see NOTE 1) |  |  |
| NID value is coded as specified in 3GPP TS 23.003 [4]. |  |  |
| Time indication (bit1 of octet u+5) Bit |  |  |
| 1 |  |  |
| 0 |  | Time validity infomation not included |
| 1 |  | Time validity infomationinformation is included |
| Location assistance information indicator (LAII) (bit 2 of octet u+5) Bit |  |  |
| 2 |  |  |
| 0 |  | Location assistance information not included |
| 1 |  | Location assistance information included |
| Location validity information indicator (LVII) (bit 3 of octet u+5) Bit |  |  |
| 3 |  |  |
| 0 |  | Location validity information not included |
| 1 |  | Location validity information included |
| Tracking area code list indicator (TACI) (bit 1 of octet q+1) (see NOTE 2) Bit |  |  |
| 1 |  |  |
| 0 |  | TAC list not included |
| 1 |  | TAC list included |
| Geographical area descriptions indicator (GADI) (bit 1 of octet s+1) Bit |  |  |
| 1 |  |  |
| 0 |  | Geographical area descriptions not included |
| 1 |  | Geographical area descriptions included |
| Tracking area information of PLMNs (TAIPI) (bit 2 of octet s+1) Bit |  |  |
| 2 |  |  |
| 0 |  | Tracking area information of PLMNs not included |
| 1 |  | Tracking area information of PLMNs included |
| Tracking area information of SNPNs (TAISI) (bit 3 of octet s+1) Bit |  |  |
| 3 |  |  |
| 0 |  | Tracking area information of SNPNs not included |
| 1 |  | Tracking area information of SNPNs included |
| Location assistance information |  |  |
| Location assistance information field contains the geographical area descriptions field, the tracking area information of PLMNs field, the tracking area information of SNPNs field. or any combination of them. |  |  |
| Time period |  |  |
| The time period field is coded as the route selection descriptor component value field for "time window type" specified in 3GPP TS 24.526 [19] table 5.2.1. |  |  |
| Type of shape |  |  |
| Type of shape is coded as specified in Table 2a in 3GPP TS 23.032 [4B]. |  |  |
| Shape description |  |  |
| Shape description is coded as specified in clause 7.3 in 3GPP TS 23.032 [4B]. |  |  |
| Tracking area information of PLMNs |  |  |
| Tracking area information of PLMNs field contains one or more tracking area information of PLMN fields. |  |  |
| Tracking area information of PLMN |  |  |
| Tracking area information of PLMN field contains an MCC and MNC of a PLMN identity and a TAC list field containing one or more tracking area code fields, each indicating a TAC of a PLMN identified by the PLMN identity. |  |  |
| Tracking area information of SNPNs |  |  |
| Tracking area information of SNPNs field contains one or more tracking area information of SNPN fields. If the tracking area information of SNPNs field is included in a location assistance information field of an SNPN info field, the SNPN identity field in each tracking area information of SNPN field of the tracking area information of SNPNs field is different from the SNPN identity field in the SNPN info field. |  |  |
| Tracking area information of SNPN |  |  |
| Tracking area information of SNPN field contains SNPN identity and a TAC list containing one or more tracking area code fields, each indicating a TAC of an SNPN identified by the SNPN identity. |  |  |
| Tracking area code (TAC) |  |  |
| In the TAC field bit 8 of the first octet is the most significant bit and bit 1 of third octet the least significant bit. The coding of the tracking area code is the responsibility of each administration. Coding using full hexadecimal representation may be used. The tracking area code consists of 3 octets. |  |  |
| NOTE 1: NID coding deviates from coding of value part of NID IE as specified in clause 9.2.7 of 3GPP TS 24.502 [18], coding of the NID field of the SNPN list IE as specified in clause 9.11.3.92, and coding of the NID field of the SNPN list with trusted 5G connectivity IE as specified in clause H.2.4.7 of 3GPP TS 24.302 [16]. NOTE 2: If the location validity information indicator is set to 'Location validity information included' but TAC list indicator is set to 'TAC list not included', the UE shall ignore the location validity information. |  |  |

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of CH controlled prioritized list of GINs contents |  |  |  |  |  |  |  | octet t+1 octet t+2 |
| GIN 1 |  |  |  |  |  |  |  | octet (t+3)* octet (t+11)* |
| GIN 2 |  |  |  |  |  |  |  | octet (t+12)* octet (t+20)* |
| ... |  |  |  |  |  |  |  | octet (t+21)* octet (t+n*9-5)* |
| GIN n |  |  |  |  |  |  |  | octet (t+n*9-6)* octet (t+n*9+2)* = octet u* |

Figure 9.11.3.51.12: CH controlled prioritized list of GINs

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of CH controlled prioritized list of preferred GINs for access for localized services in SNPN contents |  |  |  |  |  |  |  | octet v+1 octet v+2 |
| GIN info 1 |  |  |  |  |  |  |  | octet (v+3)* octet v1* |
| GIN info 2 |  |  |  |  |  |  |  | octet (v1+1)* octet v2* |
| ... |  |  |  |  |  |  |  | octet (v2+1)* octet v3* |
| GIN info n |  |  |  |  |  |  |  | octet (v3+1)* octet w* |

Figure 9.11.3.51.12A: CH controlled prioritized list of preferred GINs for access for localized services in SNPN

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Length of GIN info |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet v+3 octet v+4 |  |
|  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | LVII |  | LAII |  | Time Ind |  | octet v+5 |  |
|  | GIN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (v+6)* octet (v+14)* |  |
|  | Time validity information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (v+15)* octet s* |  |
|  | Location assistance information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (s+1)* octet q* |  |
|  | Location validity information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (q+1)* octet v1* |  |

Figure 9.11.3.51.12B: GIN info

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet t+12 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet t+13 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet t+14 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | NID assignment mode |  |  |  | octet t+15 |
| NID value digit 2 |  |  |  | NID value digit 1 |  |  |  | octet t+16 |
| NID value digit 4 |  |  |  | NID value digit 3 |  |  |  | octet t+17 |
| NID value digit 6 |  |  |  | NID value digit 5 |  |  |  | octet t+18 |
| NID value digit 8 |  |  |  | NID value digit 7 |  |  |  | octet t+19 |
| NID value digit 10 |  |  |  | NID value digit 9 |  |  |  | octet t+20 |

Figure 9.11.3.51.13: GIN

Table 9.11.3.51.6: CH controlled prioritized list of GINs and CH controlled prioritized list of preferred GINs for access for localized services in SNPN

| Mobile country code (MCC): The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A. |  |  |
| --- | --- | --- |
| Mobile network code (MNC): The coding of MNC field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, MNC digit 3 shall be coded as "1111". |  |  |
| NID assignment mode (see NOTE) |  |  |
| NID assignment mode is coded as specified in 3GPP TS 23.003 [4]. |  |  |
| NID value (see NOTE) |  |  |
| NID value is coded as specified in 3GPP TS 23.003 [4]. |  |  |
| Time indication (bit1 of octet v+5) Bit |  |  |
| 1 |  |  |
| 0 |  | Time validity information not included |
| 1 |  | Time validity information included |
| Location assistance information indicator (LAII) (bit 2 of octet v+5) Bit |  |  |
| 2 |  |  |
| 0 |  | Location assistance information not included |
| 1 |  | Location assistance information included |
| Location validity information indicator (LVII) (bit 3 of octet v+5) Bit |  |  |
| 3 |  |  |
| 0 |  | Location validity information not included |
| 1 |  | Location validity information included |
| Time period |  |  |
| The time period field is coded as the route selection descriptor component value field for "time window type" specified in 3GPP TS 24.526 [19] table 5.2.1. |  |  |
| Location assistance information (oct(s+1)* to (q)*) |  |  |
| The Location assistance information is coded according to Figure 9.11.3.51.11B and Table 9.11.3.51.5. |  |  |
| NOTE: NID coding deviates from coding of value part of NID IE as specified in clause 9.2.7 of 3GPP TS 24.502 [18], coding of the NID field of the SNPN list IE as specified in clause 9.11.3.92, and coding of the NID field of the SNPN list with trusted 5G connectivity IE as specified in clause H.2.4.7 of 3GPP TS 24.302 [16]. |  |  |

#### 9.11.3.51A	Supported codec list

See clause 10.5.4.32 in 3GPP TS 24.008 [12].

#### 9.11.3.52	Time zone

See clause 10.5.3.8 in 3GPP TS 24.008 [12].

#### 9.11.3.53	Time zone and time

See clause 10.5.3.9 in 3GPP TS 24.008 [12].

#### 9.11.3.53A	UE parameters update transparent container

The purpose of the UE parameters update transparent container when sent from the network to the UE is to provide UE parameters update data, optional acknowledgement request and optional re-registration request. The purpose of the UE parameters update transparent container when sent from the UE to the network is to indicate the UE acknowledgement of successful reception of the UE parameters update transparent container.

The UE parameters update transparent container information element is coded as shown in figure 9.11.3.53A.1, figure 9.11.3.53A.2, figure 9.11.3.53A.3, figure 9.11.3.53A.4, figure 9.11.3.53A.4B, figure 9.11.3.53A.4C, figure 9.11.3.53A.5, figure 9.11.3.53A.6, figure 9.11.3.53A.7 and table 9.11.3.53A.1.

The UE parameters update transparent container is a type 6 information element with a minimum length of 20 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE parameters update transparent container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of UE parameters update transparent container contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| UE parameters update header |  |  |  |  |  |  |  | octet 4 |
| UPU-MAC-IAUSF |  |  |  |  |  |  |  | octet 5-20 |
| CounterUPU |  |  |  |  |  |  |  | octet 21-22 |
| UE parameters update list |  |  |  |  |  |  |  | octet 23* - n* |

Figure 9.11.3.53A.1: UE parameters update transparent container information element for UE parameters update data type with value "0"

|  | 8 |  | 7 |  | 6 |  | 5 |  | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | UE parameters update data set 1 type |  |  |  |  | octet 23* |  |  |
| Length of UE parameters update data set 1 |  |  |  |  |  |  |  |  |  |  |  |  | octet 24*- 25* |  |  |
| UE parameters update data set 1 |  |  |  |  |  |  |  |  |  |  |  |  | octet 26*- x* |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | UE parameters update data set n type |  |  |  |  | octet y* |  |  |
| Length of UE parameters update data set n |  |  |  |  |  |  |  |  |  |  |  |  | octet y+1*- y+2* |  |  |
| UE parameters update data set n |  |  |  |  |  |  |  |  |  |  |  |  | octet y+3*- n* |  |  |

Figure 9.11.3.53A.2: UE parameters update list

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Secured packet |  |  |  |  |  |  |  | octet a* - a+z* |

Figure 9.11.3.53A.3: UE parameters update data set for UE parameters update data set type with value "0001"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default configured NSSAI |  |  |  |  |  |  |  | octet b* - c* |

Figure 9.11.3.53A.4: UE parameters update data set for UE parameters update data set type with value "0010"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | DREIEPS | AOL | DREI | octet d* |

Figure 9.11.3.53A.4A: UE parameters update data set for UE parameters update data set type with value "0011"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Routing indicator digit 2 |  |  |  | Routing indicator digit 1 |  |  |  | octet e* |
| Routing indicator digit 4 |  |  |  | Routing indicator digit 3 |  |  |  | octet (e+1)* |

Figure 9.11.3.53A.4B: UE parameters update data set for UE parameters update data set type with value "0100"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | REG | ACK | UPU data type | octet d* |

Figure 9.11.3.53A.4C: UE parameters update data set for UE parameters update data set type with value "0101"

| UE parameters update transparent container IEI | octet 1 |
| --- | --- |
| Length of UE parameters update transparent container contents | octet 2 octet 3 |
| UE parameters update header | octet 4 |
| UPU-MAC-IUE | octet 5 – 20 |

Figure 9.11.3.53A.5: UE parameters update transparent container information element for UE parameters update data type with value "1"

|  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | REG |  | ACK |  | UPU data type |  | octet 4 |  |  |

Figure 9.11.3.53A.6: UE parameters update header for UE parameters update data type with value "0"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | UHP | UPU data type | octet 4 |

Figure 9.11.3.53A.7: UE parameters update header for UE parameters update data type with value "1"

Table 9.11.3.53A.1: UE parameters update transparent container information element

| UPU-MAC-IAUSF, UPU-MAC-IUE and CounterUPU are coded as specified in 3GPP TS 33.501 [24] |  |
| --- | --- |
| UE parameters update (UPU) data type (octet 4, bit 1) |  |
| 0 | The UE parameters update transparent container carries a UE parameters update list (NOTE 2) |
| 1 | The UE parameters update transparent container carries an acknowledgement of successful reception of a UE parameters update list (NOTE 2, NOTE 4) |
| Acknowledgement (ACK) value (octet 4, bit 2) |  |
| 0 | acknowledgement not requested |
| 1 | acknowledgement requested |
| UE parameters update header protection (UHP) value (octet 4, bit 2) |  |
| 0 | UE parameters update header protection not supported |
| 1 | UE parameters update header protection supported |
| Re-registration (REG) value (octet 4, bit 3) |  |
| 0 | re-registration not requested |
| 1 | re-registration requested |
| UE parameters update data set type |  |
| Bits 4 3 2 1 |  |
| 0 0 0 1 Routing indicator update data |  |
| 0 0 1 0 Default configured NSSAI update data |  |
| 0 0 1 1 Disaster roaming information update data |  |
| 0 1 0 0 ME routing indicator update data |  |
| 0 1 0 1 Protected UE parameters update header information (NOTE 5) |  |
| All other values are reserved |  |
| Disaster Roaming Enabled Indication (DREI) value (octet d*, bit 1) |  |
| 0 | Disaster roaming is disabled in the UE |
| 1 | Disaster roaming is enabled in the UE |
| Indication of 'applicability of "lists of PLMN(s) to be used in disaster condition" provided by a VPLMN' (AOL) value (octet d*, bit 2) |  |
| 0 | false |
| 1 | true |
| Disaster Roaming Enabled Indication in EPS (DREIEPS) value (octet d*, bit 3) |  |
| 0 | Disaster roaming in EPS is disabled in the UE |
| 1 | Disaster roaming in EPS is enabled in the UE |
| The secured packet is coded as specified in 3GPP TS 31.115 [22B]. |  |
| The default configured NSSAI is encoded as the value part of the NSSAI IE (see clause 9.11.3.37). |  |
| Routing indicator |  |
| Routing indicator is encoded as the routing indicator field of the 5GS mobile identity IE (see clause 9.11.3.4). |  |
| NOTE 1: When the routing indicator is updated, if the SNPN uses the EAP based primary authentication and key agreement procedure using the EAP-AKA' or the 5G AKA based primary authentication and key agreement procedure, then UE parameter update data set type is set to "routing indicator update data", otherwise, UE parameter update data set type is set to "ME routing indicator update data". NOTE 2: The UPU data type set to "0" is used in the network to UE direction. The UPU data type set to "1" is used in the UE to network direction. NOTE 4: In the UE parameters update data set with the UE parameters update data set type indicating "protected UE parameters update header information", the UPU data type shall be set to "0". NOTE 5: If the UE supports UE parameters update header protection and the UE parameters update list includes a UE parameters update data set with the UE parameters update data set type indicating "protected UE parameters update header information", then the UE uses header information from the UE parameters update data set with the UE parameters update data set type indicating "protected UE parameters update header information". |  |

#### 9.11.3.54	UE security capability

The UE security capability information element is used by the UE and by the network to indicate which security algorithms are supported by the UE in N1 mode for NAS security as well as which security algorithms are supported over NR and E-UTRA connected to 5GCN for AS security.

The UE security capability information element is coded as shown in figure 9.11.3.54.1 and table 9.11.3.54.1.

The UE security capability is a type 4 information element with a minimum length of 4 octets and a maximum length of 10 octets.

Octets 5 to 10 are optional. If octet 5 is included, then also octet 6 shall be included.

If the UE does not support any security algorithm for AS security over E-UTRA connected to 5GCN, it shall not include octets 5 and 6. The UE shall not include octets 7 to 10.

If the UE does not support any security algorithm for AS security over E-UTRA connected to 5GCN, and if the network includes octets 7 to 10, then the network shall also include octets 5 to 6.

If the network includes octet 7, then it shall include also octet 8. If the network includes octet 9, then it shall include also octet 10.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE security capability IEI |  |  |  |  |  |  |  | octet 1 |
| Length of UE security capability contents |  |  |  |  |  |  |  | octet 2 |
| 5G-EA0 | 128- 5G-EA1 | 128- 5G-EA2 | 128- 5G-EA3 | 5G-EA4 | 5G-EA5 | 5G-EA6 | 5G-EA7 | octet 3 |
| 5G-IA0 | 128- 5G-IA1 | 128- 5G-IA2 | 128- 5G-IA3 | 5G-IA4 | 5G-IA5 | 5G-IA6 | 5G-IA7 | octet 4 |
| EEA0 | 128- EEA1 | 128- EEA2 | 128- EEA3 | EEA4 | EEA5 | EEA6 | EEA7 | octet 5* |
| EIA0 | 128- EIA1 | 128- EIA2 | 128- EIA3 | EIA4 | EIA5 | EIA6 | EIA7 | octet 6* |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Spare |  |  |  |  |  |  |  | octet 7* -10* |

Figure 9.11.3.54.1: UE security capability information element

Table 9.11.3.54.1: UE security capability information element

| 5GS encryption algorithms supported (see NOTE 1) (octet 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5GS encryption algorithm 5G-EA0 supported (octet 3, bit 8) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 5G-EA0 not supported |
| 1 |  |  |  | 5GS encryption algorithm 5G-EA0 supported |
| 5GS encryption algorithm 128-5G-EA1 supported (octet 3, bit 7) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 128-5G-EA1 not supported |
| 1 |  |  |  | 5GS encryption algorithm 128-5G-EA1 supported |
| 5GS encryption algorithm 128-5G-EA2 supported (octet 3, bit 6) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 128-5G-EA2 not supported |
| 1 |  |  |  | 5GS encryption algorithm 128-5G-EA2 supported |
| 5GS encryption algorithm 128-5G-EA3 supported (octet 3, bit 5) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 128-5G-EA3 not supported |
| 1 |  |  |  | 5GS encryption algorithm 128-5G-EA3 supported |
| 5GS encryption algorithm 5G-EA4 supported (octet 3, bit 4) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 5G-EA4 not supported |
| 1 |  |  |  | 5GS encryption algorithm 5G-EA4 supported |
| 5GS encryption algorithm 5G-EA5 supported (octet 3, bit 3) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 5G-EA5 not supported |
| 1 |  |  |  | 5GS encryption algorithm 5G-EA5 supported |
| 5GS encryption algorithm 5G-EA6 supported (octet 3, bit 2) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 5G-EA6 not supported |
| 1 |  |  |  | 5GS encryption algorithm 5G-EA6 supported |
| 5GS encryption algorithm 5G-EA7 supported (octet 3, bit 1) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 5G-EA7 not supported |
| 1 |  |  |  | 5GS encryption algorithm 5G-EA7 supported |
| 5GS integrity algorithms supported (see NOTE 2) (octet 4) |  |  |  |  |
| 5GS integrity algorithm 5G-IA0 supported (octet 4, bit 8) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 5G-IA0 not supported |
| 1 |  |  |  | 5GS integrity algorithm 5G-IA0 supported |
| 5GS integrity algorithm 128-5G-IA1 supported (octet 4, bit 7) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 128-5G-IA1 not supported |
| 1 |  |  |  | 5GS integrity algorithm 128-5G-IA1 supported |
| 5GS integrity algorithm 128-5G-IA2 supported (octet 4, bit 6) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 128-5G-IA2 not supported |
| 1 |  |  |  | 5GS integrity algorithm 128-5G-IA2 supported |
| 5GS integrity algorithm 128-5G-IA3 supported (octet 4, bit 5) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 128-5G-IA3 not supported |
| 1 |  |  |  | 5GS integrity algorithm 128-5G-IA3 supported |
| 5GS integrity algorithm 5G-IA4 supported (octet 4, bit 4) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 5G-IA4 not supported |
| 1 |  |  |  | 5GS integrity algorithm 5G-IA4 supported |
| 5GS integrity algorithm 5G-IA5 supported (octet 4, bit 3) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 5G-IA5 not supported |
| 1 |  |  |  | 5GS integrity algorithm 5G-IA5 supported |
| 5GS integrity algorithm 5G-IA6supported (octet 4, bit 2) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 5G-IA6 not supported |
| 1 |  |  |  | 5GS integrity algorithm 5G-IA6 supported |
| 5GS integrity algorithm 5G-IA7 supported (octet 4, bit 1) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 5G-IA7 not supported |
| 1 |  |  |  | 5GS integrity algorithm 5G-IA7 supported |
| EPS encryption algorithms supported (see NOTE 3) (octet 5) |  |  |  |  |
| EPS encryption algorithm EEA0 supported (octet 5, bit 8) |  |  |  |  |
| 0 |  |  |  | EPS encryption algorithm EEA0 not supported |
| 1 |  |  |  | EPS encryption algorithm EEA0 supported |
| EPS encryption algorithm 128-EEA1 supported (octet 5, bit 7) |  |  |  |  |
| 0 |  |  |  | EPS encryption algorithm 128-EEA1 not supported |
| 1 |  |  |  | EPS encryption algorithm 128-EEA1 supported |
| EPS encryption algorithm 128-EEA2 supported (octet 5, bit 6) |  |  |  |  |
| 0 |  |  |  | EPS encryption algorithm 128-EEA2 not supported |
| 1 |  |  |  | EPS encryption algorithm 128-EEA2 supported |
| EPS encryption algorithm 128-EEA3 supported (octet 5, bit 5) |  |  |  |  |
| 0 |  |  |  | EPS encryption algorithm 128-EEA3 not supported |
| 1 |  |  |  | EPS encryption algorithm 128-EEA3 supported |
| EPS encryption algorithm EEA4 supported (octet 5, bit 4) |  |  |  |  |
| 0 |  |  |  | EPS encryption algorithm EEA4 not supported |
| 1 |  |  |  | EPS encryption algorithm EEA4 supported |
| EPS encryption algorithm EEA5 supported (octet 5, bit 3) |  |  |  |  |
| 0 |  |  |  | EPS encryption algorithm EEA5 not supported |
| 1 |  |  |  | EPS encryption algorithm EEA5 supported |
| EPS encryption algorithm EEA6 supported (octet 5, bit 2) |  |  |  |  |
| 0 |  |  |  | EPS encryption algorithm EEA6 not supported |
| 1 |  |  |  | EPS encryption algorithm EEA6 supported |
| EPS encryption algorithm EEA7 supported (octet 5, bit 1) |  |  |  |  |
| 0 |  |  |  | EPS encryption algorithm EEA7 not supported |
| 1 |  |  |  | EPS encryption algorithm EEA7 supported |
| EPS integrity algorithms supported (see NOTE 4) (octet 6) |  |  |  |  |
| EPS integrity algorithm EIA0 supported (octet 6, bit 8) |  |  |  |  |
| 0 |  |  |  | EPS integrity algorithm EIA0 not supported |
| 1 |  |  |  | EPS integrity algorithm EIA0 supported |
| EPS integrity algorithm 128-EIA1 supported (octet 6, bit 7) |  |  |  |  |
| 0 |  |  |  | EPS integrity algorithm 128-EIA1 not supported |
| 1 |  |  |  | EPS integrity algorithm 128-EIA1 supported |
| EPS integrity algorithm 128-EIA2 supported (octet 6, bit 6) |  |  |  |  |
| 0 |  |  |  | EPS integrity algorithm 128-EIA2 not supported |
| 1 |  |  |  | EPS integrity algorithm 128-EIA2 supported |
| EPS integrity algorithm 128-EIA3 supported (octet 6, bit 5) |  |  |  |  |
| 0 |  |  |  | EPS integrity algorithm 128-EIA3 not supported |
| 1 |  |  |  | EPS integrity algorithm 128-EIA3 supported |
| EPS integrity algorithm EIA4 supported (octet 6, bit 4) |  |  |  |  |
| 0 |  |  |  | EPS integrity algorithm EIA4 not supported |
| 1 |  |  |  | EPS integrity algorithm EIA4 supported |
| EPS integrity algorithm EIA5 supported (octet 6, bit 3) |  |  |  |  |
| 0 |  |  |  | EPS integrity algorithm EIA5 not supported |
| 1 |  |  |  | EPS integrity algorithm EIA5 supported |
| EPS integrity algorithm EIA6 supported (octet 6, bit 2) |  |  |  |  |
| 0 |  |  |  | EPS integrity algorithm EIA6 not supported |
| 1 |  |  |  | EPS integrity algorithm EIA6 supported |
| EPS integrity algorithm EIA7 supported (octet 6, bit 1) |  |  |  |  |
| 0 |  |  |  | EPS integrity algorithm EIA7 not supported |
| 1 |  |  |  | EPS integrity algorithm EIA7 supported |
| For the UE not supporting any security algorithm for AS security over E-UTRA connected to 5GCN, all bits in octets 5 to 10 are spare and shall be ignored, if the respective octet is received with the information element. For the UE supporting at least one security algorithm for AS security over E-UTRA connected to 5GCN all bits in octets 7 to 10 are spare and shall be ignored, if the respective octet is received with the information element. If the AMF receives any of the octets 7 to 10 (NOTE 5), it shall store the octets as received and include them when sending the UE security capability information element to the UE. |  |  |  |  |
| NOTE 1: The code points in octet 3 are used to indicate support for 5GS encryption algorithms for NAS security in N1 mode and support for 5GS encryption algorithms for AS security over NR. NOTE 2: The code points in octet 4 are used to indicate support for 5GS integrity algorithms for NAS security in N1 mode and support for 5GS integrity algorithms for AS security over NR. NOTE 3: The code points in octet 5 are used to indicate support for EPS encryption algorithms for AS security over E-UTRA connected to 5GCN. NOTE 4: The code points in octet 6 are used to indicate support for EPS integrity algorithms for AS security over E-UTRA connected to 5GCN. NOTE 5: The AMF can receive this information element also from another AMF or MME during N1 mode to N1 mode or S1 mode to N1 mode handover preparation. |  |  |  |  |

#### 9.11.3.55	UE's usage setting

The purpose of the UE's usage setting information element is to provide the network with the UE's usage setting as defined in 3GPP TS 24.301 [15]. The network uses the UE's usage setting to select the RFSP index.

The UE's usage setting information element is coded as shown in figure 9.11.3.55.1 and table 9.11.3.55.1.

The UE's usage setting is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE's usage setting IEI |  |  |  |  |  |  |  | octet 1 |
| Length of UE's usage setting contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | UE's usage setting | octet 3 |

Figure 9.11.3.55.1: UE's usage setting information element

Table 9.11.3.55.1: UE's usage setting information element

| UE's usage setting (octet 3, bit 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 0 |  |  |  | voice centric |
| 1 |  |  |  | data centric |
| All other bits in the octet 3 are spare and shall be coded as zero, |  |  |  |  |

#### 9.11.3.56	UE status

The purpose of the UE status information element is to provide the network with information concerning aspects of the current UE registration status which is used for interworking with EPS.

The UE status information element is coded as shown in figure 9.11.3.56.1 and table 9.11.3.56.1.

The UE status is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE status IEI |  |  |  |  |  |  |  | octet 1 |
| Length of UE status contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | N1 mode reg | S1 mode reg | octet 3 |

Figure 9.11.3.56.1: UE status information element

Table 9.11.3.56.1: UE status information element

| EMM registration status (S1 mode reg) (octet 3, bit 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 0 |  |  |  | UE is not in EMM-REGISTERED state |
| 1 |  |  |  | UE is in EMM-REGISTERED state |
| 5GMM registration status (N1 mode reg) (octet 3, bit 2) |  |  |  |  |
| 0 |  |  |  | UE is not in 5GMM-REGISTERED state |
| 1 |  |  |  | UE is in 5GMM-REGISTERED state |
| All other bits in the octet 3 are spare and shall be coded as zero. |  |  |  |  |

#### 9.11.3.57	Uplink data status

The purpose of the Uplink data status information element is to indicate to the network which preserved PDU session(s) have uplink data pending or which preserved PDU session(s) are associated with active multicast MBS session(s).

The Uplink data status information element is coded as shown in figure 9.11.3.57.1 and table 9.11.3.57.1.

The Uplink data status information element is a type 4 information element with minimum length of 4 octets a maximum length of 34 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink data status IEI |  |  |  |  |  |  |  | octet 1 |
| Length of uplink data status contents |  |  |  |  |  |  |  | octet 2 |
| PSI (7) | PSI (6) | PSI (5) | PSI (4) | PSI (3) | PSI (2) | PSI (1) | PSI (0) | octet 3 |
| PSI (15) | PSI (14) | PSI (13) | PSI (12) | PSI (11) | PSI (10) | PSI (9) | PSI (8) | octet 4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| spare |  |  |  |  |  |  |  | octet 5* -34* |

Figure 9.11.3.57.1: Uplink data status information element

Table 9.11.3.57.1: Uplink data status information element

PSI(x) shall be coded as follows: PSI(0): Bit 1 of octet 3 is spare and shall be coded as zero. PSI(1) – PSI(15): 0 indicates that no uplink data are pending for the corresponding PDU session identity or the PDU session is in PDU SESSION INACTIVE state or is in PDU SESSION ACTIVE state with user-plane resources already established. 1 indicates that uplink data are pending for the corresponding PDU session identity and the user-plane resources for the corresponding PDU session are not established, or the UE has active multicast MBS session(s) associated with the corresponding PDU session. All bits in octet 5 to 34 are spare and shall be coded as zero, if the respective octet is included in the information element.

#### 9.11.3.58	Void

#### 9.11.3.59	Void

#### 9.11.3.60	Void

#### 9.11.3.61	Void

#### 9.11.3.62	Void

#### 9.11.3.63	Void

#### 9.11.3.64	Void

#### 9.11.3.65	Void

#### 9.11.3.66	Void

#### 9.11.3.67	Void

#### 9.11.3.68	UE radio capability ID

The purpose of the UE radio capability ID information element is to carry a UE radio capability ID.

The UE radio capability ID information element is coded as shown in figure 9.11.3.68.1 and table 9.11.3.68.1.

The UE radio capability ID is a type 4 information element with a length of n octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE radio capability ID IEI |  |  |  |  |  |  |  | octet 1 |
| Length of UE radio capability ID contents |  |  |  |  |  |  |  | octet 2 |
| UE radio capability ID |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet n |

Figure 9.11.3.68.1: UE radio capability ID information element

Table 9.11.3.68.1: UE radio capability ID information element

UE radio capability ID (octets 3 to n)

The UE radio capability ID contents contain the UE radio capability ID as specified in 3GPP TS 23.003 [4], with each hexadecimal digit coded over 4 bits, starting with the first hexadecimal digit coded in bits 4 to 1 of octet 3, the second hexadecimal digit coded in bits 8 to 5 of octet 3, and so on. If the UE radio capability ID contains an odd number of hexadecimal digits, bits 8 to 5 of the last octet (octet n) shall be coded as "1111".

#### 9.11.3.69	UE radio capability ID deletion indication

The purpose of the UE radio capability ID deletion indication information element is to indicate to the UE that deletion of UE radio capability IDs is requested.

The UE radio capability ID deletion indication is a type 1 information element.

The UE radio capability ID deletion indication information element is coded as shown in figure 9.11.3.69.1 and table 9.11.3.69.1.

| 8 | 7 | 6 | 5 |  | 4 | 3 |  | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE radio capability ID deletion indication IEI |  |  |  | 0 spare |  |  | Deletion request |  |  | octet 1 |

Figure 9.11.3.69.1: UE radio capability ID deletion indication information element

Table 9.11.3.69.1: UE radio capability ID deletion indication information element

| Deletion requested (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 0 |  | UE radio capability ID deletion not requested |
| 0 | 0 | 1 |  | Network-assigned UE radio capability IDs deletion requested |
| All other values are unused and shall be interpreted as "UE radio capability ID deletion not requested", if received by the UE. |  |  |  |  |

#### 9.11.3.70	Truncated 5G-S-TMSI configuration

The purpose of the Truncated 5G-S-TMSI configuration information element is to provide the size of the components of the truncated 5G-S-TMSI to the UE in NB-N1 mode to create the truncated 5G-S-TMSI.

The Truncated 5G-S-TMSI configuration information element is coded as shown in figure 9.11.3.70.1 and table 9.11.3.70.1.

The Truncated 5G-S-TMSI configuration is a type 4 information element with 3 octets length.

| 8 | 7 | 6 | 5 |  | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Truncated 5G-S-TMSI configuration IEI |  |  |  |  |  |  |  |  | octet 1 |
| Length of Truncated 5G-S-TMSI configuration contents |  |  |  |  |  |  |  |  | octet 2 |
| Truncated AMF Set ID value |  |  |  | Truncated AMF Pointer value |  |  |  |  | octet 3 |

Figure 9.11.3.70.1: Truncated 5G-S-TMSI configuration information element

Table 9.11.3.70.1: Truncated 5G-S-TMSI configuration information element

| Truncated AMF Pointer value (bits 4 to 1 of octet 3) This field represents the size of the least significant bits of the AMF Pointer. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | reserved |
| 0 | 0 | 0 | 1 | 1 least significant bit of the AMF Pointer |
| 0 | 0 | 1 | 0 | 2 least significant bits of the AMF Pointer |
| 0 | 0 | 1 | 1 | 3 least significant bits of the AMF Pointer |
| 0 | 1 | 0 | 0 | 4 least significant bits of the AMF Pointer |
| 0 | 1 | 0 | 1 | 5 least significant bits of the AMF Pointer |
| 0 | 1 | 1 | 0 | 6 least significant bits of the AMF Pointer |
| All other values shall be interpreted as "6 least significant bits of the AMF Pointer" by this version of the protocol. |  |  |  |  |
| Truncated AMF Set ID value (bits 8 to 5 of octet 3) This field represents the size of the least significant bits of the AMF Set ID. |  |  |  |  |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | reserved |
| 0 | 0 | 0 | 1 | 1 least significant bit of the AMF Set ID |
| 0 | 0 | 1 | 0 | 2 least significant bits of the AMF Set ID |
| 0 | 0 | 1 | 1 | 3 least significant bits of the AMF Set ID |
| 0 | 1 | 0 | 0 | 4 least significant bits of the AMF Set ID |
| 0 | 1 | 0 | 1 | 5 least significant bits of the AMF Set ID |
| 0 | 1 | 1 | 0 | 6 least significant bits of the AMF Set ID |
| 0 | 1 | 1 | 1 | 7 least significant bits of the AMF Set ID |
| 1 | 0 | 0 | 0 | 8 least significant bits of the AMF Set ID |
| 1 | 0 | 0 | 1 | 9 least significant bits of the AMF Set ID |
| 1 | 0 | 1 | 0 | 10 least significant bits of the AMF Set ID |
| All other values shall be interpreted as "10 least significant bits of the AMF Set ID" by this version of the protocol. |  |  |  |  |
| NOTE: Total sum of the "Truncated AMF Set ID value" and the "Truncated AMF Pointer value" in the Truncated 5G-S-TMSI configuration IE is specified in 3GPP TS 23.003 [4] and 3GPP TS 36.300 [25B]. |  |  |  |  |

#### 9.11.3.71	WUS assistance information

See clause 9.9.3.62 in 3GPP TS 24.301 [15].

#### 9.11.3.72	N5GC indication

The purpose of the N5GC indication information element is to indicate to the network that the registration request by the W-AGF is on behalf of an N5GC device.

The N5GC indication information element is coded as shown in figure 9.11.3.72.1.

The N5GC indication is a type 1 information element.

| 8 | 7 | 6 | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N5GC indication IEI |  |  |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | N5GCREG |  | octet 1 |  |  |

Figure 9.11.3.72.1: N5GC indication

Table 9.11.3.72.1: N5GC indication

| N5GC device indication bit (N5GCREG) (octet 1, bit 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | N5GC device registration is not requested |
| 1 | N5GC device registration is requested |
| Bits 2 to 4 are spare and shall be coded as zero. |  |

#### 9.11.3.73	NB-N1 mode DRX parameters

The purpose of the NB-N1 mode DRX parameters information element is to indicate that the UE wants to use DRX in NB-N1 mode and for the network to indicate the DRX cycle value to be used at paging in NB-N1 mode.

The NB-N1 mode DRX parameters is a type 4 information element with a length of 3 octets.

The NB-N1 mode DRX parameters information element is coded as shown in figure 9.11.3.73.1 and table 9.11.3.73.1.

The value part of a DRX parameter information element is coded as shown in table 9.11.3.73.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NB-N1 mode DRX parameters IEI |  |  |  |  |  |  |  | octet 1 |
| Length of NB-N1 mode DRX parameters contents |  |  |  |  |  |  |  | octet 2 |
| 0 | 0 | 0 | 0 | NB-N1 mode DRX value |  |  |  |  |
| Spare |  |  |  |  |  |  |  | octet 3 |

Figure 9.11.3.73.1: NB-N1 mode DRX parameters information element

Table 9.11.3.73.1: NB-N1 mode DRX parameters information element

| NB-N1 mode DRX value (octet 3, bits 1 to 4) This field represents the DRX cycle parameter 'T', for NB-N1 mode, as defined in 3GPP TS 36.304 [25C]. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | DRX value not specified |
| 0 | 0 | 0 | 1 | DRX cycle parameter T = 32 |
| 0 | 0 | 1 | 0 | DRX cycle parameter T = 64 |
| 0 | 0 | 1 | 1 | DRX cycle parameter T = 128 |
| 0 | 1 | 0 | 0 | DRX cycle parameter T = 256 |
| 0 | 1 | 0 | 1 | DRX cycle parameter T = 512 |
| 0 | 1 | 1 | 1 | DRX cycle parameter T = 1024 |
| All other values shall be interpreted as "DRX value not specified" by this version of the protocol. Bits 5 to 8 of octet 3 are spare and shall be coded as zero. |  |  |  |  |

#### 9.11.3.74	Additional configuration indication

The purpose of the Additional configuration indication information element is to indicate additional information associated with the generic UE configuration update procedure.

The Additional configuration indication information element is coded as shown in figure 9.11.3.74.1 and table 9.11.3.74.1.

The Additional configuration indication is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Additional configuration indication IEI |  |  |  | 0 Spare | 0 Spare | 0 Spare | SCMR | octet 1 |

Figure 9.11.3.74.1: Additional configuration indication

Table 9.11.3.74.1: Additional configuration indication

| Signalling connection maintain request (SCMR) (octet 1, bit 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | no additional information |
| 1 | release of N1 NAS signalling connection not required |
| Bits 2 to 4 are spare and shall be coded as zero, |  |

#### 9.11.3.75	Extended rejected NSSAI

The purpose of the Extended rejected NSSAI information element is to identify a collection of rejected S-NSSAIs if UE supports extended rejected NSSAI.

The Extended rejected NSSAI information element is coded as shown in figure 9.11.3.75.1, figure 9.11.3.75.2 and table 9.11.3.75.1.

The Extended rejected NSSAI is a type 4 information element with a minimum length of 5 octets and a maximum length of 90 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended rejected NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Extended rejected NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| Partial extended rejected NSSAI list 1 |  |  |  |  |  |  |  | octet 3 octet m |
| Partial extended rejected NSSAI list 2 |  |  |  |  |  |  |  | octet m+1* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* octet u* |
| Partial extended rejected NSSAI list n |  |  |  |  |  |  |  | octet u+1* octet v* |

Figure 9.11.3.75.1: Extended rejected NSSAI information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| spare | Type of list |  |  | Number of elements |  |  |  | octet 3 |
| Rejected S-NSSAI 1 |  |  |  |  |  |  |  | octet 4 octet j |
| Rejected S-NSSAI 2 |  |  |  |  |  |  |  | octet j+1* octet k* |
| … |  |  |  |  |  |  |  | octet k+1 octet p* |
| Rejected S-NSSAI n |  |  |  |  |  |  |  | octet p+1* octet m* |

Figure 9.11.3.75.2: Partial extended rejected NSSAI list – type of list = 000

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| spare | Type of list |  |  | Number of elements |  |  |  | octet 3 |
| Back-off timer value |  |  |  |  |  |  |  | octet 4 |
| Rejected S-NSSAI 1 |  |  |  |  |  |  |  | octet 5 octet j |
| Rejected S-NSSAI 2 |  |  |  |  |  |  |  | octet j+1* octet k* |
| … |  |  |  |  |  |  |  | octet k+1* octet p* |
| Rejected S-NSSAI n |  |  |  |  |  |  |  | octet p+1* octet m* |

Figure 9.11.3.75.3: Partial extended rejected NSSAI list – type of list = 001

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of rejected S-NSSAI |  |  |  | Cause value |  |  |  | octet 4 |
| SST |  |  |  |  |  |  |  | octet 5 |
| SD |  |  |  |  |  |  |  | octet 6* octet 8* |
| Mapped HPLMN SST |  |  |  |  |  |  |  | octet 9* |
| Mapped HPLMN SD |  |  |  |  |  |  |  | octet 10* octet 12* |

Figure 9.11.3.75.4: Rejected S-NSSAI

Table 9.11.3.75.1: Extended rejected NSSAI information element

| Value part of the Extended rejected NSSAI information element (octet 3 to v) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The value part of the Extended rejected NSSAI information element consists of one or more partial extended rejected NSSAI lists. The length of each partial extended rejected NSSAI list can be determined from the 'type of list' field and the 'number of elements' field in the first octet of the partial extended rejected NSSAI list. Each rejected S-NSSAI consists of one S-NSSAI and an associated cause value. Each rejected S-NSSAI also includes the mapped HPLMN S-NSSAI if available. The length of each rejected S-NSSAI can be determined by the 'length of rejected S-NSSAI' field in the first octet of the rejected S-NSSAI. |  |  |  |  |  |
| The UE shall store the complete list received (NOTE 0). If more than 8 rejected S-NSSAIs are included in this information element, the UE shall store the first 8 rejected S-NSSAIs and ignore the remaining octets of the information element. |  |  |  |  |  |
| Partial extended rejected NSSAI list: |  |  |  |  |  |
| Number of elements (octet 3, bits 1 to 4) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 1 element |  |
| 0 | 0 | 0 | 1 | 2 element |  |
|  |  | … |  |  |  |
| 0 | 1 | 1 | 0 | 7 element |  |
| 0 | 1 | 1 | 1 | 8 element |  |
| All other values are unused and shall be interpreted as 8, if received by the UE. |  |  |  |  |  |
| Type of list (octet 3, bits 5 to 7) (NOTE 7) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 7 | 6 | 5 |  |  |  |
| 0 | 0 | 0 |  | list of S-NSSAIs without any associated back-off timer value |  |
| 0 | 0 | 1 |  | list of S-NSSAIs with one associated back-off timer value that applies to all S-NSSAIs in the list |  |
| All other values are reserved. |  |  |  |  |  |
| Bit 8 of octet 3 is spare and shall be coded as zero. |  |  |  |  |  |
| Back-off timer value (octet 4): |  |  |  |  |  |
| Back-off timer value is coded as the value part of GPRS timer 3 in clause 10.5.7.4a in 3GPP TS 24.008 [12]. |  |  |  |  |  |
| Rejected S-NSSAI: |  |  |  |  |  |
| Cause value (octet x) (NOTE 8) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 |  | S-NSSAI not available in the current PLMN or SNPN |
| 0 | 0 | 0 | 1 |  | S-NSSAI not available in the current registration area |
| 0 | 0 | 1 | 0 |  | S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization |
| 0 | 0 | 1 | 1 |  | S-NSSAI not available due to maximum number of UEs reached |
| All other values are reserved. |  |  |  |  |  |
| Slice/service type (SST) (octet x+1) |  |  |  |  |  |
| This field contains the 8 bit SST value. The coding of the SST value part is defined in 3GPP TS 23.003 [4]. (NOTE 5) |  |  |  |  |  |
| Slice differentiator (SD) (octet x+2 to octet x+4) |  |  |  |  |  |
| This field contains the 24 bit SD value. The coding of the SD value part is defined in 3GPP TS 23.003 [4]. (NOTE 6) If the SST encoded in octet x+1 is not associated with a valid SD value, and the sender needs to include a mapped HPLMN SST (octet x+5) and a mapped HPLMN SD (octets x+6 to x+8), then the sender shall set the SD value (octets x+2 to x+4) to "no SD value associated with the SST". mapped HPLMN Slice/service type (SST) (octet x+5) This field contains the 8 bit SST value of an S-NSSAI in the S-NSSAI(s) of the HPLMN to which the SST value is mapped. The coding of the SST value part is defined in 3GPP TS 23.003 [4]. mapped HPLMN Slice differentiator (SD) (octet x+6 to octet x+8) This field contains the 24 bit SD value of an S-NSSAI in the S-NSSAI(s) of the HPLMN to which the SD value is mapped. The coding of the SD value part is defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |
| NOTE 0: The number of rejected S-NSSAI(s) shall not exceed eight. NOTE 1: Octet x and octet x+1 shall always be included. NOTE 2: If the octet x+2 is included, then octet x+3 and octet x+4 shall be included. NOTE 3: If the octet x+5 is included, then octets x+6, x+7, and x+8 may be included. NOTE 4: If the octet x+6 is included, then octet x+7 and octet x+8 shall be included. NOTE 5: If the Cause value is "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization", this field shall contain the 8 bit SST value of an S-NSSAI in the S-NSSAI(s) of the HPLMN and octets x+5, x+6, x+7, and x+8 shall not be included. NOTE 6: If the Cause value is "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization", this field shall contain the 24 bit SD value of an S-NSSAI in the S-NSSAI(s) of the HPLMN and octets x+5, x+6, x+7, and x+8 shall not be included. NOTE 7: The partial extended rejected NSSAI with type of list = 001 shall only be used for rejected S-NSSAI(s) with the rejection cause "S-NSSAI not available due to maximum number of UEs reached". |  |  |  |  |  |
| NOTE 8: Octet x can be 4 or 5. |  |  |  |  |  |

#### 9.11.3.76	UE request type

See clause 9.9.3.65 in 3GPP TS 24.301 [15].

#### 9.11.3.77	Paging restriction

The purpose of the Paging restriction information element is to request the network to restrict paging.

The Paging restriction information element is coded as shown in figure 9.11.3.77.1, figure 9.11.3.77.2 and table 9.11.3.77.1.

The Paging restriction is a type 4 information element with a minimum length of 3 octets and a maximum length of 35 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Paging restriction IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Paging restriction contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | Paging restriction type |  |  |  | octet 3 |

Figure 9.11.3.77.1:	Paging restriction information element for Paging restriction type = "All paging is restricted" and for Paging restriction type = "All paging is restricted except voice"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Paging restriction IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Paging restriction contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | Paging restriction type |  |  |  | octet 3 |
| PSI (7) | PSI (6) | PSI (5) | PSI (4) | PSI (3) | PSI (2) | PSI (1) | PSI (0) | octet 4 |
| PSI (15) | PSI (14) | PSI (13) | PSI (12) | PSI (11) | PSI (10) | PSI (9) | PSI (8) | octet 5 |
| 0 0 0 0 0 0 0 0 spare |  |  |  |  |  |  |  | octet 6*-35* |

Figure 9.11.3.77.2:	Paging restriction information element for Paging restriction type = "All paging is restricted except for specified PDU session(s)" and for Paging restriction type = "All paging is restricted except for voice service and specified PDU session(s)"

Table 9.11.3.77.1:	Paging restriction information element

| Paging restriction type (bits 4 to 1 of octet 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | reserved |
| 0 | 0 | 0 | 1 | All paging is restricted |
| 0 | 0 | 1 | 0 | All paging is restricted except for voice service |
| 0 | 0 | 1 | 1 | All paging is restricted except for specified PDU session(s) |
| 0 | 1 | 0 | 0 | All paging is restricted except for voice service and specified PDU session(s) |
| All other values are reserved. |  |  |  |  |
| Bits 5 to 8 of octet 3 are spare and shall be coded as zero. PSI(x) (bits 8 to 1 of octet 4 and octet 5): This field indicates the PDU session identity of the PDU session for which paging is restricted. PSI(0): (bit 1 of octet 4) Spare and shall be coded as zero. PSI(1) – PSI(15): 0 indicates that paging is restricted for the PDU session associated with the PDU session identity. 1 indicates that paging is not restricted for the PDU session associated with the PDU session identity. |  |  |  |  |
| All bits in octet 6 to 35 are spare and shall be coded as zero, if the respective octet is included in the information element. |  |  |  |  |

#### 9.11.3.78	Void

#### 9.11.3.79	NID

See clause 9.2.7 in 3GPP TS 24.502 [18].

#### 9.11.3.80	PEIPS assistance information

The purpose of the PEIPS assistance information, information element is to transfer the required assistance information for PEIPS.he PEIPS assistance information, information element is coded as shown in figure 9.11.3.80.1, figure 9.11.3.80.2, figure 9.11.3.80.3 and table 9.11.3.80.1.

The PEIPS assistance information is a type 4 information element, with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PEIPS assistance information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PEIPS assistance information contents |  |  |  |  |  |  |  | octet 2 |
| PEIPS assistance information type |  |  |  |  |  |  |  | octet 3 |

Figure 9.11.3.80.1: PEIPS assistance information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of information |  |  | Paging subgroup ID value |  |  |  |  | octet 3 |

Figure 9.11.3.80.2: PEIPS assistance information type –type of information= "000"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of information |  |  | UE paging probability information value |  |  |  |  | octet 3 |

Figure 9.11.3.80.3: PEIPS assistance information type –type of information= "001"

Table 9.11.3.80.1: PEIPS assistance information information element

| Value part of the PEIPS assistance information information element (octet 3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The value part of the PEIPS assistance information information element consists of one PEIPS assistance information type parameter. |  |  |  |  |  |
| PEIPS assistance information type: |  |  |  |  |  |
| Type of information (octet 3, bits 6 to 8) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 8 | 7 | 6 |  |  |  |
| 0 | 0 | 0 | Paging subgroup ID |  |  |
| 0 | 0 | 1 | UE paging probability information |  |  |
| All other values are reserved. |  |  |  |  |  |
| Paging subgroup ID value (octet 3, bits 1 to 5) |  |  |  |  |  |
| This field contains the binary encoded value (in decimal) of paging subgroup ID that is assigned by the AMF for paging the UE. This field has a valid range of values from (0-7). All other values are unused and shall be interpreted as 0 by this version of the protocol. |  |  |  |  |  |
| UE paging probability information value (octet 3, bits 1 to 5) |  |  |  |  |  |
| This field contains the value of UE paging probability information provided by the UE to the AMF. It represents the probability of the UE receiving the paging. |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 | UE paging probability information value |
| 0 | 0 | 0 | 0 | 0 | p00 (UE calculated paging probability is 0%) |
| 0 | 0 | 0 | 0 | 1 | p05 (UE calculated paging probability > 0% and <= 5%) |
| 0 | 0 | 0 | 1 | 0 | p10 (UE calculated paging probability > 5% and <= 10%) |
| 0 | 0 | 0 | 1 | 1 | p15 (UE calculated paging probability > 10% and <= 15%) |
| 0 | 0 | 1 | 0 | 0 | p20 (UE calculated paging probability > 15% and <= 20%) |
| 0 | 0 | 1 | 0 | 1 | p25 (UE calculated paging probability > 20% and <= 25%) |
| 0 | 0 | 1 | 1 | 0 | p30 (UE calculated paging probability > 25% and <= 30%) |
| 0 | 0 | 1 | 1 | 1 | p35 (UE calculated paging probability > 30% and <= 35%) |
| 0 | 1 | 0 | 0 | 0 | p40 (UE calculated paging probability > 35% and <= 40%) |
| 0 | 1 | 0 | 0 | 1 | p45 (UE calculated paging probability > 40% and <= 45%) |
| 0 | 1 | 0 | 1 | 0 | p50 (UE calculated paging probability > 45% and <= 50%) |
| 0 | 1 | 0 | 1 | 1 | p55 (UE calculated paging probability > 50% and <= 55%) |
| 0 | 1 | 1 | 0 | 0 | p60 (UE calculated paging probability > 55% and <= 60%) |
| 0 | 1 | 1 | 0 | 1 | p65 (UE calculated paging probability > 60% and <= 65%) |
| 0 | 1 | 1 | 1 | 0 | p70 (UE calculated paging probability > 65% and <= 70%) |
| 0 | 1 | 1 | 1 | 1 | p75 (UE calculated paging probability > 70% and <= 75%) |
| 1 | 0 | 0 | 0 | 0 | p80 (UE calculated paging probability > 75% and <= 80%) |
| 1 | 0 | 0 | 0 | 1 | p85 (UE calculated paging probability > 80% and <= 85%) |
| 1 | 0 | 0 | 1 | 0 | p90 (UE calculated paging probability > 85% and <= 90%) |
| 1 | 0 | 0 | 1 | 1 | p95 (UE calculated paging probability > 90% and <= 95%) |
| 1 | 0 | 1 | 0 | 0 | p100 (UE calculated paging probability > 95% and <= 100%) |
| All other values shall be interpreted as 10100 by this version of the protocol. |  |  |  |  |  |

#### 9.11.3.81	5GS additional request result

The purpose of the 5GS additional request result information element is to inform the UE about the result of additional request.

The 5GS additional request result information element is coded as shown in figure 9.11.3.81.1 and table 9.11.3.81.1.

The 5GS additional request result is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS additional request result IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS additional request result contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | PRD |  | octet 3 |

Figure 9.11.3.81.1: 5GS additional request result information element

Table 9.11.3.81.1: 5GS additional request result information element

| Paging restriction decision (PRD) (bits 2 to 1 of octet 3) |  |  |
| --- | --- | --- |
| Bits |  |  |
| 2 | 1 |  |
| 0 | 0 | no additional information |
| 0 | 1 | paging restriction is accepted |
| 1 | 0 | paging restriction is rejected |
| All other values are reserved. |  |  |
| Bits 3 to 8 of octet 3 are spare and shall be coded as zero. |  |  |

#### 9.11.3.82	NSSRG information

The purpose of the NSSRG information information element is to identify one or more NSSRG values associated with each of the HPLMN S-NSSAIs in a configured NSSAI.

The NSSRG information information element is coded as shown in figure 9.11.3.82.1, figure 9.11.3.82.2 and table 9.11.3.82.1.

The NSSRG information is a type 6 information element with minimum length of 7 octets and maximum length of 4099 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NSSRG information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of NSSRG information contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| NSSRG values for S-NSSAI 1 |  |  |  |  |  |  |  | octet 4 octet m |
| NSSRG values for S-NSSAI 2 |  |  |  |  |  |  |  | octet m+1* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* octet u* |
| NSSRG values for S-NSSAI x |  |  |  |  |  |  |  | octet u+1* octet v* |

Figure 9.11.3.82.1: NSSRG information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NSSRG values for S-NSSAI |  |  |  |  |  |  |  | octet 4 |
| S-NSSAI value |  |  |  |  |  |  |  | octet 5 octet w |
| NSSRG value 1 for the S-NSSAI |  |  |  |  |  |  |  | octet w+1 |
| NSSRG value 2 for the S-NSSAI |  |  |  |  |  |  |  | octet w+2* |
| … |  |  |  |  |  |  |  | octet w+3* octet w+15 |
| NSSRG value y for the S-NSSAI |  |  |  |  |  |  |  | octet w+16 |

Figure 9.11.3.82.2: NSSRG values for S-NSSAI

Table 9.11.3.82.1: NSSRG information information element

Value part of the NSSRG information information element (octet 4 to v) The value part of the NSSRG information information element consists of one or more NSSRG values for each S-NSSAI in the Configured NSSAI IE. S-NSSAI value (octet 5 to w) (see NOTE 2) S-NSSAI value is coded as the length and value part of S-NSSAI information element as specified in clause 9.11.2.8 starting with the second octet. See NOTE 1.

NSSRG value for the S-NSSAI (octet w+1)

This field contains the 8 bit NSSRG value.

NOTE 1: If a mapped HPLMN SST is included in a S-NSSAI value, then the NSSRG value(s) are associated with the Mapped HPLMN SST, and the Mapped HPLMN SD, if included. NOTE 2: The NSSRG information IE shall contain the complete set of S-NSSAI(s) included in the configured NSSAI. NOTE 3: The number of NSSRG values associated with an S-NSSAI cannot exceed 16. If there are more than 16 NSSRG values for an S-NSSAI in the NSSRG information, then the UE shall retain only the first 16 NSSRG values and ignore the rest.

#### 9.11.3.83	List of PLMNs to be used in disaster condition

The purpose of the list of PLMNs to be used in disaster condition information element is to provide the "list of PLMN(s) to be used in disaster condition" associated with the serving PLMN to the UE.

The list of PLMNs to be used in disaster condition information element is coded as shown in figures 9.11.3.83.1 and 9.11.3.83.2 and table 9.11.3.83.1.

The list of PLMNs to be used in disaster condition is a type 4 information element, with a minimum length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| List of PLMNs to be used in disaster condition list IEI |  |  |  |  |  |  |  | octet 1 |
| Length of list of PLMNs to be used in disaster condition contents |  |  |  |  |  |  |  | octet 2 |
| PLMN ID 1 |  |  |  |  |  |  |  | octet 3* octet 5* |
| PLMN ID 2 |  |  |  |  |  |  |  | octet 6* octet 8* |
| … |  |  |  |  |  |  |  | octet 9* octet q* |
| PLMN ID n |  |  |  |  |  |  |  | octet q+1* octet q+3* |

Figure 9.11.3.83.1: List of PLMNs to be used in disaster condition information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet q+1 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet q+2 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet q+3 |

Figure 9.11.3.83.2: PLMN ID n

Table 9.11.3.83.1: List of PLMNs to be used in disaster condition information element

MCC, Mobile country code (octet q+1 and bits 1 to 4 octet q+2) The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A.

MNC, Mobile network code (bits 5 to 8 of octet q+2 and octet q+3) The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet q+2 shall be coded as "1111".

The MCC and MNC digits are coded as octets 6 to 8 of the Temporary mobile group identity IE in figure 10.5.154 of 3GPP TS 24.008 [12].

NOTE: The PLMN IDs are provided in decreasing order of priority, i.e. PLMN ID 1 indicates highest priority and PLMN ID n indicates lowest priority.

#### 9.11.3.84	Registration wait range

The purpose of the registration wait range information element is to provide the disaster roaming wait range or the disaster return wait range to the UE.

The registration wait range information element is coded as shown in figure 9.11.3.84.1 and table 9.11.3.84.1.

The registration wait range is a type 4 information element, with a length of 4 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Registration wait range IEI |  |  |  |  |  |  |  | octet 1 |
| Length of registration wait range |  |  |  |  |  |  |  | octet 2 |
| Minimum registration wait time |  |  |  |  |  |  |  | octet 3 |
| Maximum registration wait time |  |  |  |  |  |  |  | octet 4 |

Figure 9.11.3.84.1: Registration wait range information element

Table 9.11.3.84.1: Registration wait range information element

Minimum registration wait time (octet 3) The minimum registration wait time contains the minimum duration of the registration wait time, encoded as octet 2 of the GPRS timer information element (see 3GPP TS 24.008 [12] clause 10.5.7.3).

Maximum registration wait time (octet 4) The maximum registration wait time contains the maximum duration of the registration wait time, encoded as octet 2 of the GPRS timer information element (see 3GPP TS 24.008 [12] clause 10.5.7.3).

#### 9.11.3.85	PLMN identity

The purpose of the PLMN identity information element is to provide a PLMN identity.

The PLMN identity information element is coded as shown in figure 9.11.3.85.1, and table 9.11.3.85.1.

The PLMN identity is a type 4 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PLMN identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PLMN identity contents |  |  |  |  |  |  |  | octet 2 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 3 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 4 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 5 |

Figure 9.11.3.85.1: PLMN identity information element

Table 9.11.3.85.1: PLMN identity information element

MCC, Mobile country code (octet 3, octet 4 bits 1 to 4)

The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A.

MNC, Mobile network code (octet 4 bits 5 to 8, octet 5)

The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet 4 shall be coded as "1111".

The MCC and MNC digits are coded as octets 6 to 8 of the Temporary mobile group identity IE in figure 10.5.154 of 3GPP TS 24.008 [12].

#### 9.11.3.86	Extended CAG information list

The purpose of the Extended CAG information list information element is to provide "CAG information list" or to delete the "CAG information list" at the UE.

The Extended CAG information list information element is coded as shown in figures 9.11.3.86.1, figure 9.11.3.86.2, figure 9.11.3.86.3, figure 9.11.3.86.4, figure 9.11.3.86.5 and table 9.11.3.86.1.

The Extended CAG information list is a type 6 information element, with a minimum length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended CAG information list IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Extended CAG information list contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Entry 1 |  |  |  |  |  |  |  | octet 4* octet a* |
| Entry 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* octet g* |
| Entry n |  |  |  |  |  |  |  | octet g+1* octet h* |

Figure 9.11.3.86.1: Extended CAG information list information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of entry contents |  |  |  |  |  |  |  | octet q |
|  |  |  |  |  |  |  |  | octet q+1 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet q+2 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet q+3 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet q+4 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | CAILI | LCI | 0 Spare | CAG only | octet q+5 |
| Length of CAG-ID without additional information list |  |  |  |  |  |  |  | octet (q+6)* octet (q+7)* |
| CAG-ID 1 |  |  |  |  |  |  |  | octet r* (see NOTE) octet (r+3)* |
| CAG-ID 2 |  |  |  |  |  |  |  | octet (r+4)* octet (r+7)* |
| … |  |  |  |  |  |  |  | octet (r+8)* octet (r+4*m-5)* |
| CAG-ID m |  |  |  |  |  |  |  | octet (r+4*m-4)* octet (r+4*m-1)* |
| CAG-ID with additional information list |  |  |  |  |  |  |  | octet t* (see NOTE) octet u* |

NOTE:	The field is placed immediately after the last present preceding field.

Figure 9.11.3.86.2: Entry n

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of CAG-ID with additional information list contents |  |  |  |  |  |  |  | octet t |
|  |  |  |  |  |  |  |  | octet t+1 |
| CAG-ID with additional information 1 |  |  |  |  |  |  |  | octet t+2 octet v |
| CAG-ID with additional information 2 |  |  |  |  |  |  |  | octet (v+1)* octet (w)* |
| … |  |  |  |  |  |  |  | octet (w+1)* octet (x)* |
| CAG-ID with additional information o |  |  |  |  |  |  |  | octet (x+1)* octet u* |

Figure 9.11.3.86.3: CAG-ID with additional information list

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of CAG-ID with additional information contents |  |  |  |  |  |  |  | octet x+1 |
|  |  |  |  |  |  |  |  | octet x+2 |
| CAG-ID |  |  |  |  |  |  |  | octet x+3 octet x+6 |
| 0 Spare | SVII | SVII | SVII | SVII | SVII | SVII | TVII | octet x+7 |
| Time validity information |  |  |  |  |  |  |  | octet (x+8)* octet (x+23)* |

NOTE:	The field is placed immediately after the last present preceding field.

Figure 9.11.3.86.4: CAG-ID with additional information

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of time periods |  |  |  |  |  |  |  | octet (x+8) |
| Time period 1 |  |  |  |  |  |  |  | octet (x+9) octet (x+24) |
| Time period 2 |  |  |  |  |  |  |  | octet (x+25)* octet (x+40)* |
| ... |  |  |  |  |  |  |  | octet (x+41)* octet (x-8+o*16)* |
| Time period o |  |  |  |  |  |  |  | octet (x-7+o*16) octet (x+8+o*16)* = octet u* |

Figure 9.11.3.86.5: Time validity infomation

Table 9.11.3.86.1: Extended CAG information list information element

| Value part of the Extended CAG information list information element (octet 4 to h) The value part of the Extended CAG information list information element consists of one or more entries. Entry n: Length of entry contents (octet q and q+1) MCC, Mobile country code (octet q+2 and bits 1 to 4 octet q+3) The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A. |  |
| --- | --- |
| MNC, Mobile network code (bits 5 to 8 of octet q+3 and octet q+4) The coding of this field is the responsibility of each administration, but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet q+2 shall be coded as "1111". |  |
| The MCC and MNC digits are coded as octets 6 to 8 of the Temporary mobile group identity IE in figure 10.5.154 of 3GPP TS 24.008 [12]. |  |
| Indication that the UE is only allowed to access 5GS via CAG cells (CAGonly) (bit 1 of octet q+5) |  |
| Bit |  |
| 1 |  |
| 0 | "Indication that the UE is only allowed to access 5GS via CAG cells" is not set (i.e., the UE is allowed to access 5GS via non-CAG cells) |
| 1 | "Indication that the UE is only allowed to access 5GS via CAG cells" is set (i.e., the UE is not allowed to access 5GS via non-CAG cells) |
| Length of CAG-ID without additional information list indicator (LCI) (bit 3 of octet q+5) |  |
| Bit |  |
| 3 |  |
| 0 | Length of CAG-ID without additional information list field is absent |
| 1 | Length of CAG-ID without additional information list field is present |
| If the UE does not support enhanced CAG information, the LCI bit shall be set to "Length of CAG-ID without additional information list field is absent". |  |
| CAG-ID with additional information list indicator (CAILI) (bit 4 of octet q+5) |  |
| Bit |  |
| 4 |  |
| 0 | CAG-ID with additional information list field is absent |
| 1 | CAG-ID with additional information list field is present |
| If the UE does not support enhanced CAG information or the LCI bit is set to "Length of CAG-ID without additional information list field is absent", the CAILI bit shall be set to "CAG-ID with additional information list field is absent". |  |
| Length of CAG-ID without additional information list (octet q+6 to octet q+7) |  |
| This field indicates length of CAG-ID fields in octet r to octet (r+4*m-1). |  |
| CAG-ID m (octet r+4*m-4 to octet r+4*m-1) This field contains the 32 bit CAG-ID which is not associated with additional information. The coding of the CAG-ID is defined as the CAG-Identifier in 3GPP TS 23.003 [4]. See NOTE 4. |  |
| If the length of entry contents field indicates a length bigger than indicated in figure 9.11.3.86.1, receiving entity shall ignore any superfluous octets located at the end of the entry contents. |  |
| CAG-ID (octet x+3 to octet x+6) This field contains the 32 bit CAG-ID which is associated with additional information. The additional information is indicated in remaining fields of figure 9.11.3.86.4. The coding of the CAG-ID is defined as the CAG-Identifier in 3GPP TS 23.003 [4]. |  |
| Time validity information indicator (TVII) (bit 1 of octet x+7) |  |
| Bit |  |
| 1 |  |
| 0 | Time validity information field is absent |
| 1 | Time validity information field is present |
| Spare validity information indicator (SVII) |  |
| 0 | Spare validity information is absent |
| 1 | Spare validity information is present |
| The SVII bit indicates presence of a validity information not specified in the present version of the present document. See NOTE 5. |  |
| If the SVII bit is set to "Spare validity information is present", the receiving entity shall ignore the CAG-ID with additional information field. |  |
| Time period (octet x+25 to octet x+40) |  |
| The time period field is coded as the route selection descriptor component value field for "time window type" specified in 3GPP TS 24.526 [19] table 5.2.1. |  |
| If the length of CAG-ID with additional information contents field indicates a length bigger than indicated in figure 9.11.3.86.4, receiving entity shall ignore any superfluous octets located at the end of the CAG-ID with additional information contents. |  |
| NOTE 1: The length of extended CAG information list contents field shall be 0 if no subscription data for CAG information list exists. |  |
| NOTE 2: The length of entry contents field shall be 4 if there is no allowed CAG-ID for the PLMN. |  |
| NOTE 3: For a given PLMN ID, there shall be up to one entry field containing the MCC value and the MNC value of the PLMN ID. |  |
| NOTE 4: CAG-ID field containing a CAG-ID which is not associated with additional information, can be provided regardless whether the LCI bit is set to "Length of CAG-ID without additional information list field is absent" or "Length of CAG-ID without additional information list field is present". |  |
| NOTE 5: In a future version of the present document, semantic of this bit can be changed to indicate presence or absence of an additional validity information. |  |

#### 9.11.3.87	NSAG information

The purpose of the NSAG information information element is to provide the NSAG information to the UE.

The NSAG information information element is coded as shown in figures 9.11.3.87.1, 9.11.3.87.2 and 9.11.3.87.3, and table 9.11.3.87.1.

The NSAG information information element can contain a maximum of 32 NSAG entries.

In the NSAG information information element, at most 4 NSAG entries can contain a TAI list.

The NSAG information is a type 6 information element, with a minimum length of 9 octets and a maximum length of 3143 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NSAG information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of NSAG information contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| NSAG 1 |  |  |  |  |  |  |  | octet 4 octet m |
| NSAG 2 |  |  |  |  |  |  |  | octet m+1* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* octet u* |
| NSAG x |  |  |  |  |  |  |  | octet u+1* octet v* |

Figure 9.11.3.87.1: NSAG information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NSAG |  |  |  |  |  |  |  | octet 4 |
| NSAG identifier |  |  |  |  |  |  |  | octet 5 |
| S-NSSAI list of NSAG |  |  |  |  |  |  |  | octet 6 octet j |
| NSAG priority |  |  |  |  |  |  |  | octet j+1 |
| TAI list |  |  |  |  |  |  |  | octet j+2* octet m* |

Figure 9.11.3.87.2: NSAG

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of S-NSSAI list of NSAG |  |  |  |  |  |  |  | octet 6 |
| S-NSSAI value 1 |  |  |  |  |  |  |  | octet 7 octet k |
| S-NSSAI value 2 |  |  |  |  |  |  |  | octet k+1* octet s* |
| … |  |  |  |  |  |  |  | octet s+1* octet i-1* |
| S-NSSAI value x |  |  |  |  |  |  |  | octet i* octet j* |

Figure 9.11.3.87.3: S-NSSAI list of NSAG

Table 9.11.3.87.1: NSAG information information element

NSAG part of the NSAG information information element (octet 4 to m) Each entry of the NSAG information information element consists of one NSAG in the NSAG information IE.

NSAG identifier(octet 5) NSAG identifier field contains an 8 bits NSAG ID value.

S-NSSAI list of NSAG (octet 6 to j) S-NSSAI list of NSAG field consists of one or more S-NSSAIs in the configured NSSAI. Each S-NSSAI in S-NSSAI list of NSAG field is coded as the length and value part of S-NSSAI information element as specified in clause 9.11.2.8 starting with the second octet, without the mapped HPLMN SST field and without the mapped HPLMN SD field.

NSAG priority (octet j+1) The NSAG priority field represents the binary coded value of NSAG priority for cell reselection (see 3GPP TS 38.304 [28]) and random access (see 3GPP TS 38.321 [58]). The range of the NSAG priority is 0 to 255. A lower value indicates a higher priority, with 0 as the highest priority.

TAI list (octet j+2 to m) The TAI list field is coded as the length and value part of the 5GS tracking area identity list IE defined in clause 9.11.3.9 starting with the second octet.

#### 9.11.3.88	ProSe relay transaction identity

The purpose of the ProSe relay transaction identity (PRTI) information element is to uniquely identify an authentication and key agreement procedure for 5G ProSe UE-to-network relay or 5G ProSe UE-to-UE relay. The PRTI allows distinguishing up to 254 different bi-directional messages.

The ProSe relay transaction identity information element is coded as shown in figure 9.11.3.88.1 and table 9.11.3.88.1.

The ProSe relay transaction identity is a type 3 information element with a length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ProSe relay transaction identity IEI |  |  |  |  |  |  |  | octet 1 |
| ProSe relay transaction identity value |  |  |  |  |  |  |  | octet 2 |

Figure 9.11.3.88.1: ProSe relay transaction identity information element

Table 9.11.3.88.1: ProSe relay transaction identity information element

| ProSe relay transaction identity value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | No ProSe relay transaction identity assigned |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | \ |
| to |  |  |  |  |  |  |  |  | } ProSe relay transaction identity value |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  | / |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | Reserved |

#### 9.11.3.89	Relay key request parameters

The purpose of the relay key request parameters information element is to transport the parameters of the key request for 5G ProSe UE-to-network relay or 5G ProSe UE-to-UE relay as specified in 3GPP TS 33.503 [56].

The relay key request parameters information element is coded as shown in figure 9.11.3.89.1, figure 9.11.3.89.2 and table 9.11.3.89.1.

The relay key request parameters is a type 6 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relay key request parameters IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Relay key request parameters |  |  |  |  |  |  |  | octet 2 octet 3 |
| Relay service code |  |  |  |  |  |  |  | octet 4 octet 6 |
| Nonce_1 |  |  |  |  |  |  |  | octet 7 octet 22 |
| UE identity |  |  |  |  |  |  |  | octet 23 octet n |

Figure 9.11.3.89.1: Relay key request parameters information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | UIT | octet 23 |
| UE ID |  |  |  |  |  |  |  | octet 23+1 octet n |

Figure 9.11.3.89.2: UE identity

Table 9.11.3.89.1: Relay key request parameters information element

Relay service code (octet 4 to 6) The relay service code contains 24-bit relay service code as defined in 3GPP TS 24.554 [19E]. Nonce_1 (octet 7 to 22) Nonce_1 is the 128-bit nonce value as defined in 3GPP TS 24.554 [19E]. UE ID type (UIT) (octet 23, bit 1) Bit 1 0 SUCI 1 CP-PRUK ID UE ID (octet 23+1 to n) UE ID indicates the value of the 5G ProSe remote UE identity or the 5G ProSe end UE identity. If the UE ID type is set to SUCI, the UE ID is coded as 5GS mobile identity IE starting from octet 2 with the Type of identity set to "SUCI" (see clause 9.11.3.4). If the UE ID type is set to CP-PRUK ID, the UE ID is coded as the CP-PRUK ID as defined in 3GPP TS 33.503 [56].

#### 9.11.3.90	Relay key response parameters

The purpose of the relay key response parameters information element is to transport the parameters of the key response for 5G ProSe UE-to-network relay or 5G ProSe UE-to-UE relay as specified in 3GPP TS 33.503 [56].

The relay key response parameters information element is coded as shown in figure 9.11.3.90.1 and table 9.11.3.90.1.

The relay key response parameters is a type 6 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relay key response parameters IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Relay key response parameters |  |  |  |  |  |  |  | octet 2 octet 3 |
| Key KNR_ProSe |  |  |  |  |  |  |  | octet 4 octet 35 |
| Nonce_2 |  |  |  |  |  |  |  | octet 36 octet 51 |
| CP-PRUK ID |  |  |  |  |  |  |  | octet 52 octet m |

Figure 9.11.3.90.1: Relay key response parameters information element

Table 9.11.3.90.1: Relay key response parameters information element

Key KNR_ProSe (octet 5 to 35) Key KNR_ProSe contains a 256-bit root key that is established between the two entities that communicating using NR PC5 unicast link as defined in 3GPP TS 33.503 [56]. Nonce_2 (octet 36 to 51) Nonce_2 is the 128-bit nonce value as defined in 3GPP TS 24.554 [19E]. CP-PRUK ID (octet 52 to m) The CP-PRUK ID is defined in 3GPP TS 33.503 [56].

#### 9.11.3.91	Priority indicator

The purpose of the Priority indicator information element is to convey a priority indication to the UE.

The Priority indicator information element is coded as shown in figure 9.11.3.91.1 and table 9.11.3.91.1.

The Priority indicator is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Priority indicator IEI |  |  |  | 0 Spare | 0 Spare | MCSI | MPSI | octet 1 |

Figure 9.11.3.91.1: Priority indicator

Table 9.11.3.91.1: Priority indicator

| MPS indicator (MPSI) (octet 1, bit 1) |  |  |
| --- | --- | --- |
| Bit 1 |  |  |
| 0 1 |  | Access identity 1 not valid Access identity 1 valid |
| MCS indicator (MCSI) (octet 1, bit 2) |  |  |
| Bit 2 |  |  |
| 0 | Access identity 2 not valid |  |
| 1 | Access identity 2 valid |  |
| Bits 3, 4 are spare and shall be coded as zero. |  |  |

#### 9.11.3.92	SNPN list

The purpose of the SNPN list information element is to provide a list of SNPN identities.

The SNPN list information element is coded as shown in figure 9.11.3.92.1, table 9.11.3.92.1, figure 9.11.3.92.2 and table 9.11.3.92.2.

The SNPN list is a type 4 information element with a minimum length of 11 octets and a maximum length of 137 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNPN list IEI |  |  |  |  |  |  |  | octet 1 |
| Length of SNPN list contents |  |  |  |  |  |  |  | octet 2 |
| SNPN identity 1 |  |  |  |  |  |  |  | octet 3 octet 11 |
| SNPN identity 2 |  |  |  |  |  |  |  | octet 12* octet 20* |
| … |  |  |  |  |  |  |  | octet 21* octet (n*9-7)* |
| SNPN identity n |  |  |  |  |  |  |  | octet (n*9-6)* octet (n*9+2)* |

Figure 9.11.3.92.1: SNPN list information element

Table 9.11.3.92.1: SNPN list information element

Each SNPN identity field is coded according to figure 9.11.3.92.2 and table 9.11.3.92.2.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet (i*9-6) |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet (i*9-5) |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet (i*9-4) |
| NID |  |  |  |  |  |  |  | octet (i*9-3) octet (i*9+2) |

Figure 9.11.3.92.2: SNPN identity i

Table 9.11.3.92.2: SNPN identity i

MCC, Mobile country code (octet (i*9-6), octet (i*9-5) bits 1 to 4)

The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A.

MNC, Mobile network code (octet (i*9-5) bits 5 to 8, octet (i*9-4))

The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet 6 shall be coded as "1111"

The MCC and MNC digits are coded as octets 6 to 8 of the Temporary mobile group identity IE in figure 10.5.154 of 3GPP TS 24.008 [12].

NID (octet (i*9-3) to octet (i*9+2))

NID field is coded as the NID field of NID IE as specified in figure 9.2.7-2 and table 9.2.7-2 of 3GPP TS 24.502 [18] starting with the octet 3 and ending with the octet 8.

#### 9.11.3.93	N3IWF identifier

The purpose of the N3IWF identifier information element is to enable the network to assign the UE, a suitable N3IWF for the requested NSSAI.

The N3IWF identifier information element is coded as shown in figure 9.11.3.93.1, figure 9.11.3.93.2, figure 9.11.3.93.3 and table 9.11.3.93.1.

The N3IWF identifier information element is a type 4 information element with a minimum length of 7 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N3IWF identifier IEI |  |  |  |  |  |  |  | octet 1 |
| Length of N3IWF identifier contents |  |  |  |  |  |  |  | octet 2 |
| N3IWF identifier type |  |  |  |  |  |  |  | octet 3 |
| N3IWF identifier entry |  |  |  |  |  |  |  | octet 4 octet k |

Figure 9.11.3.93.1: N3IWF identifier information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N3IWF IP address |  |  |  |  |  |  |  | octet 4 octet m |

Figure 9.11.3.93.2: N3IWF address entry (N3IWF identifier type = "IPv4", "IPv6" or "IPv4v6")

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N3IWF FQDN |  |  |  |  |  |  |  | octet 4 octet n |

Figure 9.11.3.93.3: N3IWF identifier entry (N3IWF identifiertype ="FQDN")

Table 9.11.3.93.1: N3IWF address entry

| N3IWF identifier type (octet 3) is set as follows: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | IPv4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | IPv6 |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | IPv4v6 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | FQDN |
| All other values are reserved. |  |  |  |  |  |  |  |  |  |
| If the N3IWF identifier type indicates IPv4, then the N3IWF IP address field contains an IPv4 address in octet 4 to octet 7. |  |  |  |  |  |  |  |  |  |
| If the N3IWF identifier type indicates IPv6, then the N3IWF IP address field contains an IPv6 address in octet 4 to octet 19. |  |  |  |  |  |  |  |  |  |
| If the N3IWF identifier type indicates IPv4v6, then the N3IWF IP address field contains two IP addresses. The first IP address is an IPv4 address in octet 4 to octet 7. The second IP address is an IPv6 address in octet 8 to octet 23. |  |  |  |  |  |  |  |  |  |
| If the N3IWF identifier type indicates FQDN, the N3IWF FQDN field in octet 4 to octet n is encoded as defined in clauses 28.3.2.2.2, 28.3.2.2.3, 28.3.2.2.8 or 28.3.2.2.9 in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |

#### 9.11.3.94	TNAN information

The purpose of the TNAN information information element is to enable the network to assign the UE, a suitable TNAN information (SSID and TNGF ID) for the requested NSSAI.

The TNAN information information element is coded as shown in figure 9.11.3.94.1 and table 9.11.3.94.1.

The TNAN information information element is a type 4 information element with a minimum length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TNAN information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of TNAN information contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare |  |  |  |  |  | SSID ind | TNGF ID ind | octet 3 |
| TNGF ID length |  |  |  |  |  |  |  | octet 4* |
| TNGF ID |  |  |  |  |  |  |  | octet 5* octet m* |
| SSID length |  |  |  |  |  |  |  | octet m+1* |
| SSID |  |  |  |  |  |  |  | octet m+2* octet u* |

Figure 9.11.3.94.1: TNAN information information element

Table 9.11.3.94.1: TNAN information information element

| TNGF ID indication (TNGF ID ind) (bit 1 of octet 3) |  |
| --- | --- |
| The TNGF ID ind indicates whether the TNGF ID length field and the TNGF ID field are included in the TNAN information or not. |  |
| Bit |  |
| 1 |  |
| 0 | TNGF ID length and TNGF ID not included |
| 1 | TNGF ID length and TNGF ID included |
| SSID indication (SSID ind) (bit 2 of octet 3) |  |
| The SSID ind indicates whether the SSID length field and the SSID field are included in the TNAN information or not. |  |
| Bit |  |
| 2 |  |
| 0 | SSID length and SSID not included |
| 1 | SSID length and SSID included |
| Bits 3 to 8 of octet 3 are spare and shall be coded as zero |  |
| TNGF ID length (octet 4) indicates the length of the TNGF ID field. |  |
| TNGF ID (octets 5 to m) |  |
| The TNGF ID field is an octet string that indicates the TNGF ID. |  |
| SSID length (octet m+1) indicates the length of the SSID field. |  |
| SSID (octets m+2 to u) |  |
| The SSID field is an octet string which shall have a maximum length of 32 octets (see IEEE Std 802.11 [59]). |  |

#### 9.11.3.95	RAN timing synchronization

The purpose of the RAN timing synchronization IE is to provide information related to the RAN timing synchronization.

The RAN timing synchronization information element is coded as shown in figure 9.11.3.95.1 and table 9.11.3.95.1.

The RAN timing synchronization is a type 4 information element with a length of 3 octets.

|  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAN timing synchronization IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |  |
| Length of RAN timing synchronization |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | Rec Req |  | octet 3 |  |  |

Figure 9.11.3.95.1: RAN timing synchronization information element

Table 9.11.3.95.1: RAN timing synchronization information element

| Request to reconnect to the network upon receiving an indication of a change in the RAN timing synchronization status (RecReq) (octet 3, bit 1) |  |  |
| --- | --- | --- |
| Bit |  |  |
| 1 |  |  |
| 0 |  | Reconnection not requested |
| 1 |  | Reconnection requested |
| Bits 2 to 8 of octet 3 are spare and shall be coded as zero. |  |  |

#### 9.11.3.96	Extended LADN information

The purpose of the Extended LADN information information element is to provide the UE with the LADN service area for each available LADN associated for an LADN DNN and an S-NSSAI in the current registration area or to delete the Extended LADN information at the UE.

The Extended LADN information information element is coded as shown in figure 9.11.3.96.1, figure 9.11.3.96.2 and table 9.11.3.96.1.

The Extended LADN information is a type 6 information element with a minimum length of 3 octets and a maximum length of 1787 octets.

The Extended LADN information information element can contain a minimum of 0 and a maximum of 8 different LADNs each including a DNN, an S-NSSAI and a 5GS tracking area identity list.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended LADN information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Extended LADN information contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| LADN 1 |  |  |  |  |  |  |  | octet 4 octet a |
| LADN 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* octet g* |
| LADN n |  |  |  |  |  |  |  | octet g+1* octet h* |

Figure 9.11.3.96.1: Extended LADN information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of DNN value |  |  |  |  |  |  |  | octet 4 |
| DNN value |  |  |  |  |  |  |  | octet 5 octet m |
| S-NSSAI |  |  |  |  |  |  |  | octet m+1 octet n |
| 5GS tracking area identity list |  |  |  |  |  |  |  | octet n+1 octet a |

Figure 9.11.3.96.2: LADN

Table 9.11.3.96.1: Extended LADN information information element

Value part of the Extended LADN information information element (octet 4 to octet h) The value part of the Extended LADN information information element consists of one or several LADNs. Each LADN (e.g. octet 4 to octet a) consists of one DNN value, one S-NSSAI and one 5GS tracking area identity list. The length of each LADN is determined by the length of DNN value field, the length of S-NSSAI field and the length of 5GS tracking area identity list field. The UE shall store the complete list as received. If more than 8 LADNs are included in this information element, the UE shall store the first 8 LADNs and ignore the remaining octets of the information element. DNN value (octet 5 to octet m): DNN value field is coded as DNN value part of DNN information element as specified in clause 9.11.2.1B starting with the third octet.

S-NSSAI (octet m+1 to n) (see NOTE 1) S-NSSAI is coded as the length and value part of S-NSSAI information element as specified in clause 9.11.2.8 starting with the second octet. 5GS tracking area identity list (octet m+1 to octet a):

5GS tracking area identity list field is coded as the length and the value part of the 5GS Tracking area identity list information element as specified in clause 9.11.3.9 starting with the second octet.

NOTE 1: The S-NSSAI included in the Extended LADN information information element shall be an S-NSSAI from, an allowed NSSAI or a partially allowed NSSAI provided to the UE.

#### 9.11.3.97	Alternative NSSAI

The purpose of the Alternative NSSAI information element is to identify a list of mapping information between the S-NSSAI to be replaced and the alternative S-NSSAI.

The Alternative NSSAI information element is coded as shown in figure 9.11.3.97.1, figure 9.11.3.97.2 and table 9.11.3.97.1.

The Alternative NSSAI is a type 4 information element with minimum length of 2 octets and maximum length of 146 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alternative NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Alternative NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| Entry 1 |  |  |  |  |  |  |  | octet 3* octet a* |
| Entry 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* octet c* |
| Entry n |  |  |  |  |  |  |  | octet c+1* octet d* |

Figure 9.11.3.97.1: Alternative NSSAI information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-NSSAI to be replaced |  |  |  |  |  |  |  | octet 3 octet x |
| Alternative S-NSSAI |  |  |  |  |  |  |  | octet x+1 octet a |

Figure 9.11.3.97.2: Entry

Table 9.11.3.97.1: Alternative NSSAI information element

Value part of the Alternative NSSAI information element (octet 3 to d) The value part of the Alternative NSSAI information element consists of one or more entries, each entry consists of one S-NSSAI to be replaced and one alternative S-NSSAI. The number of entries shall not exceed eight. S-NSSAI to be replaced (octet 3 to x) (see NOTE) S-NSSAI to be replaced is coded as the length and value part of S-NSSAI information element as specified in clause 9.11.2.8 starting with the second octet.

Alternative S-NSSAI (octet x+1 to a)

Alternative S-NSSAI is coded as the length and value part of S-NSSAI information element as specified in clause 9.11.2.8 starting with the second octet.

NOTE: The S-NSSAI to be replaced shall be one S-NSSAI included in the allowed NSSAI or partially allowed NSSAI.

#### 9.11.3.98	Type 6 IE container

The purpose of the Type 6 IE container information element is to transfer type 6 IEs of format TLV-E explicitly specified for inclusion in this information element for the respective message.

NOTE:	Use of this information element is intended only for type 6 IEs added to a message in Rel-18 or later.

The rules for the IEI value encoding specified in 3GPP TS 24.007 [11], clause 11.2.4, are not applicable for the IEIs of the type 6 IEs within the Type 6 IE container information element. These IEIs can take any value in the range 00 to FF (hexadecimal).

The type 6 IE container information element is coded as shown in figure 9.11.3.98.1, figure 9.11.3.98.2 and table 9.11.3.98.1.

The type 6 IE container is a type 6 information element with a minimum length of 6 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type 6 IE container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Type 6 IE container contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet 4 |
| Type 6 IE container contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 9.11.3.98.1: Type 6 IE container information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type 6 IE 1 |  |  |  |  |  |  |  | octet 4 octet j |
| Type 6 IE 2 |  |  |  |  |  |  |  | octet j + 1* octet k* |
| ….. |  |  |  |  |  |  |  | octet k+1* octet m* |
| Type 6 IE z |  |  |  |  |  |  |  | octet m+1* octet n* |

Figure 9.11.3.98.2: Type 6 IE container contents

Table 9.11.3.98.1: Type 6 IE container contents

Type 6 IE container contents (octets 4 to n) The Type 6 IE container is coded according to figure 9.11.3.98.2.

The sender of this information element shall encode each type 6 IE included in the contents in format TLV-E.

For the coding of each of the type 6 IEs in the type 6 IE container, see the definition of the respective type 6 IE in clause 9.11

#### 9.11.3.99	Non-3GPP access path switching indication

The purpose of the Non-3GPP access path switching indication information element is to indicate whether the UE supports the non-3GPP access path switching for the PDU session.

The Non-3GPP access path switching indication information element is coded as shown in figure 9.11.3.99.1 and table 9.11.3.99.1.

The Non-3GPP access path switching indication is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 |  | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-3GPP access path switching indication IEI |  |  |  |  |  |  |  |  |  | octet 1 |
| Non-3GPP access path switching indication length |  |  |  |  |  |  |  |  |  | octet 2 |
| 0 | 0 | 0 | 0 | 0 |  | 0 |  | 0 | NAPS | octet 3 |
| Spare |  |  |  |  |  |  |  |  |  |  |

Figure 9.11.3.99.1: Non-3GPP access path switching indication information element

Table 9.11.3.99.1 : Non-3GPP access path switching indication information element

| Non-3GPP access path switching (NAPS) (octet 3, bit 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| This bit indicates whether non-3GPP access path switching is supported. |  |  |  |  |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | non-3GPP access path switching not supported |
| 1 |  |  |  | non-3GPP access path switching supported |
| Bits 2 to 8 of octet 3 are spare and shall be coded as zero. |  |  |  |  |

#### 9.11.3.100	S-NSSAI location validity information

The purpose of the S-NSSAI location validity information information element is to provide the S-NSSAI location validity information to the UE.

The S-NSSAI location validity information information element is coded as shown in figures 9.11.3.100.1, 9.11.3.100.2, 9.11.3.100.3, and 9.11.3.100.4 and table 9.11.3.100.1.

The S-NSSAI location validity information information element can contain per-S-NSSAI location validity information for maximum 16 S-NSSAIs.

The S-NSSAI location validity information information is a type 6 information element with a minimum length of 17 octets and a maximum length of 38611 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-NSSAI location validity information information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of S-NSSAI location validity information contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Per-S-NSSAI location validity information for S-NSSAI 1 |  |  |  |  |  |  |  | octet 4 octet a |
| Per-S-NSSAI location validity information for S-NSSAI 2 |  |  |  |  |  |  |  | octet (a+1)* octet b* |
| … |  |  |  |  |  |  |  | octet (b+1)* octet c* |
| Per-S-NSSAI location validity information for S-NSSAI n |  |  |  |  |  |  |  | octet (c+1)* octet d* |

Figure 9.11.3.100.1: S-NSSAI location validity information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Per-S-NSSAI location validity information for S-NSSAI |  |  |  |  |  |  |  | octet 4 octet 5 |
| S-NSSAI |  |  |  |  |  |  |  | octet 6 octet e |
| NS-AoS |  |  |  |  |  |  |  | octet e+1 octet a |

Figure 9.11.3.100.2: Per-S-NSSAI location validity information for S-NSSAI

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of NR CGIs |  |  |  |  |  |  |  | octet (e+1) octet (e+2) |
| NR CGI 1 |  |  |  |  |  |  |  | octet (e+3) octet (e+10) |
| NR CGI 2 |  |  |  |  |  |  |  | octet (e+11)* octet (e+18)* |
| … |  |  |  |  |  |  |  | octet (e+19)* octet f* |
| NR CGI w |  |  |  |  |  |  |  | octet (f+1)* octet a=(f+8)* |

Figure 9.11.3.100.3: NS-AoS

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Cell ID |  |  |  |  |  |  |  | octet (e+3) octet (e+7) |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet (e+8) |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet (e+9) |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet (e+10) |

Figure 9.11.3.100.4: NR CGI

Table 9.11.3.100.1: S-NSSAI location validity information information element

S-NSSAI (octet 6 to e) S-NSSAI value is coded as the length and value part of S-NSSAI information element as specified in clause 9.11.2.8 starting with the second octet.

NS-AoS (octet e+1 to octet a)

NS-AoS field consists of the Number of NR CGIs field and at least one NR CGI.

Number of NR CGIs (octet e+1 to octet e+2)

The field indicates the number of NR CGIs included in octets e+3 to octet a. (NOTE).

NR CGI (octet e+3 to e+10)

The NR CGI globally identifies an NR cell. It contains the NR Cell ID and the PLMN ID of that cell.

NR Cell ID (octet e+3 to e+7)

The NR Cell ID consists of 36 bits identifying an NR Cell ID as specified in clause 9.3.1.7 of 3GPP TS 38.413 [31], in hexadecimal representation. Bit 8 of octet e+3 is the most significant bit and bit 5 of octet e+7 is the least significant bit. Bits 1 to 4 of octet e+7 are spare and shall be coded as zero.

MCC, Mobile country code (octet e+8 and bits 1 to 4 octet e+9) The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A.

MNC, Mobile network code (bits 5 to 8 of octet e+9 and octet e+10) The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet e+9 shall be coded as "1111".

The MCC and MNC digits are coded as octets 6 to 8 of the Temporary mobile group identity IE in figure 10.5.154 of 3GPP TS 24.008 [12].

NOTE: In this version of the specification, the maximum number of NR CGIs is 300.

#### 9.11.3.101	S-NSSAI time validity information

The purpose of the S-NSSAI time validity information information element is to provide S-NSSAI time validity information of one or more S-NSSAIs to the UE.

The S-NSSAI time validity information information element is coded as shown in figures 9.11.3.101.1 and 9.11.3.101.2 and table 9.11.3.101.1.

The S-NSSAI time validity information information element can contain per-S-NSSAI time validity information for maximum 16 S-NSSAIs.

The S-NSSAI time validity information information is a type 4 information element with a minimum length of 23 octets and a maximum length of 257 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-NSSAI time validity information information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of S-NSSAI time validity information contents |  |  |  |  |  |  |  | octet 2 |
| Per-S-NSSAI time validity information for S-NSSAI 1 |  |  |  |  |  |  |  | octet 3 octet a |
| Per-S-NSSAI time validity information for S-NSSAI 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* octet c* |
| Per-S-NSSAI time validity information for S-NSSAI n |  |  |  |  |  |  |  | octet c+1* octet d* |

Figure 9.11.3.101.1: S-NSSAI time validity information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Per-S-NSSAI time validity information for S-NSSAI |  |  |  |  |  |  |  | octet 3 |
| S-NSSAI |  |  |  |  |  |  |  | octet 4 octet e |
| Per-S-NSSAI time validity information for the S-NSSAI |  |  |  |  |  |  |  | octet e+1 octet a |

Figure 9.11.3.101.2: Per-S-NSSAI time validity information for S-NSSAI 1

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Per-S-NSSAI time validity information for the S-NSSAI |  |  |  |  |  |  |  | octet e+1 |
| Time window 1 |  |  |  |  |  |  |  | octet e+2 octet f |
| Time window 2 |  |  |  |  |  |  |  | octet f+1* octet g* |
| … |  |  |  |  |  |  |  | octet g+1* octet h* |
| Time window m |  |  |  |  |  |  |  | octet h+1* octet i* |

Figure 9.11.3.101.3: Per-S-NSSAI time validity information for the S-NSSAI

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Time window 1 |  |  |  |  |  |  |  | octet e+2 |
| Start time |  |  |  |  |  |  |  | octet e+3 octet e+10 |
| Stop time |  |  |  |  |  |  |  | octet e+11 octet e+18 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | Recurrence pattern |  |  |  | octet e+19* |
| Recurrence end time |  |  |  |  |  |  |  | octet e+20* octet e+27* |

Figure 9.11.3.101.4: Time window 1

Table 9.11.3.101.1: S-NSSAI time validity information information element

| S-NSSAI (octet 6 to octet e) S-NSSAI value is coded as the length and value part of S-NSSAI information element as specified in clause 9.11.2.8 starting with the second octet. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Start time (octet e+3 to octet e+10) |  |  |  |  |  |
| The field indicates the time when the network slice identified by the S-NSSAI becomes available (for the first time if octet e+20 is included) and is represented by the number of seconds since 00:00:00 on 1 January 1970 UTC and is encoded as the 64-bit NTP timestamp format defined in IETF RFC 5905 [36A], where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. |  |  |  |  |  |
| Stop time (octet e+11 to octet e+18) |  |  |  |  |  |
| The field indicates the time when the network slice identified by the S-NSSAI becomes unavailable (for the first time if octet e+20 is included) and is represented by the number of seconds since 00:00:00 on 1 January 1970 UTC and is encoded as the 64-bit NTP timestamp format defined in IETF RFC 5905 [36A], where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. |  |  |  |  |  |
| Recurrence pattern (bit 1 to bit 4 of octet e+19) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 |  | Everyday |
| 0 | 0 | 0 | 1 |  | Every weekday |
| 0 | 0 | 1 | 0 |  | Every week |
| 0 | 0 | 1 | 1 |  | Every 2 weeks |
| 0 | 1 | 0 | 0 |  | Every month (absolute) |
| 0 | 1 | 0 | 1 |  | Every month (relative) |
| 0 | 1 | 1 | 0 |  | Every quarter (absolute) |
| 0 | 1 | 1 | 1 |  | Every quarter (relative) |
| 1 | 0 | 0 | 0 |  | Every 6 months (absolute) |
| 1 | 0 | 0 | 1 |  | Every 6 months (relative) |
| All other values are reserved. The recurrence pattern indicates how often the time window is repeated. For example, if the time window starts at 13:00 on Wednesday January 1st 2020 and stops at 13:30 on Wednesday January 1st 2020 and the recurrent pattern is set to: - "Everyday", the time window repeats everyday from 13:00 to 13:30; - "Every week", the time window repeats every Wednesday from 13:00 to 13:30; - "Every month (absolute)", the time window repeats every 1st day of the month from 13:00 to 13:30; and - "Every month (relative)", the time window repeats every month on the first Wednesday from 13:00 to 13:30. |  |  |  |  |  |
| Recurrence end time (octet e+20 to octet e+27) |  |  |  |  |  |
| The field indicates the time when the repetition of the time window ends. If the field is not included and octet e+19 is included in the IE, the time window is repeated indefinitely. The field is represented by the number of seconds since 00:00:00 on 1 January 1970 UTC and is encoded as the 64-bit NTP timestamp format defined in IETF RFC 5905 [36A], where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. |  |  |  |  |  |

#### 9.11.3.102	Non-3GPP path switching information

The purpose of the Non-3GPP path switching information information element is to request from the network to keep using the user plane resources of the old non-3GPP access during path switching to the new non-3GPP access.

The Non-3GPP path switching information information element is coded as shown in figure 9.11.3.102.1 and table 9.11.3.102.1.

The Non-3GPP path switching information is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-3GPP path switching information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Non-3GPP path switching information |  |  |  |  |  |  |  | octet 2 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | NSONR | octet 3 |
| Spare |  |  |  |  |  |  |  |  |

Figure 9.11.3.102.1: Non-3GPP path switching information information element

Table 9.11.3.102.1: Non-3GPP path switching information information element

| Non-3GPP path switching while using old non-3GPP resources (NSONR) (octet 3, bit 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | non-3GPP path switching while using old non-3GPP resources not requested |
| 1 |  |  |  | non-3GPP path switching while using old non-3GPP resources requested |
| Bits 8 to 2 of octet 3 are spare and shall be coded as zero. |  |  |  |  |

#### 9.11.3.103	Partial NSSAI

The purpose of the Partial NSSAI information element is to deliver one or more S-NSSAIs in a set of tracking areas of a registration area from the network to the UE.

The Partial NSSAI information element is coded as shown in figure 9.11.3.103.1, and table 9.11.3.103.1.

The Partial NSSAI information element is a type 6 information element, with a minimum length of 3 octets and a maximum length of 808 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Partial NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of partial NSSAI contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| S-NSSAI value1 |  |  |  |  |  |  |  | octet 4* octet i* |
| Partial tracking area identity list 1 |  |  |  |  |  |  |  | octet (i+1)* octet j* |
| … |  |  |  |  |  |  |  | octet (j+1)* octet l* |
| S-NSSAI value n |  |  |  |  |  |  |  | octet (l+1)* octet k* |
| Partial tracking area identity list n |  |  |  |  |  |  |  | octet (k+1)* octet m* |

Figure 9.11.3.103.1: Partial NSSAI information element

Table 9.11.3.103.1: Partial NSSAI information element

S-NSSAI value (octet 4 to i) (NOTE 1)

S-NSSAI value is coded as the length and value part of S-NSSAI information element as specified in clause 9.11.2.8 starting with the second octet.

Partial tracking area identity list (octet i+1 to j)

The partial tracking area identity list field is coded as the length and value part of the 5GS tracking area identity list IE defined in clause 9.11.3.9 starting with the second octet (NOTE 2).

NOTE 1: The maximum number of S-NSSAIs included in this information element is 7. NOTE 2: A registration area contains maximum 16 different tracking areas, therefore the partial tracking area identity list can contain at the most 15 tracking area identities. NOTE 3: The AMF shall set the Length of partial NSSAI contents to 0 if there are no S-NSSAIs to deliver in a set of tracking areas of a registration area and, the UE shall delete any existing stored partially allowed NSSAI for the current registration area or partially rejected NSSAI for the current registration area.

#### 9.11.3.104	AUN3 indication

The purpose of the AUN3 indication information element is to indicate to the network that the registration request by the 5G-RG is on behalf of an AUN3 device.

The AUN3 indication information element is coded as shown in figure 9.11.3.104.1 and table 9.11.3.104.1.

The AUN3 indication is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AUN3 indication IEI |  |  |  |  |  |  |  | octet 1 |
| Length of AUN3 indication |  |  |  |  |  |  |  | octet 2 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | AUN3REG | octet 3 |
| Spare |  |  |  |  |  |  |  |  |

Figure 9.11.3.104.1: AUN3 indication information element

Table 9.11.3.104.1: AUN3 indication information element

| AUN3 device indication bit (AUN3REG) (octet 3, bit 1) |  |  |
| --- | --- | --- |
| Bit |  |  |
| 1 |  |  |
| 0 |  | AUN3 device registration is not requested |
| 1 |  | AUN3 device registration is requested |
| Bits 8 to 2 of octet 3 are spare and shall be coded as zero. |  |  |

#### 9.11.3.105	Feature authorization indication

The purpose of the Feature authorization indication information element is to indicate whether the UE that is authorized to operate certain feature.

The Feature authorization indication is a type 4 information element with a minimum length of 3 octets and maximum length of 257 octets.

The Feature authorization indication information element is coded as shown in Figure 9.11.3.105.1 and Table 9.11.3.105.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Feature authorization indication IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Feature authorization indication contents |  |  |  |  |  |  |  | octet 2 |
| 0 | 0 | 0 | 0 | 0 | HPASE | MBSRAI |  | octet 3 |
| Spare | Spare | Spare | Spare | Spare |  |  |  |  |
| Spare |  |  |  |  |  |  |  | octet 4 octet n |

Figure 9.11.3.105.1: Feature authorization indication information element

Table 9.11.3.105.1: Feature authorization indication information element

| MBSR authorization indication (MBSRAI) (octet 3, bit 1 to bit 2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| This field indicates whether UE is authorized or not to operate as an MBSR node |  |  |  |  |
| Bits |  |  |  |  |
| 2 | 1 |  |  |  |
| 0 | 0 |  |  | no information |
| 0 | 1 |  |  | not authorized to operate as MBSR but allowed to operate as a UE |
| 1 | 0 |  |  | authorized to operate as MBSR |
| 1 | 1 |  |  | spare |
| High priority access service area restriction exempt (HPASE) (octet 3, bit 3) |  |  |  |  |
| This field indicates the operator policy for high priority access exemption for service area restrictions |  |  |  |  |
| Bit |  |  |  |  |
| 3 |  |  |  |  |
| 0 |  |  |  | High priority access UEs are exempt from service area restrictions or the network does not support operator policy for high priority access exemptions for service area restrictions |
| 1 |  |  |  | High priority access UEs are not exempt from service area restrictions |
| Bits 4 to 8 of octet 3 are spare and shall be coded as zero. |  |  |  |  |

#### 9.11.3.106	Payload container information

The purpose of the Payload container information information element is to provide information related to a payload container.

The Payload container information is a type 1 information element.

The Payload container information information element is coded as shown in figure 9.11.3.106.1 and table 9.11.3.106.1.

| 8 | 7 | 6 | 5 |  | 4 | 3 |  | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Payload container information IEI |  |  |  | 0 |  |  | 0 spare |  | 0 | PRU | octet 1 |

Figure 9.11.3.106.1: Payload container information information element

Table 9.11.3.106.1: Payload container information information element

| PRU related content (PRU) (octet 1, bit 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | Payload container not related to PRU |
| 1 |  |  |  | Payload container related to PRU |
| Bits 2 to 4 are spare and shall be coded as zero. |  |  |  |  |

#### 9.11.3.107	AUN3 device security key

The purpose of the AUN3 device security key information element is to provide the security keys to the 5G-RG that is acting on behalf of an AUN3 device.

The AUN3 device security key information element is coded as shown in figure 9.11.3.107.1 and table 9.11.3.107.1.

The AUN3 device security key is a type 4 information element with a minimum length of 36 octets.

| 8 | 7 |  | 6 |  |  | 5 | 4 |  | 3 | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AUN3 device security key IEI |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of AUN3 device security key content |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |
| 0 |  | 0 |  | 0 | 0 |  |  | 0 | 0 |  | ASKT |  | octet 3 |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length of key content |  |  |  |  |  |  |  |  |  |  |  |  | octet 4 |
| Key content |  |  |  |  |  |  |  |  |  |  |  |  | octet 5 octet n |

Figure 9.11.3.107.1: AUN3 device security key information element

Table 9.11.3.107.1: AUN3 device security key information element

| AUN3 device security key type (ASKT) (bits 1 and 2 of octet 3) |  |  |
| --- | --- | --- |
| The ASKT indicates which AUN3 device security key is included in the IE. Bits |  |  |
| 2 | 1 |  |
| 0 | 0 | Master session key is included |
| 0 | 1 | KWAGF key is included |
| All other values are unused and shall be interpreted as "Master session key is included", if received by the UE. |  |  |
| Key content (octets 5 to n) If ASKT is set to "Master session key included", the Key content contains the value of the Master session key as defined in 3GPP TS 33.501 [24]. If ASKT is set to "KWAGF key included", the Key content contains the value of the KWAGF key as defined in 3GPP TS 33.501 [24]. |  |  |

#### 9.11.3.108	On-demand NSSAI

The purpose of the On-demand NSSAI information element is to provide a list of one or more on-demand S-NSSAIs and the associated slice deregistration inactivity timer value per the on-demand S-NSSAI to the UE.

The On-demand NSSAI information element is coded as shown in figure 9.11.3.108.1, figure 9.11.3.108.2, and table 9.11.3.108.1.

The On-demand NSSAI is a type 4 information element with a minimum length of 5 octets and a maximum length of 210 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| On-demand NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of On-demand NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| Information of on-demand S-NSSAI 1 |  |  |  |  |  |  |  | octet 3 octet a |
| Information of on-demand S-NSSAI 2 |  |  |  |  |  |  |  | octet (a+1)* octet b* |
| … |  |  |  |  |  |  |  | octet (b+1)* octet c* |
| Information of on-demand S-NSSAI n |  |  |  |  |  |  |  | octet (c+1)* octet d* |

Figure 9.11.3.108.1: On-demand NSSAI information element

| Length of Information of on-demand S-NSSAI | octet 3 |
| --- | --- |
| On-demand S-NSSAI | octet 4 octet a-3 |
| Slice deregistration inactivity timer | octet (a-2)* octet a* |

Figure 9.11.3.108.2: Information of on-demand S-NSSAI

Table 9.11.3.108.1: On-demand NSSAI information element

Value part of the On-demand NSSAI information element (octet 3 to d) The value part of the On-demand NSSAI information element consists of one or more Information of on-demand S-NSSAIs. Each Information of on-demand S-NSSAI consists of one on-demand S-NSSAI and optionally the slice deregistration inactivity timer of the on-demand S-NSSAI. The number of information of on-demand S-NSSAIs shall not exceed 16. On-demand S-NSSAI (octet 4 to a-3) On-demand S-NSSAI is coded as the length and value part of S-NSSAI information element as specified in clause 9.11.2.8 starting with the second octet. Slice deregistration inactivity timer (octet (a-2)* to a*) Slice deregistration inactivity timer is coded as the value part of Time duration information element as specified in clause 9.9.3.68 of 3GPP TS 24.301 [15] starting with the third octet.

#### 9.11.3.109	Extended 5GMM cause

The purpose of the Extended 5GMM cause information element is to indicate additional information associated with a 5GMM cause.

The Extended 5GMM cause information element is coded as shown in figure 9.11.3.109.1 and table 9.11.3.109.1.

The Extended 5GMM cause is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended 5GMM cause IEI |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of Extended 5GMM cause contents |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | Sat-NR |  | octet 3 |  |

Figure 9.11.3.109.1: Extended 5GMM cause information element

Table 9.11.3.109.1: Extended 5GMM cause information element

| Sat-NR value (octet 3, bit 1) |  |  |
| --- | --- | --- |
| Bit |  |  |
| 1 |  |  |
| 0 |  | Satellite NG-RAN allowed in PLMN |
| 1 |  | Satellite NG-RAN not allowed in PLMN |
| Bit 2 to 8 of octet 3 are spare and shall be coded as zero. |  |  |

#### 9.11.3.110	Access technology utilization control

See clause 9.9.3.3A in 3GPP TS 24.301 [15].

#### 9.11.3.111	LP-WUSPS assistance information

The purpose of the LP-WUSPS assistance information IE is to transfer the required assistance information for LP-WUSPS.

The LP-WUSPS assistance information, information element is coded as shown in figure 9.11.3.111.1, figure 9.11.3.111.2, figure 9.11.3.111.3 and table 9.11.3.111.1.

The LP-WUSPS assistance information is a type 4 information element, with a minimum length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LP-WUSPS assistance information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of LP-WUSPS assistance information contents |  |  |  |  |  |  |  | octet 2 |
| LP-WUSPS assistance information type |  |  |  |  |  |  |  | octet 3* |

Figure 9.11.3.111.1: LP-WUSPS assistance information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of information |  |  | LP-WUSPS paging subgroup ID value |  |  |  |  | octet 3 |

Figure 9.11.3.111.2: LP-WUSPS assistance information type – type of information= "000"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of information |  |  | UE paging probability information value |  |  |  |  | octet 3 |

Figure 9.11.3.111.3: LP-WUSPS assistance information type – type of information= "001"

Table 9.11.3.111.1: LP-WUSPS assistance information information element

|  | Value part of the LP-WUSPS assistance information information element (octet 3) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | The value part of the LP-WUSPS assistance information information element consists of one LP-WUSPS assistance information type parameter. |  |  |  |  |  |  |
|  | LP-WUSPS assistance information type: |  |  |  |  |  |  |
|  | Type of information (octet 3, bits 6 to 8) |  |  |  |  |  |  |
|  | Bits |  |  |  |  |  |  |
|  | 8 | 7 | 6 |  |  |  |  |
|  | 0 | 0 | 0 | LP-WUSPS paging subgroup ID |  |  |  |
|  | 0 | 0 | 1 | UE paging probability information |  |  |  |
|  | All other values are reserved. |  |  |  |  |  |  |
|  | LP-WUSPS paging subgroup ID value (octet 3, bits 1 to 5) |  |  |  |  |  |  |
|  | This field contains one out of 31 (decimal) LP-WUSPS paging subgroup IDs that is assigned by the AMF for paging the UE. |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |
| 5 |  | 4 | 3 | 2 | 1 |  | LP-WUSPS paging subgroup ID value |
| 0 |  | 0 | 0 | 0 | 0 |  | LP-WUSPS paging subgroup ID 0 (decimal) |
|  |  |  | to |  |  |  |  |
| 1 |  | 1 | 1 | 1 | 0 |  | LP-WUSPS paging subgroup ID 30 (decimal) |
| 1 |  | 1 | 1 | 1 | 1 |  | reserved |
|  | UE paging probability information value (octet 3, bits 1 to 5) |  |  |  |  |  |  |
|  | This field contains the value of UE paging probability information provided by the UE to the AMF. It represents the probability of the UE receiving the paging. |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |
|  | 5 | 4 | 3 | 2 | 1 |  | UE paging probability information value |
|  | 0 | 0 | 0 | 0 | 0 |  | p00 (UE calculated paging probability is 0%) |
|  | 0 | 0 | 0 | 0 | 1 |  | p05 (UE calculated paging probability > 0% and <= 5%) |
|  | 0 | 0 | 0 | 1 | 0 |  | p10 (UE calculated paging probability > 5% and <= 10%) |
|  | 0 | 0 | 0 | 1 | 1 |  | p15 (UE calculated paging probability > 10% and <= 15%) |
|  | 0 | 0 | 1 | 0 | 0 |  | p20 (UE calculated paging probability > 15% and <= 20%) |
|  | 0 | 0 | 1 | 0 | 1 |  | p25 (UE calculated paging probability > 20% and <= 25%) |
|  | 0 | 0 | 1 | 1 | 0 |  | p30 (UE calculated paging probability > 25% and <= 30%) |
|  | 0 | 0 | 1 | 1 | 1 |  | p35 (UE calculated paging probability > 30% and <= 35%) |
|  | 0 | 1 | 0 | 0 | 0 |  | p40 (UE calculated paging probability > 35% and <= 40%) |
|  | 0 | 1 | 0 | 0 | 1 |  | p45 (UE calculated paging probability > 40% and <= 45%) |
|  | 0 | 1 | 0 | 1 | 0 |  | p50 (UE calculated paging probability > 45% and <= 50%) |
|  | 0 | 1 | 0 | 1 | 1 |  | p55 (UE calculated paging probability > 50% and <= 55%) |
|  | 0 | 1 | 1 | 0 | 0 |  | p60 (UE calculated paging probability > 55% and <= 60%) |
|  | 0 | 1 | 1 | 0 | 1 |  | p65 (UE calculated paging probability > 60% and <= 65%) |
|  | 0 | 1 | 1 | 1 | 0 |  | p70 (UE calculated paging probability > 65% and <= 70%) |
|  | 0 | 1 | 1 | 1 | 1 |  | p75 (UE calculated paging probability > 70% and <= 75%) |
|  | 1 | 0 | 0 | 0 | 0 |  | p80 (UE calculated paging probability > 75% and <= 80%) |
|  | 1 | 0 | 0 | 0 | 1 |  | p85 (UE calculated paging probability > 80% and <= 85%) |
|  | 1 | 0 | 0 | 1 | 0 |  | p90 (UE calculated paging probability > 85% and <= 90%) |
|  | 1 | 0 | 0 | 1 | 1 |  | p95 (UE calculated paging probability > 90% and <= 95%) |
|  | 1 | 0 | 1 | 0 | 0 |  | p100 (UE calculated paging probability > 95% and <= 100%) |
|  | All other values shall be interpreted as 10100 by this version of the protocol. |  |  |  |  |  |  |

#### 9.11.3.112	LP-WUS status

The purpose of the LP-WUS status information element is to indicate LP-WUS status.

The LP-WUS status information element is coded as shown in figure 9.11.3.112.1 and table 9.11.3.112.1.

The LP-WUS status is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LP-WUS status IEI |  |  |  | 0 Spare | 0 Spare | 0 Spare | LWS | octet 1 |

Figure 9.11.3.112.1: LP-WUS status indication

Table 9.11.3.112.1: LP-WUS status indication

| LP-WUS status indication (LWS) (octet 1, bit 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | LP-WUS enabled |
| 1 | LP-WUS disabled |
| Bits 2 to 4 are spare and shall be coded as zero. |  |

### 9.11.4	5GS session management (5GSM) information elements

#### 9.11.4.1	5GSM capability

The purpose of the 5GSM capability information element is to indicate UE capability related to the PDU session management.

The 5GSM capability information element is coded as shown in figure 9.11.4.1.1 and table 9.11.4.1.1.

The 5GSM capability is a type 4 information element with a minimum length of 3 octets and a maximum length of 15 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GSM capability IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GSM capability contents |  |  |  |  |  |  |  | octet 2 |
| TPMIC | ATSSS-ST |  |  |  | EPT-S1 | MH6-PDU | RQoS | octet 3 |
| MPQUIC-IP | MPQUIC-UDP | MPTCP | ATSSS-LL |  | RTPMMII | SDNAEPC | APMQF | octet 4* |
| 0 spare | 0 spare | 0 spare | 0 spare |  | 0 spare | E8PCPDEI | MPQUIC-E | octet 5* |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | octet 6* -15* |
| spare |  |  |  |  |  |  |  |  |

Figure 9.11.4.1.1: 5GSM capability information element

Table 9.11.4.1.1: 5GSM capability information element

| 5GSM capability value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RQoS (octet 3, bit 1) |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the 5GSM capability to support reflective QoS. |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  | Reflective QoS not supported |  |  |
| 1 |  |  |  |  |  |  |  | Reflective QoS supported |  |  |
| Multi-homed IPv6 PDU session (MH6-PDU) (octet 3, bit 2) |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the 5GSM capability for Multi-homed IPv6 PDU session. |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  | Multi-homed IPv6 PDU session not supported |  |  |
| 1 |  |  |  |  |  |  |  | Multi-homed IPv6 PDU session supported |  |  |
| Ethernet PDN type in S1 mode (EPT-S1) (octet 3, bit 3) |  |  |  |  |  |  |  |  |  |  |
| This bit indicates UE's 5GSM capability for Ethernet PDN type in S1 mode. |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  | Ethernet PDN type in S1 mode not supported |  |  |
| 1 |  |  |  |  |  |  |  | Ethernet PDN type in S1 mode supported |  |  |
| Supported ATSSS steering functionalities and steering modes (ATSSS-ST) (octet 3, bits 4 to 7) (NOTE 1) |  |  |  |  |  |  |  |  |  |  |
| These bits indicate the 5GSM capability of ATSSS steering functionalities and steering modes |  |  |  |  |  |  |  |  |  |  |
| 0 |  | 0 |  | 0 |  | 0 |  | ATSSS not supported |  |  |
| 0 |  | 0 |  | 0 |  | 1 |  | ATSSS Low-Layer functionality with any steering mode allowed for ATSSS-LL supported |  |  |
| 0 |  | 0 |  | 1 |  | 0 |  | MPTCP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported |  |  |
| 0 |  | 0 |  | 1 |  | 1 |  | MPTCP functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported |  |  |
| All other values are reserved. |  |  |  |  |  |  |  |  |  |  |
| Transfer of port management information containers (TPMIC) (octet 3, bit 8) |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the 5GSM capability to support transfer of port management information containers |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  | Transfer of port management information containers not supported |  |  |
| 1 |  |  |  |  |  |  |  | Transfer of port management information containers supported |  |  |
| Access performance measurements per QoS flow rule (APMQF) (octet 4, bit1) |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the 5GSM capability to support access performance measurements using the QoS flow of the non default QoS rule, that is used by the service data flow (SDF) traffic. |  |  |  |  |  |  |  |  |  |  |
| 0; Access performance measurements per QoS flow not supported.. |  |
| 1; Access performance measurements per QoS flow supported.. |  |
| Secondary DN authentication and authorization over EPC (SDNAEPC) (octet 4, bit 2). This bit indicates the 5GSM capability to support secondary DN authentication and authorization over EPC. 0; Secondary DN authentication and authorization over EPC not supported. 1; Secondary DN authentication and authorization over EPC supported. |  |
| (S)RTP multiplexed media identification information (RTPMMII) (octet 4, bit 3) |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  | (S)RTP multiplexed media identification information not supported. |  |  |  |
| 1 |  |  |  |  |  |  | (S)RTP multiplexed media identification information supported. |  |  |  |
| ATSSS-LL functionality support (ATSSS-LL) (octet 4 bits 4 and 5) |  |  |  |  |  |  |  |  |  |  |
| This capability indicates the support for ATSSS-LL functionality. Bit |  |  |  |  |  |  |  |  |  |  |
| 5 |  | 4 |  |  |  |  |  |  |  |  |
| 0 |  | 0 |  |  |  |  |  | ATSSS-LL functionality not supported (NOTE 2) |  |  |
| 0 |  | 1 |  |  |  |  |  | ATSSS-LL functionality with only active-standby steering mode supported |  |  |
| 1 |  | 0 |  |  |  |  |  | ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported |  |  |
| 1 |  | 1 |  |  |  |  |  | spare |  |  |
| Supporting MPTCP functionality with any steering mode (MPTCP) (octet 4, bit 6) |  |  |  |  |  |  |  |  |  |  |
| This bit indicates whether the MPTCP functionality with any steering mode is supported (NOTE 3). |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  | MPTCP functionality with any steering mode not supported (NOTE 2) |  |  |
| 1 |  |  |  |  |  |  |  | MPTCP functionality with any steering mode supported |  |  |
| Supporting MPQUIC-UDP functionality with any steering mode (MPQUIC-UDP) (octet 4, bit 7) |  |  |  |  |  |  |  |  |  |  |
| This bit indicates whether the MPQUIC-UDP functionality with any steering mode is supported (NOTE 3). |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  | MPQUIC-UDP functionality with any steering mode not supported (NOTE 2) |  |  |
| 1 |  |  |  |  |  |  |  | MPQUIC-UDP functionality with any steering mode supported |  |  |
| Supporting MPQUIC-IP functionality with any steering mode (MPQUIC-IP) (octet 4, bit 8) |  |  |  |  |  |  |  |  |  |  |
| This bit indicates whether the MPQUIC-IP functionality with any steering mode is supported. |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  | MPQUIC-IP functionality with any steering mode not supported (NOTE 2) |  |  |
| 1 |  |  |  |  |  |  |  | MPQUIC-IP functionality with any steering mode supported |  |  |
| Supporting MPQUIC-E functionality with any steering mode (MPQUIC-E) (octet 5, bit 1) |  |  |  |  |  |  |  |  |  |  |
| This bit indicates whether the MPQUIC-E functionality with any steering mode is supported (NOTE 3). |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  | MPQUIC-E functionality with any steering mode not supported (NOTE 2) |  |  |
| 1 |  |  |  |  |  |  |  | MPQUIC-E functionality with any steering mode supported |  |  |
| Supporting extended 802.1Q C-TAG/S-TAG PCP/DEI (E8PCPDEI) (octet 5, bit 2) |  |  |  |  |  |  |  |  |  |  |
| This bit indicates whether to support the extended 802.1Q C-TAG/S-TAG PCP/DEI packet filter component type. |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  | Extended 802.1Q C-TAG/S-TAG PCP/DEI packet filter component type not supported |  |  |
| 1 |  |  |  |  |  |  |  | Extended 802.1Q C-TAG/S-TAG PCP/DEI packet filter component type supported |  |  |
| All other bits in octet 5 and bits in octets 6 to 15 are spare and shall be coded as zero, if the respective octet is included in the information element. |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UE shall populate these bits in the same way as a UE from a previous release. NOTE 2: For MA PDU session, at least one of the following fields: ATSSS-ST, ATSSS-LL, MPTCP, MPQUIC-UDP, MPQUIC-IP and MPQUIC-E shall be set to a non-zero value. NOTE 3: If ATSSS-LL is set to 00 then this bit shall be set to 0. |  |  |  |  |  |  |  |  |  |  |

#### 9.11.4.2	5GSM cause

The purpose of the 5GSM cause information element is to indicate the reason why a 5GSM request is rejected.

The 5GSM cause information element is coded as shown in figure 9.11.4.2.1 and table 9.11.4.2.1.

The 5GSM cause is a type 3 information element with a length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GSM cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 9.11.4.2.1: 5GSM cause information element

Table 9.11.4.2.1: 5GSM cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  | Operator determined barring |
| 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |  | Insufficient resources |
| 0 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |  | Missing or unknown DNN |
| 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 |  | Unknown PDU session type |
| 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |  | User authentication or authorization failed |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |  | Request rejected, unspecified |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |  | Service option not supported |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |  | Requested service option not subscribed |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |  | PTI already in use |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |  | Regular deactivation |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |  | 5GS QoS not accepted |
| 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 |  | Network failure |
| 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 |  | Reactivation requested |
| 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 |  | Semantic error in the TFT operation |
| 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |  | Syntactical error in the TFT operation |
| 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |  | Invalid PDU session identity |
| 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |  | Semantic errors in packet filter(s) |
| 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 |  | Syntactical error in packet filter(s) |
| 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 |  | Out of LADN service area |
| 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 |  | PTI mismatch |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |  | PDU session type IPv4 only allowed |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |  | PDU session type IPv6 only allowed |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 |  | PDU session does not exist |
| 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |  | PDU session type IPv4v6 only allowed |
| 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 |  | PDU session type Unstructured only allowed |
| 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 |  | Unsupported 5QI value |
| 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |  | PDU session type Ethernet only allowed |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | Insufficient resources for specific slice and DNN |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | Not supported SSC mode |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | Insufficient resources for specific slice |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |  | Missing or unknown DNN in a slice |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  | Invalid PTI value |
| 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |  | Maximum data rate per UE for user-plane integrity protection is too low |
| 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |  | Semantic error in the QoS operation |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |  | Syntactical error in the QoS operation |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  | Invalid mapped EPS bearer identity |
| 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |  | UAS services not allowed |
| 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |  | QoS differentiation for non-3GPP device identifier(s) not available |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |  | Semantically incorrect message |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |  | Invalid mandatory information |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |  | Message type non-existent or not implemented |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |  | Message type not compatible with the protocol state |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |  | Information element non-existent or not implemented |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |  | Conditional IE error |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |  | Message not compatible with the protocol state |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Protocol error, unspecified |
| Any other value received by the UE shall be treated as 0001 1111, " Request rejected, unspecified ". Any other value received by the network shall be treated as 0110 1111, "protocol error, unspecified". |  |  |  |  |  |  |  |  |  |

#### 9.11.4.3	Always-on PDU session indication

The purpose of the Always-on PDU session indication information element is to indicate whether a PDU session is established as an always-on PDU session.

The Always-on PDU session indication information element is coded as shown in figure 9.11.4.3.1 and table 9.11.4.3.1.

The Always-on PDU session indication is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Always-on PDU session indication IEI |  |  |  | 0 Spare | 0 Spare | 0 Spare | APSI | octet 1 |

Figure 9.11.4.3.1: Always-on PDU session indication

Table 9.11.4.3.1: Always-on PDU session indication

| Always-on PDU session indication (APSI) (octet 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | Always-on PDU session not allowed |
| 1 | Always-on PDU session required |
| Bits 2, 3 and 4 are spare and shall be coded as zero, |  |

#### 9.11.4.4	Always-on PDU session requested

The purpose of the Always-on PDU session requested information element is to indicate whether a PDU session is requested to be established as an always-on PDU session.

The Always-on PDU session requested information element is coded as shown in figure 9.11.4.4.1 and table 9.11.4.4.1.

The Always-on PDU session requested is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Always-on PDU session requested IEI |  |  |  | 0 Spare | 0 Spare | 0 Spare | APSR | octet 1 |

Figure 9.11.4.4.1: Always-on PDU session requested

Table 9.11.4.4.1: Always-on PDU session requested

| Always-on PDU session requested (APSR) (octet 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | Always-on PDU session not requested |
| 1 | Always-on PDU session requested |
| Bits 2, 3 and 4 are spare and shall be coded as zero, |  |

#### 9.11.4.5	Allowed SSC mode

The purpose of the Allowed SSC mode information element is to indicate the SSC modes allowed to be used by the UE for the PDU session.

The Allowed SSC mode information element is coded as shown in figure 9.11.4.5.1 and table 9.11.4.5.1.

The Allowed SSC mode is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allowed SSC mode IEI |  |  |  | 0 Spare | SSC3 | SSC2 | SSC1 | octet 1 |

Figure 9.11.4.5.1: Allowed SSC mode information element

Table 9.11.4.5.1: Allowed SSC mode information element

| SSC1 (octet 1, bit 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | SSC mode 1 not allowed |
| 1 | SSC mode 1 allowed |
| SSC2 (octet 1, bit 2) |  |
| Bit |  |
| 2 |  |
| 0 | SSC mode 2 not allowed |
| 1 | SSC mode 2 allowed |
| SSC3 (octet 1, bit 3) |  |
| Bit |  |
| 3 |  |
| 0 | SSC mode 3 not allowed |
| 1 | SSC mode 3 allowed |
| Bit 4 is spare and shall be encoded as zero. |  |

#### 9.11.4.6	Extended protocol configuration options

See clause 10.5.6.3A in 3GPP TS 24.008 [12].

#### 9.11.4.7	Integrity protection maximum data rate

The purpose of the integrity protection maximum data rate information element is for the UE to indicate to the network the maximum data rate per UE for user-plane integrity protection for uplink and the maximum data rate per UE for user-plane integrity protection for downlink that are supported by the UE.

The integrity protection maximum data rate is coded as shown in figure 9.11.4.7.1 and table 9.11.4.7.2.

The integrity protection maximum data rate is a type 3 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integrity protection maximum data rate IEI |  |  |  |  |  |  |  | octet 1 |
| Maximum data rate per UE for user-plane integrity protection for uplink |  |  |  |  |  |  |  | octet 2 |
| Maximum data rate per UE for user-plane integrity protection for downlink |  |  |  |  |  |  |  | octet 3 |

Figure 9.11.4.7.1: Integrity protection maximum data rate information element

Table 9.11.4.7.2: Integrity protection maximum data rate information element

| Maximum data rate per UE for user-plane integrity protection for uplink (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 64 kbps (NOTE 3) |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | NULL (NOTE 1) |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | Full data rate (NOTE 2) |
| All other values are spare and shall not be used by a UE compliant to the present version of this specification. If received they shall be interpreted as "64 kbps". |  |  |  |  |  |  |  |  |  |
| Maximum data rate per UE for user-plane integrity protection for downlink (octet 3) |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 64 kbps (NOTE 3) |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | NULL (NOTE 1) |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | Full data rate (NOTE 2) |
| All other values are spare and shall not be used by a UE compliant to the present version of this specification. If received they shall be interpreted as "64 kbps". |  |  |  |  |  |  |  |  |  |
| NOTE 1: This value shall be used when N3 data transfer is not supported by the UE or when the UE does not support standalone NR connected to 5GCN. |  |  |  |  |  |  |  |  |  |
| NOTE 2: If the UE supports N3 data transfer and supports standalone NR connected to 5GCN (this includes UEs supporting NR-NR dual connectivity, NR-E-UTRA dual connectivity with MN terminated bearers or both of them as described in 3GPP TS 37.340 [51]), then the UE shall use this value. |  |  |  |  |  |  |  |  |  |
| NOTE 3: The network can receive this value from a UE compliant to an earlier version of this specification. |  |  |  |  |  |  |  |  |  |

#### 9.11.4.8	Mapped EPS bearer contexts

The purpose of the mapped EPS bearer contexts information element is to indicate a set of EPS bearer contexts for a PDU session, as described in clause 6.1.4.1.

The mapped EPS bearer contexts information element is a type 6 information element with a minimum length of 7 octet and a maximum length of 65538 octets.

The mapped EPS bearer contexts information element is coded as shown in figure 9.11.4.8.1, figure 9.11.4.8.2, figure 9.11.4.8.3 and table 9.11.4.8.1.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mapped EPS bearer contexts IEI |  |  |  |  |  |  |  | octet 1 |
|  | Length of Mapped EPS bearer contexts contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  |  | octet 3 |
|  | Mapped EPS bearer context 1 |  |  |  |  |  |  |  | octet 4 octet u |
|  | Mapped EPS bearer context 2 |  |  |  |  |  |  |  | octet u+1 octet v |
|  | … |  |  |  |  |  |  |  | octet v+1 octet w |
|  | Mapped EPS bearer context n |  |  |  |  |  |  |  | octet w+1 octet x |

Figure 9.11.4.8.1: Mapped EPS bearer contexts

|  | 8 | 7 | 6 | 5 |  | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EPS bearer identity |  |  |  |  |  |  |  |  | octet 4 |
|  | Length of Mapped EPS bearer context |  |  |  |  |  |  |  |  | octet 5 octet 6 |
|  | Operation code |  | 0 Spare |  | E bit | Number of EPS parameters |  |  |  | octet 7 |
|  | EPS parameters list |  |  |  |  |  |  |  |  | octet 8* octet u* |

Figure 9.11.4.8.2: Mapped EPS bearer context

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EPS parameter identifier 1 |  |  |  |  |  |  |  | octet 8 |
|  | Length of EPS parameter contents 1 |  |  |  |  |  |  |  | octet 9 |
|  | EPS parameter contents 1 |  |  |  |  |  |  |  | octet 10 octet h |
|  | EPS parameter identifier 2 |  |  |  |  |  |  |  | octet h+1 |
|  | Length of EPS parameter contents 2 |  |  |  |  |  |  |  | octet h+2 |
|  | EPS parameter contents 2 |  |  |  |  |  |  |  | octet h+3 octet i |
|  | … |  |  |  |  |  |  |  | octet i+1 octet j |
|  | EPS parameter identifier N |  |  |  |  |  |  |  | octet j+1 |
|  | Length of EPS parameter contents N |  |  |  |  |  |  |  | octet j+2 |
|  | EPS parameter contents N |  |  |  |  |  |  |  | octet j+3 octet u |

Figure 9.11.4.8.3: EPS parameters list

Table 9.11.4.8.1: Mapped EPS bearer contexts information element

EPS bearer identity (octet 4) Bits 5 to 8 contain the EPS bearer identity, and are coded as specified in clause 9.3.2 of 3GPP TS 24.301 [15]. Bits 1 to 4 are spare and shall be coded as zero. Operation code (bits 8 to 7 of octet 7) Bits 8 7 0 0 Reserved 0 1 Create new EPS bearer 1 0 Delete existing EPS bearer 1 1 Modify existing EPS bearer Bit 6 of octet 7 is spare and shall be coded as zero. E bit (bit 5 of octet 7) For the "create new EPS bearer" operation, the E bit is encoded as follows: Bit 5 0 parameters list is not included (NOTE) 1 parameters list is included For the "modify existing EPS bearer" operation, the E bit is encoded as follows: Bit 5 0 extension of previously provided parameters list 1 replacement of all previously provided parameters list If the E bit is set to "parameters list is included", the number of EPS parameters field has non-zero value. If the E bit is set to "extension of previously provided parameters list" or "replacement of previously provided parameters list", the number of parameters field has non-zero value. If the E bit is set to "extension of previously provided parameters" and one of the parameters in the new parameters list already exists in the previously provided parameters, the parameter shall be set to the new value. If the E bit is set to "replacement of all previously provided parameters list" and a parameter in the previously provided parameters is not provided in the new parameters list, the parameter shall be deleted. For the "create new EPS bearer" operation and "delete existing EPS bearer" operation, bit 5 of octet 7 is ignored. Number of EPS parameters (bits 4 to 1 of octet 7) The number of EPS parameters contains the binary coding for the number of EPS parameters in the EPS parameters list field. The number of EPS parameters field is encoded in bits 4 through 1 of octet 7 where bit 4 is the most significant and bit 1 is the least significant bit. EPS parameters list (octets 8 to u) The EPS parameters list contains a variable number of EPS parameters. Each EPS parameter included in the EPS parameters list is of variable length and consists of: - an EPS parameter identifier (1 octet); - the length of the EPS parameter contents (1 octet); and - the EPS parameter contents itself (variable amount of octets). The EPS parameter identifier field is used to identify each EPS parameter included in the EPS parameters list and it contains the hexadecimal coding of the EPS parameter identifier. Bit 8 of the EPS parameter identifier field contains the most significant bit and bit 1 contains the least significant bit. In this version of the protocol, the following EPS parameter identifiers are specified: - 01H (Mapped EPS QoS parameters); - 02H (Mapped extended EPS QoS parameters); and - 03H (Traffic flow template). - 04H (APN-AMBR). - 05H (extended APN-AMBR). If the EPS parameters list contains an EPS parameter identifier that is not supported by the receiving entity the corresponding EPS parameter shall be discarded. The length of EPS parameter contents field contains the binary coded representation of the length of the EPS parameter contents field. The first bit in transmission order is the most significant bit. When the parameter identifier indicates mapped EPS QoS parameters, the length and parameter contents field are coded as specified in clause 9.9.4.3 of 3GPP TS 24.301 [15]. When the parameter identifier indicates mapped extended EPS QoS parameters, the length and parameter contents field are coded as specified in clause 9.9.4.30 of 3GPP TS 24.301 [15]. When the parameter identifier indicates traffic flow template, the length and parameter contents field are coded from octet 2 as shown figure 10.5.144 and table 10.5.162 of 3GPP TS 24.008 [12]. When the parameter identifier indicates APN-AMBR, the length and parameter contents field are coded as specified in clause 9.9.4.2 of 3GPP TS 24.301 [15]. When the parameter identifier indicates Extended APN-AMBR, the length and parameter contents field are coded as specified in clause 9.9.4.29 of 3GPP TS 24.301 [15].

NOTE: This value shall not be used In this version of the specification.

#### 9.11.4.9	Maximum number of supported packet filters

The purpose of the Maximum number of supported packet filters information element is for the UE to indicate to the network the maximum number of packet filters, associated with signaled QoS rules, that can be supported by the UE for the PDU session that is being established, when the PDU session type "IPv4", "IPv6", "IPv4v6" or "Ethernet".

The Maximum number of supported packet filters is coded as shown in figure 9.11.4.9.1 and table 9.11.4.9.1.

The Maximum number of supported packet filters is a type 3 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum number of supported packet filters IEI |  |  |  |  |  |  |  | octet 1 |
| Maximum number of supported packet filters |  |  |  |  |  |  |  | octet 2 |
| Maximum number of supported packet filters (continued) |  |  | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet 3 |

Figure 9.11.4.9.1: Maximum number of supported packet filters information element

Table 9.11.4.9.1: Maximum number of supported packet filters information element

Maximum number of supported packet filters (octet 2 to 3)

In the Maximum number of supported packet filters field bit 8 of the first octet is the most significant bit and bit 6 of second octet is the least significant bit. Bit 5 to bit 1 of the second octet are spare bits and shall be coded as zero. The number of supported packet filters shall be in the range of 17 to 1024.

#### 9.11.4.10	PDU address

The purpose of the PDU address information element is to assign to the UE:

-	an IPv4 address associated with a PDU session;

-	an interface identifier for the IPv6 link local address associated with the PDU session; or

-	an IPv4 address and an interface identifier for the IPv6 link local address, associated with the PDU session.

This purpose of the PDU address information element is also to enable the W-AGF acting on behalf of the FN-RG to provide an interface identifier for the IPv6 link local address associated with the PDU session suggested to be allocated to the FN-RG, and to enable the SMF to provide SMF's IPv6 link local address to the W-AGF acting on behalf of the FN-RG.

The PDU address information element is coded as shown in figure 9.11.4.10.1 and table 9.11.4.10.1.

The PDU address is a type 4 information element with minimum length of 7 octets and a maximum length of 31 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU address IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PDU address contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | SI6LLA | PDU session type value |  |  | octet 3 |
| PDU address information |  |  |  |  |  |  |  | octet 4 octet n |
| SMF's IPv6 link local address |  |  |  |  |  |  |  | octet (n+1)* octet (n+16)* |

Figure 9.11.4.10.1: PDU address information element

Table 9.11.4.10.1: PDU address information element

| PDU session type value (octet 3, bits 1 to 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 1 |  | IPv4 |
| 0 | 1 | 0 |  | IPv6 |
| 0 | 1 | 1 |  | IPv4v6 |
| All other values are reserved. |  |  |  |  |
| SI6LLA (SMF's IPv6 link local address) bit (octet 3, bit 4) (see NOTE) |  |  |  |  |
| Bit |  |  |  |  |
| 4 |  |  |  |  |
| 0 |  |  |  | SMF's IPv6 link local address field is absent |
| 1 |  |  |  | SMF's IPv6 link local address field is present |
| Bits 5 to 8 of octet 3 are spare and shall be coded as zero. |  |  |  |  |
| PDU address information (octet 4 to n) |  |  |  |  |
| If the PDU session type value indicates IPv4, the PDU address information in octet 4 to octet 7 contains an IPv4 address. |  |  |  |  |
| If the PDU session type value indicates IPv6, the PDU address information in octet 4 to octet 11 contains an interface identifier for the IPv6 link local address. |  |  |  |  |
| If the PDU session type value indicates IPv4v6, the PDU address information in octet 4 to octet 11 contains an interface identifier for the IPv6 link local address and in octet 12 to octet 15 contains an IPv4 address. |  |  |  |  |
| SMF's IPv6 link local address (octet n+1 to n+16) |  |  |  |  |
| SMF's IPv6 link local address field contains SMF's IPv6 link local address. |  |  |  |  |
| NOTE: In the UE to network direction, the SI6LLA bit shall be set to "SMF's IPv6 link local address field is absent". |  |  |  |  |

#### 9.11.4.11	PDU session type

The purpose of the PDU session type information element is to indicate type of the PDU session.

The PDU session type information element is coded as shown in figure 9.11.4.11.1 and table 9.11.4.11.1.

The PDU session type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU session type IEI |  |  |  | 0 Spare | PDU session type value |  |  | octet 1 |

Figure 9.11.4.11.1: PDU session type information element

Table 9.11.4.11.1: PDU session type information element

| PDU session type value (octet 1, bit 1 to bit 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 1 |  | IPv4 |
| 0 | 1 | 0 |  | IPv6 |
| 0 | 1 | 1 |  | IPv4v6 |
| 1 | 0 | 0 |  | Unstructured |
| 1 | 0 | 1 |  | Ethernet |
| 1 | 1 | 1 |  | reserved |
| All other values are unused and shall be interpreted as "IPv4v6", if received by the UE or the network. |  |  |  |  |

#### 9.11.4.12	QoS flow descriptions

The purpose of the QoS flow descriptions information element is to indicate a set of QoS flow descriptions to be used by the UE, where each QoS flow description is a set of parameters as described in clause 6.2.5.1.1.4.

The QoS flow descriptions information element is a type 6 information element with a minimum length of 6 octets. The maximum length for the information element is 65538 octets.

The QoS flow descriptions information element is coded as shown in figure 9.11.4.12.1, figure 9.11.4.12.2, figure 9.11.4.12.3, figure 9.11.4.12.4, and table 9.11.4.12.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QoS flow descriptions IEI |  |  |  |  |  |  |  | octet 1 |
| Length of QoS flow descriptions contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| QoS flow description 1 |  |  |  |  |  |  |  | octet 4 octet u |
| QoS flow description 2 |  |  |  |  |  |  |  | octet u+1 octet v |
| ... |  |  |  |  |  |  |  | octet v+1 octet w |
| QoS flow description n |  |  |  |  |  |  |  | octet w+1 octet x |

Figure 9.11.4.12.1: QoS flow descriptions information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare | 0 Spare | QFI |  |  |  |  |  | octet 4 |
| Operation code |  |  | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet 5 |
| 0 Spare | E | Number of parameters |  |  |  |  |  | octet 6 |
| Parameters list |  |  |  |  |  |  |  | octet 7* octet u* |

Figure 9.11.4.12.2: QoS flow description

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter 1 |  |  |  |  |  |  |  | octet 7 octet m |
| Parameter 2 |  |  |  |  |  |  |  | octet m+1 octet n |
| ... |  |  |  |  |  |  |  | octet n+1 octet o |
| Parameter n |  |  |  |  |  |  |  | octet o+1 octet u |

Figure 9.11.4.12.3: Parameters list

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter identifier |  |  |  |  |  |  |  | octet 7 |
| Length of parameter contents |  |  |  |  |  |  |  | octet 8 |
| Parameter contents |  |  |  |  |  |  |  | octet 9 octet m |

Figure 9.11.4.12.4: Parameter

Table 9.11.4.12.1: QoS flow descriptions information element

QoS flow identifier (QFI) (bits 6 to 1 of octet 4) QFI field contains the QoS flow identifier. Bits 6 5 4 3 2 1 0 0 0 0 0 0 no QoS flow identifier assigned 0 0 0 0 0 1 QFI 1 to 1 1 1 1 1 1 QFI 63 The network shall not set the QFI value to 0.

Operation code (bits 8 to 6 of octet 5) Bits 8 7 6 0 0 1 Create new QoS flow description 0 1 0 Delete existing QoS flow description 0 1 1 Modify existing QoS flow description All other values are reserved.

E bit (bit 7 of octet 6) For the "create new QoS flow description" operation, the E bit is encoded as follows: Bit 7 0 reserved 1 parameters list is included For the "Delete existing QoS flow description" operation, the E bit is encoded as follows: Bit 7 0 parameters list is not included 1 reserved For the "modify existing QoS flow description" operation, the E bit is encoded as follows: Bit 7 0 extension of previously provided parameters 1 replacement of all previously provided parameters If the E bit is set to "parameters list is not included", the number of parameters field has zero value. If the E bit is set to "parameters list is included", the number of parameters field has non-zero value. If the E bit is set to "extension of previously provided parameters" or "replacement of all previously provided parameters", the number of parameters field has non-zero value. If the E bit is set to "extension of previously provided parameters" and one of the parameters in the new parameters list already exists in the previously provided parameters, the parameter shall be set to the new value. If the E bit is set to "replacement of all previously provided parameters list" and a parameter in the previously provided parameters is not provided in the new parameters list, the parameter shall be deleted. Number of parameters (bits 6 to 1 of octet 6) The number of parameters field contains the binary coding for the number of parameters in the parameters list field. The number of parameters field is encoded in bits 6 through 1 of octet 6 where bit 6 is the most significant and bit 1 is the least significant bit. Parameters list (octets 7 to u) The parameters list contains a variable number of parameters. Each parameter included in the parameters list is of variable length and consists of: - a parameter identifier (1 octet); - the length of the parameter contents (1 octet); and - the parameter contents itself (variable amount of octets). The parameter identifier field is used to identify each parameter included in the parameters list and it contains the hexadecimal coding of the parameter identifier. Bit 8 of the parameter identifier field contains the most significant bit and bit 1 contains the least significant bit. In this version of the protocol, the following parameter identifiers are specified: - 01H (5QI); - 02H (GFBR uplink); - 03H (GFBR downlink); - 04H (MFBR uplink); - 05H (MFBR downlink); - 06H (Averaging window); and - 07H (EPS bearer identity). If the parameters list contains a parameter identifier that is not supported by the receiving entity the corresponding parameter shall be discarded. The length of parameter contents field contains the binary coded representation of the length of the parameter contents field. The first bit in transmission order is the most significant bit. When the parameter identifier indicates 5QI, the parameter contents field contains the binary representation of 5G QoS identifier (5QI) that is one octet in length. 5QI: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 5QI 1 0 0 0 0 0 0 1 0 5QI 2 0 0 0 0 0 0 1 1 5QI 3 0 0 0 0 0 1 0 0 5QI 4 0 0 0 0 0 1 0 1 5QI 5 0 0 0 0 0 1 1 0 5QI 6 0 0 0 0 0 1 1 1 5QI 7 0 0 0 0 1 0 0 0 5QI 8 0 0 0 0 1 0 0 1 5QI 9 0 0 0 0 1 0 1 0 5QI 10 0 0 0 0 1 0 1 1 to Spare 0 1 0 0 0 0 0 0 0 1 0 0 0 0 0 1 5QI 65 0 1 0 0 0 0 1 0 5QI 66 0 1 0 0 0 0 1 1 5QI 67 0 1 0 0 0 1 0 0 Spare 0 1 0 0 0 1 0 1 5QI 69 0 1 0 0 0 1 1 0 5QI 70 0 1 0 0 0 1 1 1 5QI 71 0 1 0 0 1 0 0 0 5QI 72 0 1 0 0 1 0 0 1 5QI 73 0 1 0 0 1 0 1 0 5QI 74 0 1 0 0 1 0 1 1 5QI 75 0 1 0 0 1 1 0 0 5QI 76 0 1 0 0 1 1 0 1 to Spare 0 1 0 0 1 1 1 0 0 1 0 0 1 1 1 1 5QI 79 0 1 0 1 0 0 0 0 5QI 80 0 1 0 1 0 0 0 1 Spare 0 1 0 1 0 0 1 0 5QI 82 0 1 0 1 0 0 1 1 5QI 83 0 1 0 1 0 1 0 0 5QI 84 0 1 0 1 0 1 0 1 5QI 85 0 1 0 1 0 1 1 0 5QI 86 0 1 0 1 0 1 1 1 5QI 87 0 1 0 1 1 0 0 0 5QI 88 0 1 0 1 1 0 0 1 5QI 89 0 1 0 1 1 0 1 0 5QI 90 0 1 0 1 1 0 1 1 to Spare 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 to Operator-specific 5QIs 1 1 1 1 1 1 1 0 1 1 1 1 1 1 1 1 Reserved The network shall consider all other values not explicitly defined in this version of the protocol as unsupported. If the UE receives a 5QI value (excluding the reserved 5QI values) that it does not understand, the UE shall choose a 5QI value from the set of 5QI values defined in this version of the protocol (see 3GPP TS 23.501 [8]) and associated with: - GBR QoS flows, if the QoS flow includes a GFBR uplink parameter, a GFBR downlink parameter, a MFBR uplink parameter and a MFBR downlink parameter; and - non-GBR QoS flows, if the QoS flow does not include any one of a GFBR uplink parameter, a GFBR downlink parameter, a MFBR uplink parameter or a MFBR downlink parameter. The UE shall use this chosen 5QI value for internal operations only. The UE shall use the received 5QI value in subsequent NAS signalling procedures. When the parameter identifier indicates "GFBR uplink", the parameter contents field contains one octet indicating the unit of the guaranteed flow bit rate for uplink followed by two octets containing the value of the guaranteed flow bit rate for uplink. Unit of the guaranteed flow bit rate for uplink (octet 1) Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 value is not used (see NOTE 2) 0 0 0 0 0 0 0 1 value is incremented in multiples of 1 Kbps 0 0 0 0 0 0 1 0 value is incremented in multiples of 4 Kbps 0 0 0 0 0 0 1 1 value is incremented in multiples of 16 Kbps 0 0 0 0 0 1 0 0 value is incremented in multiples of 64 Kbps 0 0 0 0 0 1 0 1 value is incremented in multiples of 256 Kbps 0 0 0 0 0 1 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 1 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 1 0 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 1 0 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 1 0 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 1 0 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 1 1 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 1 1 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 1 1 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 1 1 1 1 value is incremented in multiples of 256 Gbps 0 0 0 1 0 0 0 0 value is incremented in multiples of 1 Tbps 0 0 0 1 0 0 0 1 value is incremented in multiples of 4 Tbps 0 0 0 1 0 0 1 0 value is incremented in multiples of 16 Tbps 0 0 0 1 0 0 1 1 value is incremented in multiples of 64 Tbps 0 0 0 1 0 1 0 0 value is incremented in multiples of 256 Tbps 0 0 0 1 0 1 0 1 value is incremented in multiples of 1 Pbps 0 0 0 1 0 1 1 0 value is incremented in multiples of 4 Pbps 0 0 0 1 0 1 1 1 value is incremented in multiples of 16 Pbps 0 0 0 1 1 0 0 0 value is incremented in multiples of 64 Pbps 0 0 0 1 1 0 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Value of the guaranteed flow bit rate for uplink (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the guaranteed flow bit rate for uplink in units defined by the unit of the guaranteed flow bit rate for uplink. When the UE indicates subscribed GFBR for uplink, the "GFBR uplink" parameter is not included in the "Parameters list". When the parameter identifier indicates "GFBR downlink", the parameter contents field contains one octet indicating the unit of the guaranteed flow bit rate for downlink followed by two octets containing the value of the guaranteed flow bit rate for downlink. Unit of the guaranteed flow bit rate for downlink (octet 1) The coding is identical to that of the unit of the guaranteed flow bit rate for uplink. Value of the guaranteed flow bit rate for downlink (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the guaranteed flow bit rate for downlink in units defined by the unit of the guaranteed flow bit rate for downlink. When the UE indicates subscribed GFBR for downlink, the "GFBR downlink" parameter is not included in the "Parameters list". When the parameter identifier indicates "MFBR uplink", the parameter contents field contains the one octet indicating the unit of the maximum flow bit rate for uplink followed by two octets containing the value of maximum flow bit rate for uplink. Unit of the maximum flow bit rate for uplink (octet 1) The coding is identical to that of the unit of the guaranteed flow bit rate for uplink. Value of the maximum flow bit rate for uplink (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the maximum flow bit rate for uplink in units defined by the unit of the maximum flow bit rate for uplink. When the UE indicates subscribed MFBR for uplink, the "MFBR uplink" parameter is not included in the "Parameters list". When the parameter identifier indicates "MFBR downlink", the parameter contents field contains one octet indicating the unit of the maximum flow bit rate for downlink followed by two octets containing the value of the maximum flow bit rate for downlink. Unit of the maximum flow bit rate for downlink (octet 1) The coding is identical to that of the unit of the guaranteed flow bit rate for uplink. Value of the maximum flow bit rate for downlink (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the maximum flow bit rate for downlink in units defined by the unit of the maximum flow bit rate for downlink. When the UE indicates subscribed MFBR for downlink, the "MFBR downlink" parameter is not included in the "Parameters list". In this version of the protocol, for messages specified in the present document, the sending entity shall not request 0 kbps for both the maximum flow bit rate for downlink and the maximum flow bit rate for uplink at the same time. Any entity receiving a request for 0 kbps in both the maximum flow bit rate for downlink and the maximum flow bit rate for uplink shall consider that as a syntactical error (see clause 7). When the parameter identifier indicates "averaging window", the parameter contents field contains the binary representation of the averaging window for both uplink and downlink in milliseconds and the parameter contents field is two octets in length. When the parameter identifier indicates EPS bearer identity, the length of EPS bearer identity is one octet, bits 5 to 8 of the parameter contents contain the EPS bearer identity as specified in clause 9.3.2 of 3GPP TS 24.301 [15] and bits 1 to 4 of the parameter contents are spare and shall be coded as zero. The UE shall not include the EPS bearer identity parameter in any mobile originated 5GSM messages (see NOTE 1).

NOTE 1: The total number of EPS bearer identities included in all QoS flow descriptions of a UE cannot exceed fifteen. NOTE 2: In this release of the specifications if received it shall be interpreted as value is incremented in multiples of 1 Kbps. In earlier releases of specifications, the interpretation of this value is up to implementation.

#### 9.11.4.13	QoS rules

The purpose of the QoS rules information element is to indicate a set of QoS rules to be used by the UE, where each QoS rule is a set of parameters as described in clause 6.2.5.1.1.2:

a)	for classification and marking of uplink user traffic; and

b)	for identification of a QoS flow which the network is to use for a particular downlink user traffic.

NOTE:	The UE needs to be aware of a QoS flow which the network is to use for a particular downlink user traffic e.g. to determine whether a resource is available for downlink media of a media stream of an SDP media description provided by the UE in an IMS session.

The QoS rules may contain a set of packet filters consisting of zero or more packet filters for UL direction, zero or more packet filters for DL direction, zero or more packet filters for both UL and DL directions or any combinations of these. The set of packet filters determine the traffic mapping to QoS flows.

The QoS rules information element is a type 6 information element with a minimum length of 7 octets. The maximum length for the information element is 65538 octets.

The QoS rules information element is coded as shown in figure 9.11.4.13.1, figure 9.11.4.13.2, figure 9.11.4.13.3, figure 9.11.4.13.4 and table 9.11.4.13.1.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QoS rules IEI |  |  |  |  |  |  |  | octet 1 |
|  | Length of QoS rules IE |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  |  | octet 3 |
|  | QoS rule 1 |  |  |  |  |  |  |  | octet 4 octet u |
|  | QoS rule 2 |  |  |  |  |  |  |  | octet u+1 octet v |
|  | … |  |  |  |  |  |  |  | octet v+1 octet w |
|  | QoS rule n |  |  |  |  |  |  |  | octet w+1 octet x |

Figure 9.11.4.13.1: QoS rules information element

|  | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QoS rule identifier |  |  |  |  |  |  |  |  | octet 4 |
|  | Length of QoS rule |  |  |  |  |  |  |  |  | octet 5 |
|  |  |  |  |  |  |  |  |  |  | octet 6 |
|  | Rule operation code |  |  | DQR bit |  | Number of packet filters |  |  |  | octet 7 |
|  | Packet filter list |  |  |  |  |  |  |  |  | octet 8* octet m* |
|  | QoS rule precedence |  |  |  |  |  |  |  |  | octet m+1* |
|  | 0 Spare | Segregation | QoS flow identifier (QFI) |  |  |  |  |  |  | octet m+2* |

Figure 9.11.4.13.2: QoS rule (u=m+2)

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | Packet filter identifier 1 |  |  |  | octet 8 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | 0 | 0 | 0 | 0 | Packet filter identifier 2 |  |  |  | octet 9 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | … |  |  |  |  |  |  |  |  |
|  | 0 | 0 | 0 | 0 | Packet filter identifier N |  |  |  | octet N+7 |
|  | Spare |  |  |  |  |  |  |  |  |

Figure 9.11.4.13.3: Packet filter list when the rule operation is "modify existing QoS rule and delete packet filters" (m=N+7)

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | Packet filter direction 1 |  | Packet filter identifier 1 |  |  |  | octet 8 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | Length of packet filter contents 1 |  |  |  |  |  |  |  | octet 9 |
|  | Packet filter contents 1 |  |  |  |  |  |  |  | octet 10 octet m |
|  | 0 | 0 | Packet filter direction 2 |  | Packet filter identifier 2 |  |  |  | octet k+1 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | Length of packet filter contents 2 |  |  |  |  |  |  |  | octet k+2 |
|  | Packet filter contents 2 |  |  |  |  |  |  |  | octet k+3 octet n |
|  | … |  |  |  |  |  |  |  | octet n+1 octet y |
|  | 0 | 0 | Packet filter direction N |  | Packet filter identifier N |  |  |  | octet y+1 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | Length of packet filter contents N |  |  |  |  |  |  |  | octet y+2 |
|  | Packet filter contents N |  |  |  |  |  |  |  | octet y+3 octet m |

Figure 9.11.4.13.4: Packet filter list when the rule operation is "create new QoS rule", or "modify existing QoS rule and add packet filters" or "modify existing QoS rule and replace all packet filters"

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Number of (S)RTP multiplexed media identification information entries |  |  |  |  |  |  |  | octet 10 |
|  | (S)RTP multiplexed media identification information entry 1 |  |  |  |  |  |  |  | octet 11 octet h |
|  | (S)RTP multiplexed media identification information entry 2 |  |  |  |  |  |  |  | octet (h+1)* octet i* |
|  | … |  |  |  |  |  |  |  | octet (i+1)* octet j* |
|  | (S)RTP multiplexed media identification information entry n |  |  |  |  |  |  |  | octet (j+1)* octet k* |

Figure 9.11.4.13.5: Value part of (S)RTP multiplexed media identification information component

| 8 | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of value part of (S)RTP multiplexed media identification information |  |  |  |  |  |  |  |  |  |  |  |  | octet 11 |
| 0 |  | 0 | 0 | RPTPI |  | RSHEMPI |  | MITPI |  | PTPI | SSRCPI |  | octet 12 |
| Spare |  | Spare | Spare |  |  |  |  |  |  |  |  |  |  |
| Synchronization Source (SSRC) |  |  |  |  |  |  |  |  |  |  |  |  | octet 13* octet 16* |
| Payload type |  |  |  |  |  |  |  |  |  |  |  |  | octet p*(see NOTE) |
| MID identification-tag |  |  |  |  |  |  |  |  |  |  |  |  | octet q*(see NOTE) octet r* |
| RTP SDES header extension id for MID |  |  |  |  |  |  |  |  |  |  |  |  | octet s* (see NOTE) octet t* |
| RTCP packet type |  |  |  |  |  |  |  |  |  |  |  |  | octet h*(see NOTE) |

NOTE:	The field is placed immediately after the last present preceding field.

Figure 9.11.4.13.6: (S)RTP multiplexed media identification information entry

| 8 | 7 |  | 6 |  | 5 |  | 4 | 3 | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | 0 | PCP indicator | DEI indicator |  | PCP |  |  |  | DEI |  | octet 1 |
| Spare |  | Spare |  |  |  |  |  |  |  |  |  |  |

Figure 9.11.4.13.7: Value part of Extended 802.1Q C-TAG PCP/DEI type

| 8 | 7 |  | 6 |  | 5 |  | 4 | 3 | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | 0 | PCP indicator | DEI indicator |  | PCP |  |  |  | DEI |  | octet 1 |
| Spare |  | Spare |  |  |  |  |  |  |  |  |  |  |

Figure 9.11.4.13.8: Value part of Extended 802.1Q S-TAG PCP/DEI type

Table 9.11.4.13.1: QoS rules information element

QoS rule identifier (octet 4) The QoS rule identifier field is used to identify the QoS rule. Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 no QoS rule identifier assigned 0 0 0 0 0 0 0 1 QRI 1 to 1 1 1 1 1 1 1 1 QRI 255 The network shall not set the QRI value to 0. QoS rule precedence (octet m+1) The QoS rule precedence field is used to specify the precedence of the QoS rule among all QoS rules (both the signalled QoS rules as described in clause 6.2.5.1.1.2 and the derived QoS rules as described in clause 6.2.5.1.1.3) associated with the PDU session of the QoS flow. This field includes the binary coded value of the QoS rule precedence in the range from 0 to 255 (decimal). The higher the value of the QoS rule precedence field, the lower the precedence of that QoS rule is. For the "delete existing QoS rule" operation, the QoS rule precedence value field shall not be included. For the "create new QoS rule" operation, the QoS rule precedence value field shall be included. The value 80 (decimal) is reserved. Segregation bit (bit 7 of octet m+2) (see NOTE 1) In the UE to network direction the segregation bit indicates whether the UE is requesting the network to bind service data flows described by the QoS rule to a dedicated QoS Flow and it is encoded as follows. In the network to UE direction this bit is spare. Bit 7 0 Segregation not requested 1 Segregation requested QoS flow identifier (QFI) (bits 6 to 1 of octet m+2) (see NOTE 1) The QoS flow identifier (QFI) field contains the QoS flow identifier. Bits 6 5 4 3 2 1 0 0 0 0 0 0 no QoS flow identifier assigned 0 0 0 0 0 1 QFI 1 to 1 1 1 1 1 1 QFI 63 The network shall not set the QFI value to 0. For the "delete existing QoS rule" operation, the QoS flow identifier value field shall not be included. For the "create new QoS rule" operation, the QoS flow identifier value field shall be included. DQR bit (bit 5 of octet 7) The DQR bit indicates whether the QoS rule is the default QoS rule and it is encoded as follows: Bit 5 0 the QoS rule is not the default QoS rule. 1 the QoS rule is the default QoS rule. Rule operation code (bits 8 to 6 of octet 7) Bits 8 7 6 0 0 0 Reserved 0 0 1 Create new QoS rule 0 1 0 Delete existing QoS rule 0 1 1 Modify existing QoS rule and add packet filters 1 0 0 Modify existing QoS rule and replace all packet filters 1 0 1 Modify existing QoS rule and delete packet filters 1 1 0 Modify existing QoS rule without modifying packet filters 1 1 1 Reserved Number of packet filters (bits 4 to 1 of octet 7) The number of packet filters contains the binary coding for the number of packet filters in the packet filter list. The number of packet filters field is encoded in bits 4 through 1 of octet 7 where bit 4 is the most significant and bit 1 is the least significant bit. For the "delete existing QoS rule" operation and for the "modify existing QoS rule without modifying packet filters" operation, the number of packet filters shall be coded as 0. For the "create new QoS rule" operation and the "modify existing QoS rule and replace all packet filters" operation, the number of packet filters shall be greater than or equal to 0 and less than or equal to 15. For all other operations, the number of packet filters shall be greater than 0 and less than or equal to 15. Packet filter list (octets 8 to m) The packet filter list contains a variable number of packet filters. For the "delete existing QoS rule" operation, the length of QoS rule field is set to one. For the "delete existing QoS rule" operation and the "modify existing QoS rule without modifying packet filters" operation, the packet filter list shall be empty. For the "modify existing QoS rule and delete packet filters" operation, the packet filter list shall contain a variable number of packet filter identifiers. This number shall be derived from the coding of the number of packet filters field in octet 7. For the "create new QoS rule" operation and for the "modify existing QoS rule and replace all packet filters" operation, the packet filter list shall contain 0 or a variable number of packet filters. This number shall be derived from the coding of the number of packet filters field in octet 7. For the "modify existing QoS rule and add packet filters" operation, the packet filter list shall contain a variable number of packet filters. This number shall be derived from the coding of the number of packet filters field in octet 7. Each packet filter is of variable length and consists of - a packet filter direction (2 bits); - a packet filter identifier (4 bits); - the length of the packet filter contents (1 octet); and - the packet filter contents itself (variable amount of octets). The packet filter direction field is used to indicate for what traffic direction the filter applies. Bits 6 5 0 0 reserved 0 1 downlink only (see NOTE 2) 1 0 uplink only 1 1 bidirectional The packet filter identifier field is used to identify each packet filter in a QoS rule. The least significant 4 bits are used. When the UE requests to "create new QoS rule", "modify existing QoS rule and replace all packet filters" or "modify existing QoS rule and add packet filters", the packet filter identifier values shall be set to 0. The length of the packet filter contents field contains the binary coded representation of the length of the packet filter contents field of a packet filter. The first bit in transmission order is the most significant bit. The packet filter contents field is of variable size and contains a variable number (at least one) of packet filter components. Each packet filter component shall be encoded as a sequence of a one octet packet filter component type identifier and a packet filter component value field. Furthermore, for all packet filter component type identifier values except for "(S)RTP multiplexed media identification information type", each of the other packet filter component value field has a fixed length. The packet filter component type identifier shall be transmitted first. In each packet filter, there shall not be more than one occurrence of each packet filter component type. Among the "IPv4 remote address type" and "IPv6 remote address/prefix length type" packet filter components, only one shall be present in one packet filter. Among the "IPv4 local address type" and "IPv6 local address/prefix length type" packet filter components, only one shall be present in one packet filter. Among the "single local port type" and "local port range type" packet filter components, only one shall be present in one packet filter. Among the "single remote port type" and "remote port range type" packet filter components, only one shall be present in one packet filter. Among the "destination MAC address type" and "destination MAC address range type" packet filter components, only one shall be present in one packet filter. Among the "source MAC address type" and "source MAC address range type" packet filter components, only one shall be present in one packet filter. If the "match-all type" packet filter component is present in the packet filter, no other packet filter component shall be present in the packet filter and the length of the packet filter contents field shall be set to one. If the "Ethertype type" packet filter component is present in the packet filter and the "Ethertype type" packet filter component value is neither "0800H" (for IPv4) nor "86DDH" (for IPv6), no IP packet filter component shall be present in the packet filter. The term "IP packet filter component" refers to "IPv4 remote address type", "IPv4 local address type", "IPv6 remote address/prefix length type", "IPv6 local address/prefix length type", "Protocol identifier/Next header type", "Single local port type", "Local port range type", "Single remote port type", "Remote port range type", "Security parameter index type", "Type of service/Traffic class type" and "Flow label type". The "(S)RTP multiplexed media identification information type" packet filter component can not be present in the packet filter with no "IP packet filter component". The term local refers to the UE and the term remote refers to an external network entity. Packet filter component type identifier Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 1 Match-all type (see NOTE 2) 0 0 0 1 0 0 0 0 IPv4 remote address type 0 0 0 1 0 0 0 1 IPv4 local address type 0 0 1 0 0 0 0 1 IPv6 remote address/prefix length type 0 0 1 0 0 0 1 1 IPv6 local address/prefix length type 0 0 1 1 0 0 0 0 Protocol identifier/Next header type 0 1 0 0 0 0 0 0 Single local port type 0 1 0 0 0 0 0 1 Local port range type 0 1 0 1 0 0 0 0 Single remote port type 0 1 0 1 0 0 0 1 Remote port range type 0 1 1 0 0 0 0 0 Security parameter index type 0 1 1 1 0 0 0 0 Type of service/Traffic class type 1 0 0 0 0 0 0 0 Flow label type 1 0 0 0 0 0 0 1 Destination MAC address type 1 0 0 0 0 0 1 0 Source MAC address type 1 0 0 0 0 0 1 1 802.1Q C-TAG VID type 1 0 0 0 0 1 0 0 802.1Q S-TAG VID type 1 0 0 0 0 1 0 1 802.1Q C-TAG PCP/DEI type 1 0 0 0 1 0 1 0 Extended 802.1Q C-TAG PCP/DEI type 1 0 0 0 0 1 1 0 802.1Q S-TAG PCP/DEI type 1 0 0 0 1 0 1 1 Extended 802.1Q S-TAG PCP/DEI type 1 0 0 0 0 1 1 1 Ethertype type 1 0 0 0 1 0 0 0 Destination MAC address range type 1 0 0 0 1 0 0 1 Source MAC address range type 1 0 0 1 0 0 0 1 (S)RTP multiplexed media identification information type (see NOTE 3, NOTE 4) All other values are reserved. The description and valid combinations of packet filter component type identifiers in a packet filter are defined in 3GPP TS 23.501 [8]. For "match-all type", the packet filter component shall not include the packet filter component value field. For "IPv4 remote address type", the packet filter component value field shall be encoded as a sequence of a four octet IPv4 address field and a four octet IPv4 address mask field. The IPv4 address field shall be transmitted first. For "IPv4 local address type", the packet filter component value field shall be encoded as defined for "IPv4 remote address type". For "IPv6 remote address/prefix length type", the packet filter component value field shall be encoded as a sequence of a sixteen octet IPv6 address field and one octet prefix length field. The IPv6 address field shall be transmitted first. For "IPv6 local address/prefix length type", the packet filter component value field shall be encoded as defined for "IPv6 remote address /prefix length". For "protocol identifier/Next header type", the packet filter component value field shall be encoded as one octet which specifies the IPv4 protocol identifier or Ipv6 next header. For "single local port type" and "single remote port type", the packet filter component value field shall be encoded as two octets which specify a port number. For "local port range type" and "remote port range type", the packet filter component value field shall be encoded as a sequence of a two octet port range low limit field and a two octet port range high limit field. The port range low limit field shall be transmitted first. For "security parameter index", the packet filter component value field shall be encoded as four octets which specify the IPSec security parameter index. For "type of service/traffic class type", the packet filter component value field shall be encoded as a sequence of a one octet type-of-service/traffic class field and a one octet type-of-service/traffic class mask field. The type-of-service/traffic class field shall be transmitted first. For "flow label type", the packet filter component value field shall be encoded as three octets which specify the IPv6 flow label. The bits 8 through 5 of the first octet shall be spare whereas the remaining 20 bits shall contain the IPv6 flow label. For "destination MAC address type" and "source MAC address type", the packet filter component value field shall be encoded as 6 octets which specify a MAC address. When the packet filter direction field indicates "bidirectional", the destination MAC address is the remote MAC address and the source MAC address is the local MAC address. For "802.1Q C-TAG VID type", the packet filter component value field shall be encoded as two octets which specify the VID of the customer-VLAN tag (C-TAG). The bits 8 through 5 of the first octet shall be spare whereas the remaining 12 bits shall contain the VID. If there are more than one C-TAG in the Ethernet frame header, the outermost C-TAG is evaluated. For "802.1Q S-TAG VID type", the packet filter component value field shall be encoded as two octets which specify the VID of the service-VLAN tag (S-TAG). The bits 8 through 5 of the first octet shall be spare whereas the remaining 12 bits shall contain the VID. If there are more than one S-TAG in the Ethernet frame header, the outermost S-TAG is evaluated. For "802.1Q C-TAG PCP/DEI type", the packet filter component value field shall be encoded as one octet which specifies the 802.1Q C-TAG PCP and DEI. The bits 8 through 5 of the octet shall be spare, the bits 4 through 2 contain the PCP and bit 1 contains the DEI. If there are more than one C-TAG in the Ethernet frame header, the outermost C-TAG is evaluated. For "extended 802.1Q C-TAG PCP/DEI type", the packet filter component value field shall be encoded as one octet which specifies the 802.1Q C-TAG PCP and DEI as shown in figure 9.11.4.13.7. The bits 8 and 7 of the octet shall be spare and set to 0. The bit 6 indicates the presence of the PCP field, when bit 6 is set to 1, the bits 4 through 2 contain the PCP. When the bit 6 is set to 0, the bits 4 through 2 are spare and not used. The bit 5 indicates the presence of the EDI field, when bit 6 is set to 1, bit 1 contains the DEI. When the bit 5 is set to 0, the bit 1 is spare and not used. If there are more than one C-TAG in the Ethernet frame header, the outermost C-TAG is evaluated. For "802.1Q S-TAG PCP/DEI type", the packet filter component value field shall be encoded as one octet which specifies the 802.1Q S-TAG PCP and DEI. The bits 8 through 5 of the octet shall be spare, the bits 4 through 2 contain the PCP and bit 1 contains the DEI. If there are more than one S-TAG in the Ethernet frame header, the outermost S-TAG is evaluated. For "extended 802.1Q S-TAG PCP/DEI type", the packet filter component value field shall be encoded as one octet which specifies the 802.1Q S-TAG PCP and DEI as shown in figure 9.11.4.13.8. The bits 8 and 7 of the octet shall be spare and set to 0. The bit 6 indicates the presence of the PCP field, when bit 6 is set to 1, the bits 4 through 2 contain the PCP. When the bit 6 is set to 0, the bits 4 through 2 are spare and not used. The bit 5 indicates the presence of the EDI field, when bit 6 is set to 1, bit 1 contains the DEI. When the bit 5 is set to 0, the bit 1 is spare and not used. If there are more than one S-TAG in the Ethernet frame header, the outermost C-TAG is evaluated. For "ethertype type", the packet filter component value field shall be encoded as two octets which specify an ethertype. For "destination MAC address range type", the packet filter component value field shall be encoded as a sequence of a 6 octet destination MAC address range low limit field and a 6 octet destination MAC address range high limit field. The destination MAC address range low limit field shall be transmitted first. When the packet filter direction field indicates "bidirectional", the destination MAC address range is the remote MAC address range. For "source MAC address range type", the packet filter component value field shall be encoded as a sequence of a 6 octet source MAC address range low limit field and a 6 octet source MAC address range high limit field. The source MAC address range low limit field shall be transmitted first. When the packet filter direction field indicates "bidirectional", the source MAC address is the local MAC address range. For "(S)RTP multiplexed media identification information type", the packet filter component value field shall be encoded as figure 9.11.4.13.5 and figure 9.11.4.13.6. An (S)RTP multiplexed media identification information entry for (s)RTCP shall contain the RTCP packet type field and at least one of the SSRC field and MID identification-tag field. An (S)RTP multiplexed media identification information entry for (s)RTP shall contain payload type field, additionally, it may contain: a) SSRC field; b) MID identification-tag field and RTP SDES header extension id for MID; or c) both a) and b). An (S)RTP multiplexed media identification information entry for (s)RTP and (s)RTCP shall contain: a) SSRC field; b) MID identification-tag field and RTP SDES header extension id for MID field; or c) both a) and b). SSRC presence indicator (SSRCPI) (bit 1 of octet 12) The SSRCPI field indicates whether the SSRC field is included or not. Bit 1 0 SSRC not included 1 SSRC included Payload type presence indicator (PTPI) (bit 2 of octet 12) The PTPI field indicates whether the payload type field is included or not. Bit 2 0 Payload type not included 1 Payload type included MID identification-tag presence indicator (MITPI) (bit 3 of octet 12) The MITPI field indicates whether the MID identification-tag field is included or not. Bit 3 0 MID identification-tag not included 1 MID identification-tag included RTP SDES header extension id for MID presence indicator (RSHEMPI) (bit 4 of octet 12) The RSHEMPI field indicates whether the RTP SDES header extension id for MID field is included or not. Bit 4 0 RTP SDES header extension id for MID not included 1 RTP SDES header extension id for MID included RTCP packet type presence indicator (RPTPI) (bit 5 of octet 12) The RPTPI field indicates whether the RTCP packet type field is included or not. Bit 5 0 RTCP packet type not included 1 RTCP packet type included The synchronization source (SSRC) field shall be encoded as 4 octet SSRC field which specify the synchronization source identifier in the RTP header as specified in IETF RFC 3550 [71]. The payload type field shall be encoded as octet payload type field which contains the binary representation of an integer between 0(inclusive) and 127(inclusive) as specified in IETF RFC 3550 [71] (see NOTE 5). The other values are spare. If a spare value is received, the UE shall ignore the corresponding (S)RTP multiplexed media identification information entry. The MID identification-tag field shall be encoded as length of the MID identification-tag which is one octet and followed by value of the MID identification-tag which is with a maximum length of 255 octets as specified in IETF RFC 9143 [72]. The RTP SDES header extension id for MID field contains the RTP SDES header extension id for MID, it shall be coded as the RTP SDES header extension id of the RTP SDES header extension for MID as specified in IETF RFC 9143 [72]. The RTCP packet type field shall be encoded as one octet and contains the binary representation of an unsigned integer in the range 0 to 255 inclusive as specified in IETF RFC 3550 [71] (see NOTE 5).

NOTE 1: Octet m+2 shall not be included without octet m+1. NOTE 2: The "Match-all type" packet filter component type identifier shall not be used with packet filter direction "downlink only". NOTE 3: When the "(S)RTP multiplexed media identification information type" packet filter component type identifier is present in an uplink packet filter, if an UL user data packet matches any (S)RTP multiplexed media identification information entry, the UL user data packet is considered to match the (S)RTP multiplexed media identification information component of the UL packet filter. How to determine an UL user data packet matches the (S)RTP multiplexed media identification information entry is left to implementation. NOTE 4: At least one of the SSRCPI, PTPI, MITPI, RSHEMPI, and RPTPI shall be set to 1 in the (S)RTP multiplexed media identification information entry. NOTE 5: As specified in clause 4 of IETF RFC 5761 [73], the RTP payload type values and RTCP packet type values in the multiplexed media shall not collide. The RTP payload type values plus 128 and RTCP packet type values in the multiplexed media shall not collide.

#### 9.11.4.14	Session-AMBR

The purpose of the Session-AMBR information element is to indicate the initial subscribed PDU session aggregate maximum bit rate when the UE establishes a PDU session or to indicate the new subscribed PDU session aggregate maximum bit rate if it is changed by the network.

The Session-AMBR information element is coded as shown in figure 9.11.4.14.1 and table 9.11.4.14.1.

The Session-AMBR is a type 4 information element with a length of 8 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Session-AMBR IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Session-AMBR contents |  |  |  |  |  |  |  | octet 2 |
| Unit for Session-AMBR for downlink |  |  |  |  |  |  |  | octet 3 |
| Session-AMBR for downlink |  |  |  |  |  |  |  | octet 4-5 |
| Unit for Session-AMBR for uplink |  |  |  |  |  |  |  | octet 6 |
| Session-AMBR for uplink |  |  |  |  |  |  |  | octet 7-8 |

Figure 9.11.4.14.1: Session-AMBR information element

Table 9.11.4.14.1: Session-AMBR information element

Unit for Session-AMBR for downlink (octet 3) 0 0 0 0 0 0 0 0 value is not used (see NOTE) 0 0 0 0 0 0 0 1 value is incremented in multiples of 1 Kbps 0 0 0 0 0 0 1 0 value is incremented in multiples of 4 Kbps 0 0 0 0 0 0 1 1 value is incremented in multiples of 16 Kbps 0 0 0 0 0 1 0 0 value is incremented in multiples of 64 Kbps 0 0 0 0 0 1 0 1 value is incremented in multiples of 256 kbps 0 0 0 0 0 1 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 1 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 1 0 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 1 0 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 1 0 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 1 0 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 1 1 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 1 1 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 1 1 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 1 1 1 1 value is incremented in multiples of 256 Gbps 0 0 0 1 0 0 0 0 value is incremented in multiples of 1 Tbps 0 0 0 1 0 0 0 1 value is incremented in multiples of 4 Tbps 0 0 0 1 0 0 1 0 value is incremented in multiples of 16 Tbps 0 0 0 1 0 0 1 1 value is incremented in multiples of 64 Tbps 0 0 0 1 0 1 0 0 value is incremented in multiples of 256 Tbps 0 0 0 1 0 1 0 1 value is incremented in multiples of 1 Pbps 0 0 0 1 0 1 1 0 value is incremented in multiples of 4 Pbps 0 0 0 1 0 1 1 1 value is incremented in multiples of 16 Pbps 0 0 0 1 1 0 0 0 value is incremented in multiples of 64 Pbps 0 0 0 1 1 0 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Session-AMBR for downlink (octets 4 and 5) Octets 4 and 5 represent the binary coded value of PDU session aggregated maximum bit rate for downlink in units defined by octet 3. Unit for Session-AMBR for uplink (octet 6) The coding is identical to the unit coding defined for Session-AMBR for downlink (octet 3) Session-AMBR for uplink (octets 7 and 8) Octets 7 and 8 represent the binary coded value of PDU session aggregated maximum bit rate for uplink in units defined by octet 6.

NOTE: In this release of the specifications if received it shall be interpreted as value is incremented in multiples of 1 Kbps. In earlier releases of specifications, the interpretation of this value is up to implementation.

#### 9.11.4.15	SM PDU DN request container

The purpose of the SM PDU DN request container information element is to carry a DN-specific identity of the UE in the network access identifier (NAI) format.

The SM PDU DN request container information element is coded as shown in figure 9.11.4.15.1 and table 9.11.4.15.1.

The SM PDU DN request container is a type 4 information element with minimal length of 3 octets and maximum length of 255 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SM PDU DN request container information IEI |  |  |  |  |  |  |  | octet 1 |
| SM PDU DN request container information length |  |  |  |  |  |  |  | octet 2 |
| DN-specific identity |  |  |  |  |  |  |  | octets 3*-n* |

Figure 9.11.4.15.1: SM PDU DN request container information element

Table 9.11.4.15.1: SM PDU DN request container information element

DN-specific identity (octet 3 to octet n) A DN-specific identity of the UE in the network access identifier (NAI) format according to IETF RFC 7542 [37], encoded as UTF-8 string.

#### 9.11.4.16	SSC mode

The purpose of the SSC mode information element is to indicate SSC mode.

The SSC mode information element is coded as shown in figure 9.11.4.16.1 and table 9.11.4.16.1.

The SSC mode is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSC mode IEI |  |  |  | 0 Spare | SSC mode value |  |  | octet 1 |

Figure 9.11.4.16.1: SSC mode information element

Table 9.11.4.16.1: SSC mode information element

| SSC mode value (octet 1, bit 1 to bit 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 1 |  | SSC mode 1 |
| 0 | 1 | 0 |  | SSC mode 2 |
| 0 | 1 | 1 |  | SSC mode 3 |
| 1 | 0 | 0 |  | unused; shall be interpreted as "SSC mode 1", if received by the network |
| 1 | 0 | 1 |  | unused; shall be interpreted as "SSC mode 2", if received by the network |
| 1 | 1 | 0 |  | unused; shall be interpreted as "SSC mode 3", if received by the network |
| All other values are reserved. |  |  |  |  |

#### 9.11.4.17	Re-attempt indicator

The purpose of the Re-attempt indicator information element is to indicate a condition under which the UE is allowed in the current PLMN or its equivalent PLMN(s) or the current SNPN or its equivalent SNPNs for the same DNN, to re-attempt a session management procedure (see 3GPP TS 24.301 [15]) corresponding to the 5GS session management procedure which was rejected by the network.

The Re-attempt indicator information element is coded as shown in figure 9.11.4.17.1 and table 9.11.4.17.1.

The Re-attempt indicator is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Re-attempt indicator IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Re-attempt indicator contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | EPLMNC | RATC | octet 3 |

Figure 9.11.4.17.1: Re-attempt indicator

Table 9.11.4.17.1: Re-attempt indicator

| RATC (octet 3, bit 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | UE is allowed to re-attempt the procedure in S1 mode |
| 1 | UE is not allowed to re-attempt the procedure in S1 mode |
| EPLMNC (octet 3, bit 2) |  |
| Bit |  |
| 2 |  |
| 0 | UE is allowed to re-attempt the procedure in an equivalent PLMN or equivalent SNPN |
| 1 | UE is not allowed to re-attempt the procedure in an equivalent PLMN or equivalent SNPN |
| Bits 3 to 8 of octet 3 are spare and shall be encoded as zero. |  |

#### 9.11.4.18	5GSM network feature support

The purpose of the 5GSM network feature support information element is to indicate whether certain session management related features are supported by the network.

The 5GSM network feature support information element is coded as shown in figure 9.11.4.18.1 and table 9.11.4.18.1.

The 5GSM network feature support is a type 4 information element with a minimum length of 3 octets and a maximum length of 15 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GSM network feature support IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GSM network feature support contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | NAPS | EPT-S1 | octet 3 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | octet 4* -15* |
| Spare |  |  |  |  |  |  |  |  |

Figure 9.11.4.18.1: 5GSM network feature support information element

Table 9.11.4.18.1: 5GSM network feature support information element

| 5GSM network feature support contents |  |  |  |  |
| --- | --- | --- | --- | --- |
| Ethernet PDN type in S1 mode (IEPT-S1) (octet 3, bit 1) |  |  |  |  |
| This bit indicates network's capability for Ethernet PDN type in S1 mode. |  |  |  |  |
| 0 |  |  |  | Ethernet PDN type in S1 mode not supported |
| 1 |  |  |  | Ethernet PDN type in S1 mode supported |
| Non-3GPP access path switching (NAPS) (octet 3, bit 2) |  |  |  |  |
| This bit indicates whether non-3GPP access path switching is supported. |  |  |  |  |
| Bit |  |  |  |  |
| 2 |  |  |  |  |
| 0 |  |  |  | non-3GPP access path switching not supported |
| 1 |  |  |  | non-3GPP access path switching supported |
| All other bits in octet 3 to 15 are spare and shall be coded as zero, if the respective octet is included in the information element. |  |  |  |  |

#### 9.11.4.19	Void

#### 9.11.4.20	Serving PLMN rate control

See clause 9.9.4.28 in 3GPP TS 24.301 [15].

#### 9.11.4.21	5GSM congestion re-attempt indicator

The purpose of the 5GSM congestion re-attempt indicator information element is to indicate whether the back-off timer is applied in the registered PLMN or all PLMNs or in the registered SNPN or all equivalent SNPNs, and additionally to indicate whether the back-off timer is applied in the current access type or both 3GPP access type and non-3GPP access type.

The 5GSM congestion re-attempt indicator information element is coded as shown in figure 9.11.4.21.1 and table 9.11.4.21.1.

The 5GSM congestion re-attempt indicator is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GSM congestion re-attempt indicator IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GSM congestion re-attempt indicator contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | CATBO | ABO | octet 3 |

Figure 9.11.4.21.1: 5GSM congestion re-attempt indicator

Table 9.11.4.21.1: 5GSM congestion re-attempt indicator

| ABO (All PLMNs Back-off timer) (octet 3, bit 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | The back-off timer is applied in the registered PLMN or registered SNPN. |
| 1 | The back-off timer is applied in all PLMNs or all equivalent SNPNs. |
| CATBO (Current Access Type Back-off Timer) (octet 3, bit 2) Bit 2 |  |
| 0 | The back-off timer is applied in both 3GPP access type and non-3GPP access type |
| 1 | The back-off timer is applied in the current access type |
| Bits 3 to 8 of octet 3 are spare and shall be encoded as zero. |  |

#### 9.11.4.22	ATSSS container

The purpose of the ATSSS container information element is to transfer parameters associated with ATSSS.

The ATSSS container information element is coded as shown in figure 9.11.4.22.1 and table 9.11.4.22.1.

The ATSSS container is a type 6 information element with a minimum length of 3 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ATSSS container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of ATSSS container contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| ATSSS container contents |  |  |  |  |  |  |  | octet 4* octet x* |

Figure 9.11.4.22.1: ATSSS container information element

Table 9.11.4.22.1: ATSSS container information element

ATSSS container contents are defined in 3GPP TS 24.193 [13B].

#### 9.11.4.23	Control plane only indication

The purpose of the control plane only indication information element is to indicate that a PDU session is only for control plane CIoT 5GS optimization.

The control plane only indication information element is coded as shown in figure 9.11.4.23.1.

The control plane only indication is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Control plane only indication IEI |  |  |  | 0 Spare | 0 Spare | 0 Spare | CPOI value | octet 1 |

Figure 9.11.4.23.1: Control plane only indication information element

Table 9.11.4.23.1: Control plane only indication information element

| Control plane only indication value (CPOI) (octet 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | reserved |
| 1 | PDU session can be used for control plane CIoT 5GS optimization only |
| The value 0 is reserved. If received, it shall be interpreted as if the control plane only indication IE was not included in the message. |  |
| Bits 4 to 2 of octet 1 are spare and shall be all encoded as zero. |  |

#### 9.11.4.24	IP header compression configuration

The purpose of the IP header compression configuration information element is to negotiate ROHC channel setup parameters specified in IETF RFC 5795 [39B] and, optionally, provide additional header compression context setup parameters.

The IP header compression configuration information element is coded as shown in figure 9.11.4.24.1 and table 9.11.4.24.1.

The IP header compression configuration is a type 4 information element with a minimum length of 5 octets and a maximum length of 257 octets.

The optional Additional IP header compression parameters container field conveys the additional header compression context setup parameters as specified in 3GPP TS 23.501 [8] in a generic container. This field corresponds to the profile-specific information in the header of the ROHC IR packet type in IETF RFC 5795 [39B].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IP header compression configuration IEI |  |  |  |  |  |  |  | octet 1 |
| Length of IP header compression configuration contents |  |  |  |  |  |  |  | octet 2 |
| Spare | P0x0104 | P0x0103 | P0x0102 | P0x0006 | P0x0004 | P0x0003 | P0x0002 | octet 3 |
| MAX_CID |  |  |  |  |  |  |  | octet 4 |
|  |  |  |  |  |  |  |  | octet 5 |
| Additional IP header compression context setup parameters type |  |  |  |  |  |  |  | octet 6* |
| Additional IP header compression context setup parameters container |  |  |  |  |  |  |  | octet 7* |
|  |  |  |  |  |  |  |  | octet n* |

Figure 9.11.4.24.1: IP header compression configuration information element

Table 9.11.4.24.1: IP header compression configuration information element

ROHC Profiles (octet 3) The ROHC Profiles shall indicate which of the ROHC profiles is supported. When a particular bit is set to 1, this indicates that the corresponding profile is supported. The No Compression profile 0x0000 (see IETF RFC 5795 [39B]) shall always be supported. When all the bits are set to 0, this indicates that only the No Compression profile 0x0000 is supported. Profile 0x0002 support indicator (see IETF RFC 3095 [33A] and IETF RFC 4815 [38A]) (octet 3 bit 1) 0 RoHC profile 0x0002 (UDP/IP) is not supported 1 RoHC profile 0x0002 (UDP/IP) is supported Profile 0x0003 support indicator (see IETF RFC 3095 [33A] and IETF RFC 4815 [38A]) (octet 3 bit 2) 0 RoHC profile 0x0003 (ESP/IP) is not supported 1 RoHC profile 0x0003 (ESP/IP) is supported Profile 0x0004 support indicator (see IETF RFC 3843 [34A] and IETF RFC 4815 [38A]) (octet 3 bit 3) 0 RoHC profile 0x0004 (IP) is not supported 1 RoHC profile 0x0004 (IP) is supported Profile 0x0006 support indicator (see IETF RFC 6846 [40B]) (octet 3 bit 4) 0 RoHC profile 0x0006 (TCP/IP) is not supported 1 RoHC profile 0x0006 (TCP/IP) is supported Profile 0x0102 support indicator (see IETF RFC 5225 [39A]) (octet 3 bit 5) 0 RoHC profile 0x0102 (UDP/IP) is not supported 1 RoHC profile 0x0102 (UDP/IP) is supported Profile 0x0103 support indicator (see IETF RFC 5225 [39A]) (octet 3 bit 6) 0 RoHC profile 0x0103 (ESP/IP) is not supported 1 RoHC profile 0x0103 (ESP/IP) is supported Profile 0x0104 support indicator (see IETF RFC 5225 [39A]) (octet 3 bit 7) 0 RoHC profile 0x0104 (IP) is not supported 1 RoHC profile 0x0104 (IP) is supported Bits 8 is spare and shall be set to 0. MAX_CID (octet 4 and octet 5) This is the MAX_CID value as specified in 3GPP TS 36.323 [25]. It is encoded in binary coding with a value in the range from 1 to 16383. Additional IP header compression context parameters type (octet 6). The Additional IP header compression context parameters type octet indicates the profile associated with the profile-specific information in the Additional IP header compression context parameters container. Bits 8 7 6 5 4 3 2 1 Type 0 0 0 0 0 0 0 0 0x0000 (No Compression) 0 0 0 0 0 0 0 1 0x0002 (UDP/IP) 0 0 0 0 0 0 1 0 0x0003 (ESP/IP) 0 0 0 0 0 0 1 1 0x0004 (IP) 0 0 0 0 0 1 0 0 0x0006 (TCP/IP) 0 0 0 0 0 1 0 1 0x0102 (UDP/IP) 0 0 0 0 0 1 1 0 0x0103 (ESP/IP) 0 0 0 0 0 1 1 1 0x0104 (IP) 0 0 0 0 1 0 0 0 Other 0 0 0 0 1 0 0 1 to 1 1 1 1 1 1 1 1 Spare Additional IP header compression context parameters container (octets 7 to n). Additional IP header compression context parameters container carries the profile-specific information (see IETF RFC 5795 [39B]). The maximum size is 251 octets. NOTE: If the Additional IP header compression context setup parameters container is included, then the Additional IP header compression context parameters type shall be included in the octet 6.

#### 9.11.4.25	DS-TT Ethernet port MAC address

The purpose of the DS-TT Ethernet port MAC address information element is to signal the MAC address of the DS-TT Ethernet port used for a PDU session of "Ethernet" PDU session type.

The DS-TT Ethernet port MAC address information element is coded as shown in figure 9.11.4.25.1 and table 9.11.4.25.1.

The DS-TT Ethernet port MAC address is a type 4 information element with a length of 8 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DS-TT Ethernet port MAC address IEI |  |  |  |  |  |  |  | octet 1 |
| Length of DS-TT Ethernet port MAC address contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| DS-TT Ethernet port MAC address contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 8 |

Figure 9.11.4.25.1: DS-TT Ethernet port MAC address information element

Table 9.11.4.25.1: DS-TT Ethernet port MAC address information element

DS-TT Ethernet port MAC address contents (octets 3 to 8) The DS-TT Ethernet port MAC address contents consist of the binary representation of the MAC address of the DS-TT Ethernet port used for the PDU session, starting with the LSB bit of the first octet of the MAC address included in bit 1 of octet 3.

#### 9.11.4.26	UE-DS-TT residence time

The purpose of the UE-DS-TT residence time information element is to signal the time taken within the UE and the DS-TT to forward a packet i.e. between the ingress of the UE and the DS-TT port in the DL direction, or between the DS-TT port and the egress of the UE in the UL direction.

The UE- DS-TT residence time information element is coded as shown in figure 9.11.4.26.1 and table 9.11.4.26.1.

The UE-DS-TT residence time is a type 4 information element with a length of 10 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE-DS-TT residence time IEI |  |  |  |  |  |  |  | octet 1 |
| Length of UE-DS-TT residence time contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| UE-DS-TT residence time contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 10 |

Figure 9.11.4.26.1: UE-DS-TT residence time information element

Table 9.11.4.26.1: UE-DS-TT residence time information element

UE-DS-TT residence time contents (octets 3 to 10) The UE-DS-TT residence time contents contain the UE-DS-TT residence time encoded as specified for the correctionField in IEEE Std 1588-2019 [43B], with the LSB bit of the first octet of the UE-DS-TT residence time included in bit 1 of octet 3. If the UE-DS-TT residence time.is too big to be represented, all bits of octets 3 to 10 shall be coded as "1" except the MSB bit of octet 10.

#### 9.11.4.27	Port management information container

The purpose of the Port management information container information element is to transport a port management service message as specified in clause 8 of 3GPP TS 24.539 [19BA].

The Port management information container information element is coded as shown in figure 9.11.4.27.1 and table 9.11.4.27.1.

The Port management information container is a type 6 information element with a minimum length of 4 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port management information container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Port management information container contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Port management information container |  |  |  |  |  |  |  | octet 4 octet n |

Figure 9.11.4.27.1: Port management information container information element

Table 9.11.4.27.1: Port management information container information element

Port management information container (octet 4 to n)

A port management service message as specified in clause 8 of 3GPP TS 24.539 [19BA].

#### 9.11.4.28	Ethernet header compression configuration

The purpose of the Ethernet header compression configuration information element is to negotiate the use of EHC and the length of the CID field in the EHC packet (see 3GPP TS 38.323 [29]).

The Ethernet header compression configuration information element is coded as shown in figure 9.11.4.28.1 and table 9.11.4.28.1.

The Ethernet header compression configuration is a type 4 information element with the length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ethernet header compression configuration IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Ethernet header compression configuration contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | CID Length |  | octet 3 |

Figure 9.11.4.28.1: Ethernet header compression configuration information element

Table 9.11.4.28.1: Ethernet header compression configuration information element

| Length of CID field value (CID Length) (octet 3 bits 1 and 2) |  |  |
| --- | --- | --- |
| Bit |  |  |
| 2 | 1 |  |
| 0 | 0 | Ethernet header compression not used |
| 0 | 1 | 7 bits |
| 1 | 0 | 15 bits |
| All other values shall be interpreted as "7 bits". Bits 3 to 8 of octet 3 are spare and shall be coded as zero. |  |  |

#### 9.11.4.29	Remote UE context list

The purpose of the Remote UE context list information element is to provide identity and optionally IP address of a 5G ProSe remote UE connected to, or disconnected from, a UE acting as a 5G ProSe layer-3 UE-to-network relay.

The Remote UE context list information element is coded as shown in figure 9.11.4.29.1, figure 9.11.4.29.2, table 9.11.4.29.1 and table 9.11.4.29.2.

The Remote UE context list is a type 6 information element with a minimum length of 16 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Remote UE context list IEI |  |  |  |  |  |  |  | octet 1 |
| Length of remote UE context list contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| Number of remote UE contexts |  |  |  |  |  |  |  | octet 4 |
| Remote UE context 1 |  |  |  |  |  |  |  | octet 5 |
|  |  |  |  |  |  |  |  | octet a |
| … |  |  |  |  |  |  |  | octet a+1* octet b* |
| Remote UE context k |  |  |  |  |  |  |  | octet b+1* |
|  |  |  |  |  |  |  |  | octet c* |

Figure 9.11.4.29.1: Remote UE context list

Table 9.11.4.29.1: Remote UE context list

Remote UE context (octet 5 etc)

The contents of remote UE context are applicable for one individual UE and are coded as shown in figure 9.11.4.29.2 and table 9.11.4.29.2.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of remote UE context |  |  |  |  |  |  |  | octet 5 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | Remote UE ID format | Remote UE ID type |  |  | octet 6 |
| Length of remote UE ID |  |  |  |  |  |  |  | octet 7 |
| Remote UE ID |  |  |  |  |  |  |  | octet 8 |
|  |  |  |  |  |  |  |  | Octet j* |
| Spare |  |  | UPRI4I | TPRI4I | Protocol used by remote UE |  |  | octet j+1* |
| Address information |  |  |  |  |  |  |  | octet j+2* octet j+k* |
| HPLMN ID |  |  |  |  |  |  |  | octet (j+k+1)* octet (j+k+3)* |

Figure 9.11.4.29.2: Remote UE context

Table 9.11.4.29.2: Remote UE context

| Remote UE ID type (bits 1 to 3 of octet 6) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |
| 3 | 2 | 1 |  |  |  |
| 0 | 0 | 1 |  | UP-PRUK ID |  |
| 0 | 1 | 0 |  | CP-PRUK ID |  |
| 0 | 1 | 1 |  | IMEI |  |
| 1 | 0 | 0 |  | IMEISV |  |
| All other values are reserved. |  |  |  |  |  |
| Remote UE ID format (bit 4 of octet 6) (NOTE) |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 0 |  |  |  | Network access identifier (NAI) |  |
| 1 |  |  |  | 64-bit string |  |
| Bits 5 to 8 of octet 6 are spare and shall be coded as zero. |  |  |  |  |  |
| Remote UE ID (octet 8 to octet j) |  |  |  |  |  |
| The UP-PRUK ID as specified in 3GPP TS 33.503 [56], the CP-PRUK ID as specified in 3GPP TS 33.503 [56], the IMEI or the IMEISV of the 5G ProSe Remote UE. If the remote UE ID type field indicates "UP-PRUK ID" or "CP-PRUK ID", and the remote UE ID format field indicates "NAI", the remote UE ID field contains the UP-PRUK ID or the CP-PRUK ID in the NAI format as defined in 3GPP TS 23.003 [4], encoded as UTF-8 string. If the remote UE ID type field indicates "UP-PRUK ID", and the remote UE ID format field indicates "64-bit string", the remote UE ID field contains the UP-PRUK ID as a 64-bit string, encoded using binary encoding. If the remote UE ID type field indicates "IMEI" or "IMEISV", the remote UE ID field contains the IMEI or the IMEISV encoded as 5GS mobile identity information element for type of identity "IMEI" or "IMEISV" as specified in clause 9.11.3.4, starting with the fourth octet. |  |  |  |  |  |
| Protocol used by remote UE (octet j+1, bits 1 to 3) Bits |  |  |  |  |  |
| 3 | 2 | 1 |  |  |  |
| 0 | 0 | 0 |  | No IP info |  |
| 0 | 0 | 1 |  | IPv4 |  |
| 0 | 1 | 0 |  | IPv6 |  |
| 1 | 0 | 0 |  | Unstructured |  |
| 1 | 0 | 1 |  | Ethernet |  |
| All other values are reserved. |  |  |  |  |  |
| TCP port range for IPv4 indicator (TPRI4I) (octet j+1, bits 4) |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 0 |  | TCP port range for IPv4 absent |  |  |  |
| 1 |  | TCP port range for IPv4 present |  |  |  |
| UDP port range for IPv4 indicator (UPRI4I) (octet j+1, bits 5) |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 0 |  | UDP port range for IPv4 absent |  |  |  |
| 1 |  | UDP port range for IPv4 present |  |  |  |
| Bits 4 to 8 of octet j+1 are spare and shall be coded as zero. |  |  |  |  |  |
| The length of remote UE ID field contains the binary coded representation of the length of the remote UE ID field. The first bit in transmission order is the most significant bit. |  |  |  |  |  |
| If the Protocol used by remote UE indicates IPv4 and: - TPRI4I bit indicates "TCP port range for IPv4 absent" and UPRI4I bit indicates "UDP port range for IPv4 absent", the Address information in octet j+2 to octet j+5 contains the IPv4 address. - TPRI4I bit indicates "TCP port range for IPv4 present" and UPRI4I bit indicates "UDP port range for IPv4 absent", the Address information in octet j+2 to octet j+9 contains the IPv4 address followed by the TCP port range field. - TPRI4I bit indicates "TCP port range for IPv4 absent" and UPRI4I bit indicates "UDP port range for IPv4 present", the Address information in octet j+2 to octet j+9 contains the IPv4 address followed by the UDP port range field. - TPRI4I bit indicates "TCP port range for IPv4 present" and UPRI4I bit indicates "UDP port range for IPv4 present", the Address information in octet j+2 to octet j+13 contains the IPv4 address followed by the UDP port range field followed by the TCP port range field. See NOTE. The UDP port range field consists of the lowest UDP port number field followed by the highest UDP port number field, of the UDP port range assigned to the remote UE in the NAT function of 5G ProSe layer-3 UE-to-network relay. The TCP port range field consists of the lowest TCP port number field followed by highest TCP port number field, of the TCP port range assigned to the remote UE in the NAT function of 5G ProSe layer-3 UE-to-network relay. Each port number field is two octets long and bit 8 of first octet of the port number field represents the most significant bit of the port number and bit 1 of second octet of the port number field the least significant bit. If the Protocol used by remote UE indicates IPv6, the Address information in octet j+2 to octet j+9 contains the /64 IPv6 prefix of a remote UE. Bit 8 of octet j+2 represents the most significant bit of the /64 IPv6 prefix and bit 1 of octet j+9 the least significant bit. If the Protocol used by remote UE indicates Ethernet, the Address information in octet j+2 to octet j+7 contains the remote UE MAC address. Bit 8 of octet j+2 represents the most significant bit of the MAC address and bit 1 of octet j+7 the least significant bit. If the Protocol used by remote UE indicates Unstructured, the Address information octets are not included. If the Protocol used by remote UE indicates No IP info, the Address information octets are not included |  |  |  |  |  |
| If the Remote UE ID type field indicates "PRUK ID" and the Remote UE ID format field indicates "64-bit string", then the HPLMN ID field is present otherwise the HPLMN ID field is absent. The HPLMN ID field indicates HPLMN ID of the 5G ProSe remote UE and is coded as value part of the PLMN ID information element as specified in 3GPP TS 24.554 [19E] clause 11.3.33 starting with the second octet. |  |  |  |  |  |
| NOTE: In the present release of the specification, providing information for IP protocols other than UDP or TCP is not specified |  |  |  |  |  |

#### 9.11.4.30	Requested MBS container

The purpose of the Requested MBS container information element is for UE to request to join or leave one or more multicast MBS sessions.

The Requested MBS container information element is coded as shown in figure 9.11.4.30.1, figure 9.11.4.30.2, figure 9.11.4.30.3, figure 9.11.4.30.4 and table 9.11.4.30.1.

The Requested MBS container is a type 6 information element with a minimum length of 8 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Requested MBS container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Requested MBS container contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| multicast MBS session information 1 |  |  |  |  |  |  |  | octet 4 octet i |
| multicast MBS session information 2 |  |  |  |  |  |  |  | octet i+1* octet l* |
| … |  |  |  |  |  |  |  | octet l+1* octet m* |
| multicast MBS session information p |  |  |  |  |  |  |  | octet m+1* octet n* |

Figure 9.11.4.30.1: Requested MBS container information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | MBS operation |  | Type of multicast MBS session ID |  | octet 4 |
| spare |  |  |  |  |  |  |  |  |
| multicast MBS session ID |  |  |  |  |  |  |  | octet 5 octet i |

Figure 9.11.4.30.2: multicast MBS session information

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TMGI |  |  |  |  |  |  |  | octet 5 |
|  |  |  |  |  |  |  |  | octet i |

Figure 9.11.4.30.3: multicast MBS session ID for Type of multicast MBS session ID = "Temporary Mobile Group Identity (TMGI)"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source IP address information |  |  |  |  |  |  |  | octet 5 octet v |
| Destination IP address information |  |  |  |  |  |  |  | Octet v+1 Octet i |

Figure 9.11.4.30.4: multicast MBS session ID for Type of multicast MBS session ID = "Source specific IP multicast address for IPv4" or "Source specific IP multicast address for IPv6"

Table 9.11.4.30.1: Requested MBS container information element

| Type of multicast MBS session ID (bits 1 to 2 of octet 4) |  |  |  |
| --- | --- | --- | --- |
| Bits |  |  |  |
| 2 | 1 |  |  |
| 0 | 1 |  | Temporary Mobile Group Identity (TMGI) |
| 1 | 0 |  | Source specific IP multicast address for IPv4 |
| 1 | 1 |  | Source specific IP multicast address for IPv6 |
| All other values are reserved. |  |  |  |
| MBS operation (bits 3 to 4 of octet 4) |  |  |  |
| Bits |  |  |  |
| 4 | 3 |  |  |
| 0 | 1 |  | Join multicast MBS session |
| 1 | 0 |  | Leave multicast MBS session |
| All other values are reserved. |  |  |  |
| Bits 5 to 8 of octet 4 are spare and shall be coded as zero. |  |  |  |
| If Type of multicast MBS session ID is set to "Temporary Mobile Group Identity (TMGI)", the multicast MBS session ID contains the TMGI (octet 5 to i) and is coded as described in clause 10.5.6.13 in 3GPP TS 24.008 [12] starting from octet 2. The structure of the TMGI is defined in 3GPP TS 23.003 [4]. |  |  |  |
| If Type of multicast MBS session ID is set to "Source specific IP multicast address for IPv4" or " Source specific IP multicast address for IPv6", the multicast MBS session ID contains the Source IP address information and the Destination IP address information. |  |  |  |
| Source IP address information (octet 5 to v) |  |  |  |
| This field contains the IP unicast address used as source address in IP packets for identifying the source of the multicast service. |  |  |  |
| If the type of multicast MBS session ID indicates "Source specific IP multicast address for IPv4", the Source IP address information in octet 5 to octet 8 contains an IPv4 address. If the type of multicast MBS session ID indicates "Source specific IP multicast address for IPv6", the Source IP address information in octet 5 to octet 20 contains an IPv6 address. |  |  |  |
| Destination IP address information (octet v+1 to i) |  |  |  |
| This field contains the IP multicast address used as destination address in related IP packets for identifying a multicast service associated with the source. |  |  |  |
| If the type of multicast MBS session ID indicates "Source specific IP multicast address for IPv4", the Destination IP address information in octet v+1 to octet v+4 contains an IPv4 address. If the type of multicast MBS session ID indicates "Source specific IP multicast address for IPv6", the Source IP address information in octet v+1 to octet v+16 contains an IPv6 address. |  |  |  |

#### 9.11.4.31	Received MBS container

The purpose of the Received MBS container information element is to indicate to the UE the information of the multicast MBS sessions that the network accepts or rejects the UE to join, the information of the multicast MBS sessions that the UE is removed from, or the information of the updated MBS service area.

The Received MBS container information element is coded as shown in figure 9.11.4.31.1, figure 9.11.4.31.2, figure 9.11.4.31.3, figure 9.11.4.31.4, figure 9.11.4.31.5, figure 9.11.4.31.6, figure 9.11.4.31.7, figure 9.11.4.31.8, figure 9.11.4.31.9, figure 9.11.4.31.10, figure 9.11.4.31.11 and table 9.11.4.31.1.

The Received MBS container is a type 6 information element with a minimum length of 9 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Received MBS container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Received MBS container contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Received MBS information 1 |  |  |  |  |  |  |  | octet 4 octet e |
| Received MBS information 2 |  |  |  |  |  |  |  | octet (e+1)* octet l* |
| … |  |  |  |  |  |  |  | octet (l+1)* octet m* |
| Received MBS information p |  |  |  |  |  |  |  | octet (m+1)* octet n* |

Figure 9.11.4.31.1: Received MBS container information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rejection cause |  |  | MSAI |  | MD |  |  | octet 4 |
| 0 | 0 | 0 | IPAT | MSCI | MTI |  | IPAE | octet 5 |
| spare |  |  |  |  |  |  |  |  |
| TMGI |  |  |  |  |  |  |  | octet 6 octet j |
| Source IP address information |  |  |  |  |  |  |  | octet (j+1)* octet v* |
| Destination IP address information |  |  |  |  |  |  |  | octet (v+1)* octet k* |
| MBS service area |  |  |  |  |  |  |  | octet (k+1)* octet s* |
| MBS timers |  |  |  |  |  |  |  | octet (s+1)* octet i* |
| MBS security container |  |  |  |  |  |  |  | octet (i+1)* octet e* |

Figure 9.11.4.31.2: Received MBS information

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MBS TAI list |  |  |  |  |  |  |  | Octet (k+1)* octet s* |

Figure 9.11.4.31.3: MBS service area for MBS service area indication = "MBS service area included as MBS TAI list"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CGI list |  |  |  |  |  |  |  | octet (k+1)* octet s* |

Figure 9.11.4.31.4: MBS service area for MBS service area indication = "MBS service area included as NR CGI list"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MBS TAI list |  |  |  |  |  |  |  | octet (k+1)* octet y* |
| NR CGI list |  |  |  |  |  |  |  | octet (y+1)* octet s* |

Figure 9.11.4.31.5: MBS service area for MBS service area indication = "MBS service area included as MBS TAI list and NR CGI list"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NR CGI list contents |  |  |  |  |  |  |  | octet (k+1)* |
| NR CGI 1 |  |  |  |  |  |  |  | octet (k+2)* octet (k+9)* |
| NR CGI 2 |  |  |  |  |  |  |  | octet (k+10)* octet (k+17)* |
| … |  |  |  |  |  |  |  | octet (k+18)* octet c* |
| NR CGI w |  |  |  |  |  |  |  | octet (c+1)* octet s* |

Figure 9.11.4.31.6: NR CGI list

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Cell ID |  |  |  |  |  |  |  | Octet (k+2)* |
|  |  |  |  |  |  |  |  | octet (k+6)* |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet (k+7)* |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet (k+8)* |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet (k+9)* |

Figure 9.11.4.31.7: NR CGI

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MBS start time |  |  |  |  |  |  |  | octet (s+1)* octet (s+6)* |  |

Figure 9.11.4.31.8: MBS timers for MBS timer indication = "MBS start time"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MBS back-off timer |  |  |  |  |  |  |  | octet (s+1)* |  |

Figure 9.11.4.31.9: MBS timers for MBS timer indication = "MBS back-off timer"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of MBS security keys sets |  |  |  |  |  |  |  | Octet (i+1)* |
| MBS security keys set 1 |  |  |  |  |  |  |  | octet (i+2)* octet t* |
| MBS security keys set 2 |  |  |  |  |  |  |  | octet (t+1)* octet g* |
| … |  |  |  |  |  |  |  | octet (g+1)* octet v* |
| MBS security keys set q |  |  |  |  |  |  |  | octet (v+1)* octet e* |

Figure 9.11.4.31.10: MBS security container

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 spare |  |  |  |  |  |  | MTKI | Octet (i+2)* |
| Key domain ID |  |  |  |  |  |  |  | octet (i+3)* octet (i+5)* |
| MSK ID |  |  |  |  |  |  |  | octet (i+6)* octet (i+9)* |
| MSK |  |  |  |  |  |  |  | octet (i+10)* octet (i+25)* |
| MTK ID |  |  |  |  |  |  |  | octet (i+26)* octet (i+27)* |
| Encrypted MTK |  |  |  |  |  |  |  | octet (i+28)* octet (i+43)* |

Figure 9.11.4.31.11: MBS security keys set

Table 9.11.4.31.1: Received MBS container information element

| MBS decision (MD) (bits 1 to 3 of octet 4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The MD indicates the network decision of the join requested by the UE, the network requests to remove the UE from the multicast MBS session or the network request to update the MBS service area or the security information of multicast MBS session. |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 3 | 2 | 1 |  |  |  |
| 0 | 0 | 1 |  | MBS service area update |  |
| 0 | 1 | 0 |  | MBS join is accepted |  |
| 0 | 1 | 1 |  | MBS join is rejected |  |
| 1 | 0 | 0 |  | Remove UE from multicast MBS session |  |
| 1 | 0 | 1 |  | MBS security information update |  |
| All other values are unused in this version of the specification and interpreted as 000 if received. |  |  |  |  |  |
| If MD is set to "MBS join is rejected" or “Remove UE from multicast MBS session”, bits 6 to 8 of octet 4 shall contain the Rejection cause which indicates the reason of rejecting the MBS join request or the reason of removing the UE from multicast MBS session, respectively, otherwise bits 6 to 8 of octet 4 are spare and shall be coded as zero. |  |  |  |  |  |
| MBS service area indication (MSAI) (bits 4 and 5 of octet 4) |  |  |  |  |  |
| The MSAI indicates whether and how the MBS service area is included in the IE. |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 5 | 4 |  |  |  |  |
| 0 | 0 | MBS service area not included |  |  |  |
| 0 | 1 | MBS service area included as MBS TAI list |  |  |  |
| 1 | 0 | MBS service area included as NR CGI list |  |  |  |
| 1 | 1 | MBS service area included as MBS TAI list and NR CGI list |  |  |  |
| Rejection cause (bits 6 to 8 of octet 4) |  |  |  |  |  |
| The Rejection cause indicates the reason of rejecting the join request or the reason of removing the UE from the MBS session. |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 8 | 7 | 6 |  |  |  |
| 0 | 0 | 0 |  |  | No additional information provided |
| 0 | 0 | 1 |  |  | Insufficient resources |
| 0 | 1 | 0 |  |  | User is not authorized to use MBS service |
| 0 | 1 | 1 |  |  | multicast MBS session has not started or will not start soon |
| 1 | 0 | 0 |  |  | User is outside of local MBS service area |
| 1 | 0 | 1 |  |  | Session context not found |
| 1 | 1 | 0 |  |  | multicast MBS session is released |
| All other values are unused in this version of the specification and interpreted as 000 if received. |  |  |  |  |  |
| IP address existence (IPAE) (bit1 of octet 5) |  |  |  |  |  |
| The IPAE indicates whether the Source IP address information and Destination IP address information are included in the IE or not. |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 1 |  |  |  |  |  |
| 0 |  | Source and destination IP address information not included |  |  |  |
| 1 |  | Source and destination IP address information included |  |  |  |
| If IPAE is set to "Source and destination IP address information included", Source IP address information and Destination IP address information shall be included in the IE, otherwise Source IP address information and Destination IP address information shall not be included in the IE (NOTE 1). |  |  |  |  |  |
| MBS timer indication (MTI) (bits 2 and 3 of octet 5) |  |  |  |  |  |
| The MTI indicates whether there is MBS timer included in the IE or not. |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 3 | 2 |  |  |  |  |
| 0 | 0 | No MBS timers included |  |  |  |
| 0 | 1 | MBS start time included |  |  |  |
| 1 | 0 | MBS back-off timer included |  |  |  |
| All other values are unused in this version of the specification and interpreted as 00 if received |  |  |  |  |  |
| MBS security container indication (MSCI) (bit 4 of octet 5) |  |  |  |  |  |
| The MSCI indicates whether the MBS security container is included in the IE or not |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 0 | MBS security container not included |  |  |  |  |
| 1 | MBS security container included |  |  |  |  |
| IP address type (IPAT) (bit 5 of octet 5) |  |  |  |  |  |
| The IPAT indicates the type of the source IP address information and destination IP address information. This field is ignored when IPAE is set to "Source and destination IP address information not included". |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 0 | Source IP address information and destination IP address information are IPv4 |  |  |  |  |
| 1 | Source IP address information and destination IP address information are IPv6 |  |  |  |  |
| Bits 6 to 8 of octet 5 are spare and shall be coded as zero. |  |  |  |  |  |
| TMGI (octets 6 to j) |  |  |  |  |  |
| The TMGI is coded as described in clause 10.5.6.13 in 3GPP TS 24.008 [12] starting from octet 2. The structure of the TMGI is defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |
| Source IP address information (octet j+1 to v) |  |  |  |  |  |
| This field contains the IP unicast address used as source address in IP packets for identifying the source of the multicast service. The value of this field is copied from the corresponding source IP address information in the requested MBS container. If the IPAT indicates "Source and destination IP address information are IPv4", the Source IP address information in octet j+1 to octet j+4 contains an IPv4 address. If the IPAT indicates "Source and destination IP address information are IPv6", the Source IP address information in octet j+1 to octet j+16 contains an IPv6 address |  |  |  |  |  |
| Destination IP address information (octet v+1 to k) |  |  |  |  |  |
| This field contains the IP multicast address used as destination address in related IP packets for identifying a multicast service associated with the source. The value of this field is copied from the corresponding destination IP address information in the requested MBS container. If the IPAT indicates "Source and destination IP address information are IPv4", the Destination IP address information in octet v+1 to octet v+4 contains an IPv4 address. If the IPAT indicates "Source and destination IP address information are IPv6", the Destination IP address information in octet v+1 to octet v+16 contains an IPv6 address. |  |  |  |  |  |
| MBS service area (octet k+1 to s) |  |  |  |  |  |
| The MBS service area contains the MBS TAI list, the NR CGI list or both, that identifies the service area(s) for a local MBS service. |  |  |  |  |  |
| MBS TAI list (octet k+1 to s) |  |  |  |  |  |
| The MBS TAI list is coded as octet 2 and above of the 5GS tracking area identity list IE defined in clause 9.11.3.9. |  |  |  |  |  |
| NR CGI (octet k+2 to k+9) |  |  |  |  |  |
| The NR CGI globally identifies an NR cell. It contains the NR Cell ID and the PLMN ID of that cell. |  |  |  |  |  |
| NR Cell ID (octet k+2 to k+6) |  |  |  |  |  |
| The NR Cell ID consists of 36 bits identifying an NR Cell ID as specified in clause 9.3.1.7 of 3GPP TS 38.413 [31], in hexadecimal representation. Bit 8 of octet k+2 is the most significant bit and bit 5 of octet k+6 is the least significant bit. Bits 1 to 4 of octet k+6 are spare and shall be coded as zero. |  |  |  |  |  |
| MCC, Mobile country code (octet k+7 and bits 1 to 4 octet k+8) The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A. |  |  |  |  |  |
| MNC, Mobile network code (bits 5 to 8 of octet k+8 and octet k+9) The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet k+8 shall be coded as "1111". |  |  |  |  |  |
| The MCC and MNC digits are coded as octets 6 to 8 of the Temporary mobile group identity IE in figure 10.5.154 of 3GPP TS 24.008 [12]. |  |  |  |  |  |
| MBS start time (octets s+1 to s+6) |  |  |  |  |  |
| The MBS start time is coded as described in clause 10.5.3.9 in 3GPP TS 24.008 [12] starting from octet 2 till octet 7. |  |  |  |  |  |
| MBS back-off timer (octet s+1) |  |  |  |  |  |
| The MBS back-off timer is coded as octet 3 described in clause 10.5.7.4a in 3GPP TS 24.008 [12]. |  |  |  |  |  |
| MTK indication (MTKI) (bit1 of octet i+2) |  |  |  |  |  |
| The MTKI indicates whether the MTK ID and Encrypted MTK are included in the MBS security keys set or not. |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 1 |  |  |  |  |  |
| 0 | MTK ID and Encrypted MTK not included |  |  |  |  |
| 1 | MTK ID and Encrypted MTK included |  |  |  |  |
| Bits 2 to 8 of octet i+2 are spare and shall be coded as zero |  |  |  |  |  |
| Key domain ID (octet i+3 to i+5) The key domain ID is 3 bytes long and is defined in 3GPP TS 33.246 [57] (NOTE 2). |  |  |  |  |  |
| MBS Service Key Identifier (MSK ID) (octets i+6 to i+9) |  |  |  |  |  |
| The MSK ID is 4 bytes long and is defined in 3GPP TS 33.246 [57]. |  |  |  |  |  |
| MBS Service Key (MSK) (octets i+10 to i+25) |  |  |  |  |  |
| The MSK is 16 bytes long and is defined in 3GPP TS 33.246 [57]. |  |  |  |  |  |
| MBS Traffic Key Identifier (MTK ID) (octets i+26 to i+27) |  |  |  |  |  |
| The MTK ID is 2 bytes long and is defined in 3GPP TS 33.246 [57]. |  |  |  |  |  |
| Encrypted MBS Traffic Key (Encrypted MTK) (octets i+28 to i+43) |  |  |  |  |  |
| The Encrypted MTK is 16 bytes long and contains the encrypted version of MTK using MSK as defined in 3GPP TS 33.246 [57]. |  |  |  |  |  |
| NOTE 1: The IPAE bit is not expected to be set to "Source and destination IP address information included" when the MBS decision (MD) indicates "Remove UE from multicast MBS session". NOTE 2: As specified in annex W in 3GPP TS 33.501 [24], the UE should not try to use the MCC and MNC constructing the key domain ID in another context, e.g., the UE should not compare those MCC and MNC to parameters received from lower layers. |  |  |  |  |  |

#### 9.11.4.32	PDU session pair ID

The purpose of the PDU session pair ID information element is to indicate a PDU session pair ID.

The PDU session pair ID information element is coded as shown in figure 9.11.4.32.1 and table 9.11.4.32.1.

The PDU session pair ID is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU session pair ID IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PDU session pair ID IE |  |  |  |  |  |  |  | octet 2 |
| PDU session pair ID |  |  |  |  |  |  |  | octet 3 |

Figure 9.11.4.32.1: PDU session pair ID information element

Table 9.11.4.32.1: PDU session pair ID information element

| PDU session pair ID (octet 3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | PDU session pair ID 0 |
| to |  |  |  |  |  |  |  |  | to |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | PDU session pair ID 6 |
| All other values are reserved. |  |  |  |  |  |  |  |  |  |

#### 9.11.4.33	RSN

The purpose of the RSN information element is to indicate an RSN.

The RSN information element is coded as shown in figure 9.11.4.33.1 and table 9.11.4.33.1.

The RSN is a type 4 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSN IEI |  |  |  |  |  |  |  | octet 1 |
| Length of RSN IE |  |  |  |  |  |  |  | octet 2 |
| RSN |  |  |  |  |  |  |  | octet 3 |

Figure 9.11.4.33.1: RSN information element

Table 9.11.4.33.1: RSN information element

| RSN (octet 3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | v1 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | v2 |
| All other values are spare and shall not be used by a UE compliant to the present version of this specification. |  |  |  |  |  |  |  |  |  |

#### 9.11.4.34	ECS address

The purpose of the ECS address information element is to indicate the ECS address (either IPv4 address, IPv6 address, or FQDN) and the associated spatial validity condition.

The ECS address information element is coded as shown in figure 9.11.4.34.1, figure 9.11.4.34.2, figure 9.11.4.34.3, figure 9.11.4.34.4, figure 9.11.4.34.5, figure 9.11.4.34.6, table 9.11.4.34.1, table 9.11.4.34.2, table 9.11.4.34.4 and table 9.11.4.34.5.

The ECS address information element is a type 6 information element with minimum length of 8 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 |  | 5 |  |  | 4 |  | 3 |  | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ECS address IEI |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of ECS address contents |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 octet 3 |
| Type of ECS address |  |  |  |  |  |  | Type of spatial validity condition |  |  |  |  |  |  | octet 4 |
| ECS address |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 5 octet a |
| Spatial validity condition contents |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (a+1)* octet n* |
| 0 Spare | 0 Spare |  | 0 Spare |  | 0 Spare | 0 Spare |  | 0 Spare |  | ESPILI |  | EAMI |  | Octet m* |
| ECS authentication methods |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o* |
| ECS supported PLMNs information list |  |  |  |  |  |  |  |  |  |  |  |  |  | octet p* octet w* |

Figure 9.11.4.34.1: ECS address information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of spatial validity condition contents |  |  |  |  |  |  |  | octet (a+1) octet (a+2) |
| Spatial validity information 1 |  |  |  |  |  |  |  | octet b octet c |
| Spatial validity information 2 |  |  |  |  |  |  |  | octet (c+1)* octet d* |
| … |  |  |  |  |  |  |  | octet (d+1)* octet e* |
| Spatial validity information N |  |  |  |  |  |  |  | octet (e+1)* octet n* |

Figure 9.11.4.34.2: Spatial validity condition contents

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | TLSGI | TLSAI | TLSCSCI | octet o* |

Figure 9.11.4.34.3: ECS authentication methods

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ECS supported PLMNs information list contents |  |  |  |  |  |  |  | octet p octet (p+2) |
| Supported PLMN information 1 |  |  |  |  |  |  |  | octet (p+3) octet q |
| Supported PLMN information 2 |  |  |  |  |  |  |  | octet (q+1)* octet u* |
| … |  |  |  |  |  |  |  | octet (u+1)* octet v* |
| Supported PLMN information N |  |  |  |  |  |  |  | octet (v+1)* octet w* |

Figure 9.11.4.34.4: ECS supported PLMNs information list contents

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Supported PLMN information contents |  |  |  |  |  |  |  | octet (q+1) octet (q+3) |
| PLMN identity |  |  |  |  |  |  |  | octet (q+4) octet (q+6) |
| ECSP information 1 |  |  |  |  |  |  |  | octet (q+7)* octet r* |
| ECSP information 2 |  |  |  |  |  |  |  | octet (r+1)* octet s* |
| … |  |  |  |  |  |  |  | octet (s+1)* octet t* |
| ECSP information N |  |  |  |  |  |  |  | octet (t+1)* octet u* |

Figure 9.11.4.34.5: Supported PLMN information contents

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ECSP identifier contents |  |  |  |  |  |  |  | octet (r+1) octet (r+2) |
| ECSP identifier |  |  |  |  |  |  |  | octet (r+3)* octet s* |

Figure 9.11.4.34.6: ECSP information contents

Table 9.11.4.34.1: ECS address information element

| Type of ECS address (octet 4, bit 1 to 4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |
| 4 |  | 3 | 2 | 1 |  |
| 0 |  | 0 | 0 | 0 | IPv4 |
| 0 |  | 0 | 0 | 1 | IPv6 |
| 0 |  | 0 | 1 | 0 | FQDN |
| 1 |  | 1 | 1 | 1 | Unspecified |
| All other values are spare. The receiving entity shall ignore an ECS address IE with type of ECS address containing a spare value. |  |  |  |  |  |
| Type of spatial validity condition (octet 4, bit 5 to 8) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 8 |  | 7 | 6 | 5 |  |
| 0 |  | 0 | 0 | 0 | No spatial validity condition |
| 0 |  | 0 | 0 | 1 | Geographical service area |
| 0 |  | 0 | 1 | 0 | Tracking area |
| 0 |  | 0 | 1 | 1 | Country-wide |
| All other values are spare. The receiving entity shall ignore a spatial validity condition with type of spatial validity condition containing an unknown value. |  |  |  |  |  |
| If the type of ECS address indicates IPv4, then the ECS address field contains an IPv4 address in octet 5 to octet 8. |  |  |  |  |  |
| If the type of ECS address indicates IPv6, then the ECS address field contains an IPv6 address in octet 5 to octet 20 and is encoded according to IETF RFC 4291 [66]. |  |  |  |  |  |
| If the type of ECS address indicates FQDN, then the ECS address field contains in octet 5 the length of FQDN value and in octet 6 to octet a an FQDN value encoded as defined in clause 19.4.2 in 3GPP TS 23.003 [4]. |  |  |  |  |  |
| If the type of ECS address indicates unspecified, then the remaining fields of ECS address information element shall be passed to the upper layers. |  |  |  |  |  |
| Spatial validity condition contents (octet (a+1)* to n*) |  |  |  |  |  |
| The spatial validity condition contents contain a variable number of spatial validity condition information. |  |  |  |  |  |
| ECS authentication methods indicator (EAMI) (octet m*, bit 1) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 1 |  |  |  |  |  |
| 0 |  |  |  |  | ECS authentication methods field is not included |
| 1 |  |  |  |  | ECS authentication methods field is included |
| If the EAMI bit is set to "ECS authentication methods field is included" then the ECS authentication methods field is included otherwise the ECS authentication methods field is not included. ECS authentication methods is an optional field and is included based on operator requirements. |  |  |  |  |  |
| ECS supported PLMNs information list indication (ESPILI) (octet m*, bit 2) Bits 2. |
| 0 | ECS supported PLMNs information list field is not included |  |  |  |  |
| 1 | ECS supported PLMNs information list field is included |  |  |  |  |
| If the ESPILI bit is set to "ECS supported PLMNs information list field is included" then the ECS supported PLMNs information list field is included otherwise the ECS supported PLMNS information list field is not included. ECS supported PLMNs information list (octet p* to w*) The ECS supported PLMNs information list contains a variable number of supported PLMN information field. The supported PLMN information field is coded according to figure 9.11.4.34.5. |  |  |  |  |  |

Table 9.11.4.34.2: Spatial validity condition contents

If the type of spatial validity condition of the ECS address indicates No spatial validity condition, then the spatial validity condition information field is empty.

If the type of spatial validity condition of the ECS address indicates geographical service area, then the spatial validity condition information field contains a geographical service area which is specified by geographical descriptions as defined in 3GPP TS 23.032 [4B].

If the type of spatial validity condition of the ECS address indicates tracking area, then the spatial validity condition information field contains a TAI as defined in clause 9.11.3.8 starting from octet 2.

If the type of spatial validity condition of the ECS address indicates country-wide, then the spatial validity condition information field contains an MCC as defined in in ITU-T Recommendation E.212 [42], annex A. The first MCC digit is coded in bit 1 to 4 of the octet b, the second MCC digit is coded in bit 5 to 8 of the octet b, and the third MCC digit is coded in bit 1 to 4 of the octet b+1. Bit 5 to bit 8 of the octet b+1 shall be padded with 1. If only two digits are used for for MCC, octet b+1 shall be padded with 1.

Table 9.11.4.34.3: ECS authentication methods contents

| ECS authentication methods (octet o*) |  |  |  |  |
| --- | --- | --- | --- | --- |
| TLS client server certificate indicator (TLSCSCI) (octet o*, bit 1) |  |  |  |  |
| 0 |  |  |  | TLS client server certificate not supported |
| 1 |  |  |  | TLS client server certificate supported |
| TLS with AKMA indicator (TLSAI) (octet o*, bit 2) |  |  |  |  |
| 0 |  |  |  | TLS with AKMA not supported |
| 1 |  |  |  | TLS with AKMA supported |
| TLS with GBA indicator (TLSGI) (octet o*, bit 3) |  |  |  |  |
| 0 |  |  |  | TLS with GBA not supported |
| 1 |  |  |  | TLS with GBA supported |

Table 9.11.4.34.4: Supported PLMN information contents

PLMN identity: The PLMN identity for which the EDN configuration information can be provided by the ECS. This is encoded as octet 3 to octet 5 in figure 9.11.3.85.1.

ECSP information: The identifier of the ECSP(s) associated with the PLMN and whose information is available at the ECS. The ECSP information is coded according to figure 9.11.4.34.6 and table 9.11.4.34.5.

Table 9.11.4.34.5: ECSP information contents

ECSP identifier This field contains one ECSP identifier. The ECSP identifier is encoded as a UTF-8 string.

#### 9.11.4.35	Void

#### 9.11.4.36	N3QAI

The purpose of the N3QAI information element is to indicate a set of QoS parameters to be used by the UE for non-3GPP access network resource management behind the UE.

The N3QAI information element is a type 6 information element with a minimum length of 9 octets. The maximum length for the information element is 65538 octets.

The N3QAI information element is coded as shown in figure 9.11.4.36.1, figure 9.11.4.36.2, figure 9.11.4.36.3, figure 9.11.4.36.4, and table 9.11.4.36.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N3QAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of N3QAI contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| N3QAI 1 |  |  |  |  |  |  |  | octet 4 octet u* |
| N3QAI 2 |  |  |  |  |  |  |  | octet (u+1)* octet v* |
| ... |  |  |  |  |  |  |  | octet (v+1)* octet w* |
| N3QAI n |  |  |  |  |  |  |  | octet (w+1)* octet x* |

Figure 9.11.4.36.1: N3QAI information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of QFIs |  |  |  |  |  |  |  | octet 4 |
| List of QFIs |  |  |  |  |  |  |  | octet 5 octet m |
| Number of N3QAI parameters |  |  |  |  |  |  |  | octet (m+1) |
| N3QAI parameters list |  |  |  |  |  |  |  | octet (m+2) octet u* |

Figure 9.11.4.36.2: N3QAI

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N3QAI parameter 1 |  |  |  |  |  |  |  | octet (m+2) octet n* |
| N3QAI parameter 2 |  |  |  |  |  |  |  | octet (n+1)* octet o* |
| ... |  |  |  |  |  |  |  | octet (o+1)* octet p* |
| N3QAI parameter n |  |  |  |  |  |  |  | octet (p+1)* octet u* |

Figure 9.11.4.36.3: N3QAI parameters list

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N3QAI parameter identifier |  |  |  |  |  |  |  | octet (m+2) |
| Length of N3QAI parameter contents |  |  |  |  |  |  |  | octet (m+3) |
| N3QAI parameter contents |  |  |  |  |  |  |  | octet (m+4) octet n* |

Figure 9.11.4.36.4: N3QAI parameter

Table 9.11.4.36.1: N3QAI information element

Number of QFIs (octet 4) The number of QFIs field contains the binary coding for the number of QFIs associated with the same N3QAI parameters. This field is encoded in bits 8 through 1 of octet 4 where bit 8 is he most significant and bit 1 is the least significant bit. List of QFIs (octet 5 to octet m) This field indicates QoS flow(s) associated with the same N3QAI parameters. This field contains QFI values encoded as below: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 QFI 1 to 0 0 1 1 1 1 1 1 QFI 63 The other values are spare. If spare value is used, the UE shall ignore the value.

Number of N3QAI parameters (bits 8 to 1 of octet (m+1)) The number of N3QAI parameters field contains the binary coding for the number of N3QAI parameters in the N3QAI parameters list field. The number of N3QAI parameters field is encoded in bits 8 through 1 of octet (m+1) where bit 8 is the most significant and bit 1 is the least significant bit.

N3QAI parameters list (octets (m+2) to q*) The N3QAI parameters list field contains a variable number of N3QAI parameters. Each N3QAI parameter included in the N3QAI parameters list is of variable length and consists of: - a N3QAI parameter identifier (1 octet); - the length of the N3QAI parameter contents (1 octet); and - the N3QAI parameter contents itself (variable number of octets). The N3QAI parameter identifier field is used to identify each parameter included in the N3QAI parameters list and it contains the hexadecimal coding of the parameter identifier. Bit 8 of the parameter identifier field contains the most significant bit and bit 1 contains the least significant bit. In this version of the protocol, the following parameter identifiers are specified: - 01H (5QI); - 02H (GFBR uplink); - 03H (GFBR downlink); - 04H (MFBR uplink); - 05H (MFBR downlink); - 06H (Averaging window); - 07H (Resource type); - 08H (Priority level); - 09H (Packet delay budget); - 0AH (Packet error rate); - 0BH (Maximum data burst volume); - 0CH (Maximum packet loss rate downlink); - 0DH (Maximum packet loss rate uplink); - 0EH (ARP); and - 0FH (Periodicity). If the N3QAI parameters list contains a N3QAI parameter identifier that is not supported by the receiving entity, the corresponding parameter shall be discarded. The length of N3QAI parameter contents field contains the binary coded representation of the length of the parameter contents field. The first bit in transmission order is the most significant bit. For the N3QAI parameter identifiers indicating "5QI", "GFBR uplink", "GFBR downlink", "MFBR uplink", "MFBR downlink", and "Averaging window", the format of the N3QAI parameter contents follows the table 9.11.4.12.1 of clause 9.11.4.12 of this specification. For the N3QAI parameter identifiers indicating "Resource type", "Priority level", "Packet delay budget", "Packet error rate", "Maximum data burst volume", "Maximum packet loss rate downlink", and "Maximum packet loss rate uplink", the format of the N3QAI parameter contents follows the table 9.3.1.1-2 of clause 9.3.1.1 of 3GPP TS 24.502 [18]. When the N3QAI parameter identifier indicates "ARP", the N3QAI parameter contents field contains the binary representation of ARP that is one octet in length. The range of the ARP priority level is 1 to 15 with 1 as the highest priority as specified in clause 5.7.2.2 of 3GPP TS.23.501 [8]. When the N3QAI parameter identifier indicates "Periodicity", the N3QAI parameter contents field contains the binary representation of the periodicity for the traffic with a unit of microsecond. (NOTE)

NOTE: The periodicity refers to the time interval between start of two data bursts for supporting consumer real time applications e.g., XR.

#### 9.11.4.37	Non-3GPP delay budget

The purpose of the Non-3GPP delay budget information element is to indicate the non-3GPP delay budget for the non-3GPP network behind the UE to the network.

The Non-3GPP delay budget information element is a type 6 information element with a minimum length of 8 octets. The maximum length for the information element is 65538 octets.

The Non-3GPP delay budget information element is coded as shown in figure 9.11.4.37.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-3GPP delay budget IEI |  |  |  |  |  |  |  | octet 1 |
| Length of non-3GPP delay budget contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| non-3GPP delay budget 1 |  |  |  |  |  |  |  | octet 4 octet u* |
| non-3GPP delay budget 2 |  |  |  |  |  |  |  | octet u+1* octet v* |
| ... |  |  |  |  |  |  |  | octet v+1* octet w* |
| non-3GPP delay budget n |  |  |  |  |  |  |  | octet w+1* octet x* |

Figure 9.11.4.37.1: Non-3GPP delay budget information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-3GPP delay budget value |  |  |  |  |  |  |  | octet 4 octet 5 |
| 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | QFIPI | PFPI | octet 6 |
| Number of QFIs |  |  |  |  |  |  |  | octet n* |
| List of QFIs |  |  |  |  |  |  |  | octet (n+1)* octet m* |
| Packet filter list |  |  |  |  |  |  |  | octet o* octet u* |

Figure 9.11.4.37.2: Non-3GPP delay budget

Table 9.11.4.37.1: Non-3GPP delay budget information element

| The Non-3GPP delay budget value field contains the binary representation of the Non-3gpp delay budget in units of 0.5ms. Bits 8 7 6 5 4 3 2 1 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 thru 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 |  |
| --- | --- |
| Packet filter presence indicator (PFPI) (bit 1 of octet 6) Bit 1 |  |
| 0 | Packet filter list associated with the Non-3GPP delay budget value is not present |
| 1 | Packet filter list associated with the Non-3GPP delay budget value is present |
| QoS flow identifier presence indicator (QFIPI) (bit 2 of octet 6) Bit 2 |  |
| 0 | QoS flow identifier associated with the Non-3GPP delay budget value is not present |
| 1 | QoS flow identifier associated with the Non-3GPP delay budget value is present |
| Number of QFIs (octet n*) The number of QFIs field is present if QFIPI is set to 1. If QFIPI is not set to 1, this field shall not be included in the non-3GPP delay budget. The number of QFIs field contains the binary coding for the number of QFIs associated with the same non-3GPP delay budget value. This field is encoded in bits 8 through 1 of octet 4 where bit 8 is he most significant and bit 1 is the least significant bit. |  |
| List of QFIs (octet (n+1)* to octet m*) The list of QFIs field is present if QFIPI is set to 1. If QFIPI is not set to 1, this field shall not be included in the non-3GPP delay budget. This field indicates QoS flow(s) associated with the same non-3GPP delay budget value. This field contains QFI values encoded as below Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 QFI 1 to 0 0 1 1 1 1 1 1 QFI 63 The other values are spare. If spare value is used, the UE shall ignore the value. |  |
| Packet filter list (octet 7 to u) The packet filter list is present if PFPI is set to 1. If not present, this field shall not be included in the non-3GPP delay budget. The encoding of the packet filter list follows the figure 9.11.4.13.4 and the table 9.11.4.13.1. |  |

#### 9.11.4.38	URSP rule enforcement reports

The purpose of the URSP rule enforcement reports information element is to provide one or more URSP rule enforcement reports to the network. Each URSP rule enforcement report includes all the connection capabilities contained in the traffic descriptor of each URSP rule associated to the PDU session.

The URSP rule enforcement reports information element is coded as shown in figure 9.11.4.38.1, figure 9.11.4.38.2, and table 9.11.4.38.1.

The URSP rule enforcement reports is a type 4 information element with a minimum length of 4 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| URSP rule enforcement reports IEI |  |  |  |  |  |  |  | octet 1 |
| Length of URSP rule enforcement reports contents |  |  |  |  |  |  |  | octet 2 |
| URSP rule enforcement report 1 |  |  |  |  |  |  |  | octet 3 octet a* |
| URSP rule enforcement report 2 |  |  |  |  |  |  |  | octet (a+1)* octet b* |
| … |  |  |  |  |  |  |  | octet (b+1)* octet g* |
| URSP rule enforcement report n |  |  |  |  |  |  |  | octet (g+1)* octet h* |

Figure 9.11.4.38.1: URSP rule enforcement reports information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of connection capability identifiers |  |  |  |  |  |  |  | octet 3 |
| Connection capability identifier 1 |  |  |  |  |  |  |  | octet 4 |
| Connection capability identifier 2 |  |  |  |  |  |  |  | octet 5* |
| … |  |  |  |  |  |  |  |  |
| Connection capability identifier n |  |  |  |  |  |  |  | octet a* |

Figure 9.11.4.38.2: URSP rule enforcement report

Table 9.11.4.38.1: URSP rule enforcement reports information element

URSP rule enforcement report (octet 3 to octet a)

The URSP rule enforcement report field contains all the connection capabilities contained in the traffic descriptor of one reported URSP rule.

Number of connection capability identifiers (octet 3)

The number of connection capability identifiers field indicates number of indicated connection capability identifiers in binary representation. The value of this field shall be set to at least one, and the receiving entity shall ignore the URSP rule enforcement reports IE with "Number of connection capability identifiers" field set to zero.

Connection capability identifier

Connection capability identifier is encoded as defined in 3GPP TS 24.526 [19] table 5.2.1.

#### 9.11.4.39	Protocol description

The purpose of the Protocol description information element is to provide protocol description for UL PDU set based handling to the UE.

The Protocol description information element is a type 6 information element with a minimum length of 6 octets.

The Protocol description information element is coded as shown in figure 9.11.4.39.1, figure 9.11.4.39.2, figure 9.11.4.39.3, figure 9.11.4.39.4, and table 9.11.4.39.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protocol description IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Protocol description contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Protocol description 1 |  |  |  |  |  |  |  | octet 4 octet u |
| Protocol description 2 |  |  |  |  |  |  |  | octet (u+1)* octet w* |
| ... |  |  |  |  |  |  |  |  |
| Protocol description n |  |  |  |  |  |  |  | octet (w+1)* octet s* |

Figure 9.11.4.39.1: Protocol description information element

| 8 | 7 |  | 6 |  | 5 |  | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of protocol description |  |  |  |  |  |  |  |  |  |  | octet 4 octet 5 |
| QoS Rule Identifier (QRI) |  |  |  |  |  |  |  |  |  |  | octet 6 |
| 0 |  | 0 | PILPI | HEIPI |  | Transport Protocol |  |  |  |  | octet 7* |
| Spare |  | Spare |  |  |  |  |  |  |  |  |  |
| RTP header extension type |  |  |  |  |  |  |  |  |  |  | octet 8* |
| RTP header extension id |  |  |  |  |  |  |  |  |  |  | octet 9* |
| RTP payload information list |  |  |  |  |  |  |  |  |  |  | octet 10* octet u* |

Figure 9.11.4.39.2: Protocol description

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of RTP payload information list |  |  |  |  |  |  |  | octet 10 octet 11 |
| RTP payload information 1 |  |  |  |  |  |  |  | octet 12 octet o |
| RTP payload information 2 |  |  |  |  |  |  |  | octet (o+1)* octet p* |
| ... |  |  |  |  |  |  |  |  |
| RTP payload information m |  |  |  |  |  |  |  | octet q* octet r* |

Figure 9.11.4.39.3: RTP payload information list

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RTP payload format |  |  |  |  |  |  |  | octet 12 |
| Number of RTP payload type |  |  |  |  |  |  |  | octet 13 |
| RTP payload type 1 |  |  |  |  |  |  |  | octet 14 |
| RTP payload type 2 |  |  |  |  |  |  |  | octet 15* |
| ... |  |  |  |  |  |  |  |  |
| RTP payload type t |  |  |  |  |  |  |  | octet (13+t)* |

Figure 9.11.4.39.4: RTP payload information

Table 9.11.4.39.1: Protocol description information element

|  | Length of protocol description (octet 4 and octet 5) (see NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | The length of protocol description field indicates the length of the protocol description entry. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QoS Rule Identifier (QRI) (octet 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The QoS Rule Identifier (QRI) field contains the QoS rule identifier as specified in clause 9.11.4.13. Each protocol description entry is associated with the QoS rule identified by the QRI field. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Transport Protocol (octet 7, bits 1 to 4) The Transport Protocol field indicates the transport protocol used by the media flow, e.g., RTP or SRTP as specified in 3GPP TS 26.522 [69]. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  | 3 |  | 2 | 1 |  |  |  |  |  |  |  |
|  | 0 |  |  | 0 |  | 0 | 1 | RTP |  |  |  |  |  |  |
|  | 0 |  |  | 0 |  | 1 | 0 | SRTP |  |  |  |  |  |  |
|  | All other values are spare. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If Transport Protocol field indicates a spare value, the corresponding entry for the protocol description shall be ignored. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RTP header extension information presence indicator (HEIPI) (bit 5 of octet 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The HEIPI field indicates whether the RTP header extension information (RTP header extension type field and RTP header extension id field) is included in the IE or not. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  | RTP header extension information not included |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  | RTP header extension information included |  |  |  |  |  |  |  |  |  |
|  | RTP payload information list presence indicator (PILPI) (bit 6 of octet 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The PILPI field indicates whether the RTP payload information list is included in the IE or not. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  | RTP payload information list not included |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  | RTP payload information list included |  |  |  |  |  |  |  |  |
|  | RTP header extension type (octet 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The RTP header extension type field contains the RTP header extension type, i.e the RTP Header Extension for PDU Set Marking as specified in clause 4.2 of 3GPP TS 26.522 [69]. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
|  | 0 |  |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 1 | RTP Header Extension for PDU Set Marking |  |  |
|  | All other values are spare. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RTP header extension id (octet 9) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | If RTP header extension type has a value other than spare values, the RTP header extension id field contains the RTP header extension id which is coded as binary representation of an integer between 1(inclusive) and 255(inclusive) as defined in IETF RFC 8285 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RTP payload information list (octets 10 to u) (see NOTE 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The RTP payload information list contains the RTP payload information for the RTP stream, which can be used to derive the PDU set information. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RTP payload format (octet 12) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
|  | 0 |  |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 1 | RTP payload format for H.264/AVC codec as specified in clause A.2.2 of 3GPP TS 26.522 [69] |  |  |
|  | 0 |  |  | 0 |  | 0 | 0 | 0 | 0 | 1 | 0 | RTP payload format for H.265/HEVC codec as specified in clause A.2.2 of 3GPP TS 26.522 [69] |  |  |
|  | All other values are spare. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | If the RTP payload format indicates a spare value, the corresponding entry for the RTP payload information shall be ignored. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RTP payload type (octet 14) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The RTP payload type field indicates the RTP or SRTP payload type, it contains the binary representation of an integer between 0(inclusive) and 127(inclusive). The other values are spare. If spare value is used, the UE shall ignore the value. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: If the value of the length of protocol description field is set to 1, the corresponding entry for the protocol description shall be deleted for the associated QoS rule. If the value of the length of protocol description field is greater than 1, the corresponding entry for the protocol description shall be added or replaced for the associated QoS rule. NOTE 2: In this release of the specification, the RTP payload information list shall contain only one RTP payload information entry. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 9.11.4.40	ECN marking for L4S indication

The purpose of the ECN marking for L4S indication information element is to provide ECN marking for L4S indication to the 5G-RG.

The ECN marking for L4S indication information element is a type 4 information element with a minimum length of 2 octets.

The ECN marking for L4S indication information element is coded as shown in figure 9.11.4.40.1 and table 9.11.4.40.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ECN marking for L4S indication IEI |  |  |  |  |  |  |  | octet 1 |
| Length of ECN marking for L4S indication contents |  |  |  |  |  |  |  | octet 2 |
| ECN marking for L4S indication entry 1 |  |  |  |  |  |  |  | octet 3* |
| ECN marking for L4S indication entry 2 |  |  |  |  |  |  |  | octet 4* |
| ... |  |  |  |  |  |  |  | ... |
| ECN marking for L4S indication entry n |  |  |  |  |  |  |  | octet n* |

Figure 9.11.4.40.1: ECN marking for L4S indication information element

Table 9.11.4.40.1: ECN marking for L4S indication information element

Length of ECN marking for L4S indication contents (octet 2)

The length of ECN marking for L4S indication contents field indicates the length of the list of the ECN marking for L4S indication entries.

ECN marking for L4S indication entry (octet 3)

The ECN marking for L4S indication entry only contains the QRI required to perform the ECN marking for L4S for 5G-RG. The QRI contains the QoS rule identifier as specified in clause 9.11.4.13.

#### 9.11.4.41	Non-3GPP device information

The purpose of the Non-3GPP device information information element is to provide one or more non-3GPP device identifiers for which QoS differentiation of traffic is requested or for which suspend of QoS differentiation of traffic is requested.

The Non-3GPP device information information element is coded as shown in figure 9.11.4.41.1, figure 9.11.4.41.2, figure 9.11.4.41.3, figure 9.11.4.41.4, figure 9.11.4.41.5, figure 9.11.4.41.6, and table 9.11.4.41.1.

The Non-3GPP device information information element is a type 6 information element with minimum length of 7 octets.

| 8 | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-3GPP device information IEI |  |  |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of non-3GPP device information contents |  |  |  |  |  |  |  |  |  |  |  | octet 2 octet 3 |
| 0 Spare |  | 0 Spare | 0 Spare | 0 Spare |  | 0 Spare |  | PDU session type value |  |  |  | octet 4 |
| Non-3GPP device information 1 |  |  |  |  |  |  |  |  |  |  |  | octet 5 octet u |
| Non-3GPP device information 2 |  |  |  |  |  |  |  |  |  |  |  | octet (u+1)* octet w* |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |
| Non-3GPP device information n |  |  |  |  |  |  |  |  |  |  |  | octet (w+1)* octet m* |

Figure 9.11.4.41.1: Non-3GPP device information information element

| 8 | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of non-3GPP device information |  |  |  |  |  |  |  |  | octet 5 |
| 0 Spare |  | Length of non-3GPP device identifier |  |  |  |  |  |  | octet 6 |
| Non-3GPP device identifier |  |  |  |  |  |  |  |  | octet 7 octet s |
| Connection information |  |  |  |  |  |  |  |  | octet (s+1)* octet u* |

Figure 9.11.4.41.2: Non-3GPP device information

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | I4PRPI | I4API | octet (s+1) |
| Spare | Spare | Spare | Spare | Spare | Spare |  |  |  |
| IPv4 address |  |  |  |  |  |  |  | octet (s+2)* octet (s+5)* |
| Number of port ranges for IPv4 |  |  |  |  |  |  |  | octet (s+6)* |
| IPv4 port range 1 |  |  |  |  |  |  |  | octet (s+7)* octet (s+10)* |
| … |  |  |  |  |  |  |  |  |
| IPv4 port range x |  |  |  |  |  |  |  | octet v* octet (v+3)* |

Figure 9.11.4.41.3: Connection information when PDU session type value = "IPv4"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 Spare | I6PRPI | I6ATI | octet (s+1) |
| Spare | Spare | Spare | Spare | Spare |  |  |  |  |
| IPv6 address |  |  |  |  |  |  |  | octet (s+2)* octet (s+17)* |
| IPv6 prefix |  |  |  |  |  |  |  | octet (s+2)* octet (s+18)* |
| Number of port ranges for IPv6 |  |  |  |  |  |  |  | octet a* |
| IPv6 port range 1 |  |  |  |  |  |  |  | octet (a+1)* octet (a+4)* |
| … |  |  |  |  |  |  |  |  |
| IPv6 port range y |  |  |  |  |  |  |  | octet b* octet (b+3)* |

Figure 9.11.4.41.4: Connection information when PDU session type value = "IPv6"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 Spare | I6PRPI | I6APPI |  | I4PRPI | I4API |  | octet (s+1) |
| Spare |  |  |  |  |  |  |  |  |
| IPv4 address |  |  |  |  |  |  |  | octet (s+2)* octet (s+5)* |
| Number of port ranges for IPv4 |  |  |  |  |  |  |  | octet c* |
| IPv4 port range 1 |  |  |  |  |  |  |  | octet (c+1)* octet (c+4)* |
| … |  |  |  |  |  |  |  |  |
| IPv4 port range x |  |  |  |  |  |  |  | octet d* octet (d+3)* |
| IPv6 address |  |  |  |  |  |  |  | octet e* octet (e+15)* |
| IPv6 prefix |  |  |  |  |  |  |  | octet e* octet (e+16)* |
| Number of port ranges for IPv6 |  |  |  |  |  |  |  | octet f* |
| IPv6 port range 1 |  |  |  |  |  |  |  | octet (f+1)* octet (f+4)* |
| … |  |  |  |  |  |  |  |  |
| IPv6 port range y |  |  |  |  |  |  |  | octet g* octet (g+3)* |

Figure 9.11.4.41.5: Connection information when PDU session type value = "IPv4v6"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | VTPI | octet s+1 |
| Spare | Spare | Spare | Spare | Spare | Spare | Spare |  |  |
| MAC address |  |  |  |  |  |  |  | octet s+2 octet s+7 |
| VLAN tag ID |  |  |  |  |  |  |  | octet (s+8)* octet (s+9)* |

Figure 9.11.4.41.6: Connection information when PDU session type value = "Ethernet"

Table 9.11.4.41.1: Non-3GPP device information information element

| PDU session type value (octet 4, bits 1 to 3) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |
| 3 | 2 |  | 1 |  |  |  |
| 0 | 0 |  | 1 |  |  | IPv4 |
| 0 | 1 |  | 0 |  |  | IPv6 |
| 0 | 1 |  | 1 |  |  | IPv4v6 |
| 1 | 0 |  | 1 |  |  | Ethernet |
| All other values are reserved. |  |  |  |  |  |  |
| Non-3GPP device identifier (octet 7 to octet s) |  |  |  |  |  |  |
| The non-3GPP device identifier field contains the non-3GPP device identifier associated with a non-3GPP device connecting through the UE. The structure of the non-3GPP device identifier is defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |
| Connection information (octet s+1 to octet u) |  |  |  |  |  |  |
| The connection information field contains the connection information details for QoS differentiation of the non-3GPP device identifier indicated according to figures 9.11.4.3, 9.11.4.4, 9.11.4.5 and 9.11.4.6, depending on the PDU session type. If no connection information field is included for the non-3GPP device identifier, it indicates that the UE requests to suspend QoS differentiation for the non-3GPP device identifier. |  |  |  |  |  |  |
| For PDU session type value = "IPv4" |  |  |  |  |  |  |
| IPv4 address presence indicator (I4API) (octet s+1, bit 1) |  |  |  |  |  |  |
| The I4API field indicates whether IPv4 address is included in the connection information or not. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |
| 0 |  |  |  |  | No IPv4 address included |  |
| 1 |  |  |  |  | IPv4 address included |  |
| If no IPv4 address is included in the connection information for the non-3GPP device identifier, the IPv4 address of the PDU session applies. |  |  |  |  |  |  |
| IPv4 port range presence indicator (I4PRPI) (octet s+1, bit 2) |  |  |  |  |  |  |
| The I4PRPI field indicates whether IPv4 port range fields are included in the IE or not. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 0 |  |  |  |  | No IPv4 port ranges included |  |
| 1 |  |  |  |  | One or more IPv4 port ranges included |  |
| IPv4 address (octet s+2 to octet s+5) |  |  |  |  |  |  |
| The IPv4 address field contains an IPv4 address for the QoS differentiation of the non-3GPP device identifier. |  |  |  |  |  |  |
| Number of port ranges for IPv4 (octet (s+6)) |  |  |  |  |  |  |
| The number of port ranges for IPv4 field indicates the number of IPv4 port ranges for the non-3GPP devices connecting to the UE, and shall be included if the I4PRPI field indicates "One or more IPv4 port ranges included". |  |  |  |  |  |  |
| IPv4 port range (octet s+7 to octet s+10) |  |  |  |  |  |  |
| The IPv4 port range field indicates the port range associated with the traffic for the non-3GPP device identifier. The IPv4 port range field consists of the lowest port number in octets s+7 and s+8, and the highest port number in octets s+9 and s+10. |  |  |  |  |  |  |
| For PDU session type value = "IPv6" |  |  |  |  |  |  |
| IPv6 address type indicator (I6ATI) (octet s+1, bit 1) |  |  |  |  |  |  |
| The I6ATI field indicates whether the IPv6 address field includes an IPv6 address or an IPv6 prefix. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |
| 0 |  |  |  |  | IPv6 address included |  |
| 1 |  |  |  |  | IPv6 prefix included |  |
| IPv6 port range presence indicator (I6PRPI) (octet s+1, bit 2) |  |  |  |  |  |  |
| The I6PRPI field indicates whether the IPv6 port range fields are included in the connection information or not. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 0 |  |  |  |  | No IPv6 port ranges included |  |
| 1 |  |  |  |  | One or more IPv6 port ranges included |  |
| IPv6 address (octet s+2 to octet s+17) |  |  |  |  |  |  |
| The IPv6 address field contains an IPv6 address. |  |  |  |  |  |  |
| IPv6 prefix (octet s+2 to octet s+18) |  |  |  |  |  |  |
| The IPv6 prefix field contains an IPv6 address in octets s+2 to octet s+17, and a prefix length value in octet s+18. |  |  |  |  |  |  |
| Number of port ranges for IPv6 (octet a) |  |  |  |  |  |  |
| The number of port ranges for IPv6 field indicates the number of IPv6 port ranges for the non-3GPP devices connecting to the UE, and shall be included if the I6PRPI field indicates "One or more IPv6 port ranges included". |  |  |  |  |  |  |
| IPv6 port range (octet (a+1) to octet (a+4)) |  |  |  |  |  |  |
| The IPv6 port range field indicates the port range associated with the traffic for the non-3GPP device identifier. The IPv6 port range field consists of the lowest port number in octets a+1 and a+2, and the highest port number in octets a+3 and a+4. |  |  |  |  |  |  |
| For PDU session type value = "IPv4v6" |  |  |  |  |  |  |
| IPv4 address presence indicator (I4API) (octet s+1, bits 1 to 2) |  |  |  |  |  |  |
| The I4API field indicates whether a IPv4 addess field or is applicable the connection information or not, and whether an applicable IPv4 address is included in the connection information or not. |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |
| 2 | 1 |  |  |  |  |  |
| 0 | 0 |  |  |  |  | No IPv4 address applicable |
| 0 | 1 |  |  |  |  | Applicable IPv4 address included in the connection information |
| 1 | 0 |  |  |  |  | Applicable IPv4 address not included in the connection information. PDU session IPv4 address applies. |
| 1 | 1 |  |  |  |  | reserved |
| IPv4 port range presence indicator (I4PRPI) (octet s+1, bit 3) |  |  |  |  |  |  |
| The I4PRPI field indicates whether IPv4 port range fields are included in the IE or not. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 0 |  |  |  |  | No IPv4 port ranges included |  |
| 1 |  |  |  |  | One or more IPv4 port ranges included |  |
| IPv6 address or prefix presence indicator (I6APPI) (octet s+1, bits 4 to 5) |  |  |  |  |  |  |
| The I6APPI field indicates whether a IPv6 address field or IPv6 prefix is included in the IE or not. |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |
| 5 | 4 |  |  |  |  |  |
| 0 | 0 |  |  |  |  | No IPv6 address or IPv6 prefix included |
| 0 | 1 |  |  |  |  | IPv6 address included |
| 1 | 0 |  |  |  |  | IPv6 prefix included |
| 1 | 1 |  |  |  |  | reserved |
| IPv6 port range presence indicator (I6PRPI) (octet s+1, bit 6) |  |  |  |  |  |  |
| The I6PRPI field indicates whether the IPv6 port range fields are included in the IE or not. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 0 |  |  |  |  | No IPv6 port ranges included |  |
| 1 |  |  |  |  | One or more IPv6 port ranges included |  |
| IPv4 address (octet s+2 to octet s+5) |  |  |  |  |  |  |
| The IPv4 address field contains an IPv4 address applicable for the QoS differentiation of the non-3GPP device identifier. |  |  |  |  |  |  |
| Number of port ranges for IPv4 (octet c) |  |  |  |  |  |  |
| The number of port ranges for IPv4 field indicates the number of IPv4 port ranges for the non-3GPP devices connecting to the UE, and shall be included if the I4PRPI field indicates "One or more IPv4 port ranges included". |  |  |  |  |  |  |
| IPv4 port range (octet c+1 to octet c+4) |  |  |  |  |  |  |
| The IPv4 port range field indicates the port range associated with the traffic for the non-3GPP device identifier. The IPv4 port range field consists of the lowest port number in octets c+1 and c+2, and the highest port number in octets c+3 and c+4. |  |  |  |  |  |  |
| IPv6 address (octet e to octet e+15) |  |  |  |  |  |  |
| The IPv6 address field contains an IPv6 address. |  |  |  |  |  |  |
| IPv6 prefix (octet e to octet e+16) |  |  |  |  |  |  |
| The IPv6 prefix field contains an IPv6 address in octets e to octet e+15, and a prefix length value in octet e+16. |  |  |  |  |  |  |
| Number of port ranges for IPv6 (octet f) |  |  |  |  |  |  |
| The number of port ranges for IPv6 field indicates the number of IPv6 port ranges for the non-3GPP devices connecting to the UE, and shall be included if the I6PRPI field indicates "One or more IPv6 port ranges included". |  |  |  |  |  |  |
| IPv6 port range (octet f+1 to octet f+4) |  |  |  |  |  |  |
| The IPv6 port range field indicates the port range associated with the traffic for the non-3GPP device identifier. The IPv6 port range field consists of the lowest port number in octets f+1 and f+2, and the highest port number in octets f+3 and f+4. |  |  |  |  |  |  |
| For PDU session type value = "Ethernet" |  |  |  |  |  |  |
| VLAN tag ID presence indicator (VTPI) (octet (s+1), bit 1) |  |  |  |  |  |  |
| The VTPI field indicates whether the VLAN tag ID field is included in the IE or not. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |
| 0 |  |  |  |  | VLAN tag ID not included |  |
| 1 |  |  |  |  | VLAN tag ID included |  |
| MAC address (octet s+2 to octet s+7) |  |  |  |  |  |  |
| The MAC address field indicates the MAC address associated with the traffic for the non-3GPP device identifier. The MAC address is defined as in clause 8 of IEEE Std 802 [43]. Bit 8 of octet s+2 is the most significant bit and bit 1 of octet s+7 is the least significant bit. |  |  |  |  |  |  |
| VLAN tag ID (octet s+8 to octet s+9) |  |  |  |  |  |  |
| The VLAN tag ID field indicates the VLAN tag ID associated with the traffic for the non-3GPP device identifier. The VLAN tag ID is defined as specified in IEEE Std 802.1Q [74]. Bit 8 of octet s+8 is the most significant bit and bit 1 of octet s+9 is the least significant bit. |  |  |  |  |  |  |

## 9.12	3GPP specific coding information defined within present document

### 9.12.1	Serving network name (SNN)

The serving network name (SNN) is used:

-	in the Network name field of the AT_KDF_INPUT attribute defined in IETF RFC 5448 [40];

-	in KAUSF derivation function as specified in 3GPP TS 33.501 [24] annex A; and

-	in RES* and XRES* derivation function as specified in 3GPP TS 33.501 [24] annex A.

SNN shall contain a UTF-8 string without terminating null characters.

SNN is of maximum length of 1020 octets.

SNN consists of SNN-service-code and SNN-network-identifier, delimited by a colon.

SNN-network-identifier identifies the serving PLMN or the serving SNPN.

MCC and MNC in the SNN-PLMN-ID are MCC and MNC of the serving PLMN. If the MNC of the serving PLMN has two digits, then a zero is added at the beginning.

MCC and MNC in the SNN-SNPN-ID are MCC and MNC of the serving SNPN. If the MNC of the serving SNPN has two digits, then a zero is added at the beginning.

SNN-NID contains an NID in hexadecimal digits.

ABNF syntax of SNN is specified in table 9.12.1.1

Table 9.12.1.1: ABNF syntax of SNN

SNN = SNN-service-code ":" SNN-network-identifier

SNN-service-code = %x35.47 ; "5G"

SNN-network-identifier = SNN-PLMN-ID / SNN-SNPN-ID

SNN-PLMN-ID = SNN-mnc-string SNN-mnc-digits "." SNN-mcc-string SNN-mcc-digits "." SNN-3gppnetwork-string "." SNN-org-string ; applicable when not operating in SNPN access operation mode.

SNN-SNPN-ID = SNN-mnc-string SNN-mnc-digits "." SNN-mcc-string SNN-mcc-digits "." SNN-3gppnetwork-string "." SNN-org-string ":" SNN-NID ; applicable when operating in SNPN access operation mode.

SNN-mnc-digits = DIGIT DIGIT DIGIT ; MNC of the PLMN ID

SNN-mcc-digits = DIGIT DIGIT DIGIT ; MCC of the PLMN ID

SNN-mnc-string = %x6d.6e.63 ; "mnc" in lower case

SNN-mcc-string = %x6d.63.63 ; "mcc" in lower case

SNN-3gppnetwork-string = %x33.67.70.70.6e.65.74.77.6f.72.6b ; "3gppnetwork" in lower case

SNN-org-string = %x6f.72.67 ; "org" in lower case

SNN-NID = 11SNN-hexadecimal-digit ; NID in hexadecimal digits

SNN-hexadecimal-digit = DIGIT / %x41 / %x42 / %x43 / %x44 / %x45 / %x46

NOTE:	SNN-service-code allows for distinguishing of ANID specified in 3GPP TS 24.302 [16] and SNN as either of SNN or ANID can be carried in the AT_KDF_INPUT attribute.

EXAMPLE 1:	In case of a PLMN, if PLMN ID contains MCC = 234 and MNC = 15, SNN is 5G:mnc015.mcc234.3gppnetwork.org.

EXAMPLE 2:	In case of an SNPN, if SNPN ID contains a PLMN ID of MCC = 234 and MNC = 15 and an NID of 123456ABCDEH, SNN is 5G:mnc015.mcc234.3gppnetwork.org:123456ABCDE.